3GPP TSG RAN Meeting #86								RP-192698

Sitges, Spain, December 9-12, 2019

Agenda Item:	9.1.1

Source:	Apple

Title:	R17 NR URLLC enhancements

Document for:	Discussion/Decision

# Introduction

In RAN #84, a Rel-16 URLLC enhancement WI was approved with the following objectives [1]:

- Specification of PDCCH enhancements [RAN1] - DCI format(s) with configurable sizes for some fields, with a minimum DCI size targeting a reduction of 10~16 bits relative to Rel-15 DCI format 0_0/1_0 and a maximum DCI size that can be larger than Rel-15 DCI format 0_0/1_0, and provide the possibility to align with the size of the DCI format 0_0/1_0 (including possible zero padding if any) - Increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for at least one SCS subject to restrictions including, but not necessarily limited to, those identified in TR 38.824. Enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) can be further considered. - Specification of UCI enhancements [RAN1] - More than one PUCCH for HARQ-ACK transmission within a slot - At least two HARQ-ACK codebooks simultaneously constructed, intended for supporting different service types for a UE - Specification of PUSCH enhancements for both grant-based PUSCH and configured grant based PUSCH [RAN1] - For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots - Specification of enhancements to scheduling/HARQ [RAN1] - Out-of-order HARQ-ACK associated with PDSCHs with different HARQ process IDs - Out-of-order PUSCH scheduling associated with different HARQ process IDs, including overlapping PUSCHs and non-overlapping PUSCHs in time-domain - Methods to handle DL data/data resource conflicts for overlapping PDSCHs in time-domain, scheduled by dynamic DL assignments - Specification of enhanced inter UE Tx prioritization/multiplexing [RAN1] - UL cancelation scheme (see section 7.2.1 in TR 38.824) - Enhanced UL power control scheme (see section 7.2.2 in TR 38.824) - Specification of enhanced UL configured grant transmission [RAN1, RAN2] - Multiple active configured grant type 1 and type 2 configurations for a given BWP of a serving cell - Note: V2X use cases are also considered - Specification of URLLC RAN4 core requirements based on Rel-15 URLLC functionalities [RAN4] - Investigate and specify the RLM requirements to support high reliability, if needed - Investigate and specify the RLM requirements for low latency, if needed

In RAN1 #85, a revised WID was agreed with a down-scoping of the objectives in RAN1 for NR intra-UE prioritization as follows [3]

- The detailed objectives for NR intra-UE prioritization/multiplexing are: - Specify enhancements to address resource conflicts between dynamic grant (DG) and configured grant (CG) PUSCH and conflicts involving multiple CGs [RAN2, RAN1]. - Specify PUSCH grant prioritization based on LCH priorities and LCP restrictions for the cases where MAC prioritizes the grant [RAN2]. - Address UL data/control and control/control resource collision by (L1 multiplexing of services of different priority is out of scope): - specifying a method to address resource collision between SR associating to high-priority traffic and uplink data of lower-priority traffic for the cases where MAC determines the prioritization [RAN2]. - specifying prioritization behaviour among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH [RAN1, RAN2].

In this contribution, we provide our view on the Rel-17 URLLC enhancement including the following areas:

- Remaining issues from Rel-16

- Reducing processing time

# Remaining issues from Rel-16

In this section, we focus on issues that were to be addressed in Rel-16 but were unable to be completed. These include Out-of-order (OOO) HARQ-ACK associated with PDSCHs and intra-UE L1 multiplexing of different services.

Out-of-order (OOO) HARQ-ACK associated with PDSCHs and other out-of-order topics had been discussed in RAN1 for multiple meetings. In the following, we focus on OOO PDSCHs.

Different views on the mechanisms to support out-of-order PDSCHs/PUSCHs were provided by companies:

- Some companies have held the position that eMBB and uRLLC traffic for the same UE can be supported with the same processing timeline at the same serving cell (e.g. Cap #1 or Cap #2 as specified in Rel-15). In Rel-15, additional DMRS is not supported for PDSCH with Cap #2, hence the demodulation performance of PDSCH with front load DMRS only (and no additional DMRS) over a long allocation may be poor. However, UE mobility is not seen as a major issue for those companies.

- For some other companies, eMBB and uRLLC traffic for the same UE is best supported by configuring two processing timelines on the same serving cell. As for how they are supported, there have been divergent views. In some designs, a UE’s CA capability is leveraged, so two virtual CCs are supported on one actual CC. The pros for such designs include easier UE implementation and power saving. However, companies have different understandings on whether the power saving aspect is in the scope of Rel-16 URLLC. In other designs, essentially the Rel-15 PDSCH Cap #2 is enhanced with support for fallback to Cap #1 with additional DMRS. In that design, if PDSCH at Cap #2 is too close to PDSCH at Cap #1, it is dropped. Whether that is a severe drawback depends on the traffic models for eMBB and uRLLC assumed at the same UE. To complicate the situation, as a UE needs to prioritize PDSCH processing for overlapping or out-of-order PDSCHs, signaling design to indicate PDSCH priority has also been contentious.

In addition, the decisions made in on the OOO HARQ-ACK associated with different PDSCHs and the associated UE architecture has a direct influence on the options under consideration for PDCCH monitoring in the PDCCH enhancements discussion.

In the end, companies could not reach consensus on the design to support OOO PDSCHs, and RAN1 concluded at RAN1 #99:

Conclusion: - For Rel. 16 URLLC, no support of out-of-order/overlap PDSCH/HARQ and out-of-order/overlap PUSCH operation.

It can be expected when discussion on scheduling/HARQ enhancements restarts in Rel-17, all those issues need to be treated again and enough TUs for Rel-17 URLLC should be budgeted for that important topic. In addition, there may be a requirement to re-open the PDCCH enhancements discussion to accommodate any decisions made under this topic.

We recommend RAN clarifies a few points concerning OOO to facilitate smoother discussions at working group level in Rel-17 including:

- Traffic mix between eMBB and uRLLC at the same UE

  - Targeted uRLLC traffic profiles

- Deployment scenarios

  - Related to traffic mix, what are the targeted deployment scenarios, e.g. indoor, outdoor at the pedestrian speed, outdoor at the vehicular speed;

- Aspects such as power saving

  - RAN needs to clarify whether they are in the scope of Rel-17 URLLC or not, and where they can be included, e.g. under Rel-17 URLLC WI or Rel-17 power saving WI.

At RAN #85 [2] [3], it was decided that concerning intra-UE prioritization & multiplexing: “Continue work in RAN1 and RAN2, however with the limitation that RAN1 would not consider L1 multiplexing of different services in Release Rel-16”. We also expect work on intra-UE prioritization & multiplexing in Rel-17 will address the down-scoped work from Rel-16 and will be considerable.

In recognition of the substantial leftovers from Rel-16, we propose

Proposal 1: In Rel-17 URLLC enhancement, prioritize leftover topics from Rel-16, and clarify on contentious points from Rel-16 to facilitate smoother discussions at working group level in Rel-17.

# Reducing processing time

In the RAN email discussion on Rel-17 URLLC topics and scope, some companies have proposed to reduce processing time for downlink data channel (PDSCH) and uplink data channel (PUSCH), and also reduce the processing time for CSI feedback.  We have the following concerns on those proposals:

- Latencies from radio access (including front-haul latency), backhaul, core network and transport network are all important factors in the end-to-end latency. As such, shortening radio access latency may not be the most critical part in reducing the overall end-to-end latency.

- Reducing processing time is just one of the approaches to allow more transmissions within a given time interval; the necessity/benefit of adopting it should be evaluated against other approaches, so adopting shortening processing time should not be a foregone conclusion.

- In comparison to the situation in 4G, LTE’s processing time remained the same from Rel-8 to Rel-14 before the specification work from LTE Rel-15 WI “Shortened TTI and processing time for LTE”. As processing timeline drives many decisions in the air interface design and also UE/gNB implementations, in our view, it is premature to consider reducing processing time from the third release of NR.

- We also note no conclusive observation can be drawn on the necessity/benefit with shortened processing time from Rel-16 uRLLC SI. Even in the Rel-16 uRLLC SI evaluations, we feel important aspects such as MIMO with a large number of antennas did not make into the study:

- In Rel-16 uRLLC Study Item, evaluations have assumed 4Tx antennas at the gNB for carrier frequency at 4 GHz.

- Shortened processing time enables faster scheduling decisions but at the same also leads to faster-changing interference including intercell interference/intra-interference due to MU-MIMO pairing; when that is coupled with the flash-light effect with beam forming, the case is still to be made on whether any meaningful gain in the number of uRLLC UEs served can be achieved.

Considering the above, we have the following proposal:

Proposal 2: In Rel-17 URLLC enhancement, reducing processing time is not considered.

# Conclusion

In this contribution, we provide our view on Rel-17 URLLC. We have

Proposal 1: In Rel-17 URLLC enhancement, prioritize leftover topics from Rel-16, and clarify on contentious points from Rel-16 to facilitate smoother discussions at working group level in Rel-17.

Proposal 2: In Rel-17 URLLC enhancement, reducing processing time is not considered.

# References

- RP-191584, "Revised WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) ", 3GPP TSG RAN Meeting #84, Newport Beach, CA, June 3-6, 2019

- RP-192287, “Scope of NR Industrial Internet of Things (IoT) & URLLC”, 3GPP TSG RAN Meeting #84, Newport Beach, CA, June 3-6, 2019

- RP-192324, “Revised WID: Support of NR Industrial Internet of Things (IoT)”, 3GPP TSG RAN Meeting #85, Newport Beach, CA, September 16-20, 2019