3GPP TSG RAN meeting #86                                                            RP-192703

Sitges, Spain, December 9-12, 2019

Source:	Philips International B.V.

Title:	Adding healthcare use cases to TR 38.807

Agenda Item:	9.3.2

Document for: 	Approval

# Introduction

In this contribution, we propose to add some healthcare use cases, as described in more detail in TR 22.826 “Study on communication services for critical medical applications” to TR 38.807. Given the very high data rates required for some of these use cases, 5G NR above 52.6 GHz is an interesting candidate technology to support these types of use cases.

# Text proposal for Section 2 in TR38.807

==================== Start of Text Proposal for Section 2 =====================

# 2    References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TD 	RP-181435: "New SID: Study on NR beyond 52.6 GHz".

[3]	3GPP TR 38.805: "Study on New Radio access technology; 60 GHz unlicensed spectrum".

[4]	3GPP TR 38.913: "Study on New Radio access technology; Next Generation Access Technologies".

[5]	WRC-15 Resolution 238: "Studies on frequency-related matters for International Mobile Telecommunications identification including possible additional allocations to the mobile services on a primary basis in portion(s) of the frequency range between 24.25 and 86 GHz for the future development of International Mobile Telecommunications for 2020 and beyond".

[6]		https://www.itu.int/md/R15-CPM19.02-R-0001/en

[7]		APG19-3/INP-35, Australia, "PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5 AND 9.1.8)", APG19-3, March, 2018, Australia

[8]		APG19-3/INP-87, China, "PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5, 9.1.8) ", APG19-3, March, 2018, Australia

[9]		Independent Communications Authority of South Africa. Standard procedures and guideline regarding the use of radio frequency spectrum in the E-band (71 – 76 GHz paired with 81 – 86 GHz). April 28, 2017.

[10]		Independent Communications Authority of South Africa. National Radio Frequency Plan 2018. May 25, 2018.

[11]		Independent Communications Authority of South Africa. Electronic Communications Act (781/2016): Amendment to the Radio Frequency Spectrum Regulations 2015. No. 404436, November 22, 2016.

[12]		Independent Communications Authority of South Africa. Radio Frequency Spectrum Regulations (279/2015): Amendment to the Radio Frequency Spectrum Regulations, 2015. November 22, 2016.

[13]		Canada Spectrum Management and Telecommunications. Canadian Table of Frequency Allocations 2018 Edition. April 2018.

[14]		Canada Spectrum Management and Telecommunications. Technical Requirements for Fixed Line-of-Sight Radio Systems Operating in the Bands 71-76 GHz and 81-86 GHz. May 2017.

[15]		MIIT, "Notice on wireless technical applications with very low transmission power (short distance) in 60GHz. 2006-082" (关于60GHz 频段微功率（短距离）无线电技术应用有关问题的通知 2006-082) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176263/part/3176264.pdf

[16]		MIIT, "Technical requirement of wireless devices with very low transmission power (short distance 2005-423)." (微功率（短距离）无线电设备的技术要求 2005-043) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176241/part/3176242.pdf

[17]	ARIB STD-B43 v2.1, "PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION," Jan. 2018.

[18]	ARIB STD-T69 v4.0, "LOW POWER DATA COMMUNICATION SYSTEM/MILLIMETER-WAVE VIDEO TRANSMISSION EQUIPMENT," Sept. 2016.

[19]	ARIB STD-T117 v1.0, "LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS," Sept. 2016.

[20]	ARIB STD-T48 v2.2, "MILLIMETER-WAVE RADAR EQUIPMENT FOR SPECIFIED LOW POWER RADIO STATION," Dec. 2015.

[21]	ARIB STD-T111 v1.1, "79 GHz BAND HIGH-RESOLUTION RADAR," Mar. 2017.

[22]		Republic of Korea Communications Agency. Republic of Korea Frequency Allocations Status. May 2018.

[23]		Republic of Korea National Radio Research Agency. National Radio Research Agency Publication 2014-12. July 02, 2014.

[24]		Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-3. July 02, 2014.

[25]		Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-4. July 02, 2014.

[26]	NFAP-18	"National Frequency Allocation Plan – 2018", Government of India Ministry of Communications.

[27]		IMDA (Infocomm Media Development Authority), IMDA TS SRD, April 2018. https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/ict-standards/telecommunication-standards/radio-comms/imdatssrd.pdf?la=en

[28]		ACMA (Australian Communications and Media Authority), "Radiocommunications (Low Interference Potential Devices) Class Licence 2015", 2018. https://www.legislation.gov.au/Details/F2018C00500

[29]		No. 2018-71 	Ministry of Science and ICT notice

[30]		No. 2018-26 	National Radio Research Agency notice

[31]	No. 2018-36 	Ministry of Science and ICT notice

[32]	Ministry of Transportation and Communications, “TABLE OF RADIO FREQUENCY ALLOCATIONS OF THE REPUBLIC OF CHINA,” Feb. 2017, http://www.motc.gov.tw/post/home.jsp?id=364&parentpath=0.

[33]	China, "People’s republic of China Regulations on the Radio Frequency Allocation", The People's Posts and Telecommunications Press, 2010.

[34]	IMDA (Infocomm Media Development Authority), "Singapore Spectrum Allocation Chart", https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/frameworks-and-policies/spectrum-management-and-coordination/spectrumchart.pdf?la=en

[35]	ACMA, "Australian Radiofrequency Spectrum Plan 2017-including general information", https://www.acma.gov.au/theacma/australian-radiofrequency-spectrum-plan-spectrum-planning-acma

[36]	The Wireless Planning & Coordination (WPC) Wing of the Ministry of Communication & Information Technology, "License Exemption for SRD Device GSR 1047(E) dated 18/10/ 2018",

[37]	https://www.itu.int/pub/R-REG-RR (Edition of 2016)

[38]	3GPP TR 22.826: "Study on communication services for critical medical applications".

[39]	3GPP TR 33.853: "Technical report on key issues and potential solutions for Integrity protection of the User Plane"

=================== End of Text Proposal for Section 2 ========================

# Text proposal for Section 5 in TR38.807

==================== Start of Text Proposal for Section 5 =====================

# 5	Use Cases and Deployment Scenarios

## 5.1	…

### 5.1.14	Critical medical communication

As described in TR 22.826 [38], various critical medical use cases require very high data rates and very low latencies. The huge bandwidths offered by 5G NR above 52.6 GHz could prove to be essential in order to meet the required KPIs. One of the primary examples includes a wirelessly connected operating room that enables real-time streaming of lossless compressed Ultra High Definition video (from devices, such as an endoscope), and real-time streaming of uncompressed medical imaging data (from devices, such as 3D UltraSound). These streams need to be displayed on 4K/8K displays, and possibly be combined into augmented reality video streams to help a surgeon perform precise procedures. This typically requires datarates ranging from 4 Gbit/s up to 48 Gbit/s and very low latencies of preferably less than 1ms. Compression is often not allowed by medical regulations, in order to not generate any artifacts and enable the most accurate representation to be available to a surgeon.

Despite the low latency and high datarates required, medical data in general needs to be transported in a highly secure manner, including being fully integrity protected as required by medical data protection and privacy regulations. And of course, the connection has to be highly reliable and very robust.

## 5.2	Deployment Scenarios

This section describes deployment scenarios applicable for NR operating in between 52.6GHz and 114.25GHz. The listed deployment scenarios are indoor hotspot, dense urban, urban micro, urban macro, rural, factory hall, and indoor D2D scenario. Table 5.2-1 describes potential relationship between described use cases in Section 5.1 and listed deployment scenarios.

Table 5.2-1: Potential relationship between use cases and deployment scenarios

| Deployment scenarios | Use cases |
| --- | --- |
| Indoor hotspot | High data rate eMBB, Mobile data offloading, Vertical industry factory application, Critical medical applications, Augmented reality/virtual reality headsets and other high-end wearables, Wireless Display Transfer, Radar/Positioning, Private Networks, Integrated access backhaul (IAB) |
| Dense Urban | High data rate eMBB, Mobile data offloading, Integrated access backhaul (IAB), Augmented reality/virtual reality headsets and other high-end wearables, Radar/Positioning, Private Networks |
| Urban Micro | High data rate eMBB, Mobile data offloading, Broadband distribution network, Integrated access backhaul (IAB) |
| Urban Macro | Broadband distribution network, Integrated access backhaul (IAB) |
| Rural | Broadband distribution network, Integrated access backhaul (IAB) |
| Factory Hall | Vertical industry factory application, Data center inter-rack connectivity, Factory automation/Industrial IoT(IIoT), Private Networks, Radar/Positioning, smart grid automation (note: smart grid automation use case is not limited to indoors only) |
| Indoor D2D | Short-range high-data rate D2D communications, Critical medical applications, Augmented reality/virtual reality headsets and other high-end wearables |

=================== End of Text Proposal for Section 5 ========================

# Text proposal for Section 6 in TR38.807

==================== Start of Text Proposal for Section 6 =====================

# 6	System Design Requirements

This section describe some of the system design considerations and requirements that are important for NR system operating in frequencies between 52.6 GHz and 114.25 GHz.

a)	Waveform

-	Power efficiency of power amplifier (PA): PA efficiency at higher frequency is expected to degrade and low PAPR waveforms designed to minimize PA backoff and maximize efficiency should be considered. At higher frequencies and especially in millimetre wave frequencies, output power per transistor as well as power added efficiency decrease. Waveform with high power back-off to support EVM and out-of-band emission requirements could dramatically reduce PA efficiency even further.

-	Dynamic range of ADC and DAC: The increase in PA back-off also affects the other device requirements, for example, the dynamic range of the ADC and DAC. A higher Tx DAC effective number of bits (ENOB) is required to accommodate higher PAPR, and extra oversampling in the baseband DSP, and Tx DAC may be needed to accommodate wider channel bandwidth. All of these are impacted by waveform, and therefore should be carefully evaluated.

-	Modulated signal accuracy and out-of-band emission: Power amplifier is designed and adjusted to meet RF requirements, such as spectral mask emission (SEM), adjacent channel leakage ratio (ACLR), in-band emission (IBE) and out-of-band emission (OBE) requirements, and error vector magnitude (EVM) requirements. Proper RF requirements are needed to determine appropriate in-band signal quality characteristics, minimize adjacent channel interference and impact to signals in adjacent channels. Occupied signal bandwidth and guardband for a given channel bandwidth in high carrier frequencies above 52.6 GHz require further investigation.

-	Complexity and performance of waveform: Given the high data rate and high sampling rates the system is expected to operate, the complexity and performance tradeoff for waveform generation/modulation and reception/demodulation should be considered.

-	Spectrum flexibility of waveform: Use cases and frequency allocations by various government bodies may require various bandwidth to be supported. Therefore, flexibility to support different system bandwidth should be considered in the design.

-	Robustness to frequency offset and phase noise: Carrier frequency offset and phase noise is much higher in spectrum beyond 52.6GHz because of imperfection of PA and crystal oscillator is more severe than that of lower bands. In addition, Doppler shift/spread is also larger with the carrier frequency increasing. As a result, robustness on frequency offset and phase noise is one of the key requirements for systems operating on bands above 52.6GHz. Increasing the subcarrier spacing for CP-OFDM waveform to better cope with increased phase noise could be investigated. For other potential waveforms impact from phase noise and ability to robustly handle phase noise should be investigated.

-	Feature re-usability and design commonality with existing NR specification: It would be good to be able to support features for FR1 and FR2 as defined in NR with minimal change (if possible) and support a common design structure that could support various use cases. To that extent further considerations of using an integer ratio between clock rates of NR below and NR above 52.6 GHz should be investigated. One possibility to achieve this would be to maintain the NR numerology scaling principle but extend to higher numerologies, i.e. Δf = 2μ × 15 kHz with an appropriate range of possible integer values for μ.

b)	MIMO

-	Multi-antenna technology with beamforming: Depending on deploying site and also applications, various antennas can be used in millimetre communication. Concerning implementation (e.g., cost, form factor), various type of antennas can be considered for developing NR above 52.6 GHz. There are four types of antennas can be considered in this frequency range: Parabolic, lens, waveguide array, and patch array. Some examples of types of antennas are shown in Figure 6-1 and brief comparison notes are provided in Table 6-1. The design of NR above 52.6 GHz should not limit the type of antennas and corresponding beamformed channel should be considered to design physical-layers.

Figure 6-1: millimeter wave antennas

Table 6-1: Comparison of millimetre wave antennas

| Antenna types | Applicable form factors |
| --- | --- |
| Parabolic | Outdoor macro for mid/long-distance PTP |
| Lens | Outdoor small cell AP for short/mid-distance PTP |
| Waveguide array | Small cell AP, indoor AP for short/mid-distance PTP |
| Patch array | Mobile device, small cell, indoor cell for mid-distance PTP |

-	Maximum supported layers: Up to two spatial layer could be supported using polarization diversity. Further study on maximum supported spatial layers taking into account support for even larger bandwidths and RF design challenges is needed.

-	Enhanced beam management: Larger antenna arrays is expected to support certain link budgets in high frequencies due to poorer free space pass loss, oxygen absorption, and the obstruction loss, even from people or foliage. This results in pencil-thin beams that require improvements in beam management robustness and overhead. Methods for managing narrower beams and greater number of beams should be studied. It should be noted depending on form factor and device, number of beams supported and beam codebook space could vary. Therefore, enhancement on discovery and tracking to support various beam assumptions should be investigated.

-	Enhanced path diversity: The narrow beams achieved with the larger arrays may result in a higher reliance on LOS paths.  To improve coverage at these frequencies, methods for improving path diversity—increasing the probability of LOS paths—should be studied.  Some examples may include collaborate MIMO techniques or same frequency multi-connectivity techniques.

-	Multi-cell/panel operation for high reliability and mobility: Considering propagation characteristics, it may be beneficial to support the near-LOS condition to UEs by position and angle tracking, and point-beamforming using massive antenna arrays.

c)	Power consumption

-	Compared to NR FR2, operation in the bands beyond 52.6 GHz may pose even more burden on UE power consumption due to potential large number of beams, low latency and high data rate transmission/reception (i.e., continuous processing of large code block and high order modulation) and PA efficiency losses caused by RF components.

-	Power efficiency in baseband:  Power consumption is one of the biggest problems in millimetre band communication. In addition to the process losses to the RF devices, the power dissipation in the ADCs is also very large. Widening the dynamic range required by the ADC when receiving high order modulation can consume high power. Therefore, power efficiency in both transmitter and receiver baseband should be considered for NR design above 52.6GHz. Improvement in uplink can be considered to reduce additional power consumption. For example, a low-power transmit scheme, advanced DTX, and a low-power beamforming scheme can be considered.

d)	Channelization and Bandwidth

-	Bandwidths for the system:  Different regulatory bodies are already provide licensed band allocation with certain bandwidth. For example, 250MHz blocks that can be aggregated to up to 5 GHz in Europe, South Africa, and Canada, 4.75 GHz blocks in UK, up to 5 GHz and 0.9 GHz blocks in US. Additionally, 802.11ad/ay systems currently support multiple of 2.16 GHz block in unlicensed spectrum. NR system operating in above 52.6 GHz should consider the above bandwidths in the system design. Supported bandwidth should weigh in various considerations, including channelization, and hardware complexity. Wideband implementation has various implementation challenges, such as IF conversion, I/Q mismatch, increase of noise power, flatness of filter, and ADC, DAC performance.

-	Support of configurable bandwidth: NR beyond 52.6 GHz is anticipated to operate in different bands with different maximum possible bandwidths. NR beyond 52.6 GHz should therefore – as NR does – support configurable transmission bandwidths. NR should also support relatively narrow bandwidth smaller than the configured transmission bandwidth to improve RF and baseband power consumption and potentially to improve coverage with peak power efficiency. Additionally, support of narrow bandwidth that is smaller than the configured transmission bandwidth in the UL would help benefit transmission of HARQ feedback and small payload sizes.

-	Channelization: Supported bandwidths for the system should be considered during the channelization of NR above 52.6 GHz. The channel definitions may need to factor into account co-existence with other RAT, such as IEE 802.11ad/ay systems, and various uses cases, such as ITS, IAB, and V2X.

e)	Range, availability, and connectivity

-	Use cases for NR above 52.6 GHz can be divided into three classes, a class for outdoor mid / long-distance such as backhaul or relay, a class for short / mid-distance such as indoor or outdoor small cell, and a class for adjacent, near-field device or isolated area. Long distance classes, which include backhaul, can support up to 1 to 3 km range under LOS conditions and weather conditions. The inter-cell distance for outdoor small cells is assumed to be within a few hundred meters and the size of a floor or a room is assumed to be within tens of meters. Between devices or servers, less than 5 m range would be sufficient to minimize link interference.

-	Services provided by backhaul, the reliability of the wired network should be guaranteed. For indoor scenario, can guarantee higher availability as it is not effected by weather condition but it should consider the availability depending on the application services. For example, IIoT services may require URLLC-like (e.g. low BLER of 10-5) availability.

-	Table 6-2 show an summary of range, availability, and connectivity for NR above 52.6GHz. P-to-P in the connectivity column refers to point to point connection, and P-to-MP in the connectivity column refers to point to multi-point connection.

Table 6-2: Range, availability, and connectivity

|  | Target range (m) | Availability (%) | Connectivity |
| --- | --- | --- | --- |
| Backhaul/fronthaul | < [500-3000] m | [99.99 – 99.999] % | [P-to-P] |
| Outdoor small cell | < [100] m | [99. 9 – 99.99] % | [P-to-MP] |
| Indoor hotspot/IoT | <[20] m | [99.9 – 99.9999] % | [P-to-MP] |
| Personal area network | < [5] m | [99.9 – 99.99] % | [P-to-P] |

f)	Spectrum regime considerations

-	Frequency range above 52.6 GHz covers not only unlicensed spectrum but also licensed spectrum. Additionally, ECC/CEPT has set 63- 64 GHz as harmonized implementation of ITS. The licensed spectrum in the range is often dedicated to fixed links and could be used by IAB. So spectrum for 52.6 GHz would need to support systems that enable various target use cases spanning from unlicensed operation to V2X, IAB, and eMBB. Additionally, coexistence with 802.11ad and 802.11ay on the 60GHz unlicensed spectrum may need to be considered.

-	It would be preferred if similar system framework supports wide range of uses cases without significant specification effort or at the least the initial system framework support forward compatibility that allows extension to address various and different use cases.

g)	Other design considerations

-	Latency: For some uses cases and application services, the overall latency should be small enough to facilitate nominal operation of the application services. NR above 52.6GHz should support low latency operations.

-	Mobility support: For many use cases and deployment scenarios, low mobility is common assumption. Even for ITS/V2X use cases, low relative speed between the Tx and the Rx can be expected. Support for high mobility may not need to be prioritized as high layer mobility procedures could rely on low frequency bands with CA/DC operations.

-	Standalone and Non-standalone operations: Some use cases may not require standalone operation at bands beyond 52.6 GHz. For example, high data rate eMBB and mobile data offloading scenarios where wide area coverage is provided via another frequency band (e.g. lower band in FR1/FR2). Also some uses cases are suitable for standalone operations. For example, short-range high-data rate D2D communications and/or Industrial IoT. In scenarios that require standalone operations, initial access and related procedure should be studied including study on whether existing NR features and functionality are sufficient.

-	Security and integrity protection: For several use cases it is necessary to provide excellent security and full data rate integrity protection that meet the potentially very high data rates and ultra low latencies required by these use cases. Hence, it may be necessary to consider more efficient security and integrity protection mechanisms than currently in place, such as zero-overhead user plane integrity protection on the link layer, as currently being explored in TR 33.853 [39].

=================== End of Text Proposal for Section 5 ========================