3GPP TSG-RAN Meeting #86	RP-192720

Sitges, Spain, December 9th – 12th, 2019

Agenda Item:	9.1.2

Source: 	Apple Inc.

Title:  	Views on FTEI-2 for R17 UDC

Document for:	Discussion and Decision

# Introduction

For R17, there is an ongoing phase 2 email discussion for scoping a work item on support of UDC in NR [1].  As part of this discussion, companies are discussing the priority of including the option on FTEI-2 in the scope of this work item.  FTEI-2 is meant to discuss the option to support compression for control signalling.

Based on the ongoing phase 2 email discussion for the UDC WI, RRC Signalling Compression is supported by companies as a lower priority option while scoping the work item.

In this paper, we will provide some analysis based on the previous RAN2 study to show the benefit and necessity to include FTEI-2 in R17 UDC WI scope.

# Discussion

## Background on RRC Signalling Compression Study in R16 RACS SI

In Rel-16 SI of RACS (i.e. Optimizations of UE Radio Capability Signaling), uplink capability signalling compression was studied and finally captured in TR37.873. Even though the compression method is not included in R16 WI scope due to the limited TU allocated, during the study phase, companies provided the analysis on the transmission efficiency to use the DEFLATE algorithm for the capability compression, as shown in Table-1, and there is common understanding that supporting compression on uplink capability can dramatically increase the transmission efficiency.

Table-1. Study and Analysis on the Compression Efficiency for RRC Signaling Compression

| Contribution | Description/Observation |
| --- | --- |
| R2-1814392 (Nokia) | Observation 1: On a LTE capability container, using deflate gave 60% reduction and with lzma even further. |
| R2-1814383 (Mediatek) | By applying generic compression algorithm, Deflate [2], to this example, the UE capability size is reduced to 54% of the original size. The compression gain is non-negligible. Observation 1: Compression can reduce LTE UE capability size by removing capability duplication within the same UE. Observation 2: Generic compression method can also be used to reduce duplication of NR UE capability within the same UE. Observation 3: Generic compression can work with other solutions to reduce size. |
| R2-1814482 (CATT) | For example, a 100 bytes message almost could not be compressed but for 300 bytes messages, it could be compressed to 50%-70%. The longer the UE capability message is the higher compression efficiency would be since more repeated capability information is contained (e.g. more bands and band combinations are supported which implies more repeated information would be contained in the message), higher ratio capability information can be compressed. Observation 1: the more repeated information is contained, the higher compression efficiency could be achieved. |
| R2-1818226 (Apple) | DEFLATE compression algorithm is defined in Rel-15 standards for user data compression. It is noted that DEFLATE compression can be used for same packet compression (no need to base it on cross packet), if the same deflate compression is used for UE capability string, message size can be reduced more than 50% as we evaluated. Proposal 3: DEFLATE compression algorithm without dictionary configuration should be considered as the UE capability compression algorithm. |
| R2-1817256 (MediaTek) | Table 1. UE capability size reduction by pure compression UE capability size (bytes); Size reduction. Case 1 [3]; 3161; 59%. Case 2 [2]; 1082; 46%. Case 3 [4]; 300; 30% - 50%. Observation 1: The larger the UE capability size is, the more the size reduction can be achieved by compression. |

Observation 1: During R16 RACS SI, it is common understanding that applying general compression algorithm (i.e. DEFLATE) on UE capability signalling can reduce the capability size to about 50% of the original size.

## Benefit and Necessity of RRC Signalling Compression in R17 UDC

With the RRC signalling size significantly increasing, it even can exceed the maximum PDCP SDU size. Larger RRC Signalling sizes cause transmission issues and hence segmentation method has to be introduced to address the huge message transmission problem. However, segmentation method can only help successfully transmit the huge message, it doesn’t increase the transmission efficiency.

Observation 2: Current method of RRC signalling segmentation cannot increase the transmission efficiency.

According to observation 1, it is obvious that the RRC signalling compression can bring significant benefit to the transmission efficiency. If the RRC signalling compression can be considered, RRC signalling compression will further improve the transmission efficiency and reliability of getting the message across to the network.

During the phase 2 email discussion for R17 UDC WI, many companies raised their concern that the RRC signalling compression will take some time for study so that it is not suitable to be in Rel-17 scope. But if the observations and benefits shown in previous RAN2 RACS study can be used as the baseline for the R17 UDC for RRC signaling compression, we do not need to study the benefit but instead discuss and specify the detailed solution directly.

Observation 3: The observations and benefits shown in R16 RACS study can be used for RRC signalling compression in R17 UDC.

Observation 4: In R17 UDC, RRC signalling compression can only focus on the pure compression using DEFLATE algorithm.

Based on the discussion above, the RRC signalling compression can bring significant benefit on the transmission efficiency, and the discussion can directly go to the detailed solution which will not take up much time. Therefore, we sincerely ask RAN to consider to include the RRC signaling compression in R17 UDC WI.

Proposal 1: RRC signaling compression (i.e. FTEI-2) should be included in R17 UDC WI scope.

# Summary

According to the analysis in section 2, we observe that

Observation 1: During R16 RACS SI, it is common understanding that applying general compression algorithm (i.e. DEFLATE) on UE capability signalling can reduce the capability size to about 50% of the original size.

Observation 2: Current method of RRC signalling segmentation does not increase the transmission efficiency.

Observation 3: The observations and benefits shown in R16 RACS study can be used for RRC signalling compression in R17 UDC.

Observation 4: In R17 UDC, RRC signalling compression can only focus on the pure compression using DEFLATE algorithm.

We propose that

Proposal 1: RRC signaling compression (i.e. FTEI-2) should be included in R17 UDC WI scope.

# References

- RP-192862 Summary of email discussion on NR UDC and other transmission efficiency improvements