3GPP TSG-RAN WG4 Meeting #93	R4-1915815

# Reno, Nevada, US, November 18th - November 22nd 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0063 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.104 on PUSCH requirements with CP-OFDM and FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Demodulation requirements for PUSCH FR1 in section 8 not finalized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This Rel-16 CR mirrors the Rel-15 CR [R4-1913394]. The original Rel-15 CR captures the mirroring of the endorsed draftCR [R4-1912688]. Changes coming from the endorsed draftCR are marked as “Mueller, Axel (Nokia - FR/Paris-Saclay)”. Additional changes are required on top of the ones from [R4-1912688], in particular, to remove the square brackets around requirements. New changes are marked as “Axel2”. Further new changes are test parameter naming changes and FDD/TDD pattern requirement applicability clarification note.. Agenda item: 7.12.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete, outdated and some parts TBD/square bracketed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.141-1, 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

### 8.2.1	Requirements for PUSCH with transform precoding disabled

#### 8.2.1.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table: 8.2.1.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

#### 8.2.1.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.1.2-1 to 8.2.1.2-14 at the given SNR for 1Tx or for 2Tx two-layer spatial multiplexing transmission. FRCs are defined in annex A.

Table 8.2.1.2-1: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -2.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 10.1 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 12.3 |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -5.8 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 6.2 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 8.8 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -8.7 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 3.0 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 5.6 |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | 1.0 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 18.2 |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | -2.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 11.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | -5.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 6.8 |

Table 8.2.1.2-2: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas |  | Cyclic prefix |  | Propagation conditions and correlation matrix (Annex G) |  | Fraction of maximum throughput |  | FRC (Annex A) |  | Additional DM-RS position |  | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-9 |  | pos1 |  | -2.5 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-9 |  | pos1 |  | 10.2 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-9 |  | pos1 |  | 12.2 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-9 |  | pos1 |  | -6.0 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-9 |  | pos1 |  | 6.3 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-9 |  | pos1 |  | 8.6 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-9 |  | pos1 |  | -8.7 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-9 |  | pos1 |  | 3.1 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-9 |  | pos1 |  | 5.5 |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | pos1 |  | 1.7 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | pos1 |  | 18.3 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | pos1 |  | -2.0 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | pos1 |  | 11.2 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | pos1 |  | -5.5 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | pos1 |  | 6.8 |

Table 8.2.1.2-3: Minimum requirements for PUSCH, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-10 |  | pos1 |  | -2.1 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-10 |  | pos1 |  | 10.0 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-10 |  | pos1 |  | 12.4 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-10 |  | pos1 |  | -5.5 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-10 |  | pos1 |  | 6.2 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-10 |  | pos1 |  | 8.6 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-10 |  | pos1 |  | -8.5 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-10 |  | pos1 |  | 3.0 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-10 |  | pos1 |  | 5.5 |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | pos1 |  | 2.1 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | pos1 |  | 18.3 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | pos1 |  | -1.8 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | pos1 |  | 11.1 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | pos1 |  | -5.3 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | pos1 |  | 6.9 |

Table 8.2.1.2-4: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-11 |  | pos1 |  | -2.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-11 |  | pos1 |  | 10.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-11 |  | pos1 |  | 12.8 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-11 |  | pos1 |  | -5.6 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-11 |  | pos1 |  | 6.4 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-11 |  | pos1 |  | 8.6 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-11 |  | pos1 |  | -8.6 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-11 |  | pos1 |  | 3.3 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-11 |  | pos1 |  | 5.5 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | pos1 |  | 1.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | pos1 |  | 18.4 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | pos1 |  | -2.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | pos1 |  | 11.2 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | pos1 |  | -5.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | pos1 |  | 7.0 |

Table 8.2.1.2-5: Minimum requirements for PUSCH, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-12 |  | pos1 |  | -2.9 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-12 |  | pos1 |  | 10.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-12 |  | pos1 |  | 12.5 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-12 |  | pos1 |  | -6.0 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-12 |  | pos1 |  | 6.4 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-12 |  | pos1 |  | 8.6 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-12 |  | pos1 |  | -8.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-12 |  | pos1 |  | 3.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-12 |  | pos1 |  | 5.5 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | pos1 |  | 1.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | pos1 |  | 18.1 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | pos1 |  | -2.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | pos1 |  | 11.3 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | pos1 |  | -5.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | pos1 |  | 6.9 |

Table 8.2.1.2-6: Minimum requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-13 |  | pos1 |  | -2.5 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-13 |  | pos1 |  | 10.0 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-13 |  | pos1 |  | 12.4 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-13 |  | pos1 |  | -5.8 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-13 |  | pos1 |  | 6.3 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-13 |  | pos1 |  | 8.5 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-13 |  | pos1 |  | -8.7 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-13 |  | pos1 |  | 3.1 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-13 |  | pos1 |  | 5.4 |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | pos1 |  | 1.3 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | pos1 |  | 19.5 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | pos1 |  | -2.3 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | pos1 |  | 11.3 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | pos1 |  | -5.2 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | pos1 |  | 6.9 |

Table 8.2.1.2-7: Minimum requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix |  | Propagation conditions and correlation matrix (Annex G) |  | Fraction of maximum throughput |  | FRC (Annex A) |  | Additional DM-RS position |  | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-14 |  | pos1 |  | -2.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-14 |  | pos1 |  | 10.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-14 |  | pos1 |  | 13.0 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-14 |  | pos1 |  | -5.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-14 |  | pos1 |  | 6.5 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-14 |  | pos1 |  | 9.0 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-14 |  | pos1 |  | -8.7 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-14 |  | pos1 |  | 3.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-14 |  | pos1 |  | 5.8 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | pos1 |  | 1.4 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | pos1 |  | 19.2 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | pos1 |  | -2.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | pos1 |  | 11.6 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | pos1 |  | -5.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | pos1 |  | 7.1 |

Table 8.2.1.2-8: Minimum requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-8 |  | pos1 |  | -2.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-8 |  | pos1 |  | 10.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-8 |  | pos1 |  | 12.5 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-8 |  | pos1 |  | -5.7 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-8 |  | pos1 |  | 6.3 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-8 |  | pos1 |  | 8.9 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-8 |  | pos1 |  | -8.7 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-8 |  | pos1 |  | 3.0 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-8 |  | pos1 |  | 5.7 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-22 |  | pos1 |  | 1.5 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-22 |  | pos1 |  | 18.3 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-22 |  | pos1 |  | -2.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-22 |  | pos1 |  | 11.1 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-22 |  | pos1 |  | -5.4 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-22 |  | pos1 |  | 6.8 |

Table 8.2.1.2-9: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas |  | Cyclic prefix |  | Propagation conditions and correlation matrix (Annex G) |  | Fraction of maximum throughput |  | FRC (Annex A) |  | Additional DM-RS position |  | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-9 |  | pos1 |  | -2.3 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-9 |  | pos1 |  | 10.5 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-9 |  | pos1 |  | 12.6 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-9 |  | pos1 |  | -5.7 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-9 |  | pos1 |  | 6.5 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-9 |  | pos1 |  | 8.9 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-9 |  | pos1 |  | -9.0 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-9 |  | pos1 |  | 3.2 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-9 |  | pos1 |  | 5.8 |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | pos1 |  | 2.0 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | pos1 |  | 18.7 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | pos1 |  | -2.3 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | pos1 |  | 11.3 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | pos1 |  | -5.2 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | pos1 |  | 7.0 |

Table 8.2.1.2-10: Minimum requirements for PUSCH, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-10 |  | pos1 |  | -2.1 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-10 |  | pos1 |  | 10.4 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-10 |  | pos1 |  | 12.3 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-10 |  | pos1 |  | -5.7 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-10 |  | pos1 |  | 6.3 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-10 |  | pos1 |  | 8.8 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-10 |  | pos1 |  | -8.5 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-10 |  | pos1 |  | 3.1 |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-10 |  | pos1 |  | 5.7 |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | pos1 |  | 1.6 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | pos1 |  | 18.1 |
|  |  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | pos1 |  | -2.0 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | pos1 |  | 11.2 |
|  |  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | pos1 |  | -5.3 |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | pos1 |  | 6.9 |

Table 8.2.1.2-11: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-11 |  | pos1 |  | -2.4 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-11 |  | pos1 |  | 10.1 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-11 |  | pos1 |  | 12.5 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-11 |  | pos1 |  | -5.7 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-11 |  | pos1 |  | 6.4 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-11 |  | pos1 |  | 8.6 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-11 |  | pos1 |  | -8.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-11 |  | pos1 |  | 3.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-11 |  | pos1 |  | 5.6 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | pos1 |  | 1.1 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | pos1 |  | 18.5 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | pos1 |  | -2.5 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | pos1 |  | 11.3 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | pos1 |  | -5.6 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | pos1 |  | 7.0 |

Table 8.2.1.2-12: Minimum requirements for PUSCH, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-12 |  | pos1 |  | -2.9 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-12 |  | pos1 |  | 10.1 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-12 |  | pos1 |  | 12.5 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-12 |  | pos1 |  | -6.0 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-12 |  | pos1 |  | 6.3 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-12 |  | pos1 |  | 8.6 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-12 |  | pos1 |  | -9.0 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-12 |  | pos1 |  | 3.1 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-12 |  | pos1 |  | 5.6 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | pos1 |  | 1.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | pos1 |  | 18.2 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | pos1 |  | -2.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | pos1 |  | 11.2 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | pos1 |  | -5.4 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | pos1 |  | 7.0 |

Table 8.2.1.2-13: Minimum requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-13 |  | pos1 |  | -2.5 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-13 |  | pos1 |  | 10.0 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-13 |  | pos1 |  | 12.5 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-13 |  | pos1 |  | -5.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-13 |  | pos1 |  | 6.2 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-13 |  | pos1 |  | 8.7 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-13 |  | pos1 |  | -8.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-13 |  | pos1 |  | 3.0 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-13 |  | pos1 |  | 5.5 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | pos1 |  | 1.7 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | pos1 |  | 18.7 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | pos1 |  | -2.1 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | pos1 |  | 11.2 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | pos1 |  | -5.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | pos1 |  | 6.9 |

Table 8.2.1.2-14: Minimum requirements for PUSCH, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix |  | Propagation conditions and correlation matrix (Annex G) |  | Fraction of maximum throughput |  | FRC (Annex A) |  | Additional DM-RS position |  | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-14 |  | pos1 |  | -2.5 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-14 |  | pos1 |  | 10.1 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-14 |  | pos1 |  | 13.1 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-14 |  | pos1 |  | -5.8 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-14 |  | pos1 |  | 6.3 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-14 |  | pos1 |  | 9.2 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-14 |  | pos1 |  | -8.7 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-14 |  | pos1 |  | 3.1 |
|  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-14 |  | pos1 |  | 5.9 |
| 2 | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | pos1 |  | 1.6 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | pos1 |  | 19.3 |
|  | 4 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | pos1 |  | -2.2 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | pos1 |  | 11.6 |
|  | 8 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | pos1 |  | -5.3 |
|  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | pos1 |  | 7.1 |

<<End of change>>