3GPP TSG-RAN WG4 Meeting #93	R4-1913209

Reno, NV, USA, 18 – 22 Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0009 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for REL-16 TS 38.307 for PC2 EN-DC TDD+TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ENDC_UE_PC2_TDD_TDD-Core |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | R4-1910771 draft CR which was endorsed in RAN4#92bis meeting. PC2 inter-band EN-DC (LTE TDD PC3 + NR TDD PC3) can be supported from Rel-15 in release independence manner. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC2 EN-DC TDD+TDD UE RF related requirement are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is missing UE RF related requirement for PC2 EN-DC TDD+TDD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************* Start of changes**************************************************

8	Release independent features for NR interworking between NR and E-UTRA

8.1	Additional EN-DC configurations

8.1.1	Intraband EN-DC

Requirements for a Rel-15 UE for additional EN-DC intraband configurations within FR1 compared to TS 38.101-3 of Rel-15 [4] are introduced via this clause.

Table 8.1.1-0: EN-DC intraband UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Intraband contiguous EN-DC power class 2 | TDD | Rel-15 | Table B.4.6-1 |
| Intraband contiguous EN-DC power class 3 | FDD, TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 2 | TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 3 | FDD, TDD | Rel-15 |  |

Table 8.1.1-1: EN-DC contiguous intraband configurations within FR1

| Feature | DL/UL | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| intraband contiguous EN-DC | DL | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | FDD, TDD | Rel-15 |  |

Table 8.1.1-2: EN-DC non-contiguous intraband configurations within FR1

| Feature | DL/UL | maximum number of sub-blocks | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| intraband non-contiguous EN-DC | DL | 2 | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 2 | 1 | 1 | FDD, TDD | Rel-15 |  |

8.1.2	Interband EN-DC

8.1.2.1	Interband EN-DC within frequency range 1

Requirements for a Rel-15 UE for additional EN-DC interband configurations within FR1 compared to TS 38.101-3 of Rel-15 [4] are introduced via this clause.

Table 8.1.2.1-0: EN-DC interband UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Interband EN-DC Power Class 2 | TDD | Rel-15 | Table B.4.6-1 |
| Interband EN-DC Power Class 3 | FDD, TDD | Rel-15 |  |

Table 8.1.2.1-1: EN-DC interband configurations without SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 4 | 5 | 2 | 2 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 |  |

Table 8.1.2.1-2: EN-DC interband configurations with SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 2 | 3 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | 2 | 2 | FDD, TDD, FDD and TDD and SUL | Rel-15 |  |

8.1.2.2	Interband EN-DC including frequency range 2

Requirements for a Rel-15 UE for additional EN-DC interband configurations including FR2 compared to TS 38.101-3 of Rel-15 [4] are introduced via this clause.

Table 8.1.2.2-1: EN-DC interband configurations including FR2

| Feature | DL/UL | number of E-UTRA bands | maximum number of E-UTRA CCs | number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 4 | 5 | 1 | 8 | TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 | 1 | 8 | TDD, FDD and TDD | Rel-15 |  |

8.1.2.3	Interband EN-DC including frequency range 1 and frequency range 2

Requirements for a Rel-15 UE for additional EN-DC interband configurations including FR1 and FR2 compared to TS 38.101-3 of Rel-15 [4] are introduced via this clause.

Table 8.1.2.3-1: EN-DC interband configurations including FR1 and FR2

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL FR1 | 4 | 4 | 1 | 2 | TDD, FDD | Rel-15 | Table B.4.6-1 |
|  | DL FR2 |  |  | 1 | 4 | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 1 | 1 | 1 | FDD, TDD | Rel-15 |  |
|  | UL FR2 |  |  | 1 | 1 | TDD, | Rel-15 |  |

******************************* End of changes ***********************************************