3GPP TSG RAN Meeting #86	RP-193123

# Sitges, Spain, December 9th – 12th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0072 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of integrated access and backhaul in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2019-12-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of IAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for support of IAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for IAB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, 38.212, 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1-1913638 |  |  |  |  |  |  |  |  |

# 14	Integrated access-backhaul operation

Throughout this specification, unless otherwise noted, statements using the term “UE” in clauses 4 through 13 are equally applicable to the IAB-node MT of an IAB node.

A procedure for an IAB-node MT to perform cell search, system information acquisition, or random access procedure is same as a corresponding one for a UE except for the following.

For initial cell selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

For PRACH transmission, an IAB-node MT determines frames and subframes within the frames containing PRACH occasions as described in [4, TS 38.211].

The IAB-node MT determines an association period for mapping SS/PBCH blocks to PRACH occasions based on a PRACH configuration period as described in Subclause 8.1 and according to Table 14.1 instead of Table 8.1-1. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec. A PRACH occasion in a PRACH slot is valid according to the conditions in Subclause 8.1.

Table 14.1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period for an IAB-node MT

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

If an IAB-node is provided a value Tdelta from a serving cell, the IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a “UE” in Subclause 4.2. The IAB-node may use the time difference to determine a DU transmission time.

A slot format for an IAB-node DU or an IAB-node MT includes downlink symbols, uplink symbols, and flexible symbols.

For each serving cell of an IAB-node DU, the IAB-node DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-Configuration. For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigDedicated-IAB-MT. If the IAB-node MT is provided tdd-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The tdd-UL-DL-ConfigDedicated-IAB-MT provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols where

-	if symbols = allDownlink, all symbols in the slot are downlink

-	if symbols = allUplink, all symbols in the slot are uplink

-	if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible.

-	if symbols = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

For each slot having a corresponding index provided by slotIndex, the IAB-MT applies a format provided by a corresponding symbols.

An IAB-node MT can be provided, by SlotFormatCombinationsPerCell-IAB-MT, a list of slot format combinations applicable for one serving cell and, by SlotFormatIndicator-IAB-MT, a configuration for monitor a DCI format 2_0 indicating a slot format combination, from the list of slot format combinations, over a number of slots as described in Subclause 11.1.1. In addition to the slot formats in Table 11.1.1-1, an SFI field for an IAB-node MT in DCI format 2_0 can indicate to the IAB-node MT a slot format from the slot formats in Table 14-1.

Table 14-1: Slot formats for normal cyclic prefix

| Slot Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 56 | U | U | U | U | U | U | U | U | U | U | U | U | U | F |
| 57 | U | U | U | U | U | U | U | U | U | U | U | U | F | F |
| 58 | U | U | U | U | U | U | U | U | U | U | U | F | F | F |
| 59 | U | U | U | U | U | U | U | U | U | U | F | F | F | F |
| 60 | U | U | U | U | U | U | U | U | U | F | F | F | F | F |
| 61 | U | U | U | U | U | U | U | U | F | F | F | F | F | F |
| 62 | U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| 63 | U | U | U | U | U | U | F | F | F | F | F | F | F | F |
| 64 | U | U | U | U | U | F | F | F | F | F | F | F | F | F |
| 65 | U | U | U | U | F | F | F | F | F | F | F | F | F | F |
| 66 | U | U | U | F | F | F | F | F | F | F | F | F | F | F |
| 67 | U | U | F | F | F | F | F | F | F | F | F | F | F | F |
| 68 | U | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 69 | U | F | F | F | F | F | F | F | F | F | F | F | F | D |
| 70 | U | U | F | F | F | F | F | F | F | F | F | F | F | D |
| 71 | U | U | U | F | F | F | F | F | F | F | F | F | F | D |
| 72 | U | F | F | F | F | F | F | F | F | F | F | F | D | D |
| 73 | U | U | F | F | F | F | F | F | F | F | F | F | D | D |
| 74 | U | U | U | F | F | F | F | F | F | F | F | F | D | D |
| 75 | U | F | F | F | F | F | F | F | F | F | F | D | D | D |
| 76 | U | U | F | F | F | F | F | F | F | F | F | D | D | D |
| 77 | U | U | U | F | F | F | F | F | F | F | F | D | D | D |
| 78 | U | U | U | U | U | U | U | U | U | U | U | U | F | D |
| 79 | U | U | U | U | U | U | U | U | U | U | U | F | F | D |
| 80 | U | U | U | U | U | U | U | U | U | U | F | F | F | D |
| 81 | U | U | U | U | U | U | U | U | U | U | U | F | D | D |
| 82 | U | U | U | U | U | U | U | U | U | U | F | F | D | D |
| 83 | U | U | U | U | U | U | U | U | U | F | F | F | D | D |
| 84 | U | F | D | D | D | D | D | D | D | D | D | D | D | D |
| 85 | U | U | F | D | D | D | D | D | D | D | D | D | D | D |
| 86 | U | U | U | F | D | D | D | D | D | D | D | D | D | D |
| 87 | U | F | F | D | D | D | D | D | D | D | D | D | D | D |
| 88 | U | U | F | F | D | D | D | D | D | D | D | D | D | D |
| 89 | U | U | U | F | F | D | D | D | D | D | D | D | D | D |
| 90 | U | F | F | F | D | D | D | D | D | D | D | D | D | D |
| 91 | U | U | F | F | F | D | D | D | D | D | D | D | D | D |
| 92 | U | U | U | F | F | F | D | D | D | D | D | D | D | D |
| 93 | U | U | U | U | U | U | U | U | U | F | F | F | F | D |
| 94 | U | U | U | U | U | U | F | F | F | F | F | F | D | D |
| 95 | U | U | U | U | U | U | F | F | D | D | D | D | D | D |
| 96 | U | U | U | U | U | U | U | D | D | D | D | D | D | D |

For a serving cell of an IAB-node MT, the IAB-node MT can be provided by guard-SymbolsProvided a number of symbols that will not be used for the IAB-node MT in slots where the IAB-node transitions between IAB-node MT and IAB-node DU. A SCS configuration for the number of symbols is provided by guardSymbol-SCS.

With reference to slots of an IAB-node DU serving cell, a symbol in a slot of an IAB-node DU serving cell can be configured to be of hard, soft, or unavailable type. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-node DU serving cell can respectively transmit, receive, or either transmit or receive in the symbol.

When a downlink, uplink, or flexible symbol is configured as soft, the IAB-node DU can respectively transmit, receive or either transmit or receive in the symbol only if

- for the IAB-node MT, ability to transmit or receive by the IAB-node DU in the soft symbol is equivalent to a configuration of the soft symbol as unavailable, or

- the IAB-node DU detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available for transmission or reception

When a symbol is configured as unavailable, the IAB-node DU neither transmits nor receives in the symbol.

A symbol of a slot is equivalent to being configured as hard if an IAB-node DU would either transmit a SS/PBCH block or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot.

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSize-AI. The IAB-node is also provided a search space set configuration, by SearchSpace-IAB, for monitoring PDCCH.

For each serving cell of an IAB-node DU in a set of serving cells of the IAB-node DU, the IAB-node DU can be provided:

-	an identity of the IAB-node DU serving cell by iabDuCellId-AI

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI

-	a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-node DU serving cell, and

-	a mapping for the soft symbol availability combinations provided by AvailabilityCombination to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

An AI index field value in a DCI format 2_5 indicates to an IAB-node DU a soft symbol availability in each slot for a number of slots for each DL BWP or each UL BWP starting from a slot where the IAB-node detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace-IAB. The AI index field includes  bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14.2.

Table 14.2: Mapping between values of resourceAvailability elements and types of soft symbol availability in a slot

| Value | Indication |
| --- | --- |
| 0 | No soft symbols are indicated available |
| 1 | Only DL soft symbols are indicated available |
| 2 | Only UL soft symbols are indicated available |
| 3 | Only DL and UL soft symbols are indicated available |
| 4 | Only Flexible soft symbols are indicated available |
| 5 | Only DL and Flexible soft symbols are indicated available |
| 6 | Only UL and Flexible soft symbols are indicated available |
| 7 | DL, UL, and Flexible soft symbols are indicated available |

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the UE obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the UE detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the UE expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot.