3GPP TSG-RAN WG1 Meeting #100	R1-2001449

# e-meeting, February 24th – March 6th 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0076 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of NR positioning support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Essential corrections to allow rel-16 specs to fully implement agreements reached in RAN1 like fixing PRS sequence generation and SRS prioritization rules. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections to procedures for PRS reception and SRS for positioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete or erroneous support for positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.6.5	PRS reception procedure

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters DL-PRS-ResourceSet and DL-PRS-Resource. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS Positioning Frequency Layer configuration(s) as indicated by the higher layer parameter DL-PRS-PositioningFrequencyLayer. A DL PRS Positioning Frequency Layer is defined as a collection of DL PRS Resource Sets which have common parameters configured by DL-PRS-PositioningFrequencyLayer.

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters DL-PRS-PositioningFrequencyLayer, DL-PRS-ResourceSet and DL-PRS-Resource.

A positioning frequency layer consists of one or more PRS resource sets and it is defined by:

-	DL-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-SubcarrierSpacing. The supported values of DL-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211].

-	DL-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-CyclicPrefix. The supported values of DL-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	DL-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS Resource Set have common Point A and all DL PRS Resources sets belonging to the same DL-PRS-PositioningFrequencyLayer have a common Point A.

The UE expects that it will be configured with [IDs] each of which is defined such that it is associated with multiple DL PRS Resource Sets from the same cell. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource.

A DL PRS resource set consists of one or more DL PRS resources and it is defined by:

-	DL-PRS-ResourceSetId defines the identity of the DL PRS resource set configuration.

-	DL-PRS-Periodicity defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 
TperPRS∈2μ4, 8, 16, 32, 64, 5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480slots, where μ=0, 1, 2, 3 for DL-PRS-SubcarrierSpacing=15, 30, 60 and 120kHz respectively. TperPRS=2μ⋅μ=0.All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS periodicity.

-	DL-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32,. All the DL PRS resources within one resource set have the same ResourceRepetitionFactor

-	DL-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same DL-PRS-ResourceID within a single instance of the DL PRS resource set and takes values TgapPRS∈1,2,4,8,16,32. The UE only expects to be configured with DL-PRS-ResourceTimeGap if DL-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a DL-PRS-ResourceSet is not expected to exceed the configured value of DL-PRS-Periodicity. All the DL PRS resources within one resource set have the same DL-PRS-ResourceTimeGap.

-	DL-PRS-MutingPattern defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap size can be {2, 4, 6, 8, 16, 32} bits long. The bitmap has two options for applicability. In the first option each bit in the bitmap corresponds to a configurable number provided by higher layer parameter DL-PRS.MutingBitRepetitionFactor of consecutive instances of a DL-PRS-ResourceSet where all the DL-PRS-Resources within the set are muted for the instance that is indicated to be muted. In the second option each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS-Resources within each instance of a DL-PRS-ResourceSet and the length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in clause 7.4.1.7.4 of [4, TS 38.211].

-	DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of [FFS in RAN2].

-	DL-PRS-ResourceSetSlotOffset defines the slot offset with respect to SFN0 slot 0 and takes values ToffsetPRS∈0,1,…,TperPRS-1.

-	DL-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.1 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of DL-PRS-combSizeN.

-	DL-PRS-ResourceBandwidth defines the number of resource blocks configured for PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of DL-PRS-ResourceBandwidth.

-	DL-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter DL-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of Start PRB.

A DL PRS resource is defined by:

-	DL-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set.

-	DL-PRS-ResourceId determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	DL-PRS-SequenceId is used to initialize cinit value used in pseudo random generator [4, TS38.211, 7.4.1.7.2] for generation of DL PRS sequence for a given DL PRS resource.

-	DL-PRS-ReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS38.211].

-	DL-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL-PRS-ResourceSetSlotOffset

-	DL-PRS-ResourceSymbolOffset determines the starting symbol of the DL PRS resource within the starting slot.

-	DL-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.1 of [4, TS38.211].

-	DL-PRS-QCL-Info defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be ‘QCL-Type-D’ with a DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be ‘QCL-Type-C’ with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both ‘QCL-Type-C’ and ‘QCL-Type-D’ with a SS/PBCH Block then the SSB index indicated should be the same.

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a PRS resource set ID, and optionally a single PRS resource ID or a list of PRS resource IDs. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference time for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the [ID], the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

The UE may be configured to report quality metrics corresponding to the RSTD and UE Rx-Tx time difference measurements which include the following fields:

-	TimingMeasQuality-Value which provides the best estimate of the uncertainty of the measurement

-	TimingMeasQuality-Resolution which specifies the resolution levels used in the Value field

The UE expects to be configured with higher layer parameter DL-PRS-expectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and DL-PRS-expectedRSTD-uncertainty, which defines a search window around the expectedRSTD.

For DL UE positioning measurement reporting in higher layer parameters DL-PRS-RstdMeasurementInfo or DL-PRS-UE-Rx-Tx-MeasurementInfo the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, UE Tx-Rx time difference or the DL PRS-RSRP.

The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report multiple UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers.

For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter Timestamp. The Timestamp can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by DL-PRS-RSTDReferenceInfo.

The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. When the UE is expected to measure the DL PRS resource outside the active DL BWP it may request a measurement gap in higher layer parameter [XYZ].

The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell.

The UE may be configured to report, subject to UE capability, up to 4 DL RSTD measurements per pair of cells with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those cells. The up to 4 measurements being performed on the same pair of cells and all DL RSTD measurements in the same report use a single reference timing.

The UE may be configured to measure and report up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception.

If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are from the same cell. If DL-PRS-QCL-Info is configured to the UE with ‘QCL-Type-D’ with a source DL-PRS-Resource then the DL-PRS-ResourceSetId and the DL-PRS-ResrouceId of the source DL-PRS-Resource are expected to be indicated to the UE.

The UE does not expect to process the DL PRS in the same symbol where other DL signals and channels are transmitted to the UE when there is no measurement gap configured to the UE.

<omitted text>

### 6.2.1	UE sounding procedure

The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet. For each SRS resource set, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306] except when SRS is configured with the higher layer parameter [SRS-for-positioning] in which case the maximum value of K is 16. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource.

-	srs-ResourceId determines SRS resource configuration identity.

-	Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset except when SRS is configured with the higher layer parameter [SRS-for-positioning] in which case the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource.

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource.

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0.

-	Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0.

-	Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero.

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 and 8, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb value as defined by the higher layer parameter transmissionComb described in Clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset as defined by the higher layer parameter combOffset-n2, combOffset-n4, or combOffset-n8 for transmission comb value 2, 4, or 8 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211].

-	SRS sequence ID as defined by the higher layer parameter sequenceId in Clause 6.4.1.4 of [4].

-	The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When SRS is configured by the higher layer parameter [SRS-for-positioning] the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter.

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent symbols within the last 6 symbols of the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter [SRS-for-positioning] the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

If a UE is not configured with [intraUEPrioritization] and PUSCH and SRS configured by SRS-Resource are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a UE is configured with [intraUEPrioritization] and a PUSCH transmission would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s).

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'periodic':

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo containing the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS. When the SRS is configured by the higher layer parameter [SRS-for-positioning] and if the higher layer parameter spatialRelationInfo contains the ID of a reference ’DL-PRS-ResourceId’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'semi-persistent':

-	when a UE receives an activation command, as described in clause 6.1.3.17 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the SRS is configured with the higher layer parameter [SRS-for-positioning], each ID in the list of reference signal IDs may also refer to a reference SS/PBCH block of a non-serving cell or DL PRS of a serving or non-serving cell indicated by a higher layer parameter.

-	if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo, the UE shall assume that the ID of the reference signal in the activation command overrides the one configured in spatialRelationInfo.

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS. When the SRS is configured by the higher layer parameter [SRS-for-positioning] and if the higher layer parameter spatialRelationInfo contains the ID of a reference ’DL-PRS-ResourceId’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

If the UE has an active semi-persistent SRS resource configuration and has not received a deactivation command, the semi-persistent SRS configuration is considered to be active in the UL BWP which is active, otherwise it is considered suspended.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 + 14. The minimal time interval in units of OFDM symbols is counted based on the minimum subcarrier spacing between the PDCCH and the aperiodic SRS.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and except when SRS is configured with the higher layer parameter [SRS-for-positioning], the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot  where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offsetCA and the  for the {scheduling, scheduled} carrier pair is defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter [SRS-for-positioning], the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot  where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offsetCA and the  for the {scheduling, scheduled} carrier pair is defined in [4, TS 38.211] clause 4.5.

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo contains the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS. When the SRS is configured by the higher layer parameter [SRS-for-positioning] and if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘DL-PRS-ResourceId’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.xx of [10, TS 38.321], for an SRS resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. [The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise.] When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

The UE is not expected to be configured with different time domain behavior for SRS resources in the same SRS resource set. The UE is also not expected to be configured with different time domain behavior between SRS resource and associated SRS resources set.

For single carrier operations, the UE does not expect to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter [SRS-for-positioning] and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘periodic’.

For single carrier operations, the UE does not expect to be triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter [SRS-for-positioning] and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘semi-persistent’ or ‘aperiodic’.

The SRS request field [5, TS38.212] in DCI format 0_1, 1_1, 0_2 (if SRS request field is present), 1_2 (if SRS request field is present) indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38.212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Clause 7.3 of [5, TS 38.212] if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB', or indicates the SRS transmission on a set of serving cells configured by higher layers if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA'.

For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s).

In case of intra-band carrier aggregation or in inter-band CA band-band combination where simultaneous SRS and PUCCH/PUSCH transmissions are not allowed, a UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band carrier aggregation or in inter-band CA band-band combination where simultaneous SRS and PRACH transmissions are not allowed, a UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured.

When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter [applicableCellList], the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

When the higher layer parameter enableDefaultBeamPlForSRS is set ‘enabled’, and if the higher layer parameter spatialRelationInfo for the SRS resource, except for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'beamManagement' or for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’ with configuration of associatedCSI-RS or for the SRS resource configured by the higher layer parameter [SRS-for-positioning], is not configured in FR2 and if the UE is not configured with higher layer parameter(s) pathlossReferenceRS, the UE shall transmit the target SRS resource

-	with the same spatial domain transmission filter used for the reception of the CORESET with the lowest controlResourceSetId in the active DL BWP in the CC.

-	with the same spatial domain transmission filter used for the reception of the activated TCI state with the lowest ID applicable to PDSCH in the active DL BWP of the CC if the UE is not configured with any CORESET in the CC

<omitted text>