3GPP TSG RAN meeting #87e										RP-200263

Electronic meeting, Mar 16-19, 2020

Status Report to TSG

Agenda item:			9.3.17

| WI / SI Name | RF requirements for NR frequency range 1 (FR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR1 |  |  |  |  |
| Unique ID | 830087 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-193266 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 06/2020 | Performance part: 09/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 70% | Performance Part: 30% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Zhangqian, Yangshan |
|  | Company | Huawei, HiSilicon, China Telecom |
|  | Email | Zhangqian67@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #100e meeting

- Switching period between case 1 and case 2

Agreement:

- If uplink Tx switching is triggered, the length of the additional time for PUSCH preparation procedure equals to the length of UL switching period.

Agreements:

- For SUL, if uplink Tx switching is configured, the state of Tx chains of last UL transmission is assumed in case of no UL transmission.

- Work assumption: For inter-band UL CA, if uplink Tx switching is configured, the state of Tx chains of last UL transmission is assumed in case of no UL transmission. It can be revisited in RAN1#100bis.

Clarify the following agreements with changes in red

Agreements:

- For standalone SUL, if UL switching period is configured by RRC

- The switching period is not always applicable on the carrier configured with switching period.

- The switching period is only applicable when the scheduled UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2.

  - For each UL transmission occasion on a carrier, the existence of the switching period is determined one time every occasion.

  - Note: 2Tx carrier 2 refers to an UL carrier capable of 2 Tx chains and both 1-port and 2-port UL transmissions.

Agreement:

- For inter-band UL CA, if uplink Tx switching is configured, UE is not expected to be scheduled or configured with UL transmissions that result in simultaneous 1Tx transmission on carrier 1 and 2Tx transmission on carrier 2.

Working assumption:

- For inter-band UL CA, if option 2 is supported, the following sub-option 2-3 is defined.

  - Minimize RAN1 impact

  - No new RAN4 impact

  - No new TDM pattern

- It can be revisited in future RAN1 meeting with taking into consideration any relevant RAN4 decisions on DL interruption for UL Tx switching.

- Option 2-3

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

Agreements:

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

- For inter-band UL CA, if UL switching period is configured by RRC

  - The switching period is not always applicable on the carrier configured with switching period.

  - The switching period is at least applicable between 1-port transmission in carrier 1 and 2-port transmission in carrier 2.

Agreements:

- For inter-band UL CA, UE is not expected to [be scheduled or configured to] transmit on any of the two carriers in the switching period.

  - FFS: Whether to handle the case when switching period cannot be ensured by gNB.

2.1.2	Remaining Open issues

- Switching period between case 1 and case 2

- The related issue and solutions for inter-band UL CA, including mapping between UL transmission port and Tx chain, switching mechanism, support of codebook/non-codebook based PUSCH transmission, etc.

- The related issue and solutions for inter-band EN-DC without SUL, including mapping between UL transmission ports and Tx chain, TDM pattern, switching mechanism, handling of transmission collision between 1Tx transmission in LTE and 2Tx transmission in NR, etc.

2.2	RAN2

2.2.1	Agreements

RAN2 #109e meeting

- Switching period between case 1 and case 2

- Initial discussion on 38.306 and 38.331 CR was taken place, an mail discussion was allocated as [Post-109e][R16 Other] UL TX Switching – NR-FR1, to progress CRs as far as possible.

2.2.2	Remaining Open issues

- Switching period between case 1 and case 2

- Specify the corresponding signaling support.

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.1.1	Agreements

RAN4 #94e meeting

- Almost contiguous allocation for CP-OFDM

- Almost contiguous allocation for CP-OFDM related issues was agreed in R4-2001762.

- Intra-band contiguous DL CA for FR1

  - CR R4-2002908 was agreed on corrections for n48 receiver requirements.

  - CR R4-2002810 was agreed on new BCS for intra-band contiguous CA configurations n77C and n78C.

  - CR R4-2002811 was agreed on intra-band configurations and DL requirements.

- Intra-band contiguous UL CA for FR1 power class 3

- WF for intra-band contiguous UL CA emission requirements was approved in R4-2002804, agreements on SEM and ACLR were captured.

- WF on intra-band contiguous UL CA MPR requirement was approved in R4-2002805, inner/outer allocation for contiguous and non-contiguous RB allocation agreements were captured.

- CR R4-2002051 was endorsed.

- Intra-band non-contiguous UL CA for FR1 power class 3

  - WF on RF architecture of intra-band UL non-contiguous CA was approved in R4-2002812. The consideration on the RF architecture and related impact on RF requirement were captured. Some agreements on RF requirement for intra-band UL non-contiguous CA were also captured.

- Switching period between case 1 and case 2

- RF session

  - WF on RF requirements for Tx switching between two uplink carriers was approved in R4-2002815.

  - RAN4 LS to RAN1 and RAN2,  capturing RAN4 agreements on UE Tx switching period delay and DL interruption, was approved in R4-2002816.

- RRM session

  - WF on RRM requirements for Tx switching between two uplink carriers was approved R4-2002243.

    - Remaining Open issues

- Intra-band contiguous UL CA for FR1 power class 3

- Requirements to be defined

- Intra-band non-contiguous UL CA for FR1 power class 3

- Requirements to be defined

- Intra-band EN-DC MPR/A-MPR improvement

- EN-DC 3_n3 MPR requirement to be defined.

- Modify time masks for ULSUP-TDM accounts for UL timing misalignment between the EUTRA and NR cell groups.

- Switching period between case 1 and case 2

- RF time mask requirements to be defined

- RRM interruption requirements as well as the applicability of RRM interruption requirements to be defined

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #94e

[1] R4-2000069, Time masks for ULSUP in R166, Huawei, HiSilicon

[2] R4-2000070, CR to 38.101-3 on time masks for ULSUP in R16, Huawei, HiSilicon

[3] R4-2000093, Intra-band Contiguous ULCA MPR, Qualcomm

[4] R4-2000104, Intra-band non-contiguous ULCA requirements and MPR consideration, Qualcomm

[5] R4-2000234, CR for 38.101-1 to introduce BCS1 for CA_n77C and CA_n78C, Huawei, HiSilicon

[6] R4-2000711, [NRULCA] Definition of Intra-band Contiguous UL CA Allocation Types, Skyworks

[7] R4-2000712, [NRULCA] Definition of Intra-band Contiguous UL CA Bandwidths, Skyworks

[8] R4-2000713, [NRULCA] PC3 Back-off Measurements for NR intra-band contiguous UL CA, Skyworks

[9] R4-2000754, CR for TS 38.101-1: Corrections for n48 receiver requirements, MediaTek

[10] R4-2001074, CR for 38.101-1: simply intra-band CA operating band table in clause 5.2A.1, Huawei, HiSilicon

[11] R4-2001077, CR for 38.101-1 to correct bandwidth class B, Huawei, HiSilicon

[12] R4-2001129, [NRULCA] Bandwidth Limitation in FR1 UL CA, Skyworks

[13] R4-2001385, UE behavior and time masks for ULSUP-TDM	, Ericsson

[14] R4-2001756, on FR1 UL CA MPR requirement Rel-16, Huawei, HiSilicon

[15] R4-2001759, CR on FR1 UL CA MPR requirement Rel-16, Huawei, HiSilicon

[16] R4-2001762, CR for 38.101-1 almost contiguous resource allocation, Huawei, HiSilicon

[17] R4-2001771, CR for intra-band CA configuration and DL requirement, Huawei, HiSilicon

[18] R4-2001772, CR for intra-band UL CA emission requirement, Huawei, HiSilicon

[19] R4-2001773, CR for intra-band UL CA output power, Huawei, HiSilicon

[20] R4-2001774, CR for intra-band UL CA signal quality, Huawei, HiSilicon

[21] R4-2002804, WF on emission RF requirement for intra-band contiguous UL CA, Huawei, HiSilicon

[22] R4-2002805, WF on MPR requirement for intra-band contiguous UL CA, Huawei, HiSilicon

[23] R4-2002812, WF on RF architecture for intra-band UL non-contiguous CA, Huawei, HiSilicon

[24] R4-2002051, draft CR for TS 38.101-1 intra-band UL contiguous CA combinations, Huawei, HiSilicon

[25] R4-2002810, CR for 38.101-1 to introduce BCS1 for CA_n77C and CA_n78C, Huawei, HiSilicon

[26] R4-2002811, CR for intra-band CA configuration and DL requirement, Huawei, HiSilicon

[27] R4-2002908, CR for TS 38.101-1: Corrections for n48 receiver requirements, MediaTek

[28] R4-2000064, Discussion on the switching between 1Tx carrier and 2Tx carrier, Huawei, HiSilicon

[29] R4-2000065, Views on DL interruptions during UE switching between 1Tx carrier and 2Tx carrier, Huawei, HiSilicon

[30] R4-2000066, CR to 38.101-1 on UE requirements for switching between 1Tx carrier and 2Tx carrier, Huawei, HiSilicon

[31] R4-2000067, CR to 38.101-3 on UE requirements for switching between 1Tx carrier and 2Tx carrier, Huawei, HiSilicon

[32] R4-2000068, Draft LS to RAN1 on DL reception interruption due to switching between 1Tx carrier and 2Tx carrier, Huawei, HiSilicon

[33] R4-2000113, Switching time details, Qualcomm Incorporated

[34] R4-2000125, Discussion on potential gain of large switching periods, vivo

[35] R4-2000131, RF time mask requirements to allow Tx switching between two uplink carriers, China Telecom

[36] R4-2000132, CR to TS 38.101-1: Switching time mask between two uplink carriers in UL CA and SUL, China Telecom

[37] R4-2000133, CR to TS 38.101-3: Switching time mask between two uplink carriers in EN-DC, China Telecom

[38] R4-2000134, LS on UE capability for switching between two uplink carriers, China Telecom

[39] R4-2000135, View on RRM interruption and delay requirement for switching between two uplink carriers, China Telecom

[40] R4-2000457, Interruption for Tx switching between two uplink carriers, MediaTek inc.

[41] R4-2000628, discussion on open issue for Tx switching between 2 uplink carriers, CATT

[42] R4-2000640, Discussion on DL interruption Tx switching between two uplink carriers, vivo

[43] R4-2000643, Requirements for switching between case1 and case2, CMCC

[44] R4-2000793, On RRM impact of Tx switching, Apple

[45] R4-2000810, Further consideration on uplink carrier switching, ZTE Wistron Telecom AB

[46] R4-2000991, On RRM requirements for Tx switching between two uplink carriers, OPPO

[47] R4-2001307, Switching time between NR between FR1 uplink carriers, MediaTek Inc.

[48] R4-2001428, CR to TS 38.101-1: Time mask requirements for switching between 1Tx and 2Tx transmissions for inter-band UL CA, Nokia, Nokia Shanghai Bell

[49] R4-2001429, CR to TS 38.101-3: Time mask requirements for switching between 1Tx and 2Tx transmissions for inter-band EN-DC without SUL, Nokia, Nokia Shanghai Bell

[50] R4-2001430, Switching between case 1 and case 2 for two NR FR1 carriers, Nokia, Nokia Shanghai Bell

[51] R4-2002183, Email discussion summary for RAN4#94e_#60_NR_RF_FR1_RRM, Moderator (Huawei, HiSilicon)

[52] R4-2002243, WF on RRM requirements for Tx switching between two uplink carriers, Huawei, HiSilicon

[53] R4-2002314, Email discussion summary for RAN4#94e_#60_NR_RF_FR1_RRM, Moderator (Huawei, HiSilicon)

[54] R4-2002692, Email discussion summary for RAN4#94e_#19_NR_RF_FR1_Part_2, Moderator (China Telecom)

[55] R4-2002815, WF on RF requirements for Tx switching between two uplink carriers, China Telecom

[56] R4-2002816, LS on Tx switching between two uplink carriers, Apple

[57] R4-2002817, CR to TS 38.101-1: Switching time mask between two uplink carriers in UL CA and SUL, China Telecom

[58] R4-2002818, CR to TS 38.101-3: Switching time mask between two uplink carriers in EN-DC, China Telecom

[59] R4-2002888, Email discussion summary for RAN4#94e_#19_NR_RF_FR1_Part_2, Moderator (China Telecom)

RAN1 #100e

- R1-2000302, Potential RAN1 impact to support TDM switching between the 1Tx and 2Tx UL carriers, vivo, RAN1 #100, Feb.2020.

- R1-2000354, Remaining Issues on Support of Tx Switching between Two Uplink Carriers, ZTE, RAN1 #100, Feb.2020.

- R1-2000515, Remaining issues on UL switching, CATT, RAN1 #100, Feb.2020.

- R1-2000552, On Tx switching between two uplink carriers, Nokia, Nokia Shanghai Bell, RAN1 #100, Feb.2020.

- R1-2000597, Remaining issues on support of Tx switching between two uplink carriers, China Telecom, RAN1 #100, Feb.2020.

- R1-2000792, Introduction of switched uplink operation, Nokia, RAN1 #100, Feb.2020.

- R1-2000883, RAN1 aspects of UL Tx switching, Ericsson, RAN1 #100, Feb.2020.

- R1-2000940, Discussion on Tx Switching between Two Uplink Carriers, OPPO, RAN1 #100, Feb.2020.

- R1-2000949, Discussion on switched UL impact in RAN1, Qualcomm Incorporated, RAN1 #100, Feb.2020.

- R1-2001026, Discussion on the remaining issues of supporting Tx switching between two uplink carriers, Huawei, HiSilicon, RAN1 #100, Feb.2020.

- R1-2001132, [Update] Remaining Issues on Support of Tx Switching between Two Uplink Carriers, ZTE, RAN1 #100, Feb.2020.

- R1-2000882, Summary of Tx switching between two uplink carriers, China Telecom, RAN1 #100, Feb.2020.

- R1-2001204, Summary#2 of Tx switching between two uplink carriers, China Telecom, RAN1 #100, Feb.2020.

- R1-2001274, [100e-5.1LS-TxSwitching-02] Email discussion/approval on remaining issues on PUSCH preparation procedure, China Telecom, RAN1 #100, Feb.2020.

- R1-2001275, [100e-5.1LS-TxSwitching-03] Email discussion/approval on remaining issues on inter-band UL CA, China Telecom, RAN1 #100, Feb.2020.

- R1-2001276, [100e-5.1LS-TxSwitching-04] Email discussion/approval on remaining issues on inter-band EN-DC without SUL, China Telecom, RAN1 #100, Feb.2020.

RAN2 #109e

- R2-2000019	Reply LS on Tx switching between two uplink carriers (R1-1913585; contact: China Telecom)	RAN1

- R2-2000043	LS on UE capabilities and RRC signalling on Tx switching period delay (R4-1916083; contact: Apple)	RAN4

- R2-2000861	Introduction of UL Tx switching for UL MIMO in FR1	Nokia, Nokia Shanghai Bell

- R2-2000862	RRC signalling for Tx switching for UL MIMO	Nokia, Nokia Shanghai Bell

- R2-2000863	UE capabilities for Tx switching for UL MIMO	Nokia, Nokia Shanghai Bell

- R2-2000870	RRC signalling for Tx switching for UL MIMO	Nokia, Nokia Shanghai Bell

- R2-2000871	UE capabilities for Tx switching for UL MIMO	Nokia, Nokia Shanghai Bell

- R2-2001580	Discussion on support of UL Tx swithing	China Telecom, CMCC, China Unicom, Orange, Huawei, HiSilicon, ZTE,  CATT, vivo

- R2-2001581	38306CR UE capability of supporting UL Tx switching	China Telecom, Huawei, HiSilicon, CMCC, ZTE, China Unicom

- R2-2001582	38331CR UE capability and RRC configuration of supporting UL Tx switching	ChinaTelecom, Huawei, HiSilicon, CMCC, ZTE, China Unicom

- R2-2002060	RRC signalling for Tx switching for UL MIMO	Nokia, Nokia Shanghai Bell

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template