3GPP TSG RAN Meeting #87e	                                              RP-200326

Electronic Meeting, March 16-19, 2020

Agenda Item:	9.9

Source: 	Apple Inc.

Title:  	on FR2 combinations fallback

Document for:	Discussion and Decision

# Discussion

In RAN4 #92 meeting, CRs for handling of fallbacks for combined contiguous and non-contiguous CA in FR2 in [1], [2] were agreed, and corresponding LS [3] was agreed and sent out to RAN2, informing RAN2 about RAN4 agreement shown as below.

1. Overall Description: RAN4 would like to inform RAN2 about a decision on mandatory fallbacks in FR2: A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier. Deactivating carriers within the CA or DC combination is still possible. RAN4 would like to ask RAN2 to help enabling signalling of CA/DC configurations without mandating all possible fallback combinations. 2. Actions: ACTION: RAN4 respectfully asks RAN2 to take the above decision into account and take the appropriate actions, if any, to enable not mandating support of all possible fallbacks.

Based on the LSin, RAN2 has been discussing how to reflect the RAN4 agreement into RAN2 specs since #107bis meeting (Oct. 2019). During and after RAN2#107bis, this issue was discussed online and also in a post-meeting email discussion [4]. 5 potential solutions were discussed and captured in [4]. In RAN2#108 meeting, it was further extensively discussed during the online and offline discussions. In [5], one solution proposed by the offline discussion rapporteur well captured and addressed concerns raised by several companies. Then in the latest RAN2 #109e meeting, multiple companies co-sourced a contribution [6] and proposed a solution, which is a further improvement based on the solution in [5] with necessary simplifications.

After three meetings’ discussions, in the last #109e meeting, RAN2 achieved the following agreements.

- R2 assume to follow R4 decision to not support all fall-backs

- A reply LS to RAN4 is noted and this discussion on LS is closed with no outcome

RAN2 meeting minutes is copied below.

R2-2000600 Handling of fallbacks for combined contiguous and non-contiguous CA in FR2 Apple, Nokia, Nokia Shanghai Bell, OPPO, Intel discussion Rel-15 NR_newRAT-Core R2-2001222 Handling of FR2 fallback band combinations Ericsson discussion DISCUSSION on the two tdocs above Chair’s decided way forward R2 assume to follow R4 decision to not support all fall-backs. Send an LS to R4 with questions to understand more detailed requirements for a solution, and understand better what R4 actually means with not supporting all fallbacks. Next Q expect to agree on the solution (solution could cover impact in R4 and R2). [AT109e][071][NR15] FR2 Fallback Combinations (MTK) Scope: LS to R4 on the details according to above. Intended outcome: Approved LS Deadline: Mar 5 CET R2-2002312 [DRAFT] Reply LS on Handling of Fallbacks for combined contiguous and non-contiguous CA or DC configurations in FR2 MediaTek Inc LS out Rel-15 NR_newRAT-Core noted close this discussion, no outcome

We believe it is time for RAN2 to work out corresponding solutions on FR2 fallbacks as clearly requested by the RAN4 LS, without further delay and complications back and forth.

Proposal 1: RAN2 should work out corresponding solutions on FR2 fallbacks within Rel-16 timeframe as clearly requested by the RAN4 LS.

# Summary

Based on the discussion above, here we propose:

Proposal 1: RAN2 should work out corresponding solutions on FR2 fallbacks within Rel-16 timeframe as clearly requested by the RAN4 LS.

# References

- R4-1908028, "CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc., CR TS 38.101-3

- R4-1910238, "CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc., CR TS 38.101-2

- R4-1910239  "LS on handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc.

- R2-1915507,  “Report for [107bis#36][NR R15] FR2 CA Fallbacks”,  (Nokia)

- R2-1916498,  “Report for [107bis#36] [NR R15] FR2 CA Fallbacks  (Nokia)”,	Nokia, Nokia Shanghai Bell

- R2-2000600 Handling of Fallbacks for Contiguous and Non-contiguous CA in FR2         Apple, Nokia, Nokia Shanghai Bell, OPPO, Intel