3GPP TSG RAN Meeting #87e RP-200447 # e-Meeting, March 16-19, 2020 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; 0098; rev; -; Current version:; 16.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; x; Radio Access Network; x; Core Network. Title:; Corrections to 38.213 including alignment of terminology across specifications in RAN1#100-e. Source to WG:. Source to TSG:; Samsung. Work item code:; NR_newRAT-Core; Date:; 2020-03-13. Category:; A; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; - Clarify that PRACH preambles can be left unused when, for a SSB-RO association, multiple SS/PBCH blocks are mapped to a RACH occasion. - Clarify mapping between a closed-loop power control index l and open loop power control configurations.. Summary of change:; - Add “or preambles” in the description for unmapped RACH occasions. - Delete ‘any of’ in “If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is any of the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j”.. Consequences if not approved:; Ambiguous specifications.. Clauses affected:; 7.1.1, 8.1. Y; N. Other specs; x; Other core specifications; TS/TR ... CR .... affected:; x; Test specifications; TS/TR ... CR .... (show related CRs); x; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated Impact Analysis: Isolated impact analysis: The changes in this CR represent common understanding within RAN1. There is no network or UE functionality modified or introduced by this CR.. This CR's revision history:.

### 7.1.1	UE behaviour

## *** Unchanged text is omitted ***

-	If a configuration for a corresponding  value is provided by higher layers

where  is determined from the value of  as

-	If  and the UE is provided higher SRI-PUSCH-PowerControl,  is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to

-	If  and the UE is not provided SRI-PUSCH-PowerControl or j=0,

-	If ,  is provided by the value of powerControlLoopToUse

## *** Unchanged text is omitted ***

8.1	Random access preamble

## *** Unchanged text is omitted ***

An association period, starting from frame 0, for mapping SS/PBCH blocks to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH blocks are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH blocks to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to  SS/PBCH blocks, no SS/PBCH blocks are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH blocks after an integer number of association periods, if any, are not used for PRACH transmissions.