Annex 1

[Editor’s note: It would be advisable to update the Tables in Annex 1 (Tables 2-4) to reflect IMT-2020 capabilities. As 3GPP indicated in 5D/913: “Features relevant to PPDR continue to evolve to include enhancements and fulfil new requirements. The LTE radio technology addresses the PPDR use case and LTE (Rel. 15 and onwards) is part of the 3GPP proposal for radio interface technologies to be included in IMT-2020. NR radio technology is also providing solutions that could be used to provide PPDR applications, such as those based on the URLLC use case. 3GPP technology provides support to PPDR within the IMT-2020 framework.”, it is proposed to request Tables 2-4 be updated by 3GPP to reflect Rel.15 at a minimum.]

IMT technology capabilities which can be used as a base
to develop mission critical applications

Tables 2, 3 and 4 herein provide details of the IMT specifications that support the PPDR requirements outlined in Annexes 4 and 5 of Report ITU-R M.2377. As broadband PPDR applications need to provide the capabilities of earlier narrowband and wideband applications, the separate tables in Report ITU-R M.2377 have been consolidated here to reflect the overall capabilities of IMT systems.

The use of the term ‘enabled’ in the column entitled “Support and 3GPP Release Number”, in the tables below, indicates that the specifications make it possible for an implementation to provide the functionality described, although this functionality is not listed in the specifications. Further, functionality under development but for which specifications have not been completed is listed as ‘not supported’. Some ‘not supported’ functionality and additional functionality not listed in the tables below will be added in successive releases.

TABLE 2

PPDR applications and examples

| Application | Feature | PPDR Example | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- | --- |
| Voice | Person-to-person | Selective calling and addressing | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | One-to-many | Dispatch and group communication | Supported in R13 | Support3a expected post Rel-16 |  |
|  | Talk-round/ direct mode operation | Groups of portable to portable/mobile-mobile in close proximity without infrastructure | Supported in R13 | Support3a expected post Rel-16 |  |
|  | Push-to-talk | Push-to-talk | Supported in R13 | Support3a expected post Rel-16 |  |
|  | Instantaneous access to voice path | Push-to-talk and selective priority access | Supported in R13 | Support3a expected post Rel-16 |  |
|  | Phone interconnect | Telephone call from/to radio subscriber | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Dispatcher terminal | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Multi select | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | CAD | Computer aided dispatch | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | Security | Voice encryption/scrambling | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Facsimile | Person-to-person | Status, short message | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | Emergency alert | Pressing the emergency button causes alert at the TG or dispatcher | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | Security | Data encryption/scrambling | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | One-to-many (broadcasting) | Initial dispatch alert (e.g. address, incident status) | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Messages | Person-to-person | Status, short message, short e-mail | Supported; Rel. Independent | Support3a expected post Rel-16 |  |  |  |
|  | One-to-many (broadcasting) | Initial dispatch alert (e.g. address, incident status) | Supported; Rel. Independent | Support3a expected post Rel-16 |  |  |  |

TABLE 2 (cont.)

| Application | Feature |  | PPDR Example |  |  | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security | Priority/instant-aneous access | Man down alarm button | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | Emergency alert | Pressing the emergency button causes alert at the TG or dispatcher | Supported in R13 | Support3a expected post Rel-16 |  |
|  | Emergency call | Priority voice call caused by pressing the emergency button | Supported in R13 | Support3a expected post Rel-16 |  |
| Location Telemetry | Location status | GPS latitude and longitude information | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | Sensory data | Vehicle telemetry/status | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | EKG (electrocardiograph) in field | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Environmental information including sensory data on air quality, temperature, contamination, radiation levels etc. | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Database interaction (minimal record size) | Forms based records query | Accessing vehicle license records | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Accessing criminal records/missing person | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Computer aided dispatch directly to field resources | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  | Forms based incident Report |  | Filing field Report |  |  | Supported; Rel. Independent |  | Support3a expected post Rel-16 |  |  |
| Messages | E-mail possibly with attachments |  | Routine e-mail message |  |  | Supported; Rel. Independent |  | Support3a expected post Rel-16 |  |  |
| Privacy | Security |  | Data encryption/scrambling |  |  | Supported in R13 |  | Support3a expected post Rel-16 |  |  |
| Data Talk-around/direct mode operation | Direct unit to unit communication without additional infrastructure |  | Direct handset to handset, on-scene localized communications |  |  | Supported in R12 (enabled via L3 network) Fully supported in R13 |  | Support3a expected post Rel-16 |  |  |

TABLE 2 (cont.)

| Application | Feature | PPDR Example | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- | --- |
| Database interaction (medium record size) | Forms and records query | Accessing medical records | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Lists of identified person/missing person | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Computer aided dispatch directly to field resources | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Computer aided dispatch directly to field resources | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | GIS (geographical information systems) | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Text file transfer | Data transfer | Filing report from scene of incident | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Records management system information on offenders | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Downloading legislative information | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Image transfer | Download/ upload of compressed still images | Biometrics (finger prints, facial recognition) | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | ID picture (car number plate recognition) | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Building layout maps | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Telemetry | Location status and sensory data | Vehicle status | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| OTAP | Over the air programming | UE programming through the air | Supported in R13 | Support3a expected post Rel-16 |  |
| Security | Priority access | Critical care | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Video | Download/ upload compressed video | Video clips | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Patient monitoring (may require dedicated link) | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Video feed of in-progress incident | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Interactive | Location determination | 2-way system | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
|  |  | Interactive location data | Supported; Rel. Independent | Support3a expected post Rel-16 |  |
| Direct mode operation of video and data | Direct unit to unit video and data communication without infrastructure | Direct handset to handset, on-scene localized command and control | Supported in R14 | Support3a expected post Rel-16 |  |
| Privacy | Security | Data encryption/scrambling | Supported in R13 | Support3a expected post Rel-16 |  |

TABLE 2 (end)

| Application |  | Feature |  | PPDR Example |  |  | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Database access | Intranet/Internet access | Accessing architectural plans of buildings, location of hazardous materials | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Web browsing | Browsing directory of PPDR organization for phone number | Supported; Rel. Independent | Supported; Rel. Independent |  |
| Robotics control |  | Remote control of robotic devices |  | Bomb retrieval robots, imaging/video robots |  |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
| Video |  | Video streaming, live video feed, Download/upload of video clips, Video Conferencing |  | Video communications from wireless clip-on cameras used by in building fire rescue |  |  | Supported; Rel. Independent |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Image or video to assist remote medical support |  |  | Supported; Rel. Independent |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Surveillance of incident scene by fixed or remote controlled robotic devices |  |  | Supported; Rel. Independent |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Assessment of fire/flood scenes from airborne platforms |  |  | Supported; Rel. Independent |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Multi-scene video dispatch |  |  | Supported in R14 |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Multicast of Multimedia from a BS to multiple users in a given area (e.g. Pt to MPt/Broadcast) |  |  | Supported in R14 |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Video conferencing 1 to 1, 1 to many, etc. |  |  | Supported in R14 |  | Support3a expected post Rel-16 |  |  |
|  |  |  |  | Encrypted video streaming |  |  | Supported in R14 |  | Support3a expected post Rel-16 |  |  |
| Real-time multimedia intelligence |  | Real time optimization of video or other multimedia content |  | Optimize the use of allocated bandwidth to support multiple video streams |  |  | Supported in R14 |  | Support3a expected post Rel-16 |  |  |
| Imagery |  | Download/ upload High resolution imagery |  | Downloading Earth exploration-satellite images |  |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
|  |  |  |  | Real-time medical imaging |  |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |

TABLE 3

PPDR requirements

| User requirement | Specifics | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- |
| 1. System |  |  |  |  |  |  |  |  |
| Support and integration of multiple applications |  | Integration of multiple applications (e.g. voice and low/medium speed data) at high speed network to service localized areas with intensive in scene activity. |  | Supported; Rel. Independent |  | Supported, Rel. Independent |  |  |
| Simultaneous use of multiple applications |  | Voice and data |  | Supported; Rel. Independent |  | Supported, Rel. Independent |  |  |
|  |  | Multicast and unicast services |  | Supported; Rel. Independent |  | System: Supported; Rel. Independent Radio: Full support expected post Rel-16 |  |  |
|  |  | Real time instant messaging |  | Supported; Rel. Independent |  | Supported, Rel. Independent |  |  |
|  |  | Mobile office functions |  | Enabled (there is no specific office function 3GPP standard); Rel. Independent |  | Enabled (there is no specific office function 3GPP standard); Rel. Independent |  |  |
|  |  | VPN services |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
|  |  | Telemetry |  | Enabled (there is no specific telemetry function 3GPP standard); Rel. Independent |  | Enabled; Rel. Independent. |  |  |
|  |  | Remote control |  | Enabled (there is no specific remote control function 3GPP standard); Rel. Independent |  | Enabled (there is no specific remote-control function 3GPP standard); Rel. Independent |  |  |
|  |  | Location of terminals |  | Supported; Rel. Independent |  | Supported; Rel. Independent. |  |  |
| Priority access |  | Manage high priority and low priority traffic load shedding during high traffic. |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
|  |  | Accommodate increased traffic loading during major operations and emergencies. |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
|  |  | Exclusive use of frequencies or equivalent high priority access to other systems. |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
| Grade of service |  | Suitable grade of service |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
| Quality of service |  | Quality of service. |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |
|  |  | Reduced response times of accessing network and information directly at the scene of incidence, including fast subscriber/network authentication. |  | Supported; Rel. Independent |  | Supported; Rel. Independent |  |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- |
| Reliability | Stable and resilient working platform. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Stable and easily operated management system. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Resilient service delivery. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | High level of availability. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Localized communication services (e.g. isolated base stations, relayed mode operation, direct mode operation (DMO), Device-to-Device (D2D). | Relaying supported in R10; DMO and D2D Supported in R12; Isolated base stations supported in R13 | System: Support4a expected post Rel-16 Radio: DMO, D2D and IAB relay Supported in R16; Full Support expected post Rel-16 |  |
| Coverage | PPDR system should provide complete coverage within relevant jurisdiction and/or operation. | Supported; Rel. Independent | Supported; Rel. Independent Enhancements from Rel-17 onward |  |
|  | Coverage of relevant jurisdiction and/or operation of PPDR organization whether at national, provincial/state or at local level. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Systems designed for peak loads and wide fluctuations in use. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Enhancing system capacity during PP emergency or DR by techniques such as reconfiguration of networks with intensive use of direct mode operation. | DMO and D2D Supported in R12; Isolated base stations and Relayed Mode Operation supported in R13 | DMO and D2D Supported in R16; Multi-hop IAB relay support in R16; UE relay mode operation, isolated base stations and full support are expected post Rel-16 |  |
|  | Standalone transportable site in order to support local site operation. | Supported in R13 | Multi-hop IAB supported in Rel-16, isolated base stations are expected for later releases |  |
|  | Mobile site in standalone mode or wide are mode in order to increase coverage/to enhance capacity. | Supported in R13 | Enabled; Rel. Independent |  |
|  | Air-to-ground communication. | Supported; Rel. Independent | Enabled; Rel. Independent |  |
|  | Vehicular repeaters (NB and WB) for coverage of localized areas/transportable site | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Reliable indoor/outdoor coverage including bi-directional amplifier (BDA). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Coverage of remote areas, underground and inaccessible areas including bi-directional amplifier (BDA). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Appropriate redundancy to continue operations, when equipment/infrastructure fails – Standalone site services. | Supported in R13 | Isolated base stations are expected for later releases and full support expected post Rel-16 |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- |
| Capabilities | Rapid dynamic reconfiguration of system. | Supported in R13 | Supported; Rel. Independent |  |
|  | Control of communications including centralized dispatch, access control, dispatch (talk) group configuration, priority levels and pre-emption. | Supported in R13 | Support4a expected post Rel-16 |  |
|  | Robust OAM offering status and dynamic reconfiguration. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Internet Protocol compatibility (complete system or interface with). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Robust equipment (hardware, software, operational and maintenance aspects). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Portable equipment (equipment that can transmit while in motion). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Equipment requiring special features such as high audio output, unique accessories (e.g. special microphones, operation while wearing gloves, operation in hostile environments and long battery life). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Fast call set-up and instant push-to-talk (PTT) group call operation. | Supported in R13 | Support4a expected post Rel-16 |  |
|  | Location services. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Communications to aircraft and marine equipment, control of robotic devices. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | One touch broadcasting/group call/ATG – announcement to all or some of talk groups and session establishment. | Supported; Rel. 13 | Support4a expected post Rel-16 |  |
|  | Terminal-to-terminal communications without infrastructure (e.g. direct mode operations/talk-around), vehicular repeaters. | Supported; Rel. 13 | Supported; Rel-16 |  |
|  | Emergency alert – Pressing the emergency button causes alert at the TG or dispatcher. | Supported in R13 | Support4a expected post Rel-16 |  |
|  | Emergency call – Priority voice call caused by pressing the emergency button. | Supported in R13 | Support4a expected post Rel-16 |  |
|  | Recording and monitoring of audio and video transmissions for evidential purpose, for safety reasons and lessons learned. | Enabled by Rel. 13 | Support4a expected post Rel-16 |  |
|  | Multi select TG’s – Ability to aggregate several TG’s and establish one call for all of them. | Supported; in R13 | Support4a expected post Rel-16 |  |
|  | Appropriate levels of interconnection to public telecommunication network(s). | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Stable and easy to operate management system. | Supported; Rel. Independent | Supported; Rel. Independent |  |
| 2. Security related requirements | End-to-end encrypted communications for mobile-mobile, dispatch and/or group calls communications (Voice and Data). | Supported; R13 | Support4a expected post Rel-16 |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- |
| 3. Cost related | Open standards. | Supported | Supported |  |
|  | Cost effective solution and applications | Supported | Supported |  |
|  | Competitive marketplace for supply of equipment and terminals | Supported | Supported |  |
|  | Reduction in deployment of permanent network infrastructure due to availability and commonality of equipment | Supported | Supported |  |
| 4. EMC | PPDR systems operation in accordance with national EMC regulations. | Supported | Supported |  |
| Scenario | Support operation of PPDR communications in any environment. | Supported | Supported |  |
|  | Implementable by public and/or private operator for PPDR applications. | Supported | Supported |  |
|  | Rapid deployment of systems and equipment for large emergencies, public events and disasters (e.g. large fires, Olympics, peacekeeping). | Supported | Supported |  |
|  | Information to flow to/from units in the field to the operational control centre and specialist knowledge centres. | Supported | Supported |  |
|  | Greater safety of personnel through improved communications. | Supported | Supported |  |
| Compatibility | End-user to end-user connectivity. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Compatible with existing networks used for PPDR communications (e.g. trunked radio). | Supported in R15 | Support4a expected post Rel-16 |  |
| Interoperability | Intra-system: Facilitate the use of common network channels and/or talk groups. | Supported in R13 | Support4a expected post Rel-16 |  |
|  | Inter-system: Promote and facilitate the options common between systems. | Supported in R16 | Support4a expected post Rel-16 |  |
|  | Coordinate tactical communications between on-scene or incident commanders of the multiple PPDR agencies. | Supported in R16 | Support4a expected post Rel-16 |  |
| 5. Spectrum usage and management | Share with other terrestrial mobile users | Supported; Rel. Independent | Supported; |  |
|  | Suitable spectrum availability (NB, WB, BB channels). | Supported | Supported; |  |
|  | Minimize interference to PPDR systems. | Supported | Supported; |  |
|  | Increased efficiency in use of spectrum. | Supported | Supported; |  |
|  | Appropriate channel spacing between mobile and base station frequencies. | Supported | Supported; |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6. Regulatory compliance | Comply with relevant national regulations. | Supported |  | Supported |  |  |  |
|  | Coordination of frequencies in border areas. | Supported |  | Supported |  |  |  |
|  | Provide capability of PPDR system to support extended coverage into neighbouring country (subject to agreements). | Supported |  | Supported |  |  |  |
|  | Ensure flexibility to use various types of systems in other Services (e.g. HF, satellites, amateur) at the scene of large emergency. | Supported |  | Supported |  |  |  |
|  | Adherence to principles of the Tampere Convention. | Supported |  | Supported |  |  |  |
| 7. Planning | Reduce reliance on dependencies (e.g. power supply, batteries, fuel, antennas, etc.). | Supported |  | Supported |  |  |  |
|  | As required, have readily available equipment (inventoried or through facilitation of greater quantities of equipment). | Supported |  | Supported |  |  |  |
|  | Provision to have national, state/provincial and local (e.g. municipal) systems. | Supported |  | Supported |  |  |  |
|  | Pre-coordination and pre-planning activities (e.g. specific channels identified for use during disaster relief operation, not on a permanent, exclusive basis, but on a priority basis during periods of need). | Supported |  | Supported |  |  |  |
|  | Maintain accurate and detailed information so that PPDR users can access this information at the scene. | Supported |  | Supported |  |  |  |
| Integration and Simultaneous use of multiple applications | Integration of multiple applications (e.g. Voice, data and video) on high speed network to service localized areas with intensive “at scene” activity. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Scene video transmission. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
| Quality of service (see Table 4 below) | Support of a prioritized range of services. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Guaranteed throughput. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Rapid session set up. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
| Coverage | RAN shall utilize maximum frequency reuse efficiency. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Vehicular repeaters (Broadband) for coverage of localized areas/transportable site. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Capabilities | Network system level management capability | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Network to perform basic self-recovery, expediting service restoration and a return to redundant operations. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Packet data capability. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Rapid deployment capability – Infrastructure and terminals | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The Network shall provide seamless coverage (via handoff/handover mechanisms) and continuous connectivity within the 95th percentile coverage area at stationary and vehicular speeds up to 120 kph. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | A single common air interface (CAI) shall be utilized for the mobile broadband network. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Mobile/portable station nominal transmit power shall be 0.25W ERP (24 dBm) and shall not exceed 3 W ERP (34.8 dBm) in rural areas for portable devices. | Different UE power classes are specified, including at least one that satisfies the requirement | Different UE power classes are specified, including at least one that satisfies the requirement |  |  |  |
| Support | 24-hour and 7 days-a-week (24/7) support for fixed and user equipment. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The network operations centre to operate on a 24 x 7 x 365 basis. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | 24/7 operations including field based support as necessary to maintain the availability of the network. In all cases, 24/7 access to call centre support for issue resolution and assistance is also required. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
| Reliability and adaptability | Adaptable to extreme natural and electromagnetic environments. No functional network failure during climate events, operational vibration, earthquake, EMI/ESD, and supplied power events. | Enabled | Enabled; Rel. Independent |  |  |  |
|  | Fixed, mobile and terminal equipment adaptable to a wide range of natural environments, with any physical facilities supporting network equipment meeting contemporary standards for electric surge suppression, grounding and EMP Protection. | Supported | Supported; Rel. Independent |  |  |  |
|  | Robust network. | Enabled | Enabled; Rel. Independent |  |  |  |
|  | Self-managed network. | Supported | Supported; Rel. Independent |  |  |  |
|  | Coordinated development of business continuity plans. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Resilient service delivery. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | High availability design e.g. Diversity, redundancy, automated failover protection, backup operational processes. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Network and operational testing to ensure data/call processing functionality is restored within predetermined and guaranteed time period following an outage. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The above should result in PPDR broadband networks at least matching the level of robustness displayed by the current public safety land mobile radio (i.e. P-25 or TETRA). | Partially Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
| Availability | Service availability shall not be calculated to allow a prolonged outage even in one service area. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Power backup using battery backup and/or power generation. Redundant backhaul circuits from the RAN to the core and to the base stations. High wind loading for the cell towers (Availability 99.995% at year 10) | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Highly reliable (99.999%) individual network elements. Ensuring adequate supply and easy access to spares to reduce Mean Time To Repair (MTTR). Operational readiness assured even in a maintenance window. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Redundant elements should automatically detect failure and activate to provide service upon failures of primary network components. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
| Security | End to end encryption. The network shall provide cryptographic controls to ensure that transmissions can only be decoded by the intended recipient. This must include data encryption over all wireless links. | Enabled; Rel. Independent | Support4a expected post Rel-16 |  |  |  |
|  | Support for domestic encryption arithmetic | Supported; Rel. Independent | Support4a expected post Rel-16 |  |  |  |
|  | The encryption should support both point-to-point traffic and point-to-multipoint traffic. | Supported; Rel. Independent | Support4a expected post Rel-16 |  |  |  |
|  | The network shall support periodic re-keying of devices such that traffic encryption keys may be changed without re-authentication of the device and without interruption of service. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | The network shall provide cryptographic controls to ensure that received transmissions have not been modified in transit. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Access to public safety services and applications shall be provided only to those authenticated users and/or devices as specifically authorized by each PPDR organization. | Enabled (for any applications and services over IP) Rel. Independent, for MCPTT in R13 | Support4a expected post Rel-16 |  |  |  |
|  | The network shall require each device that attempts to connect to the network to prove its identity prior to granting access to network resources. Each device shall be assigned a unique identifier, and the authentication method must provide strong assurance (e.g. by public key cryptography) of the device’s identity in a manner that requires no user interaction. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The device authentication service shall utilize an open standard protocol. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | To protect against both malicious devices and malicious network stations, the authentication must be mutual, with the device proving its identity to the network and the network proving its identity to the device. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Each PPDR organization shall be granted the option to require user authentication in addition to device authentication for certain devices assigned to that organization. When user authentication has been selected as a requirement, the network shall require each of the organization’s designated devices to prove its user’s identity prior to granting access to network resources. | Supported in R13 | Support4a expected post Rel-16 |  |  |  |
|  | For organizations requiring user authentication, the network must facilitate sequential authentication of multiple users from a single device. | Supported; Rel. Independent | Support4a expected post Rel-16 |  |  |  |
|  | System authorization management. Each organization shall be granted control over authorization by means of an administrative interface. | Supported in R13 | Support4a expected post Rel-16 |  |  |  |

TABLE 3 (cont.)

| User requirement | Specifics | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | For organizations requiring user authentication, the organization shall be granted via administrative interface (e.g. Web based) the ability to add, remove, and manage user accounts that are permitted to access the network. | Supported in R13 | Support4a expected post Rel-16 |  |  |  |
|  | For organizations requiring user authentication, the network must facilitate sequential authentication of multiple users from a single device. | Supported; Rel. Independent | Support4a expected post Rel-16 |  |  |  |
|  | The network should have dedicated PPDR system core. | Supported in R13 |  |  |  |  |
|  | 3rd party key management system. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The network shall maintain a record of all device and user access attempts and all authentication and authorization transactions, including changes to authentication and authorization data stores. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Over the air key update. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The network shall enforce a configurable time‐out, imposing a maximum time that each device may be connected to the network. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | The network shall enforce an inactivity time‐out, imposing a maximum time that each device may be connected to the network without transmitting data. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Each PPDR organization shall be granted control of the network time out and inactivity setting for individual devices assigned to that organization. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Each organization shall also be granted via administrative interface the means to manually and forcibly terminate access, including active sessions, to the network for any of its assigned devices individually. | Supported; Rel. Independent for network operators, R13 for PPDR administrators | Support4a expected post Rel-16 |  |  |  |
|  | The network shall be capable of attack monitoring. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |

TABLE 3 (cont.)

| User requirement |  | Specifics |  |  | Support by 3GPP and Release Number |  |  |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal Requirements for preventing unauthorized use |  | Devices shall support the network’s device authentication protocol. Each device shall be assigned a unique identifier, and the authentication method must provide strong assurance (e.g. by public key cryptography) of the device’s identity in a manner that requires no user interaction. |  |  | Supported; Rel. Independent |  |  | Supported; Rel. Independent |  |  |  |
|  |  | To protect against both malicious devices and malicious network stations, the authentication must be mutual, with the device proving its identity to the network and the network proving its identity to the device. The device must not permit connectivity to the PPDR network unless the network is authenticated. |  |  | Supported; Rel. Independent |  |  | Supported; Rel. Independent |  |  |  |
|  |  | Each PPDR organization shall have the option to require user authentication for device access. When user authentication has been selected as a requirement, the device shall require each user to prove his or her identity prior to granting access to applications or network resources. |  |  | Supported in R13 |  |  | Support4a expected post Rel-16 |  |  |  |
|  |  | Devices may support a means of erasing (via best practice multiple pass overwriting of data storage media) all data stored on the device. |  |  | Supported; Rel. Independent |  |  | Supported; Rel. Independent |  |  |  |
|  |  | Devices may support a means of encrypting data stored on the device such that user authentication is required for decryption. |  |  | Supported; Rel. Independent |  |  | Supported; Rel. Independent |  |  |  |
| Cost |  | Scalable system |  |  | Supported; Rel. Independent |  |  | Supported; Rel. Independent |  |  |  |
|  |  | Open system architecture |  |  | Supported in R13 |  |  | Support4a expected post Rel-16 |  |  |  |
|  | Implementable by public and/or private operator for PPDR applications. | Supported in R13 | Support4a expected post Rel-16 |  |
| Interoperability | Interoperable/Interconnection with narrowband trunked systems. Interconnection required with: – Inter RF subsystem Interface Voice service and Supplementary services – Console supplementary Interface Voice service and Supplementary services | Supported in R16 | Support4a expected post Rel-16 |  |
|  | Interoperable/Interconnection with other broadband systems. | Supported in R16 | Support4a expected post Rel-16 |  |
|  | Interoperable/Interconnection with satellite systems. | Not Supported | Not supported Support of service continuity for mobility between terrestrial system and satellite system expected post Rel-16 |  |
|  | Interconnection with other information systems. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | Interfaces that interconnect to other communication systems. | Supported; Rel. Independent | Supported; Rel. Independent |  |
|  | API compatible with standard interfaces. | Enabled; Rel. Independent | Enabled; Rel. Independent |  |

TABLE 3 (end)

| User requirement | Specifics | Support by 3GPP LTE and Release Number |  | Support by 3GPP NR and Release Number |  | Support by Technology YYY |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Appropriate levels of interconnection to public telecommunication network(s) – Fixed and mobile. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
| Spectrum usage and management | Dynamic spectrum allocation. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Suitable spectrum availability (Broadband channels for uploads at maximum data rates). | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |
|  | Reallocation of upstream and downstream rates. | Supported; Rel. Independent | Supported; Rel. Independent |  |  |  |

TABLE 4

Capabilities provided under Localized Communication Services

| Localized Communication Services: D2D/DMO/ Isolated Base Station/Relayed Mode |  | Support by 3GPP LTE and Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- |
| Voice | Person-to-person |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
|  | One-to-many |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
|  | Push-to-talk |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
|  | Priority |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
|  | Encryption |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
|  | Emergency PTT |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
| Multimedia (V+V+D) | Person-to-person |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | One-to-many |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Push-to-MM |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Priority |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Encryption |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Real time video |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
| Text Message/ Instant Message | Person-to-person |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Emergency alert |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | One-to-many |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |

TABLE 45

Capabilities provided under Localized Communication Services

| Localized Communication Services: D2D/DMO/Isolated Base Station/Relayed Mode |  | Support by 3GPP LTE Release Number | Support by 3GPP NR and Release Number | Support by Technology YYY |
| --- | --- | --- | --- | --- |
| Multi Media Message/ Instant Message | Person-to-person |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | One-to-many |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | SD |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | HD |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Presence |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
| Data Base Interaction |  |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
| Location | Interactive location data |  | Enabled in R9 |  |  |  | Support5a expected post Rel-16 |  |
| File Transfer |  |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
| Client Server App. |  |  | Enabled in R12; Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
| Peer to Peer App. |  |  | Enabled in R12; Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
| Miscellaneous | Software/Firmware update online |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | GIS maps updates |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Automatic telemetries |  | Enabled in R12 |  |  |  | Support5a expected post Rel-16 |  |
|  | Hotspot on disaster or event area |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |
|  | Alarming/Paging |  | Supported in R13 |  |  |  | Support5a expected post Rel-16 |  |

______________

____________________

This Table 2 combines the requirements listed in Report ITU-R M.2377-0 in Table A4-1 “Generic/narrowband part”, Table A4-2 “Additional wideband part”, and Table A4-3 “Additional broadband part”.

3a	3GPP MCX Stage 1 requirements available for 5G; MCX Stage 2 5G work ongoing in Rel-17. MCX applications supported for unicast LTE-based communication are applicable for 5G RAN.

This Table 3 combines the requirements listed in Report ITU-R M.2377-0 in Table A5-1, “Generic user requirements supported by PPDR narrow-, wide-, and broadband communications” and Table A5-2, “Table of additional requirements for PPDR broadband communications”.

4a	3GPP MCX Stage 1 requirements available for 5G; MCX Stage 2 5G work ongoing in Rel-17. MCX applications supported for unicast LTE-based communication are applicable for 5G RAN.

This Table 4 aligns with Table A5-3 of Report ITU-R M.2377-0, which has the same title. Further, in this Table, all separate columns of Table A5-3 of Report ITU-R M.2377-0 relating to “Localized Communication Services: D2D/DMO, Isolated Base Station and Relayed Mode”, both for “Isolated” and “Connected to the core” topologies have been combined in one column.

5a	3GPP MCX Stage 1 requirements available for 5G; MCX Stage 2 5G work ongoing in Rel-17. MCX applications supported for unicast LTE-based communication are applicable for 5G RAN.