3GPP TSG RAN Meeting #100b-e	R1-2003147

# e-Meeting, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0094 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRS-RSRP reception procedure for CLI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM-Core |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on contribution R1-2001589, it has been identified the following: In the RAN1 #100e-meeting, the agreed RAN1 assumptions are as follows. Agreements: - Send LS to RAN4 (CC RAN2) to ask their checking and confirmation on following RAN1 assumptions. - UE may performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource. - CLI-RSSI measurement is applicable for RRC_CONNECTED intra-frequency only. - When SRS-RSRP measurement resource is fully confined within BW of DL active BWP, UE operates SRS-RSRP measurement using the SRS-RSRP measurement resource. Otherwise the UE does not operate SRS-RSRP measurement using the SRS-RSRP measurement resource. UE is not expected to be configured a SRS-RSRP resource outside DL BWP. - SRS-RSRP measurement is applicable for RRC_CONNECTED intra-frequency only. Wherein, the agreements of “CLI-RSSI/SRS-RSRP measurement is applicable for RRC_CONNECTED intra-frequency only” have already been captured into 38.215 CR R1-2001440. In our opinion, the agreement of UE measurement behaviour corresponding to SRS-RSRP measurement resource configuration should also be captured in 38.214 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.1.6.4 based on email discussion [100b-e-NR-TEIs-01], and based on contribution R1-2001589, the assumption on SRS-RSRP measurement has been captured in specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for SRS-RSRP measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.6.4	SRS reception procedure for CLI

The SRS resources defined in Clause 6.4.1.4 of [4, TS 38.211] may be configured for SRS-RSRP measurement for CLI, as defined in Clause 5.1.19 of [7, TS 38.215]. The UE is not expected to measure SRS-RSRP with a subcarrier spacing other than the one configured for the active BWP confining the SRS resource. The UE is not expected to measure SRS-RSRP using the SRS-RSRP measurement resource which is not fully confined within the DL active BWP. The UE is not expected to measure more than 32 SRS resources, and the UE is not expected to receive more than 8 SRS resources in a slot.

<omitted text>