3GPP TSG RAN Meeting #101-e	R1-2005158

# e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0091 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-06-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections on V2X and NRSL For changes in section 8.1. based on email discussion [100b-e-NR-5G_V2X_NRSL-Mode-1-05], clarification on how to set HARQ process ID and NDI for Mode 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performing various corrections and clarifications as follows: Implementation of UE procedures related determination of slots and resources blocks based on “Time resources assignment” and “Frequency resources assignment” received in SCI format 0-1. The changes discussed and agreed in email discussions [100e-NR-5G_V2X_NRSL-RA_Mode2-02] and [100b-e-NR-5G_V2X_NRSL-Mode-2-05]. An editorial update thoughout this CR of the SCI formats with the latest naming, that is SCI 1-A, SCI 2-A and SCI 2-B, in order to align with 38.212 specification. In section 8 based on email discussion [101-e-NR-5G_V2X_NRSL-PHYstructure-02], as summarized in R1-2005019: The description for how to allocate slots into a resource pool according to (pre-)configuration has been added implementing the following agreements: Agreements: - The periodicity of resource pool bitmap is 10240 ms. - The (pre-)configured length of the bitmap (L_bitmap) can be one among 10, 11, 12, …, 160. Agreements: - The following WA made in RAN1#100-e is confirmed. - For derivation of the set of slots to be included in the resource pool, the set includes all the slots except the following slots: - … - (Working assumption) reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213 In section 8.1, some text re-org has been performed for SCI formats 1-A, 2-A and 2-B In section 8.1 based on email discussion [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03], as summarized in R1-2004933: RAN1 made the agreements to complete the UE procedure for sidelink HARQ operation and CSI reporting. The text proposal is to implement these agreements. In section 8.1. based on email discussion [100b-e-NR-5G_V2X_NRSL-Mode-1-05], as summarized in R1-2002988, specification text describing how to set HARQ process ID and NDI for Mode 1. In section 8.1.2 based on email discussion [101-e- NR-5G_V2X_NRSL-Mode-1-02] and as summarized in R1-2005007: a new section is introduced with the procedure for activation and release of SL CG Type-2. In sections 8.1.3.1 and 8.3 based on email discussion [101-e-NR-5G_V2X_NRSL-PHYstructure-01]: TBS determination procedure for PSSCH is added as the current specification does not have TBS determination procedure for PSSCH, and the detailed procedure was agreed in RAN1#101-e. In sections 8.1.3.2, based on email discussion [100b-e-NR-5G_V2X_NRSL-PHYstructure-04], MCS tables have been introduced. In sections 8.1.3.2, based on email discussion [101-e-NR-5G_V2X_NRSL-PHYstructure-01] and summarized in R1-2005018, TBS determination procedure for PSSCH is added.. In section 8.1.4, an editorial change fixing a reference. In section 8.1.4 based on email discussion [101-e-NR-5G_V2X_NRSL-Mode-2-01] and as summarized in R1-2004943: Values for Tproc,0 and Tproc,1 added to specification In sections 8.1.4, 8.1.5 and 8.1.7(new) based on email discussion [101-e-NR-5G_V2X_NRSL-Mode-2-05], as sumarized in R1-200494: Procedure to convert a reservation period in ms to logical slots is specified. Cresel calculation from SL_RESOURCE_RESELECTION_PERIOD is specified. In section 8.1.4 based on email discussion [101-e-NR-5G_V2X_NRSL-Mode-2-01] and as summarized in R1-2004942: Defined a procedure of pre-emption condition checking. Defined procedure of re-evaluation condition checking. Defined pre-emption and re-evaluation timeline. In section 8.1.4 based on email discussion [101-e-NR-5G_V2X_NRSL-Mode-2-03] and as summarized in R1-2004944: Ratio of resource in the set SA to the total number of candidate resources in the selection window is made configurable instead of fixing to 20%. In section 8.1.6 based on email discussion [100b-e-NR-5G_V2X_NRSL-QoS-01], it is up to UE implementation how to meet CR limits for sidelink congestion control. In section 8.2.2, removed the square brackets. In sectioni 8.2.3, added similar higher layer updates as in section 8.4.3 In section 8.3, based on email discussion [101-e-NR-5G_V2X_NRSL-PHYstructure-04], as summarized in R1-2005021 OCC number and the restriction in the RX UE side were now added. In section 8.4.2.1, based on email discussion [100b-e-NR-5G_V2X_NRSL-Mode-2-06], RSRP for resource selection in sidelink resource allocation mode 2 defined in sub-clause 8.4.2.1 is corrected so that measurement of PSSCH-RSRP and PSCCH-RSRP are made over PSSCH DM-RS and PSCCH DM-RS respectively when corresponding higher layer parameters are provided. Alignment of RRC parameter names. In sections 8.5.1.2, 8.5.2.2 and 8.5.2.3 based on email discussion [100b-e-NR-5G_V2X_NRSL-PHY-Procedure-04], new procedures have been introduced with respect to triggering of sodelink CSI reports, CSI-RS and CSI reference resource definition. In sections 8.5.1.2 and 8.5.2.2 based on email discussion [101-e-NR-5G_V2X_NRSL-PHYstructure-04], as summarized in R1-2005021, the following agreements have been implemented Agreement: - A collision between SL-CSI-RS and the corresponding PSCCH is not expected - Note: this implies that such a collision case is a mis-configuration Agreements: - A TX UE is not expected to transmit a SL CSI-RS in a same symbol with the 2nd SCI or PSSCH DMRS - Note: this implies that it’s an error case if there is such a collision Agreements: A TX UE is not expected to transmit a SL CSI-RS and a SL PT-RS which overlap - Note: this implies that it’s an error case if there is such a collision (no puncturing, and the collision is avoided by the Tx UE) - This imples a Rx UE is not expected to receive a SL CSI-RS and a SL PT-RS which overlap Sectin 8.6 is new and is based on email discussion [101-e- NR-5G_V2X_NRSL-Mode-1-01] and summarized in R1-2005006. In the new section the preparation procedure time is introduced for sidelink transmission for dynamic grant in mode 1. In sections 8.5.2.1.1, based on email discussion [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03], CQI table determination is introduced based on the indicated MCS table In sections 8.2.1, 8.4.3, 8.5.2.2, 8.5.3: aligned higher layer parameters names with 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for V2X and NRSL. For the corrections based on email discussion [100e-NR-5G_V2X_NRSL-RA_Mode2-02] and [100b-e-NR-5G_V2X_NRSL-Mode-2-05], incomplete support for V2X and NRSL. A UE cannot transmit and receive SCI format 0-1. A UE cannot perform sensing and resource selection according to Mode-2 resource allocation. For the corrections based on email discussion [100b-e-NR-5G_V2X_NRSL-Mode-2-06], PSSCH-RSRP is measured over PSCCH DM-RS and PSCCH-RSRP is measured over PSSCH DM-RS, that is contradicting to the definition of the measurements in TS 38.215. For corrections done in sections 8.1.4, 8.1.5 and 8.1.7(new): undefined procedure to calculate Cresel. Undefined procedure to convert a reservation period in ms to logical slots. As a result, NR V2X Mode-2 resource allocation does not work as expected. For corrections in section 8.1.4 based on email discussion [101-e-NR-5G_V2X_NRSL-Mode-2-01] and as summarized in R1-2004942: incorrect split between L1 and MAC functions. Undefined timeline for pre-emption and re-evaluation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 8.1, 8.1.2, 8.1.3.1, 8.1.3.2, 8.1.4, 8.1.5, 8.1.6, 8.1.7(new), 8.3, 8.4.2.1, 8.5.1.2, 8.5.2.2, 8.5.2.3, 8.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

# 8	Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

In the frequency domain, a sidelink resource pool consists of numSubchannel contiguous sub-channels. A sub-channel consists of subchannelsize contiguous PRBs, where numSubchannel and subchannelsize are higher layer parameters.

The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where

-	0≤tiSL<10240×2μ, 0≤i<Tmax,

-	the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

-	the set includes all the slots except the following slots,

-	NS_SSB slots in which S-SS/PSBCH block (S-SSB) is configured,

-	NnonSL slots in each of which at least one of Y-th, (Y+1)-th, …, (Y+X-1)-th OFDM symbols are not semi-statically configured as UL as per the higher layer parameter TDD-UL-DL-ConfigCommon, where Y and X are set by the higher layer parameters sl-StartSymbol and sl-LengthSymbols, respectively.

- The reserved slots which are determined by the following steps.

  - the remaining slots excluding NS_SSB slots and NnonSL slots from the set of all the Tmax slots are denoted by (l0,l1,⋯,l(10240×2μ-NSSSB-NnonSL-1)) arranged in increasing order of slot index.

  - a slot lr (0≤r<10240×2μ-NSSSB-NnonSL) belongs to the reserved slots if r=m∙(10240×2μ-NSSSB-NnonSL)Nreserved, here m=0,1,⋯,Nreserved-1 and Nreserved=10240×2μ-NSSSB-NnonSL mod Lbitmap where Lbitmap denotes the length of bitmap configured by higher layers.

-    The slots in the set are arranged in increasing order of slot index.

-    The UE determines the set of slots assigned to a sidelink resource pool as follows:

-  a bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used where Lbitmap the length of the bitmap is configured by higher layers.

-  a slot tkSL (0≤k<10240×2μ-NSSSB-NnonSL-Nreserved) belongs to the slot pool if bk'=1 where k'=k mod Lbitmap.

The UE determines the set of resource blocks assigned to a sidelink resource pool as follows:

-	The resource block pool consists of NPRB PRBs.

-	The sub-channel m for m=0,1,⋯,numSubchannel-1 consists of a set of nsubCHsize contiguous resource blocks with the physical resource block number nPRB=nsubCHRBstart+m∙nsubCHsize+j for j=0,1,⋯,nsubCHsize-1, where nsubCHRBstart and nsubCHsize are given by higher layer parameters sl_StartRB-Subchannel and sl-SubchannelSize, respectively

A UE is not expected to use the last NPRB mod nsubCHsize PRBs in the resource pool.

<omitted text>

## 8.1	UE procedure for transmitting the physical sidelink shared channel

Each PSSCH transmission is associated with an PSCCH transmission.

That PSCCH transmission carries the 1st stage of the SCI associated with the PSSCH transmission; the 2nd stage of the associated SCI is carried within the resource of the PSSCH.

If the UE transmits SCI format 1-A on PSCCH according to a PSCCH resource configuration in slot n and PSCCH resource m, then for the associated PSSCH transmission in the same slot

-	one transport block is transmitted with up to two layers;

-	The number of layers (ʋ) is determined according to the “Number of DMRS port” field in the SCI

-	The set of consecutive symbols within the slot for transmission of the PSSCH is determined according to clause 8.1.2.1;

-	The set of contiguous resource blocks for transmission of the PSSCH is determined according to clause 8.1.2.2;

Transform precoding is not supported for PSSCH transmission.

Only wideband precoding is supported for PSSCH transmission.

The DM-RS antenna ports  in Clause 8.4.1.1.1 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 8.3.1.1-1 in Clause 8.3.1.1 of [5, TS 38.212].

The UE shall set the contents of the SCI formats 2-A as follows:

-	the UE shall set value of the “HARQ process number” field as indicated by higher layers.

-	the UE shall set value of the “NDI” field as indicated by higher layers.

-     the UE shall set value of the “Source ID” field as indicated by higher layers.

-     the UE shall set value of the “Destination ID” field as indicated by higher layers.

-	the UE shall set value of the “HARQ feedback enabled/disabled indicator” field as indicated by higher layers.

-     the UE shall set value of the “Cast type indicator” field as indicated by higher layers.

-     the UE shall set value of the “CSI request” field as indicated by higher layers.

The UE shall set the contents of the SCI formats 2-B as follows:

-     the UE shall set value of the “HARQ process number” field as indicated by higher layers.

-     the UE shall set value of the “NDI” field as indicated by higher layers.

-     the UE shall set value of the “Source ID” field as indicated by higher layers.

-     the UE shall set value of the “Destination ID” field as indicated by higher layers.

-     the UE shall set value of the “HARQ feedback enabled/disabled indicator” field as indicated by higher layers.

-     the UE shall set value of the “Zone ID” field as indicated by higher layers.

-     the UE shall set the "Communication range requirement" field as indicated by higher layers.

<omitted text>

### 8.1.2	Resource allocation

In sidelink resource allocation mode 1:

-	for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.3 of [6, TS 38.213].

<omitted text>

### 8.1.3	Modulation order, target code rate, redundancy version and transport block size determination

#### 8.1.3.1	Modulation order and target code rate determination

IMCS is given by the “Modulation and coding scheme” field in SCI format 1-A.

The MCS table is determined as follows: Table 5.1.3.1-1 is used if no additional MCS table is configured by higher layer parameter sl-MCS-Table; otherwise an MCS table is determined according to Table 8.1.3.1-1 or Table 8.1.3.1-2 and “MCS table indicator” field in SCI format 1-A.

Table 8.1.3.1-1: Mapping of one bit of MCS table indicator to MCS table

| MCS table indicator | MCS table |
| --- | --- |
| '0' | Table 5.1.3.1-1 |
| '1' | 1st table provided by higher layer parameter sl-Additional-MCS-Table |

Table 8.1.3.1-2: Mapping of two bits of MCS table indicator to MCS table

| MCS table indicator | MCS table |
| --- | --- |
| '00' | Table 5.1.3.1-1 |
| '01' | 1st table provided by higher layer parameter sl-Additional-MCS-Table |
| '10' | 2nd table provided by higher layer parameter sl-Additional-MCS-Table |
| '11' | reserved |

The UE shall use IMCS and the MCS table determined according to the previous step to determine the modulation order (Qm) and Target code rate (R) used in the physical sidelink shared channel.

<omitted text>

#### 8.1.3.2	Transport block size determination

For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot.

-	A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where

-	NscRB=12 is the number of subcarriers in a physical resource block,

-	NsymbshNsymbslot = sl-lengthSLsymbols -2, where sl-lengthSLsymbols is the number of sidelink symbols within the slot provided by higher layers,

-	NsymbPSFCH = 3 if “PSFCH overhead indication” field of SCI format 1-A indicates “1”, and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	NohPRB is the overhead given by higher layer parameter sl-xOverhead,

-	NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePattern.

Table 8.1.3.2-1.  NREDMRS according to higher layer parameter sl-PSSCH-DMRS-TimePattern

| sl-PSSCH-DMRS-TimePattern | NREDMRS |
| --- | --- |
| {2} | 12 |
| {3} | 18 |
| {4} | 24 |
| {2,3} | 15 |
| {2,4} | 18 |
| {3,4} | 21 |
| {2,3,4} | 18 |

-	A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where

-	nPRB is the total number of allocated PRBs for the PSSCH,

-	NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DMRS.

-	NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212].

The UE determines TBS according to Steps 2), 3), and 4) in subclause 5.1.3.2

else if Table 5.1.3.1-2 is used and ,

-	the TBS is assumed to be as determined from the SCI for the same transport block using 0 ≤ IMCS ≤27..

else

-	the TBS is assumed to be as determined from the SCI for the same transport block using 0 ≤ IMCS ≤28.

<omitted text>

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of ms.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	t2min_SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter t2min_SelectionWindow for the given value of prioTX.

-	SL-ThresRSRP_pi_pj: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	RSforSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList

-	t0_SensingWindow: internal parameter T0 is defined as the number of slots corresponding to t0_SensingWindow ms.

-	sl-xPercentage: internal parameter X for a given prioTX is defined as sl-xPercentage(prioTX) converted from percentage to ratio

-	p_preemption: internal parameter priopre is set to the higher layer provided parameter p_preemption

The resource reservation interval, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

Notation:

t0SL,t1SL,t2SL,... denotes the set of slots which can belong to a sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL Tproc,1 , where Tproc,1SL Tproc,1 is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SLTproc,0) where T0 is defined above and Tproc,0SLTproc,1 is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which can belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

3)	The internal parameter Th(pi) is set to the corresponding value from higher layer parameter SL-ThresRSRP_pi_pj for  pj  equal to the given value of prioTX and each priority value pi.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than  ThprioRX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2X⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

If a resource ri' from the set (r0',r1',r2',…) is not a member of SA due to exclusion in step 6 above by comparison with the RSRP measurement for the received SCI format 1-A with an associated priority prioRX, and prioRX<priopre and prioTX>prioRX, then the UE shall report pre-emption of the resource ri' to higher layers.

Table 8.1.4-1 Tproc,0SL depending on sub-carrier spacing

| μSL | Tproc,0SL [slots] |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |

Table 8.1.4-2 Tproc,1SL depending on sub-carrier spacing

| μSL | Tproc,1SL [slots] |
| --- | --- |
| 0 | 3 |
| 1 | 5 |
| 2 | 9 |
| 3 | 17 |

<omitted text>

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1-A as described below.

"Time resource assignment" carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter numSubchannel

If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a set of sub-channels in slot tmSL is determined as the time and frequency resource for PSSCH transmission corresponding to the configured sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots tm+j×Prsvp_TX'SL  are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t0SL,t1SL,t2SL,... is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

<omitted text>

### 8.1.6	Sidelink congestion control in sidelink resource allocation mode 2

If a UE is configured with higher layer parameter sl-CR-Limit and transmits PSSCH in slot n, the UE shall ensure the following limits for any priority value k;

i≥kCRi≤CRLimitk

where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with "Priority" field in the SCI set to i, and CRLimitk corresponds to the high layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range which includes the CBR measured in slot n-N, where N is the congestion control processing time.

The congestion control processing time N is based on µ of Table 8.1.6-1 and Table 8.1.6-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH is to be transmitted. A UE shall only apply a single processing time capability in sidelink congestion control.

Table 8.1.6-1: Congestion control processing time for processing timing capability 1

| µ | Congestion control processing time N [slots] |
| --- | --- |
| 0 | 2 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Table 8.1.6-2: Congestion control processing time for processing timing capability 2

| µ | Congestion control processing time N [slots] |
| --- | --- |
| 0 | 2 |
| 1 | 4 |
| 2 | 8 |
| 3 | 16 |

It is up to UE implementation how to meet the above limits, including dropping the transmissions in slot n.

### 8.1.7	UE procedure for determining the number of logical slots for a reservation period

A given resource reservation period Prsvp in milliseconds is converted to a period Prsvp' in logical slots as:

Prsvp'=N20 ms×Prsvp

where N is the number of slots that can be used for SL transmission within 20 ms of the configured UL-DL configuration.

<omitted text>

### 8.2.1	CSI-RS transmission procedure

A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold:

-	CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and

-	the “CSI request” field in the corresponding SCI format 0-2 is set to 1.

<omitted text>

### 8.2.2	PSSCH DM-RS transmission procedure

The UE selects the DM-RS time domain pattern out of the patterns configured using the higher layer parameter TimePatternPsschDmrs for the resource pool on which the PSSCH is to be transmitted. If more than one DM-RS time domain pattern is configured, the selected pattern is indicated by the “DMRS pattern” field in the SCI format 0-1 associated with the PSSCH transmission.

If PSSCH DMRS and PSCCH are mapped to the same OFDM symbol, then this mapping within a single sub-channel is only supported if higher layer parameter subchannelsize >= 20, i.e. the sub-channel size is at least 20 PRBs.

<omitted text>

### 8.2.3	PT-RS transmission procedure

Transmission of PT-RS is only supported in FR2.

The UE PT-RS transmission procedure specified in section 6.2.3.1 applies for derivation of the PT-RS parameters LPT-RS and, KPT-RS  and for determination of PT-RS presence, with the following changes:

-	timeDensity and frequencyDensity in PTRS-UplinkConfig are replaced by sl-PTRS-TimeDensity and sl-PTRS-FreqDensity in SL-PTRS-Config respectively, and SL-PTRS-Config is (pre)configured per resource pool.

-	the number of antenna ports is the same as the number of PSSCH DM-RS antenna ports and the association between a PT-RS antenna port and a PSSCH DM-RS antenna port is fixed.

<omitted text>

## 8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A and 2-B, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A and 2-B, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A and 2-B nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

<omitted text>

#### 8.4.2.1	RSRP for resource selection in sidelink resource allocation mode 2

In sidelink resource allocation mode 2, the UE measures RSRP for resource selection as follows:

-	PSSCH-RSRP over the DM-RS resource elements for the PSSCH according to the received SCI format 1-A if higher layer parameter sl-RS-ForSensing  is set to “pssch”, and

-	PSCCH-RSRP over the DM-RS resource elements for the PSCCH carrying the received SCI format 1-A if higher layer parameter sl-RS-ForSensing is set to “pscch”.

<omitted text>

### 8.4.3	PT-RS reception procedure

Reception of PT-RS is only supported in FR2.

The UE PT-RS reception procedure specified in section 5.1.6.3 applies for derivation of the PT-RS parameters LPT-RS and KPT-RS and for determination of PT-RS presence, with the following changes:

-	timeDensity and frequencyDensity in PTRS-DownlinkConfig are replaced by sl-PTRS-TimeDensity and sl-PTRS-FreqDensity in SL-PTRS-Config respectively, and SL-PTRS-Config is (pre)configured per resource pool;

-	the number of antenna ports is the same as the number of PSSCH DMRS antenna ports and the association between a PT-RS antenna port and a PSSCH DMRS antenna port is fixed.

<omitted text>

#### 8.5.1.2	Triggering of sidelink CSI reports

The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A with the “CSI request” field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321].

An aperiodic CSI report is triggered by an SCI format 2-A with the “CSI request” field set to 1.

A UE is not expected to transmit a sidelink CSI-RS and a sidelink PT-RS which overlap.

### 8.5.2	Channel state information

#### 8.5.2.1	CSI reporting quantities

#### 8.5.2.1.1	Channel quality indicator (CQI)

The UE shall derive CQI as specified in section 5.2.2.1, with the following changes

-	PDSCH replaced by PSSCH

-	uplink slot replaced by sidelink slot

-	downlink physical resource blocks replaced by sidelink physical resource blocks

-	Transport Block Size determination according to Clause 8.1.3.2

-	CSI reference resource according to the Clause 8.5.2.3

-	interference measurements are not supported

-	sub-band differential CQI is not supported

-    cqi-Table is determined as follows

-  cqi-Table = ‘table1’ if Table 5.1.3.1-1 is determined as the MCS table according to Clause 8.1.3.1 of [6, 38.214],

-  cqi-Table = ‘table2’ if Table 5.1.3.1-2 is determined as the MCS table according to Clause 8.1.3.1 of [6, 38.214],

-  cqi-Table = ‘table3’ if Table 5.1.3.1-3 is determined as the MCS table according to Clause 8.1.3.1 of [6, 38.214]

#### 8.5.2.2	Reference signal (CSI-RS)

The UE can be configured with one CSI-RS pattern as indicated by the higher layer parameters sl-CSI-RS-FreqAllocation, sl-OneAntennaPort, sl-CSI-RS-FirstSymbol in SL-CSI-RS-Config.

Parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured according to clause 8.2.1.

A UE is not expected to be configured such that a CSI-RS and the corresponding PSCCH can be mapped to the same resource element.A UE is not expected to receive sidelink CSI-RS and PSSCH DMRS, nor CSI-RS and 2nd-stage SCI, on the same symbol.

Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A triggering a report.

#### 8.5.2.3	CSI reference resource definition

The CSI reference resource in sidelink is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of sidelink physical resource blocks containing the sidelink CSI-RS to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in sidelink slot n is defined by a single sidelink slot nCSI_ref where nCSI_ref is the same sidelink slot as the corresponding CSI request.

### 8.5.3	CSI reporting

The UE can be configured with one CSI reporting latency bound as indicated by the higher layer parameter sl-LatencyBound-CSI-Report. CSI reporting is aperiodic and is described in [10, TS 38.321].

<omitted text>

## 8.6	UE PSSCH preparation procedure time

For sidelink dynamic grant, if the first sidelink symbol in the sidelink allocation for a PSSCH for a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol, as defined by the slot offset KSL of the scheduling DCI, is no earlier than at symbol L, where L is defined as the next sidelink symbol with its CP starting Tproc=(N2+d2,1)(2048+144)⋅κ2-μ⋅TC after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the sidelink transmissions, then the UE shall transmit the PSSCH and the associated PSCCH.

-	N2 is based on µ of Table 8.6-1, where µ corresponds to the one of (µDL, µSL) resulting with the largest Tproc, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PSSCH was transmitted and µSL corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH and the associated PSCCH are to be transmitted, and κ is defined in Clause 4.1 of [4, TS 38.211].

-	d2,1 = 1.

Otherwise the UE may ignore the scheduling DCI.

The value of Tproc is used both in the case of normal and extended cyclic prefix.

Table 8.6-1: PSSCH preparation time

|  | PSSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |