3GPP TSG-RAN WG1 Meeting 101e	R1-2005165

# e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0028 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to V2X measurement definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-06-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. Current definition only allows use of demodulation reference signals associated with physical sidelink broadcast channel for PSBCH-RSRP measurements. 2. Clarify meaning of “granted” in the definition of channel occupancy ratio (SL CR) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. It has been clarified that sidelink secondary synchronization signals in addition to demodulation reference signals of PSBCH may be used for PSBCH-RSRP measurements. 2. Add a note to SL CR definition to explain that “granted” refers to the configured sidelink grant as defined in TS 38.321. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE cannot use S-SSS for PSBCH-RSRP measurement. 2. Lack of clarity about the meaning of “granted” in SL CR definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.22, 5.1.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.22	PSBCH reference signal received power (PSBCH-RSRP)

| Definition | PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). For PSBCH-RSRP sidelink secondary synchronization signals in addition to demodulation reference signals for PSBCH may be used. PSBCH-RSRP using sidelink secondary synchronization signals shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals. For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine PSBCH-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:   It is up to UE implementation to use PSBCH DMRS only or both S-SSS and PSBCH DMRS for PSBCH-RSRP.

…

### 5.1.26	Sidelink channel occupancy ratio (SL CR)

| Definition | Sidelink Channel Occupancy Ratio (SL CR) evaluated at slot n is defined as the total number of sub-channels used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b]. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	a is a positive integer and b is 0 or a positive integer; a and b are determined by UE implementation with a+b+1 = 1000 or 1000·2µ slots, according to higher layer parameter timeWindowSize-CR, b < (a+b+1)/2, and n+b shall not exceed the last transmission opportunity of the grant for the current transmission.

NOTE 2:	SL CR is evaluated for each (re)transmission.

NOTE 3:	In evaluating SL CR, the UE shall assume the transmission parameter used at slot n is reused according to the existing grant(s) in slot [n+1, n+b] without packet dropping.

NOTE 4:	The slot index is based on physical slot index.

NOTE 5:	SL CR can be computed per priority level

NOTE 6:	A resource is considered granted if it is a member of a configured sidelink grant as defined in TS 38.321 [7].