3GPP TSG RAN Meeting #88e								                                               RP-200816

Electronic Meeting, June 29 - July 3, 2020

Source: 	ZTE, Sanechips

Title:	On the scope of unlicensed band URLLC/IIoT operation

Agenda item:	9.10.5

Document for:	Discussion

- Introduction

In the RAN#86 meeting, a new work item on enhanced industrial internet of things (IoT) and URLLC support was approved [1]. One of the objectives of this work item is to identify potential enhancements to ensure Release 16 feature compatibility with unlicensed band URLLC/IIoT operation. It was planned to clarify the detailed enhancements in RAN#87, while all Rel-17 SIs/WIs discussion was postponed to this e-meeting due to impact of COVID-19 outbreak.

Identify potential enhancements to ensure Release 16 feature compatibility with unlicensed band URLLC/IIoT operation in controlled environment [RAN1, RAN2] - Detailed objectives to be clarified at RAN#87 based on essential issues to be identified in RAN#87 (if any)

In this contribution, we provide our views on the compatibility of Rel-16 URLLC/IIoT features over unlicensed band.

- Compatibility issues for the support of URLLC in unlicensed spectrum

1) Support priority configuration for e-type 2 CB and type 3 CB.

Although URLLC over unlicensed spectrum is applicable for controlled environment, there could be traffics with different priorities for some URLLC use cases. That’s why intra-UE prioritization is specified for Rel-16 URLLC. So, it is desirable to support e-type 2 CB and Type 3 CB to compensate the performance degradation due to LBT failure.

If multiple CBs with different priorities could be extended to e-type 2 CB and type 3 CB, there could be following compatibility issues

- The compatibility of NNK1 value and priority indication for DCI format 1_1/DCI format 1_2.

- The compatibility of PDSCH group index and priority indication for DCI format 1_1/DCI format 1_2

- The compatibility of One-shot HARQ-ACK request and priority indication for DCI format 1_1/DCI format 1_2.

2) Inter-UE multiplexing/prioritization

Same as above, inter-UE multiplexing/prioritization of the UEs with different priorities should also be considered.

Currently, UL power control mechanism for UL inter-UE multiplexing cannot work in unlicensed band. This is because high priority transmission may not be able to access the channel due to collision with an ongoing low priority transmission. In such case, transmission of high-priority PUSCH without LBT can be considered.

Another possibility is to only support UL CI for the inter-UE prioritization. If low priority resources have been canceled by UL CI, some enhanced mechanisms can be further considered, e.g. transmission of high-priority PUSCH with no LBT.

3) Configured grant PUSCH

Currently in Rel-16, the configured grant based UL transmission has been enhanced in both URLLC and NR-U, independently. It is unclear how to configure the CG resources for the URLLC operation in unlicensed band, due to the following compatibility issues:

- The determination of HARQ processing number and RV. In NR-U, they are determined by UE and carried by CG-UCI; while in URLLC, they are associated with the transmission occasions.

- Repetition: In NR-U, the initial transmission depends on the LBT outcome, and multiple consecutive slots are configured for the repetitions; while in URLLC, PUSCH repetition Type B is introduced with support of repetition segmentation across slot boundary or invalid symbols.

- Retransmission: In NR-U, the auto retransmission can be initiated on the CG resources which could be on a same or different CG configuration compared to the CG for initial transmission, and a retransmission timer is introduced in MAC; while in URLLC, the retransmission can be triggered only by UL grant.

- Feedback: Whether CG-DFI is supported or not.

4) Dynamic grant PUSCH

In NR-U, multi-TTI scheduling is supported for multiple TB transmissions. While, in Rel-16 URLLC, repetition number is embedded in the TDRA table for support of dynamic repetition indication. Thus, a combined TDRA table for PUSCH is needed to enable both PUSCH repetition Type B and multi-TTI scheduling.

Proposal 1:

- At least the following compatibility issues for the support of URLLC in unlicensed spectrum should be addressed in Rel-17 URLLC/IIoT WI.

  - Compatibility issues for support of priority configuration for e-type 2 CB and type 3 CB

  - Compatibility issues for support of inter-UE multiplexing/prioritization

  - Compatibility of configured grant designs between Rel-16 URLLC and NR-U

  - Compatibility of TDRA table designs for dynamic PUSCH between Rel-16 URLLC and NR-U.

- Enhancements to improve the unlicensed band URLLC operation

Other than the compatibility issues listed in section 2, there could be some additional “nice to have” enhancements to make the URLLC operate better on the unlicensed band. These enhancements with some justifications on the benefits can be further considered. While it also depends on the overall scope of the WI, so that the workload is manageable.

Proposal 2:

- A minimum set of enhancements on the URLLC or NR-U features can be considered, which can be down-selected from the Table 1 as follows.

Table 1 Possible enhancements to improve the applicability of R16 IIOT/URLLC featured in unlicensed band

| Feature | Descriptions | Justifications | Priority |
| --- | --- | --- | --- |
| PUCCH enhancements | There is no difference between eMBB and URLLC on Type 1 channel access procedures for PUCCH because CAPC p =1 is always used for PUCCH. How to improve channel access procedure for URLLC PUCCH should be studied. | This is to ensure the transmission opportunities for URLLC PUCCH transmission. | Medium |
| URLLC unlicensed band operation using FBE | Support of UL to DL COT sharing for FBE | This is to reduce the latency. The standardization effort can be limited as this has been supported for LBE. | Medium |
| PDCCH enhancements | The new bit fields introduced in Rel-16 NR-U should be extended to DCI format 0_2/1_2. | Support of compact DCI in unlicensed band is beneficial in terms of reliability. | Medium |
| 2-step RACH | MsgA PRACH and PUSCH transmitted in the same slot. | This is to reduce the required number of LBT and thus reduce the latency | Low |
| Wideband operation | Alt.2 can be supported for URLLC on licensed band. | If only Alt.1 is supported, a UE cannot transmit PUSCH until CCA is successful in all LBT bandwidths. The consequence is that URLLC traffic cannot be transmitted timely even though there are still a subset of LBT bandwidths for which CCA is successful. | Low |
| PDCP duplication enhancement | UE Autonomous PDCP Duplication | This is to reduce the latency. When UL LBT fails, UE can activate the PDCP duplication. | Medium |
| RRM measurement enhancement | Event triggering RSSI and channel occupancy reporting | This is to help gNB to discovery the interference. | Low |
| More transmission occasions | The UL grants for deprioritized logical channel can be used for URLLC. | This is to ensure the transmission opportunities for URLLC. | Low |
| UL LBT detection and recovery enhancement | UL LBT detection and recovery mechanism that needs less latency | This is to reduce the latency and recovery the connection quickly for URLLC. | Low |

- Conclusions

In this contribution, we have the following proposals.

Proposal 1:

- At least the following compatibility issues for the support of URLLC in unlicensed spectrum should be addressed in Rel-17 URLLC/IIoT WI.

  - Compatibility issues for support of priority configuration for e-type 2 CB and type 3 CB

  - Compatibility issues for support of inter-UE multiplexing/prioritization

  - Compatibility of configured grant designs between Rel-16 URLLC and NR-U

  - Compatibility of TDRA table designs for dynamic PUSCH between Rel-16 URLLC and NR-U.

Proposal 2:

- A minimum set of enhancements on the URLLC or NR-U features can be considered, which can be down-selected from the Table 1.

Reference

- 3GPP, RAN#86, RP-193233, New WID on enhanced Industrial Internet of Things (IoT) and URLLC support, Nokia, Nokia Shanghai Bell.