3GPP TSG RAN meeting #88e									        RP-200839

Electronic Meeting, June 29 – July 03, 2020

## Status Report to TSG

Agenda item:			9.3.11

| WI / SI Name | Integrated Access and Backhaul for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB |  |  |  |  |
| Unique ID | 820070 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-200084 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2020 (updated from 06/2020) | Performance part: 12/2020 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 96% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #100bis-e

Extensions of RACH occasions and periodicities for backhaul RACH resources

Agreements

The TP below is endorsed.

38.211 <Unchanged text is omitted> For the IAB-MT part of an IAB node, the following applies: - if the higher-layer parameter prach-ConfigurationPeriocdScaling is configured, the variable x used in nSFNmodx=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by xIAB , where xIAB=δx and δ is given by the higher-layer parameter prach-ConfigurationPeriodScaling and the IAB node does not expect xIAB to be larger than 64; - if the higher-layer parameter prach-ConfigurationFrameOffset is configured, the variable y used in nSFNmodx=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by yIAB=y+Δymodx where Δy is given by the higher-layer parameter prach-ConfigurationFrameOffset, and xis the value used innSFNmod x=y; - if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number sn from in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number sn from in Table 6.3.3.2-4 shall be replaced by sn+ΔsmodL where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset, and L is the number of subframes in a frame when using in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame for SCS of 60 KHz when using in Table 6.3.3.2-4. <Unchanged text is omitted>

Mechanisms for resource multiplexing among backhaul and access links

Agreements: TS 38.213 Section 14 should be updated with a TP capturing the following behavior when an IAB-MT is provided with tdd-UL-DL-ConfigurationDedication-IAB-MT:

- Clarification that the behaviors described in Section 11.1 of 38.213 for a UE provided with tdd-UL-DL-ConfigurationDedicated are also applicable apply for an IAB-node MT when provided with tdd-UL-DL-ConfigurationDedicated-IAB-MT

- The IAB-node MT does not expect to be configured with both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationDedicated-IAB-MT.

  - Note: It is up to the 38.213 editor to decide whether or not to include this behavior in case of misconfiguration in the final TP to 38.213.

Agreements: Confirm that from a RAN1 perspective all Rel-15 UE common search space types are also applicable to Rel-16 IAB nodes. Signaling details are left to RAN2.

Agreements: Confirm DCI Format 2_0 and DCI Format 2_5 can be monitored by an IAB-MT in at least a common search space. The same number of aggregation levels and candidates can be separately configured for both DCI Format 2_0 and DCI Format 2_5.

Agreements: DCI Format 2_0 is not monitored by an IAB-MT in a UE(MT)-specific search space. DCI Format 2_5 can be additionally monitored by an IAB-MT in a UE(MT)-specific search space. Signaling details (e.g. whether the configuration is in the existing UE-specific search space configuration or a new MT-specific search space configuration is left up to RAN2).

Agreements:  maxAI-DCI-PayloadSize = maxSFI-DCI-PayloadSize = 128.

Agreements: Capture the following TP in 38.213:

< Unchanged parts are omitted >

For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT.

If the IAB-node MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clauses 11.1 containing the term "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-node MT of an IAB node, with the term "tdd-UL-DL-ConfigurationDedicated" replaced with "tdd-UL-DL-ConfigurationDedicated-IAB-MT”, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

If the IAB-node MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

-     a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-     for each slot configuration from the set of slot configurations

-     a slot index for a slot provided by slotIndex

-     a set of symbols for a slot by symbols where

-     if symbols = allDownlink, all symbols in the slot are downlink

-     if symbols = allUplink, all symbols in the slot are uplink

-     if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible.

-     if symbols = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

For each slot having a corresponding index provided by slotIndex, the IAB-MT applies a format provided by a corresponding symbols.

< Unchanged parts are omitted >

Conclusion: The following note is not captured in 38.213 Section 14 in RAN1#100b-e:

[Note (up to 38.213 editor to decide whether to include or not): The IAB-node MT does not expect to be configured with both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationDedicated-IAB-MT.]

Agreements:

Adopt the following editorial udpates:

A.1

An IAB-node MT can be provided, by SlotFormatCombinationsPerCell-IAB-MT, a list of slot format combinations applicable for one serving cell and, by SlotFormatIndicator-IAB-MT, a configuration for monitor a DCI format 2_0 indicating a slot format combination, from the list of slot format combinations, over a number of slots as described in Clause 11.1.1. In addition to the slot formats in Table 11.1.1-1, an SFI field for an IAB-node MT in DCI format 2_0 can indicate to the IAB-node MT a slot format from the slot formats in Table 14-1.

A.3

a mapping for the soft symbol availability combination provided byAvailabilityCombinationresourceAvailability to a corresponding AI index field value in DCI format 2_5 provided byavailabilityCombinationId

Draft LS to RAN2 as in R1-2002954 is approved, with final LS in R1-2003043

Agreements For paired spectrum, the DU resource configuration framework is extended with the following:

Two separate per-cell D/U/F and H/S/NA configurations are provided for DL and UL respectively.

Whether this signalling is supported in Rel-16 is up to RAN3 and no additional specification impact is considered in RAN1 in Rel-16 for IAB operation in paired spectrum.

Agreements No additional specification impact for 38.213 is required for the definition of half-duplex operation in case of IAB nodes operating in paired spectrum. Further discussion of the default multiplexing capability indication for IAB nodes operating in paired spectrum can be discussed under the IAB-MT Features agenda item in the future (if needed).

Agreements:

The latest TP (38.213, section 14) is endorsed (as in summary R1-2002750, under "Email discussion agreement: To adopt following TP for T_delta mapping in TS38.213”)

Conclusion:

- In case N_TA rounding has impact to IAB case-1 timing synchronization, the issue can be handled by IAB-node implementation.

RAN1 #101-e

Agreement

The text proposals in R1- 2004871 are endorsed. Send an LS to RAN2 to request that they make the necessary specification update.

- LS is endorsed in R1-2004872.

Mechanisms for resource multiplexing among backhaul and access links

Agreement

Confirm to RAN2 that the Number of Guard Symbols can be indicated for a specific serving cell of an IAB-MT.

Additionally inform RAN2 of the following:

From a RAN1 perspective, the Guard Symbols MAC CE should support that the Number of Guard Symbols can be indicated for a specific serving cell of an IAB-MT. RAN1 also acknowledges that while in certain scenarios the Number of Guard Symbols could be the same for all the serving cells in the cell group, this condition is not a requirement.

The LS is endorsed in R1-2004784.

Agreement

Send a response LS to RAN3 with the following content. The LS is endorsed in R1-2004760.

RAN1 response: - RAN1 agrees that the following approach is feasible based on existing RAN1 agreements. “Make the CSI-RS and SR configurations as optional in the cell-specific signals/channels configurations so that they do not have to be configured if signaling storm becomes a concern.” - RAN1 was not able to reach consensus on the feasibility of the following approach. “Explicitly configure these resources used for CSI-RS and SR as Hard at the child node or Not Available at the parent node. Meanwhile, exclude CSI-RS and SR configurations from the list of cell-specific signals/channels configurations.” If this approach or any additional alternative is adopted by RAN3, RAN1 should be informed in order to determine the impact on RAN1 specifications. - No other alternatives result from RAN1 #101-e discussion. ACTION: RAN1 respectfully requests RAN3 to take the above into account, and to kindly inform RAN1 of the decision.

Conclusion

In presence of F symbols in the child DU configuration at the edge of a MT to DU transition (or vice versa), how many guard symbols the parent node will insert at the transition is left to implementation.

- Note: This does not preclude RAN1 from further considering additional solutions in Rel-17

Conclusion

No specification in Rel-16 is introduced defining rules for Guard symbol insertion at a parent node or definitions of MT to DU and DU to MT transitions.

- Note: This does not preclude RAN1 from further considering additional solutions in Rel-17

Conclusion

In Rel-16, an RRC-configured reference SCS configuration for DCI Format 2_5 is not introduced.

Conclusion

In Rel-16, no additional specification impact is introduced for monitoring DCI Format 2_5 simultaneously in both a CSS and a USS by an IAB-MT.

UE features for IAB

Agreement:

- Wide-area IAB-MTs support the following Rel. 15 layer-1 mandatory UE features (as defined in TR38.822)

  - Without capability

    - 0-1, 0-3, 0-4, 1-1 (only 1 preamble for component 1, component 2, component 3 except paging), 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 5-1 (only components 1/2/3/4/5/6/7/9/10/12), 6-1, 7-1, 8-3

  - With capability signaling which shall be set to '1'

    - 1-3, 2-22, 4-10

  - The rest of Rel-15 layer-1 UE features other than the ones as listed above are optional for wide-area IAB-MTs.

- Note: Mandatory MT capabilities are independent from DU capabilities and do not imply a corresponding mandatory DU capability.

- The UE feature list for local-area IAB-MTs is FFS

Agreements:

- The square bracket (including the text) is agreed to be removed for FG20-6/20-7.

Agreement:

- Inform RAN4 that the mentioned features can be made optional for IAB-MTs with no impact from a RAN1 perspective. Additionally in case of FG 1-2 and 1-3, there is no impact from a RAN1 perspective if the parent node supports fallback to 16 QAM during initial access or does not schedule 64 QAM during initial access, which may apply for NR UEs and IAB-MTs.

Draft LS is approved. Final LS in R1-2004954.

#### 2.1.2	Remaining Open issues

100% of the items defined in the RAN1 WID objectives have been accomplished. RAN1 has completed the WI.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #109e-bis

- Agreements on Stage-2 Correction

- The following IAB terminology is adopted:

- - 	There is no IAB-donor gNB or IAB-donor-gNB.

- - 	IAB-donor-DU and IAB-donor-CU are hyphenated in the same manner as the gNB-CU and gNB-DU.

- - 	The IAB-node holds gNB-DU functionality with IAB-specific enhancements, referred to as IAB-DU.

- - 	There is no IAB-node-DU.

- - 	The IAB-DU of a specific IAB-node, e.g., IAB-node 1, can be referred to as IAB-node-1’s IAB-DU, or IAB-DU 1.

- - 	The IAB-node holds UE functionality with IAB-specific enhancements, referred to as IAB-MT.

- - 	There is no IAB-node-MT.

- - 	The IAB-MT of a specific IAB-node, e.g., IAB-node 1, can be referred to as IAB-node-1’s IAB-MT, or IAB-MT 1.

- - 	The parent-node IAB-DU and child-node IAB-MT may be referred to as parent IAB-DU and child IAB-MT, respectively.

- - 	Hyphenation follows commonly known rules

- R2-2004151	Miscellaneous correction to 37.340 for IAB, Huawei, HiSilicon	 - Endorsed

- R2-2004133	CR to 38.300 on Integrated Access and Backhaul for NR	, Qualcomm Incorporated - Endorsed

- Agreements on BAP Open Issues and Corrections

- CU can remove F1 interface to IAB-DU without releasing IAB-MT, in orderly release case.

- CU should remove F1 interface to IAB-DU first, if IAB-MT is to be released, in orderly release case.

- BAP at IAB-DU will be released by implementation, if IAB-MT is released due to RLF, in disorderly release case.

- R2 will NOT specify delay timer for event triggered BAP flow control in R16

- The Donor CU can configure the 1:1 or N:1 mapping to BH RLC Channel on the backup egress link of IAB-node before BH RLF. Whether it is configured is up to CU implementation.

- If the regular mapping to BH RLC Channel in the backup egress link is configured by donor CU, IAB node follows the configured BH RLC channel mapping for re-routed packets.

- If the regular mapping to BH RLC Channel in the backup egress link is NOT configured by donor CU, IAB node: uses any BH RLC channel on the backup egress link for re-routed packets by implementation.

- R2-2004153	Miscellaneous corrections to 38.340 for IAB, Huawei, HiSilicon - Endorsed

- Agreements on User Plane – Open Issues and Corrections

- P8/P7 continue offline

- All of the MAC CEs introduced by the IAB WI shall have their identifiers selected from set2 of the one-byte eLCID space.

- Pre-emptive BSR procedure to be captured as a standalone Section (separate from Section 5.4.5 on “legacy” BSR).

- RAN2 to leave the decision on whether the information contained in the Guard Symbol MAC CE applies to the entire cell group, or individual cell on which it is received, to RAN1. RAN2 to send an appropriate LS to RAN1

- RAN2 to introduce a short section on procedural aspects for Timing offset adjustment for IAB.

- Skip P9, this could possibly be a capablity discussion, no need to change other TS for this,

- the IAB specific IAB RACH configuration is used by IAB MT, if configured (exceptions can be discussed), further details to be discussed offline.

- Apart from the already agreed cancellation condition (that Pre-emptive BSR shall be cancelled when a MAC PDU that contains the pre-emptive BSR MAC CE is sent), RAN2 will not standardize any additional Pre-emptive BSR cancellation conditions in Rel-16.

- Implementation-specific cancellation conditions for Pre-emptive BSR are not precluded.

- SR triggered by (the impossibility to send) Pre-emptive BSR shall be cancelled if a MAC PDU containing the relevant Pre-emptive BSR MAC CE is sent.

- R2-2004127	LS to RAN1 on Guard Symbols in IAB - Approved

- R2-2003830	CR (IAB MAC - rapporteur corrections and clarifications), Samsung Electronics GmbH

- – Endorsed (baseline for further updates)

- R2-2004126	CR (IAB MAC - rapporteur corrections and clarifications), Samsung Electronics GmbH

- – Endorsed (baseline for further updates)

- Agreements on RRC – Open Issues and Corrections

- R2-2004125	Correction for TS 38.331 Related to IAB WI, Ericsson

- – Endorsed (baseline for further updates)

- A clarification to be made in the field description of the default BH RLC channel IE in BAP configuration, indicating that, for the case that IAB-MT is in DC mode: If the IAB-MT is operating in (NG)EN-DC, the default BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel on the MCG.

- Agreements on RLF-handling – Open Issues and Corrections

- IAB-DU behavior after RLF declaration is left up to implementation. IAB-DU should be able to send RLF notification when RLF recovery fails.

- Fast MCG link recovery is supported for NRDC and ENDC.

- Agreements on IP address allocation

- [023] An IAB node explicitly requests IP address(es) during integration in the NSA case. [Explicit means here that either an existing message (e.g. RRCReconfigurationComplete) is modified to explicitly include a request, or a new message is introduced to indicate a request.]

- [023] A single RRC message/procedure is adopted for IP address request, for both SA and NSA cases.

- [023] For the IP address configuration by the CU, RRCReconfiguration message is used for both SA and NSA cases.

- Continue in the RRC discussion.

- Agreements on UE Capabilities

- All optional features remain optional for IAB-MTs.

- Clarification: EN-DC mode support is not mandatory for IAB-MT.

- The following features are optional for IAB-MT:

1. PDCP; 1-5: Short SN

3. MAC; 3-3: DRX

4. Measurements; 4-5: ANR

6. Inactive; 6-1: RRC Inactive

- The following features are mandatory for IAB-MT:

1. PDPC; 1-0 Basic PDCP procedures, at least for SRB, FFS for DRB related components

2. RLC; 2-0 Basic RLC procedures, 2-4 NR RLC SN size for SRB

3. MAC; 3-0 Basic MAC procedures

- It is FFS if in general mandatory features with capability signaling are optional for IAB-MT.

- It is FFS if UE capability signalling will be used at all for Wide Area MTs.

- We consider a min set of features for wide area MT, and whether there may be a need for more mandatory features local area MT.

- The following features should be included in the minimum capabilities set for IAB-MT: 9. RRC: 9-1 RRC buffer size, 9-2 RRC processing time.

- Mandatory IAB-MT features (minimum set of capabilities) are defined (indicated) in a dedicated sub-section in TS 38.306.

- Inclusion of iab-NodeIndication-r16 in RRCSetupComplete message is used as an indication of IAB-MT supporting the defined minimum IAB-MT capabilities set.

- Local-Area IAB-MTs have to support the UE capability signaling framework.

- Ph1-4: SDAP sublayer is always configured for a DRB for connections to 5GC (i.e. same as in Rel-15)

- Ph1-5: SDAP related features in TR 38.822 (5-1, 5-2) are optional for IAB-MT.

- Ph1-8: Introduce channel bandwidth signalling allowing the IAB-MT to indicate whether it supports 100MHz BW for FR1 and 200MHz BW for FR2. Exact modifications are FFS, but [6] is used as a baseline.

- Ph1-9: Introduce capability bits for IAB-MT to allow support indication for the features which are mandatory without capability signaling for Rel-15 UEs, but are optional for IAB-MT.

- Agreements on Other Corrections

- IAB-MT shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds, as in the current specification.

- IAB-MT ignores intraFreqReselection

- R2 make an attempt to support IAB functionality in non-public network deployments in R16 in R2#109bis and R2#110 meeting. If the agreeable CRs can be achieved before ASN.1 freeze, then it is supported. Otherwise, it is not supported in R16

- R2-2004155	Miscellaneous correction to 38.304 for IAB, Huawei, HiSilicon

- - Endorsed as baseline

- R2-2004156	Miscellaneous correction to 36.304 for IAB, Huawei, HiSilicon

- - Endorsed as baseline

RAN2 #110e

- Agreements on RRC

- R2 think no effort should be spent to standardize extensions to RRC Inactive for IAB. If RRC Inactive is supported by an IAB MT, the operation (beyond what is currently specified) is completely up to implementation.

- Agreements on IP address handling

- We use a new message for IP address allocation (can consider to have a generic name)

- R2-2006155	Way forward on IP Address handling in IAB, Samsung (Rapporteur) – Endorsed

- R2-2006351	TP for TS 38.331 on IP Address allocation for IAB nodes, Samsung – Merged with RRC CR

- Agreements on CRs

- R2-2006303	CR to 38.300 	- endorsed

- R2-2006346	CR to 37.340	- endorsed

- R2-2006265	CR to 38.321	- endorsed

- Agreements on Userplane/MAC

- Implementation-specific cancellation conditions for SR triggered by Pre-emptive BSR are not precluded. No changes to specs are needed to capture this – it is sufficient to capture this in meeting notes.

- Confirm that the MAC-CE will follow the R1 agreement: The Number of Guard Symbols is indicated per cell.

- the MAC CE contains the cell id information

- Remove all reserved values from the 2-octed IAB-specific eLCID space, and reassign all 128 values to BH RLC channel ID space.

- Agreements on RRC

- Option 2: IAB-MT applies the IAB specific IEs in RACH config generic, for RACH for SI request.

- Agreements on F1-C transport and configuration

- At most one default UL BAP routing ID and one default BH RLC channel is configured for dual-connected IAB node.

- Default UL BAP routing ID and default BH RLC channel can be (re-)configured, e.g., when IAB node’s IP address for F1-C traffic change.

- Default UL BAP routing ID and default BH RLC channel is only (re-)configured via RRCReconfiguration message. No explicit RRC signalling is supported for the release of the default UL BAP routing ID and default BH RLC channel configuration.

- All Rel-15 UE common search space types apply to IAB-MT unless explicitly specified otherwise. No further clarification is needed in TS 38.331.

- RAN2 to confirm RAN3’s agreement that IAB-MT could be configured with LTE leg only, NR leg only, or both LTE and NR leg for F1-C transfer.

- Include F1-C transfer path field in the CellGroupConfig IE in NR RRCReconfiguration message.

- R2-2006157: TP for F1-C transfer path configuration [045] Merged with the IAB RRC CR

- Agreements on Access Control

- To stick to the current RAN2 agreement on UAC that IAB MT Access Stratum shall consider its access as “allowed” as part of UAC, irrespective of AC and AI provided by NAS.

- To introduce NO further changes to RAN2 specifications on UAC bypassing.

- To introduce NO special handling for setting establishmentCause in RAN2 specifications. That is, the establishmentCause is set in accordance with the information received from upper layers as already specified in RAN2 specification.

- To introduce NO special handling for setting resumeCause in RAN2 specifications.

- NO action is taken in RAN2 to clarify UAC operation in both cases; a) IAB-MT is accessing a cell not broadcasting iab-Support indication, and b) IAB-MT capable of IAB is accessing a cell before having been authorized for IAB operation or having been configured with IAB parameters.

- Agreements on UE capabilities

- Minimum set of IAB-MT capabilities should contain:

- Features which are indispensable for IAB-MT to perform initial access and establish an RRC connection and OAM connection with the network.

- Basic BAP procedures, i.e. routing, bearer mapping

- IP signalling over RRC

- R2 to specify that IAB-MTs can make use of the UE capability signaling framework (including specification of minimum set). Whether it is actually used for e.g. Wide Area IAB-MTs may be up to implementation.

- Agree 3-1 Specify non-DRB operation for IAB-MTs, i.e. support for DRB is optional.

- R2-2006296	Draft CR for IAB capabilities introduction to TS 38.331	Nokia, Nokia Shanghai Bell

- - Endorsed, to be merged with UE capabilities

- R2-2006297	Draft CR for IAB capabilities introduction to TS 38.331	Nokia, Nokia Shanghai Bell

- - Endorsed, to be merged with UE capabilities

- Agreements on Other Corrections

- cellReservedForOtherUse is ignored by IAB-MT for cell barring determination, but still considered by NPN capable IAB-MT for determination of an NPN-only cell.

- IAB-MTs ignore the cellReservedForFutureUse.

- R2-2006320	Corrections to 38.331 for supporting IAB in NPN	Huawei, HiSilicon, Kyocera

- - Agreed unseen

- R2-2006321	Corrections to 38.304 for supporting IAB in NPN	Huawei, HiSilicon, Kyocera

- - Agreed unseen

- R2-2006149	Miscellaneous corrections to 38.304 for IAB, Huawei, HiSilicon

- - Agreed

- R2-2006151	Correction on RLC spec to support the BAP as upper layer, Huawei, HiSilicon

- - Agreed

#### 2.2.2	Remaining Open issues

100% of the items defined in the RAN2 WID objectives have been accomplished. RAN2 has completed the WI.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #108e

Endorsement of Running CRs

- R3-203002: CR to 38.423 endorsed as baseline

- R3-203003: CR to 38.463 endorsed as baseline

- R3-203004: CR to 36.413 endorsed as baseline

- R3-203005: CR to 38.413 endorsed as baseline

- R3-203018: CR to 38.474 endorsed as baseline

- R3-203036: CR to 36.420 endorsed as baseline

- R3-203037: CR to 38.425 endorsed as baseline

- R3-203066: CR to 36.423 endorsed as baseline

- R3-203067: CR to 38.401 endorsed as baseline

- R3-203068: CR to 38.470 endorsed as baseline

- R3-203069: CR to 38.473 endorsed as baseline

Agreements on IAB-DU features and PHY parameters

- R3-204306: TP to TS 38473  - Agreed

- IAB-DU capabilities are not exchanged between IAB-DU/IAB-donor-DU and IAB-donor-CU via F1 interface.

- In Rel-16, how the donor-CU and/or parent node are aware of the appropriate capabilities of a given child IAB-DU is left up to network implementation (e.g. via OAM)

- F1AP signaling needs to be enhanced to support DU resource configuration for paired spectrum, and the CHOICE structure is used to capture the configuration.

- Make the CSI-RS and SR configurations optional in the cell-specific signals/channels configurations so that they do not have to be configured if signaling storm becomes a concern.

Agreements on optional E-RAP setup

- R3-204381: TP to TS 38401  - Agreed unseen

- DRB setup is optional for NR SA since Rel-15; E-RAB setup is mandatory in LTE since Rel-8

Agreements on Cell Barring

- R3-204388: TP to TS 38473  - Agreed unseen

- IAB donor CU can send IAB node barred information to the IAB-DU or IAB donor DU

- IAB barred information exchange over X2/Xn is not supported in this release.

Agreements on IAB bearer mapping

- R3-204382: TP to TS 38401  - Agreed unseen

- maxnoofEgressLinks = 2

- Use Mapping Information Index to reference the mapping configuration

- R3-204244: TP to TS 38401  - Agreed unseen

- R3-204242: TP to TS 38473  - Agreed

- For F1AP procedure to configure the DL mapping in the Donor-DU, and to configure the UL/DL mapping in the intermediate IAB, the UE-Associated F1AP procedure is used when the BH RLC CH is impacted (e.g. add a new BH RLC CH, or modify an existing BH RLC CH), and Non-UE-Associated F1AP procedure is used otherwise; when to use UA and NUA procedures (including e.g. the avoidance of concurrent usage) shall be fully described in st2 and st3 as necessary

- In principle, use same IE (i.e. To Add IE and To Remove IE) for UA and NUA signaling

- Modify existing F1AP BH Routing Configuration procedure also for BH RLC Channel mapping configuration

- No need to add Additional Egress BH RLC CH information

- Max no of mapping entries: 2^26

- Max no of DS information: 64

Agreements on BH Misc Cleanup

- R3-204248: TP to TS 38473  - Agreed

- R3-204249: TP to TS 38300  - Agreed

- Max # of non-UP traffic mapping: 32

- Keep the presence of Non-UP Traffic Type IE as it is.

- BAP address update for IAB node and IAB donor DU is not supported in R16.

- Two following cause values are defined for BH RLC CH in F1AP:

- Multiple BH RLC CH ID Instances

- Unknown BH RLC CH ID

- Delete the semantics description for Uplink BH Non-UP Traffic Mapping IE, and correct the “rach-ConfigCommon” to be “rach-ConfigCommonIAB-r16”.

- SCTP multi-homing can be supported by IAB node with connection to one or multiple IAB donor DUs.

- To support SCTP multi-homing, multiple UL BH Information IEs can be configured to same F1-C traffic, including UE-associated F1AP and non-UE-associated F1AP.

- If more than one UL BH Information is configured to a given F1-C traffic, it is up to IAB node implementation to select a suitable one for transmitting the corresponding UL F1-C traffic.

Agreements on IAB-MT RRC INACTIVE

- The IAB-MT optionally supports the RRC INACTIVE state. Upon the IAB-MT entering the RRC INACTIVE state, it is up to implementation whether to keep or to release the F1 connection of the collocated IAB-DU.

- Attempt st2 text to capture the fact that certain functionality will need to be impacted through implementation (e.g. SCTP connection fault mgmt., IPsec keep-alive, F1AP config, BAP config, …)

- R3-204322: TP to TS 38401  - Agreed unseen

Agreements on IP address allocation in SA

- R3-204079: TP to TS 38474  - Agreed

Agreements on IAB DC Operation

- For F1-C traffic transfer for NSA IAB, the LTE leg and NR leg should use separate IP address pairs {IAB-DU’s IP address, IAB-donor-CU’s IP address}. How the IAB-DU gets the remote IP end point(s) and its own IP address for LTE leg is not specified in this release.

- The IAB-donor-CU decides to only configure LTE leg, or only configure NR leg, or configure both LTE leg and NR leg, to be used for F1-C traffic transfer. The configuration may be performed before IAB-DU part setup. IAB-donor-CU may also change the configuration after IAB-DU part setup. In case the configuration is not performed before IAB-DU part setup, the IAB node uses the NR leg as the default one. When both LTE leg and NR leg are configured, it is up to the implementation to select the leg for F1-C traffic transfer.

- Introduce a new X2AP IE (sent from MeNB to en-gNB) for IP address request/report.

- R3-204246: TP to TS 38401  - Agreed

- R3-204247: TP to TS 36423  - Agreed

- R3-204165: LS to RAN2  - Agreed unseen

Agreements on IAB Migration Functions

- R3-204250: TP to TS 38401  - Agreed

Agreements on IAB Migration under Same Donor

- R3-204251: TP to TS 38401  - Agreed

- R3-204384: TP to TS 38463  - Agreed unseen

- R3-204383: TP to TS 38473  - Agreed unseen

- F1/E1 group signaling uses an NUA F1AP/E1AP message since it may include information updates affecting multiple UEs.

- The F1/E1 group signaling message can be used for IAB node migration/RLF recovery to same or different donor-DU as well as same or different CU-UP/SeGW

- The F1/E1 group signaling can be used to update DL IP addresses for IAB node migration/ RLF recovery to different donor-DUs

- Optional IEs contains the following contents are included in the messages for NUA F1AP/ E1AP procedures:

- IAB-donor-CU-> IAB-DU:

- list of {new UL UP TNL Information, old UL UP TNL Information, UL BH Information }

- list of {new UL UP TNL address, old UL UP TNL address}

- IAB-DU-> IAB-donor-CU:

- list of { new DL UP TNL address, old DL UP TNL address }

- CU-CP->CU-UP:

- list of {new DL UP TNL address, old DL UP TNL address}

- CU-UP->CU-CP:

- list of {new UL UP TNL address, old UL UP TNL address}

- Introduce the following cause value for the New E1AP/F1AP procedure:

- F1AP：“Unknown UP TNL information”, “Unknown TNL address”

- E1AP：“Unknown TNL address”

RAN3 #107e-bis

Endorsement of Running CRs

- R3-201529: CR to 38.423 endorsed as baseline

- R3-201540: CR to 38.463 endorsed as baseline

- R3-201532: CR to 36.413 endorsed as baseline

- R3-201534: CR to 38.413 endorsed as baseline

- R3-202647: CR to 38.425 endorsed as baseline

- R3-201596: CR to 36.423 endorsed as baseline

- R3-201597: CR to 38.401 endorsed as baseline

- R3-201598: CR to 38.470 endorsed as baseline

- R3-201621: CR to 38.473 endorsed as baseline

- R3-201533: CR to 38.300 endorsed as baseline

Agreements on IAB Baselines and terminology

- There is no IAB-donor gNB or IAB-donor-gNB.

- IAB-donor-DU and IAB-donor-CU are hyphenated in the same manner as the gNB-CU and gNB-DU.

- The IAB-node holds gNB-DU functionality with IAB-specific enhancements, referred to as IAB-DU.

- There is no IAB-node-DU.

- The IAB-DU of a specific IAB-node, e.g., IAB-node 1, can be referred to as IAB-node-1’s IAB-DU, or IAB-DU 1.

- The IAB-node holds UE functionality with IAB-specific enhancements, referred to as IAB-MT.

- There is no IAB-node-MT.

- The IAB-MT of a specific IAB-node, e.g., IAB-node 1, can be referred to as IAB-node-1’s IAB-MT, or IAB-MT 1.

- The parent-node IAB-DU and child-node IAB-MT may be referred to as parent IAB-DU and child IAB-MT, respectively.

- Hyphenation follows commonly known rules.

- R3-202757: TP to TS 38.470  - Agreed

Agreements on IAB Topology Discovery

- In case IPsec tunnel mode is used, it is the outer IP addresses that are allocated to an IAB-node by the OAM or IAB donor

- In case IPsec tunnel mode is used, how an IAB-node obtains the inner IP addresses is out of RAN3 scope

- In case of IPsec transport mode or no IPsec, the IP addresses are allocated to an IAB-node by OAM or IAB-donor

- For OAM-based IP address allocation, it is necessary to inform, via RRC before F1 transport has been set up, the IAB-donor-CU about the IP addresses allocated to the IAB node, to enable the IAB-donor-CU to configure the IAB-donor-DU with the mapping between the IP addresses allocated to the IAB-node and DL BAP routing IDs

- Opt3 for IAB Topology Discovery may be used for all IAB scenarios of interest

- R3-202758: TP to TS 38.300  - Agreed

Agreements on Stage Cleanups

- R3-202836: TP to TS 38.401  - Agreed

Agreements on IAB X2 Stage 2

- R3-202836: TP to TS 36.420  - Endorsed as BL

Agreements on IAB BAP configuration of Donor DU

- For configure BAP address in the Donor-DU:

- No change to F1 SETUP REQUEST message.

- Only assign one BAP address to a Donor-DU in Rel-16.

- Agree the following TP: “If the F1 SETUP REQUEST message is received from an IAB-donor-DU, the gNB-CU shall, if supported, include the BAP Address IE in the F1 SETUP RESPONSE message. If the F1 SETUP RESPONSE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address, and use it as specified in TS 38.340[x].”

- No need to change the F1 DU Configuration Update procedure or CU Configuration Update procedure.

- R3-202747: TP to TS 38.473  - Agreed

Agreements on IAB Bearer Mapping

- WA: For F1AP procedure to configure the DL mapping in the Donor-DU, and to configure the UL/DL mapping in the intermediate IAB, the UE-Associated F1AP procedure is used when the BH RLC CH is impacted (e.g. add a new BH RLC CH, or modify an existing BH RLC CH), and Non-UE-Associated F1AP procedure is used otherwise.

- To execute the DL traffic mapping, following information is needed in the Donor-DU:

- Destination IP address

- IPv6 Flow Label

- List of DSCP

- BAP Routing ID

- Next-hop BAP address

- Egress BH RLC Channel ID

- To execute the UL and DL traffic mapping, following information is needed in the intermediate IAB:

- Prior-hop BAP address

- Ingress BH RLC CH ID

- Next hop BAP address

- Egress BH RLC Channel ID

- Support both scenarios using standalone Security Gateway, or collocated Security Gateway with Donor-CU

- R3-202788: TP to TS 38.473  - Agreed

Agreements on F1AP Cleanup

- Use the following parameters:

- maxnoofBHRLCChannels: 65536

- BH RLC CH ID: BIT STRING(16)

- maxnoofChildIABNodes: 1024

- maxnoofServedCellsIAB: 512

- For the BAP control PDU mapping configuration, adopt the following:

- For DL: Include usage indication (e.g.  for BAP control PDU transmission) rather than extending Control Plane Traffic Type IE in F1AP message to parent DU when configuring BH RLC channel.

- For UL: Reuse UL BH Non-UP Traffic Mapping IE to tell IAB-MT the BH RLC CHs configured for BAP control PDU at upstream direction.

- No new BHRLCCHDirection IE to indicate the traffic direction over the configured BH RLC channel.

- Add the following clarification on the PDB for BH RLC channel in IAB BL CR for TS38.473:

- “For a BH RLC channel, the Packet Delay Budget included in QCI defines the upper bound for the time that a packet may be delayed between the gNB-DU and its child IAB-MT.”

- IAB node determines the IP address according to the BAP address of the destination IAB donor DU contained in the UL BH information IE, which is configured by the IAB-donor-CU in the UE CONTEXT SETUP/MODIFICATION REQUEST message.

- RAN3 extend the UL BH Information IE to allow CU provide a list of egress BH RLC channels for a given upper layer traffic.

- R3-202648: TP to TS 38.473  - Agreed

Agreements on IAB IP Address Management

- Each IAB-node under an IAB-donor-DU should be assigned a unique IPv6 prefix

- The configured mapping in the IAB-donor-DU may be based on the IPv6 prefix assigned to an IAB node; the full address takes precedence over the corresponding prefix

- IP address(es) to signal

- IE(s) can include IPv4 or IPv6 address

- What they are used for

- F1-C

- F1-U

- non-F1 traffic

- IP address update is performed using DL RRC signaling (the updated address replaces the old one); signaling details are out of RAN3 scope

- R3-202842: TP to TS 38.401  - Agreed

- R3-202858: TP to TS 38.473  - Agreed

- R3-202858: LS to RAN2  - Agreed

Agreements on IAB PHY Parameter Configurations

- PDCCH-ConfigSIB1 and subCarrierSpacingCommon shall be added in the Child-Node List Item IE in the GNB-DU RESOURCE CONFIGURATION message.

- SSB FreqInfo in IAB-STC-Info Item IE only indicates the ARFCN value of the SSB central frequency.

- NR SCS IE instead of SSB Transmission Bandwidth IE is contained in IAB-STC-Info Item IE.

- Multiplex info of IAB node shall be added in the Child-Node List Item IE in the GNB-DU RESOURCE CONFIGURATION message

- R3-202850: TP to TS 38.473  - Agreed

- R3-202852: LS to RAN1  - Agreed

- R3-202859: LS to RAN1  - Agreed

Agreements on IAB DC Operation

- The F1-C IP packet over the LTE leg includes the SCTP/IP header.

- When an LTE leg is configured, it can be used for F1-C. It is out of RAN3 scope to design how to perform the configuration.

- It is up to Donor-CU to decide to only configure LTE leg, or only configure NR leg, or configure both LTE leg and NR leg, for F1-C.

- When both LTE leg and NR leg are configured, it is up to node implementation to select a leg for F1-C transfer.

- The existing security requirements for F1-C apply to F1-C over LTE leg as well. The F1-C over LTE leg reuses the existing security mechanisms for F1-C interface.

- Separate IP address pairs {IAB-DU’s IP address, CU’s IP address} should be used for NR leg and LTE leg.

- FFS on whether same SA/NSA IP address allocation procedures for the IP address assignment for F1-C over LTE leg

- No impact to X2 interface on setting up LTE leg for F1-C.

- R3-202789: TP to TS 36.423  - Agreed

Agreements on IAB-node Migration

- WA: A new class-1 non-UE associated E1AP procedure is defined for donor-CU-CP to inform donor-CU-UP to update the DL UP TNL Information for multiple UEs and child IAB-MTs.

- Donor CU configures a default UL BAP routing ID and a default BH RLC channel to migrating IAB node in order to configure UL mapping for F1-C traffic on the target path, e.g. IKE handshake, SCTP chunks, during the handover preparation procedure.

- WA: A new class-1 non-UE-associated F1AP procedure is defined for donor-CU-CP to inform IAB-DU to update UL FTEID, UL BH Info and DL FTEID for UP traffic

- R3-202751: TP to TS 38.401  - Agreed

- R3-202841: TP to TS 38.401  - Agreed

- R3-202860: TP to TS 38.401  - Agreed unseen

- R3-202851: LS to RAN2  - Agreed

#### 2.3.2	Remaining Open issues

100% of the items defined in the RAN3 WID objectives have been accomplished. RAN3 has completed the WI.

## 2.4	RAN4

#### 2.4.1	Agreements

##### IAB-MT Features Agreements

RAN4 #94bis-e

WF agreed in R4-2005606, following agreements were captured:

  - Capabilities that were optional for UE are kept optional

  - Capabilities related to CA and EN-DC support are FFS until the CA/EN-DC support framework becomes clear

  - Capabilities related to modulation (64QAM  for FR2, 256QAM for FR1) that were mandatory without capability signaling are FFS, infra-vendors proposed to have them optional based on manufacturer declaration.

  - Support for PI/2 BPSK is FFS, so far it was agreed that there won’t be any EVM requirements but it was not yet explicitly agreed that it would not be supported by IAB-MTs

  - Active BWP switching delay and 7.5kHz UL raster shift are also FFS

LS to RAN1/2 agreed in R4-2005608

  - RAN4 is asking RAN1/2 whether there are any issues with the RAN4 agreement on IAB-MT features. The main issue is whether there would be problems on a system level if some modulations that were mandatory for Rel.15 UEs would not be supported

RAN4 #95-e

WF agreed in R4-2009067, LS to RAN2 agreed in R4-2009051

Following is agreed:

  - UL and DL 64QAM(1-2 and 1-3) are mandatory

  - UL and DL 246QAM are optional (1-4 and 1-5)

  - PI/2 BPSK  and related features are not supported (1-6, 1-7,2-10)

  - DCI based BWP switching delay(1-8) – not applicable

  - MFBI is optional (2-10) – could be revisited if this contradicts RF requirements

  - UL Duty cycle related features are not supported (2-13, 2-15)

- CA, EN-DC and SUL features are TBD until specs are clear and support framework is clear

  - This will depend on the signaling framework

- Baseband related features (3-1, 3-2, 3-3), Power class(2-8) and MPR/A-MPR related features (2-11, 2-12) are TBD

  - Handling of some features depends on the RAN2 capability/feature support signaling framework:

    - For the features agreed as optional, the parent/donor should be aware of whether such features are supported by the MT or not. The detail mechanism is up to RAN2 design.

        - For IAB features agreed as optional, the conformance test mechanism will be further discussed in RAN4 performance phase.

        - Two options are considered from RAN4 aspect:

          - Option 1: optional with capability signalling

          - Option 2: optional with declaration

##### 2.4.1.1 RF Agreements

RAN4 #94Bis-e

WF on System parameters capturing several agreements in R4-2005482:

For the multi-band, multi-carrier and CA requirements, IAB-MT requirements follow BS approach for all wide area  IAB-MT class only, other IAB-MT class FFS.

Minimum number of IAB-MT transceivers:

For 1-H, no minimum limit of number of TRX for ITA IAB-MT

For 1-O, minimum limit of number of TRX for ITA IAB-MT

- Option 1: limitation is 4

- Option 2: limitation is 8

Introduce second IAB-MT class as “Local-Area” IAB-MT

- The requirements can be discussed case by case; with above naming “Local-Area” for 2nd IAB-MT class, there is no indication the requirements will be automatically reused from local-area BS requirements

Postpone the introduction of description of IAB-MT class in TS, focused on introduction of requirements for different IAB-MT classes first.

- The background of IAB-MT class can be captured in the TR for information and reference purpose

It’s RAN4 common understanding there is no test for the parameter used in the IAB-MT class definition; meanwhile the definition will be the key background information we used to define requirements.

IAB-DU and IAB-MT class combinations

IAB-MT and IAB-DU classes shall be possible to be combined freely in an IAB-Node, additional RF impact analysis is needed after two IAB-MT class RF requirement is agreed

IAB-MT Tx power dynamic range related agreements:

Define tx dynamic range depending on IAB-MT class

- Wide area IAB-MT Tx dynamic range:

  - Option 1: 0 dB Tx dynamic range

  - Option 2: 5dB Tx dynamic range

  - Option 3: 10dB Tx dynamic range

- Local area IAB-MT Tx dynamic range:

  - Option 1: 10dB Tx dynamic range

  - Option 2: 20dB Tx dynamic range

IAB-MT minimum and maximum output power related agreements (R4-2005486 and R4-2005487)

- Agreement: for FR1 and FR2 IAB-MT no minimum power requirement is specified.

- Agreement: no upper limit on the output power in FR1 for wide area IAB-MT

- Agreement: no TRP upper limit on the output power for local area IAB-MT of type 2-O

IAB-MT ACLR for FR2:

  - Wide Area IAB-MT shall re-use BS requirements

  - Local Area IAB-MT shall meet 24 dBc ACLR, FFS on different requirements and related capability for transmission during UL and DL timeslot.

IAB-MT Frequency Error (R4-2005489)

  - IAB-MT frequency error requirement is defined as relative requirement as +/-0.1 PPM relative to received signal from parent.

  - Test case configuration should be implementation agnostic and further discussion on test aspect can happen during conforming testing phase.

  - Companies are encouraged to provide opinions on potential network impact for one possible implementation option where IAB-MT is used as synchronization source

IAB-MT Reference Sensitivity (R4-2005492)

  - Declaration range of EIS_REFSENS_50M will be defined for each IAB-MT class respectively to either reuse or merge together different BS 2-O declaration ranges on EIS_REFSENS_50M as table below:

| Class | EISREFSENS_50M range (dBm) |
| --- | --- |
| Wide Area | -96 to -119 |
| [Local Area] | Option 1:-86 to -109 Option 2: -86 to -114 |

IAB-MT ACS and IBB (R4-2005493)

- FR2 IAB-MT ACS wanted signal power:

  - For WA IAB-MT use REFSENS + 6dB as wanted signal power.

  - For LA IAB-MT consider;

    - REFSENS + 14dB as the wanted power

    - REFSENS +6 dB as wanted power

    - FR2 IAB-MT In-Band Blocking wanted signal and interfering signal:

  - Reuse BS In-Band Blocking wanted signal and interference level for WA IAB-MT.

  - Reuse UE In-Band blocking wanted signal and interference level for LA IAB-MT

  - FR2 IAB-MT In-Band Blocking interfering signal type:

  - CP-OFDM in-band interferer shall be used for IAB-MT.

RAN4 #95-e

IAB-MT Class description agreed as follows (R4-2008767)

  - Wide Area IAB-MT nodes are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

  - Local Area IAB-MT nodes are characterised by requirements derived from Pico Cell and /or Micro Cell scenarios.

Pcmax definition agreements (R4-2008774)

  - PCMAX,f,c shall be defined for IAB-MT

  - The factor related to pi/2 BPSK, UL Duty cycle and P-MPR will not be applied for IAB-MT

  - Many other FFSs

IAB-MT Output Power Dynamic Range agreements (R4-2008775)

  - Dynamic range requirements:

    - two separate requirements X and Y as following

    - Dynamic range (X)-dynamic PSD: power difference between min TX and max Tx power under the same side condition with full RB transmission.

      - The side condition including modulation order, reference channel FFS

    - Dynamic range (Y)-constant PSD: 10*logNRB

      - This applied for power ratio among single RB and full RB in the same channel

  - For WA IAB-MT

    - FR1 – X= 5dB

    - FR2 – X= 5dB

    - No additional power control requirements beside dynamic range requirements will be defined in Rel-16.

    - FFS how the dynamic range requirements can guarantee the accuracy not only minimum threshold, which can be further discussed in performance phase.

  - For LA IAB-MT

    - FR1 – X= 10dB

    - FR2 – X= 10dB

    - Introduce power control requirements, FFS for the details

Unwanted emissions agreements (R4-2009065)

- IAB-MT ACLR minimum requirement in FR1

  - Wide area IAB-MT: 45 dBc

  - FFS for Local area IAB-MT, make decision in Q3 2020:

    - Option 1: 45dBc

    - Option 2: 30dBc

- OBUE and SEM

  - For FR2:

    - Wide area IAB-MT: Wide Area IAB-MT shall meet [wide area] BS OBUE requirements

    - FFS for local area IAB-MT, make decision in Q3 2020

      - Option 1: Using Local area BS OBUE requirements

      - Option 2: Using UE specification requirements

  - For FR1

    - Wide Area IAB-MT shall meet [wide area] BS OBUE requirements

    - FFS for local area IAB-MT

      - Option 1: Using Local area BS OBUE requirements

      - Option 2: Using UE specification requirements

  - FR1 and FR2 the boundary between in-band and spurious region

    - Wide-area IAB-MT: follow BS requirements

    - Local-area IAB-MT: FFS

- Absolute ACLR

  - Absolute ACLR to be discussed further in future meetings.

- Tx spurious emissions

  - Wide-area IAB-MT: The same spurious emissions requirements as base stations (category A and category B)

  - Local-area IAB-MT:FFS

IAB-MT Reference Sensitivity agreements (R4-2008785)

  - Declaration range for FR2 LA IAB-MT: -86 to -114dBm

  - LA IAB-MT type 1-H FR1 Prefsens: Reused from Local Area BS with NF =13dB, IM =2dB

  - FR1 OTA sensitivity and OTA reference sensitivity level applicability for IAB-MT

  - FR1 OTA sensitivity for IAB-MT:

    - For IAB-MT type 1-H

      - OTA sensitivity requirement is specified for IAB-MT type 1-H

    - For IAB-MT type 1-O:

      - Both OTA sensitivity and OTA REFSENS needs to be specified for 1-O

  - FRC to be used by IAB-MT:

    - Selected UE FRC can be for IAB-MT based the same criteria as BS

In-band selectivity and blocking agreements (R4-2009066)

Both  EIS_minSENS and EIS_REFSENS will be used for IBB, but only both  EIS_minSENS is used for ACS, this is the same as BS ACS type 1-O

For interference signal:

- if the interferer signal characteristic ( bandwidth , # of RB) is changed, then the interferer offset also needs to be re-evaluated, both in FR1 and FR2

- ACS and IBB of FR1 WA IAB-MT Type 1-H:

| WA IAB-MT | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- |
| ACS(45dB) | REFSENS+6dB | -52 | CP-OFDM, FFS on others |
| In-band blocking | REFSENS+6dB | -43 | CP-OFDM, FFS on others |

- ACS of FR1 WA IAB-MT Type 1-O:

| WA IAB-MT | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- |
| ACS (45dB) | EISminSENS +6dB | -52 – ΔminSENS | CP-OFDM, FFS on others |

- IBB of FR1 WA IAB-MT Type 1-O:

| WA IAB-MT | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- |
| In-band blocking | EISREFSENS +6dB | -43 - ΔOTAREFSENS | CP-OFDM, CP-OFDM, FFS on others |
|  | EISminSENS + 6 dB | -43 – ΔminSENS |  |

- FR1  LA IAB-MT Type 1-H  with below candidate options

| LA IAB-MT |  | Wanted signal level[dBm] | Interference signal level [dBm] |
| --- | --- | --- | --- |
| Option 1: ACS 33dB | ACS | REFSENS+14dB | Case 1: REFSENS+45.5 Necessity on case 2 is FFS since the dynamic range for FR1 IAB-MT is open too. |
|  | In-band blocking | REFSENS+6dB | Case 1:-56 Case 2:-44 |
| Option 2: ACS 45dB | ACS(45dB) | REFSENS+6dB | -44 |
|  | In-band blocking | REFSENS+6dB | -35 if REFSENS = -93.7dBm |

Note: in option 1 for LA IAB-MT is based on the NF of 9/10dB, if the NF is agreed as 13dB the interference level should be adjusted accordingly.

- ACS of FR1 LA IAB-MT Type 1-O with below candidate options

| LA IAB-MT | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- |
| Option 1: ACS 33dB | EISminSENS +14dB | EISminSENS +45.5 – ΔminSENS | CP-OFDM, other FFS |
| Option 2: ACS 45 dB | EISminSENS +6dB | -44 – ΔminSENS | CP-OFDM, other FFS |

- IBB of FR1 LA IAB-MT Type 1-O: with below candidate options

| LA IAB-MT | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- |
| Option 1: | EISREFSENS +6dB | -56 - ΔOTAREFSENS -44 - ΔOTAREFSENS | CP-OFDM, Others FFS |
|  | EISminSENS + 6 dB | -56 – ΔminSENS -44 – ΔminSENS |  |
| Option 2: | EISREFSENS +6dB | -35 - ΔOTAREFSENS | CP-OFDM, Others FFS |
|  | EISminSENS + 6 dB | -35 – ΔminSENS |  |

Approved TPs to TS 38.174:

  - System parameters: R4-2008769

  - Transmit On/off Power: R4-2008778

  - IAB-DU Transmitted Signal Quality: R4-2006275

  - IAB Rx IM requirements: R4-2008788

  - IAB ICS requirement: R4-2008791

  - Tx IMD: R4-2009062

  - Conducted Rx Spurious: R4-2008799

  - OTA ACS: R4-2008795

  - OTA Rx Spurious: R4-2008796

  - OTA In-band Blocking: R4-2008798

  - IAB-DU Tx Dynamic range: R4-2009063

  - IAB-DU Rx Sensitivity: R4-2008800

  - Rx Dynamic Range: R4-2008801

Approved TPs to TR 38.809

  - Transmit On/Off Power: R4-2008777

  - IAB Rx IM requirements: R4-2008787

  - IAB ICS requirement: R4-2008789

  - Tx IMD: R4-2009062

##### RRM Agreements

RAN4 #94bis-e

Issue 2-1: RRC re-establishment delay

Agreement: The time to identify target cell in RRC re-establishment delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC re-establisment by a factor of 8.

Table 2: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell for IAB-MT

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (1600 ms, 5 x TSMTC) | MAX (6400 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (8000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 6400Note1 |
| < -8 | FR2 | N/A | 28160Note1 |
| Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC >160 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Table 3: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell for IAB-MT

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (1600 ms, 6 x TSMTC, i) | MAX (6400 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (8000 ms, 104 x TSMTC, i)) |
| < -8 | FR1 | N/A | 6400Note1 |
| < -8 | FR2 | N/A | 32000Note1 |
| Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC,i >160 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Issue 2-2: RRC release with redirection delay

Agreement

- The time to identify target cell in RRC redirection delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC redirection by a factor of 8.

Table 5: Time to identify target NR cell for RRC connection release with redirection to NR for IAB-MT

| Frequency range (FR) of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (5440 ms, 11Trs) |
| FR2 | MAX (7040 ms, 811Trs) |

Sub-topic 4-1: Relaxed RLM requirement

Agreement: Downselect from following options during the 2nd round discussion:

- Relax the RLM evaluation period of IAB-MTs with respect to that of NR UEs

- Scale the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period by a factor.

- FFS: Scaling factor

- Do not relax RLM evaluation period of IAB-MTs with respect to that of NR UEs

Sub-topic 4-2: Sharing factor P.

Continue discussion in the 2nd round.  Capture conclusions in R4-2005318

Sub-topic 4-3: Density of CSI-RS to define CSI-RS based RLM/BFD/CBD requirements.

Agreement: The CSI-RS requirements shall apply only for CSI-RS density 3 over the bandwidth ≥ 24 PRBs.

Sub-topic 4-4: Other requirements

Agreement: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based RLM and CSI-RS based RLM

- Minimum requirement at transitions

- Minimum requirement for L1 indication.

- Scheduling availability during RLM

Sub-topic 5-1: Beam sweeping factor

Continue discussion in the 2nd round.

Sub-topic 5-2: Other requirements

Agreement: The following CBD related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based CBD and CSI-RS based CBD

- Scheduling availability during CBD

Inclusion of RRM requirements in IAB TR

- RRM requirements will be captured in IAB TR

- The description of RRM requirements from IAB TS will be reused in the relevant sections of IAB TR.

  - Companies are encouraged to briefly describe the rationale behind introducing these requirements

Number of SMTC windows for RRC re-establishment and release with redirection

Down-select from following options during the next meeting

- Option 1: Requirements should be derived based on Rel-16 RAN1 IAB-MT related agreements, i.e., by assuming each IAB-MT can be configured up to four SMTC windows per frequency layer.

- Option 2: Requirements should be derived based on Rel-15 UE requirements, i.e., by assuming each IAB-MT can be configured up to two and one SMTC windows per intra-frequency and inter-frequency layers respectively.

- Option 3:

  - For IAB-MTs that support four SMTC configurations per frequency layer, option 1 is supported.

  - For IAB-MTs that do not support four SMTC configurations per frequency layer, option 2 is supported.

Beam sweeping factor N for FR2 during CBD evaluation period of IAB-MTs

For IAB CBD requirement, down-select beam sweeping factor N for FR2 from following options during the next meeting

- Option 1: N = 8

  - Option 2: N = 4

  - Option 3: N = 6

RAN4 #95e

Number of supportable SMTC configurations per frequency layer

Agreement: For IAB-MTs that support four SMTC configurations per frequency layer, requirements are derived by assuming each IAB-MT can be configured up to four SMTC windows per frequency layer.

For IAB-MTs that don’t support four SMTC configurations per frequency layer, requirements are derived by assuming each IAB-MT can be configured up to two SMTC windows in intra-frequency and one SMTC window per inter-frequency layer.

Framework of RLM evaluation period

Agreement: RLM evaluation periods of IAB-MTs follow the following framework (where K1 and K2 denote the relaxation factors for FR1 and FR2 respectively):

Table 12.3.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200 K1, Ceil(10 P K1) TSSB) | Max(100 K1, Ceil(5 P K1) TSSB) |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. |  |  |

Table 12.3.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200 K2, Ceil(10 P N K2) TSSB) | Max(100 K2, Ceil(5 P N K2) TSSB) |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. |  |  |

Table 12.3.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200 K1, Ceil(Mout×P K1)×TCSI-RS) | Max(100 K1, Ceil(Min×P K1) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. |  |  |

Table 12.3.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200 K2, Ceil(Mout×P K2)×TCSI-RS) | Max(100 K2, Ceil(Min×P K2) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. |  |  |

The agreement will be captured with the following editor’s note in the spec:

[Editor’s note: K1 and K2 will eventually be replaced by their values once RAN4 finalizes these]

Beam sweeping factor N for SSB based RLM evaluation period in FR2:

Agreement: N = 8.

Beam sweeping factor N for IAB CBD requirements

Session chair: continue discussion given additional comments in the reflector

Beam sweeping factor N for IAB BFD requirements

Agreement: For IAB BFD requirement, reuse the beam sweeping factors that were defined for UEs in Rel-15.

Measurement restriction requirements and minimum requirements for L1 indication during BFD of IAB-MTs.

Agreement: Re-use the measurement restriction requirements and minimum requirements for L1 indication, that were defined for UEs in Rel-15, in IAB networks

CA scenarios in IAB-MT timing requirements

- Define transmit timing requirements of IAB-MTs, that are supporting local area IAB-MT class, in CA scenarios

  - The transmit timing requirements of IAB-MTs, that are supporting local area IAB-MT class, in CA scenarios can reuse the related requirements of Rel-15 UEs

- FFS: Whether to define transmit timing requirements of IAB-MTs, that are supporting wide area IAB-MT class, in CA scenarios.

DRX mode in IAB-MT timing requirements

- Transmit timing requirements are not defined for IAB-MTs in DRX mode

Relaxation factor K1 and K2 in FR1 and FR2 SSB based and CSI-RS based RLM evaluation period for IAB-MTs

- K1 = 5

- Down-select K2 from following options:

  - K2 = 2

  - K2 = 5

Beam sweeping factor N in CBD evaluation period for IAB-MTs

- N = 8

##### EMC Agreements

RAN4 #95e

Approved: WF on IAB EMC (R4-2008732)

- Emission requirement

- For Radiated Emission requirement, wait to see the RF spurious emission outcome for further study.

- For conducted emission requirement, it is agree to reuse the base station frame work and the wording are per requirement basis.

- Immunity requirement.

- 3V/m with 80--6000MHz range is starting point for radiated immunity requirement

- Exclusion band for RI requirement is FFS

- How to capture the IAB EMC requirement

- It is agreed to have a new TS for IAB EMC

    - Supported companies as: ZTE, Huawei, Ericsson

    - Nokia has no strong view on this

#### 2.4.2	Remaining Open issues

Roughly 85% of items defined in WID Core Part objectives for RAN4 have been accomplished. About 0% of items defined in WID Performance Part objectives for RAN4 have been accomplished. The remaining open issues are:

- Core Part: Specification of RF and RRM requirements [RAN4-led]:

  - Finalize all the details of RF requirements for the IAB-MT including requirements for co-existence.

  - Define relaxation factor in FR2 RLM evaluation period of IAB-MTs.

- Performance Part: Define RRM performance requirements.

- Performance Part: Define demodulation performance requirements for both backhaul and access links of an IAB-node. [RAN4]

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

RAN2 sent an LS to SA2/SA5 to indicate impending work to capture impact of IAB on CN (R2-1905247).

SA3 has started independent SI and WI to assess IAB security.

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #100e-bis

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2001526 | Remaining issues on resource multiplexing among backhaul and access links | Huawei, HiSilicon |
| R1-2001527 | Remaining issues on the "case-1" OTA timing alignment | Huawei, HiSilicon |
| R1-2001658 | Remaining issue on mechanisms for resource multiplexing among backhaul and access links | vivo |
| R1-2001792 | Specifications for T_delta signalling | Nokia, Nokia Shanghai Bell |
| R1-2001862 | Remaining maintenance issues for IAB resource multiplexing | AT&T |
| R1-2001863 | Summary on Mechanisms for resource multiplexing among backhaul and access links | Moderator (AT&T) |
| R1-2001874 | Remaining issues in IAB PRACH | ZTE, Sanechips |
| R1-2001882 | Remaining issues in IAB resource multiplexing | ZTE, Sanechips |
| R1-2001883 | Remaining issues in IAB case-1 timing | ZTE, Sanechips |
| R1-2001893 | FL summary on remaining issues in IAB case-1 timing | Moderator (ZTE) |
| R1-2001952 | Remaining details on IAB resource multiplexing | LG Electronics |
| R1-2001953 | Text proposals on case 1 timing alignment | LG Electronics |
| R1-2002203 | Remaining issues on mechanisms for resource multiplexing among backhaul and access links | CMCC |
| R1-2002436 | Discussion on the RAN2 LS on T_delta in IAB | NTT DOCOMO, INC |
| R1-2002535 | Remaining issues and clarifications on IAB resource management | Qualcomm Incorporated |
| R1-2002536 | Remaining issues on OTA timing | Qualcomm Incorporated |
| R1-2002650 | Definition of IAB half-duplex operation in paired spectrum | Ericsson |
| R1-2002651 | On T_delta Signaling in MAC CE | Ericsson |
| R1-2002652 | On max number of commonSearchSpaceListIAB | Ericsson |
| R1-2002750 | FL summary#2 on remaining issues in IAB case-1 timing | Moderator (ZTE) |
| R1-2002802 | Summary of [100b-e-NR-IAB-02] regarding IAB-MT Resource/Control Channel Configuration | Moderator (AT&T) |
| R1-2002803 | Summary of [100b-e-NR-IAB-03] regarding IAB Operation in Paired Spectrum | Moderator (AT&T) |
| R1-2002898 | Draft reply LS on T_delta in IAB | ZTE |
| R1-2002931 | Reply LS on T_delta in IAB | RAN1, ZTE |
| R1-2002935 | Outcome of [100b-e-NR-IAB-01] regarding TP for section 6.3.3.2 of 38.211 | Moderator (Qualcomm) |
| R1-2002954 | Draft LS on RAN1 agreements related to resource multiplexing in IAB | Ericsson |
| R1-2002991 | TP for 38.213 regarding usage of tdd-UL-DL-ConfigurationDedicated-IAB-MT | Moderator (AT&T) |
| R1-2003043 | LS on RAN1 agreements related to resource multiplexing in IAB | RAN1, Ericsson |

RAN1 #101e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2003376 | Remaining aspects for resource multiplexing among backhaul and access links | vivo |
| R1-2003505 | Remaining issues on resource multiplexing among backhaul and access links | Huawei, HiSilicon |
| R1-2003544 | Remaining issues in IAB resource multiplexing | ZTE, Sanechips |
| R1-2003732 | Discussion on soft resource availability indication | Intel Corporation |
| R1-2003948 | Remaining issues on mechanisms for resource multiplexing among backhaul and access links | CMCC |
| R1-2003974 | Discussion on IAB resource multiplexing | ETRI |
| R1-2004133 | Remainig details on IAB resource multiplexing | LG Electronics |
| R1-2004280 | Remaining maintenance issues for IAB resource multiplexing | AT&T |
| R1-2004281 | Summary of Mechanisms for resource multiplexing among backhaul and access links | Moderator (AT&T) |
| R1-2004448 | Proposed IAB updates to 38.300 | Qualcomm Incorporated |
| R1-2004449 | Remaining issues on IAB resource management | Qualcomm Incorporated |
| R1-2004582 | Mechanisms for resource multiplexing among backhaul and access links | Ericsson |
| R1-2004583 | Mechanism to support the “case-1” OTA timing alignment | Ericsson |
| R1-2004584 | On RAN4 UE features for IAB-MTs | Ericsson |
| R1-2004666 | FL summary #1 on IAB case-1 timing | Moderator (ZTE) |
| R1-2004759 | [Draft] Reply LS on cell specific signal/channel configurations | ZTE |
| R1-2004760 | Reply LS on cell specific signal/channel configurations | RAN1, ZTE |
| R1-2004784 | LS response to RAN2 LS on Guard Symbols in IAB | RAN1, Samsung |
| R1-2004871 | FL summary for [101-e-NR-IAB-05] - Updates to TS38.300 | Moderator (Qualcomm Incorporated) |
| R1-2004872 | LS on IAB updates to 38.300 | RAN1, Qualcomm |
| R1-2005085 | Session notes for 7.2.3 (Maintenance of Integrated Access and Backhaul for NR) | Ad-hoc Chair (Samsung) |

RAN2 #109e-bis

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2002522 | LS on the inter donor DU re-routing and source IP configuration (R3-201418; contact: Huawei)) | RAN3 |
| R2-2002523 | LS on IP address management in IAB network (R3-201420; contact: Samsung) | RAN3 |
| R2-2002600 | Considerations on BAP entity release | KDDI Corporation |
| R2-2002664 | PWS information handling in IAB | Sony |
| R2-2002672 | Report on email discussion Post109e_26 IAB IP address allocation (Samsung) | Samsung Electronics GmbH |
| R2-2002679 | On interpretation and use of the Guard Symbols MAC CE | Samsung Electronics GmbH |
| R2-2002680 | Open issues with IAB LCID space extension | Samsung Electronics GmbH |
| R2-2002690 | Finalising Rel-16 MAC design (IAB-related open issues) | Samsung Electronics GmbH |
| R2-2002691 | CR (IAB MAC - rapporteur corrections and clarifications) | Samsung Electronics GmbH |
| R2-2002715 | Introducing a section for handling of Tdelta MAC CE | Samsung Electronics GmbH |
| R2-2002716 | Summary of IAB User Plane open issues and corrections | Samsung Electronics GmbH |
| R2-2002717 | Optional Rel-15 UE Features for IAB-MTs | AT&T |
| R2-2002727 | IAB workplan update | Qualcomm Incorporated (Rapporteur) |
| R2-2002728 | Notation of IAB terminology | Qualcomm Incorporated (Rapporteur) |
| R2-2002729 | Report email discussion [Post109e#36][IAB] RLF Handling Open Issues | Qualcomm Incorporated |
| R2-2002730 | Optionality of mandatory Rel-15 features for IAB-MT | Qualcomm Incorporated |
| R2-2002814 | Better cell selection for IAB Nodes | Apple |
| R2-2002851 | Further consideration on bearer mapping | ZTE, Sanechips |
| R2-2002852 | Discussion on IAB User plane aspects | ZTE, Sanechips |
| R2-2002853 | Remaining issues for F1-C over LTE | ZTE, Sanechips |
| R2-2002854 | Misellaneous RRC issues for IAB | ZTE, Sanechips |
| R2-2002855 | Discussion on IAB BH RLF handling | ZTE, Sanechips |
| R2-2002856 | Remaining issues of IP address allocation | ZTE, Sanechips |
| R2-2002857 | Rel-15 mandatory UE features for Rel-16 IAB-MT | ZTE, Sanechips |
| R2-2002858 | Discussion on channel bandwidth for Rel-16 IAB-MT | ZTE, Sanechips |
| R2-2002889 | Remaining issues of DL HbH FC | vivo |
| R2-2002890 | Renamed to be: remaining issues of preemtpive BSR | vivo |
| R2-2002891 | IAB-MT Capability of Rel-15 features | vivo |
| R2-2002991 | Discussion on IAB BH RLF handling | NEC |
| R2-2002999 | Determining a cell to apply a Guard Symbols MAC CE | LG Electronics Inc. |
| R2-2003000 | Consideration on LCID set for IAB MAC CE and reserved LCID values | LG Electronics Inc. |
| R2-2003001 | TP for remaining issues on Guard Symbols MAC CE and LCID extension | LG Electronics Inc. |
| R2-2003002 | TP on clarifying a condition and aligning a terminology in BAP specification | LG Electronics Inc. |
| R2-2003011 | Miscellaneous corrections to 38.340 for IAB | Huawei, HiSilicon |
| R2-2003012 | Miscellaneous corrections to 38.304 for IAB | Huawei, HiSilicon |
| R2-2003013 | Miscellaneous corrections to 36.304 for IAB | Huawei, HiSilicon |
| R2-2003014 | Miscellaneous correction to 37.340 for IAB | Huawei, HiSilicon |
| R2-2003015 | The bearer mapping configuration on the backup link in RLF | Huawei, HiSilicon |
| R2-2003016 | Remaining issues of Guard Symbols MAC CE | Huawei, HiSilicon |
| R2-2003017 | Clarification on the RACH configuration used in MAC procedure for IAB | Huawei, HiSilicon |
| R2-2003018 | Clarification of BSR and Pre-emptive BSR | Huawei, HiSilicon |
| R2-2003019 | Discussion on the SR cancelation for pre-BSR and LCID values for IAB | Huawei, HiSilicon |
| R2-2003020 | ASN.1 issues related to L1 parameters for IAB | Huawei, HiSilicon |
| R2-2003021 | Draft LS to RAN1 on IAB L1 parameters | Huawei, HiSilicon |
| R2-2003022 | Capturing IAB capability | Huawei, HiSilicon |
| R2-2003048 | Cell information in Guard symbols MAC CE | Nokia, Nokia Shanghai Bell |
| R2-2003049 | Handling of IAB specific MAC CEs | Nokia, Nokia Shanghai Bell |
| R2-2003098 | PDCP duplication in IAB DC | Lenovo, Motorola Mobility |
| R2-2003099 | Fast MCG link recovery for IAB DC case | Lenovo, Motorola Mobility |
| R2-2003176 | Corrections to BAP | Nokia, Nokia Shanghai Bell |
| R2-2003177 | IAB-MT features list and capabilities | Nokia, Nokia Shanghai Bell |
| R2-2003178 | F1AP over LTE leg signalling correction to 37.340 | Nokia, Nokia Shanghai Bell |
| R2-2003179 | Cell re-selection handling for IAB-MT | Nokia, Nokia Shanghai Bell |
| R2-2003180 | IP address request in NSA and SA | Nokia, Nokia Shanghai Bell |
| R2-2003236 | Remaining details for Backhaul RLF Handling | Futurewei |
| R2-2003297 | Report on email discussion [Post109e][035][IAB] RRC open issues | Ericsson |
| R2-2003298 | Correction for TS 38.331 Related to IAB WI | Ericsson |
| R2-2003299 | Correction for TS 36.331 Related to IAB WI | Ericsson |
| R2-2003300 | On Multi-connectivity for IAB | Ericsson |
| R2-2003301 | Allowing an IAB configuration without DRB | Ericsson |
| R2-2003302 | Further details on Backhaul link RLF Notification Types to Downstream Node(s) | Ericsson |
| R2-2003303 | IP Address Assignment for IAB Node(s) | Ericsson |
| R2-2003304 | Backhaul RLC Channel Remapping for IAB node(s) | Ericsson |
| R2-2003314 | Possible issues on Backhaul RLF handling | Kyocera |
| R2-2003323 | Mandatory/optional features for IAB-MT | Intel Corporation |
| R2-2003346 | IAB support in NPN deployment | Kyocera |
| R2-2003359 | Change LCID to eLCID for IAB MAC CEs | Ericsson |
| R2-2003360 | Rel-15 capabilities to be supported by IAB-MT | Ericsson |
| R2-2003361 | Capability signalling for IAB | Ericsson |
| R2-2003438 | On Release of BAP Entities | CATT |
| R2-2003439 | Views on IAB MT Capability | CATT |
| R2-2003525 | IP address allocation for IAB-nodes | Futurewei |
| R2-2003561 | Summary of 6.1.3 for BAP open issues and corrections | Huawei, HiSilicon |
| R2-2003595 | On the need of a new cause in SCG Failure Information | LG Electronics France |
| R2-2003596 | No need to support RRC_INACTIVE for IAB-MT | LG Electronics France |
| R2-2003597 | Capabilities of IAB MTs | LG Electronics France |
| R2-2003598 | On RRC connection without DRB | LG Electronics France |
| R2-2003644 | Issue of SR triggered by Pre-emptive BSR | ASUSTeK |
| R2-2003720 | Remaining issues for IAB MAC | Futurewei |
| R2-2003725 | IP address assignment for IAB node DU on failure handling | Samsung R&D Institute UK |
| R2-2003726 | SCGFailureInformation procedure in IAB RLF handling | Samsung R&D Institute UK |
| R2-2003727 | Considering the optionality of Rel-15 UE features for IAB-MT use | Samsung R&D Institute UK |
| R2-2003728 | [S020],[S021] SMTC occasion calculation for smtc3 for IAB-MT | Samsung R&D Institute UK |
| R2-2003742 | [S020] Conditional presence on smtc3 in IAB | Samsung R&D Institute UK |
| R2-2003743 | [S021] Clarification on smtc3 operation in IAB | Samsung R&D Institute UK |
| R2-2003775 | Report email discussion [Post109e#36][IAB] RLF Handling Open Issues | Qualcomm Incorporated |
| R2-2003794 | Summary of 6.1.8 UE capabilities for IAB-MT | Nokia, Nokia Shanghai Bell |
| R2-2003813 | Report email discussion [Post109e#36][IAB] RLF Handling Open Issues | Qualcomm Incorporated |
| R2-2003826 | Summary of IAB User Plane open issues and corrections | Samsung Electronics GmbH |
| R2-2003829 | Summary of IAB User Plane open issues and corrections | Samsung Electronics GmbH |
| R2-2003830 | IAB MAC - rapporteur corrections and clarifications | Samsung |
| R2-2003831 | Introducing a section for handling of Tdelta MAC CE | Samsung Electronics GmbH |
| R2-2003868 | LS on UAC applicability to IABs | RAN2 |
| R2-2003941 | LS on UAC applicability to IABs | RAN2 |
| R2-2004125 | Correction for TS 38.331 Related to IAB WI | Ericsson |
| R2-2004126 | IAB MAC - rapporteur corrections and clarifications | Samsung |
| R2-2004127 | LS to RAN1 on Guard Symbols in IAB | RAN2 |
| R2-2004128 | Remaining issues from IAB User Plane | Samsung |
| R2-2004132 | [AT 109bis-e][018][IAB] IAB terminology and other issues | Qualcomm (Rapporteur) |
| R2-2004133 | CR to 38.300 on Integrated Access and Backhaul for NR | Qualcomm (Rapporteur) |
| R2-2004142 | Way forward on IP address allocation for IAB nodes | Samsung |
| R2-2004151 | Miscellaneous correction to 37.340 for IAB | Huawei, HiSilicon |
| R2-2004152 | Summary of email discussion [AT109bis-e][019][IAB] BAP | Huawei |
| R2-2004153 | Miscellaneous corrections to 38.340 for IAB | Huawei, HiSilicon |
| R2-2004154 | Summary of [AT109bis-e][024] 3X.304 CRs and IAB supporting in NPN | Huawei |
| R2-2004155 | Miscellaneous corrections to 38.304 for IAB | Huawei, HiSilicon |
| R2-2004156 | Miscellaneous corrections to 36.304 for IAB | Huawei, HiSilicon |
| R2-2004169 | Reply LS on T_delta in IAB (R1-2002931; contact: ZTE) | RAN1 |
| R2-2004225 | Miscellaneous corrections to 38.304 for IAB | Huawei, HiSilicon |
| R2-2004226 | Miscellaneous corrections to 36.304 for IAB | Huawei, HiSilicon |
| R2-2004227 | Corrections to 38.331 for supporting IAB in NPN | Huawei, HiSilicon |
| R2-2004228 | Corrections to 38.304 for supporting IAB in NPN | Huawei, HiSilicon |
| R2-2004229 | Draft LS on IAB supporting in NPN deployment | Huawei, HiSilicon |
| R2-2004243 | Correction for TS 38.331 for IAB WI | Ericsson |
| R2-2004280 | Corrections to 38.331 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2004281 | Corrections to 38.304 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2004282 | LS on IAB supporting in NPN deployment | RAN2 |
| R2-2004287 | Correction for TS 38.331 for IAB WI | Ericsson |

RAN2 #110e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2004338 | LS on UL F1-C traffic mapping for intra-CU migration scenario (R3-202851; contact: ZTE) | RAN3 |
| R2-2004353 | LS on RAN1 agreements related to resource multiplexing in IAB (R1-2003043; contact: Ericsson) | RAN1 |
| R2-2004361 | RRC Message Design for IAB IP Address Allocation (R3-202926; contact: Ericsson) | RAN3 |
| R2-2004373 | LS on RAN4 IAB-MT feature list agreement (R4-2005608; contact: Qualcomm) | RAN4 |
| R2-2004494 | On Guard Symbol MAC CE Design | vivo |
| R2-2004495 | Remaining Issues of SR Cancellation for Preemptive BSR | vivo |
| R2-2004496 | RRC signaling for IP request and indication | vivo |
| R2-2004497 | Discussion for IAB-MT Capabilities | vivo |
| R2-2004498 | TP for indicator of IAB-MT class | vivo |
| R2-2004593 | Some left over issues on IAB BH RLF handling | NEC Corporation |
| R2-2004607 | Report on email discussion [Post109bis-e][920][IAB] RRC 2 | Ericsson |
| R2-2004608 | Remaining Issues for IP Address Allocation in IAB Network | Ericsson |
| R2-2004609 | On the Issue of INACTIVE mode for IAB-MT | Ericsson |
| R2-2004610 | Further Discussion on F1-AP Transport in EN-DC | Ericsson |
| R2-2004611 | Allowing an IAB Configuration Without DRB | Ericsson |
| R2-2004612 | RIL E264 About Backhaul RLC Channel IDs | Ericsson |
| R2-2004684 | Summary of e-mail discussion: [Post109bis-e][925][IAB] UE Cap (Nokia) | Nokia, Nokia Shanghai Bell |
| R2-2004685 | Access category and establishment cause for IAB-MT [RIL: H697] | Nokia, Nokia Shanghai Bell |
| R2-2004686 | Draft LS to CT1 on IAB-MT establishment cause | Nokia, Nokia Shanghai Bell |
| R2-2004687 | RRC signalling for F1-C-over-LTE/X2 path configuration | Nokia, Nokia Shanghai Bell |
| R2-2004688 | Stage-2 correction on RLF handling by IAB-MT | Nokia, Nokia Shanghai Bell |
| R2-2004731 | Capability signalling and mandatory/optional features for IAB MT | Intel Corporation, KDDI, AT&T |
| R2-2004748 | IAB - establishment of F1-C over LTE | Qualcomm Incorporated |
| R2-2004749 | Implications of RAN3 agreements on RRC configurations for IP- and BAP-layer | Qualcomm Incorporated |
| R2-2004750 | TP for 38331 on IP address signaling | Qualcomm Incorporated |
| R2-2004780 | Better cell selection for IAB Nodes | Apple |
| R2-2004782 | Conditional handover upon BH RLF in IAB | Kyocera |
| R2-2004783 | Remaining issue on idle mode procedure for IAB-MT | Kyocera |
| R2-2004784 | Corrections to 38.331 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2004785 | Corrections to 38.304 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2004801 | Discussion on default UL mapping configuration during migration | ZTE, Sanechips |
| R2-2004802 | Further discussion on F1-C over LTE path | ZTE, Sanechips |
| R2-2004803 | Remaining issues of IP address allocation | ZTE, Sanechips |
| R2-2004804 | Considerations on IAB-MT features | ZTE, Sanechips |
| R2-2004805 | Discussion on channel bandwidth for Rel-16 IAB-MT | ZTE, Sanechips |
| R2-2004874 | [C501] Corrections to RRC Resume for IAB | CATT |
| R2-2004875 | Proposals on IAB MT Capabilities | CATT |
| R2-2004876 | Remaining issues of IAB in NPN | CATT |
| R2-2004946 | Finalising Rel-16 MAC design (IAB-related open issues) | Samsung Electronics GmbH |
| R2-2004948 | IAB MAC - rapporteur corrections and clarifications | Samsung |
| R2-2004966 | Summary of IAB User Plane open issues and corrections | Samsung Electronics GmbH |
| R2-2004977 | Further discussion on Rel-15 IAB-MT capabilities | Ericsson |
| R2-2004978 | Introduction of IAB capabilities | Ericsson |
| R2-2004979 | Introduction of IAB capabilities | Ericsson |
| R2-2004999 | TP for configuration on F1AP transport in EN-DC | vivo |
| R2-2005142 | PWS information handling in IAB | Sony |
| R2-2005157 | IP address configuration for IAB | Samsung |
| R2-2005226 | Support of Rel-15 UE features by IAB-MTs | AT&T |
| R2-2005406 | [C502] Corrections to IAB behavior in Determining the NPN-only Cell | CATT |
| R2-2005516 | Miscellaneous corrections to 38.304 for IAB | Huawei, HiSilicon |
| R2-2005517 | Miscellaneous corrections to 36.304 for IAB | Huawei, HiSilicon |
| R2-2005518 | Miscellaneous correction to 37.340 for IAB | Huawei, HiSilicon |
| R2-2005519 | Discussion on inapplicable features for IAB | Huawei, HiSilicon |
| R2-2005520 | Discussion on the R16 other WI features supporting for IAB-MT | Huawei, HiSilicon |
| R2-2005521 | Adding AI-RNTI and IAB specific RACH parameters in MAC specification | Huawei, HiSilicon |
| R2-2005522 | Differentiation between Pre-emptive BSR MAC CE and BSR MAC CE | Huawei, HiSilicon |
| R2-2005523 | Correction on RLC spec to support the BAP as upper layer | Huawei, HiSilicon |
| R2-2005524 | TP for IP addresss signaling of IAB [ToDo RIL H698] | Huawei, HiSilicon |
| R2-2005525 | Clarification on the cause value and not supporting UAC for IAB [ToDo RIL H697] | Huawei, HiSilicon |
| R2-2005526 | Default BAP configuration for non-bootstrapping cases [ToDo RIL H691] | Huawei, HiSilicon |
| R2-2005527 | Suspending BAP operation at IAB-MT during RRC re-establishment, RRC inactive state [ToDo RIL H690] | Huawei, HiSilicon |
| R2-2005528 | Correction on the TDD-UL-DL-ConfigDedicated-IAB-MT [ToDo RIL H696] | Huawei, HiSilicon |
| R2-2005563 | Remaining issue of SR triggered by Pre-emptive BSR | ASUSTeK |
| R2-2005584 | Miscellaneous corrections to 38.340 for IAB | Huawei, HiSilicon |
| R2-2005585 | Summary of email discussion [Post109bis-e][019][IAB] BAP | Huawei |
| R2-2005625 | Title: Draft CR to TS 38.331 on IP address request and configuration to IAB nodes | Futurewei |
| R2-2005653 | Clarification of access control bypasssing | LG Electronics France |
| R2-2005654 | Capabilities of IAB MTs | LG Electronics France |
| R2-2005655 | RRC message for IP allocation | LG Electronics France |
| R2-2005656 | Support of RRC_INACTIVE | LG Electronics France |
| R2-2005657 | Options for IAB node address allocation using RRC | Futurewei |
| R2-2005664 | Consideration on reserved value in two byte eLCID space | LG Electronics Inc. |
| R2-2005665 | Clarification on BAP routing ID determination | LG Electronics Inc. |
| R2-2005666 | Consideration on default configuration during IAB migration | LG Electronics Inc. |
| R2-2005667 | TP on default and BAP routing ID determination | LG Electronics Inc. |
| R2-2005669 | [S001] Adding procedural text for smtc3 occasion derivation for IAB | Samsung R&D Institute UK |
| R2-2005670 | [S002] Change of need code regarding smtc3list for IAB | Samsung R&D Institute UK |
| R2-2005671 | [S003] Failure Type extension in SCGFailureInformation msg for IAB | Samsung R&D Institute UK |
| R2-2005672 | Applicability of Conditional Handover in IAB | Samsung R&D Institute UK |
| R2-2005673 | Minor text correction for CR 38300 for IAB | Samsung R&D Institute UK |
| R2-2005992 | Reply LS on UAC applicability to IABs (S1-202274; contact: Nokia) | SA1 |
| R2-2006032 | Capability signalling and mandatory/optional features for IAB MT | Intel Corporation, KDDI, AT&T, Kyocera, Apple |
| R2-2006049 | Summary of Phase 1 of e-mail discussion: [AT110e][048][IAB] UE capabilities (Nokia) | Nokia, Nokia Shanghai Bell |
| R2-2006051 | Summary of IAB User Plane open issues and corrections | Samsung Electronics GmbH |
| R2-2006086 | LS response to RAN2 LS on Guard Symbols in IAB (R1-2004784; contact: Samsung) | RAN1 |
| R2-2006087 | LS on IAB updates to 38.300 (R1-2004872; contact: Qualcomm) | RAN1 |
| R2-2006094 | Reply LS on RAN4 IAB-MT feature list agreement (R1-2004954; contact: Qualcomm) | RAN1 |
| R2-2006128 | Reply LS on IAB-MT Features (R4-2009051; contact: Qualcomm) | RAN4 |
| R2-2006141 | Summary of IAB particular issues I Misc | ZTE Corporation |
| R2-2006145 | Summary of offline discussion [AT110e][049][IAB] Other | Huawei |
| R2-2006146 | Miscellaneous correction to 37.340 for IAB | Huawei, HiSilicon |
| R2-2006147 | Corrections to 38.331 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2006148 | Corrections to 38.304 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2006149 | Miscellaneous corrections to 38.304 for IAB | Huawei, HiSilicon |
| R2-2006150 | Miscellaneous corrections to 36.304 for IAB | Huawei, HiSilicon |
| R2-2006151 | Correction on RLC spec to support the BAP as upper layer | Huawei, HiSilicon |
| R2-2006155 | Way forward on IP Address handling in IAB | Samsung |
| R2-2006157 | TP for F1-C transfer path configuration | ZTE Corporation |
| R2-2006161 | E-mail discusson: [AT110-e][047][IAB] Particular issues III UAC | LG Electronics |
| R2-2006165 | Reply LS on IAB supporting in NPN deployment (S2-2004469; contact: Qualcomm) | SA2 |
| R2-2006210 | [AT110e][041][IAB] 3800/36300 – Open issues | Qualcomm Incorporated (Rapporteur) |
| R2-2006211 | Draft LS on establishment cause of IAB MT access | LGE |
| R2-2006226 | Summary for [AT110-e][042][IAB] BAP | Huawei, HiSilicon |
| R2-2006227 | Miscellaneous corrections to 38.340 for IAB | Huawei, HiSilicon |
| R2-2006258 | Miscellaneous corrections to 38.304 for IAB | Huawei, HiSilicon |
| R2-2006265 | IAB MAC - rapporteur corrections and clarifications | Samsung |
| R2-2006303 | CR to 38.300 on Integrated Access and Backhaul for NR | Qualcomm (Rapporteur) |
| R2-2006319 | Summary of IAB particular issues I Misc | ZTE Corporation |
| R2-2006320 | Corrections to 38.331 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2006321 | Corrections to 38.304 for supporting IAB in NPN | Huawei, HiSilicon, Kyocera |
| R2-2006351 | TP for TS 38.331 on IP address allocation for IAB nodes | Samsung |

RAN3 #107e-bis

| TDoc | Title | Source |
| --- | --- | --- |
| R3-201529 | BL CR to 38.423: Support for IAB | Samsung |
| R3-201530 | BL CR to 38.463: Support for IAB | Huawei |
| R3-201531 | BL CR to 38.425: Support for IAB | Samsung |
| R3-201532 | BL CR to 36.413: Support for IAB | Huawei |
| R3-201533 | draftCR TS 38.300 Mapping of Uplink Traffic to Backhaul RLC Channels | Ericsson |
| R3-201534 | BL CR to 38.413: Support for IAB | Nokia, Nokia Shanghai Bell |
| R3-201559 | Support for IAB | Nokia, Nokia Shanghai Bell |
| R3-201596 | BL CR to 36.423: Support for IAB | Samsung |
| R3-201597 | BL CR to 38.401 Support for IAB | Huawei |
| R3-201598 | BL CR to 38.470: Support for IAB | Ericsson |
| R3-201621 | BL CR to 38.473: Support for IAB | Ericsson |
| R3-201696 | Considerations on BAP entity release | KDDI Corporation |
| R3-201777 | IAB workplan update | Qualcomm Incorporated (Rapporteur) |
| R3-201778 | Notation of IAB terminology | Qualcomm Incorporated (Rapporteur) |
| R3-201779 | (TP for NR_IAB BL CR TS 38.401) Remaining issues on topology discovery for IAB | Qualcomm Incorporated |
| R3-201780 | DL mapping at IAB-donor-DU | Qualcomm Incorporated |
| R3-201781 | (TP to NR_IAB BL CR 38473) IAB bearer-mapping configuration | Qualcomm Incorporated |
| R3-201782 | IAB - Establishment of F1-C over LTE | Qualcomm Incorporated |
| R3-202017 | (TP for NR_IAB BL CR for TS 38.473): Left configuration issues for donor-DU F1 setup | ZTE, Sanechips |
| R3-202018 | (TP for NR_IAB BL CR for TS 38.473): Routing Selection Configuration at donor DU | ZTE, Sanechips |
| R3-202019 | (TP for NR_IAB BL CR for TS 38.473): Bearer Mapping Configuration | ZTE, Sanechips |
| R3-202020 | Discussion on topology discovery | ZTE, Sanechips |
| R3-202022 | Remaining issues on bearer mapping and routing configuration | ZTE, Sanechips |
| R3-202025 | (TP for NR_IAB BL CR for TS 38.473): IP address allocation | ZTE, Sanechips |
| R3-202027 | Discussion on the left issues for IP address allocation | ZTE, Sanechips |
| R3-202030 | (TP for NR_IAB BL CR for TS 38.473): PHY Layer parameters configuration | ZTE, Sanechips |
| R3-202032 | (TP for NR_IAB BL CR for TS 36.423): Support of F1-C signalling over LTE leg | ZTE, Sanechips |
| R3-202034 | (TP for NR_IAB BL CR for TS 38.401): Intra IAB donor-CU Topology Adaptation Procedure | ZTE,Sanechips |
| R3-202059 | Discussion on left issues on intra-CU migration | ZTE, Sanechips |
| R3-202060 | (TP for NR-IAB BL CR for 38.401) Stage 2 clean-up for IAB integration procedure | Samsung |
| R3-202061 | (TP for NR-IAB BL CR for 38.401) Discussions on topology discovery | Samsung |
| R3-202062 | BL CR to 36.420: Support for IAB | Samsung |
| R3-202063 | (TP for NR-IAB BL CR for 38.473) Mapping configuration for intra-donor and intermediate IAB node | Samsung |
| R3-202064 | (TP for NR-IAB BL CR for 38.473) Remaining issues except bearer mapping configuration | Samsung |
| R3-202065 | (TP for NR-IAB BL CR for 38.473) Remaining issues on IP address management | Samsung |
| R3-202066 | (TP for NR-IAB BL CR for 38.401) Remaining issues on IP address management | Samsung |
| R3-202067 | [Draft] LS on RRC signalling design impact of IAB | Samsung |
| R3-202068 | (TP for NR-IAB BL CR for 36.423) Further discussion on F1AP over X2 | Samsung |
| R3-202081 | (TP for NR_IAB BL CR for TS 38.401): Remaining issue of the IAB topology discovery | Huawei |
| R3-202082 | Draft LS to RAN2 on the IAB topology discovery | Huawei |
| R3-202083 | (TP for NR_IAB BL CR for TS38.473): BAP configuration for IAB donor DU | Huawei |
| R3-202084 | (TP for NR_IAB BL CR for TS38.473): BH mapping configuration for intermediate IAB nodes | Huawei |
| R3-202085 | (TP for NR_IAB BL CR for TS38.473): BH mapping configuration for control PDU | Huawei |
| R3-202086 | (TP for NR-IAB BL CR for TS 38.473): Remaining issues for BH RLC channel management | Huawei |
| R3-202087 | (TP for NR-IAB BL CR for TS 38.473) :IP address management for IAB node | Huawei |
| R3-202088 | (TP for NR-IAB BL CR for TS 38.473) : Cell-specific signals/channels configuration of child IAB-DU | Huawei |
| R3-202089 | (TP for NR-IAB BL CR for TS 38.401): Miscellaneous change for stage 2 cleanup | Huawei |
| R3-202090 | Remaining issues for F1AP over LTE | Huawei |
| R3-202091 | (TP for NR-IAB BL CR for TS 38.463): IP address update when IAB topology change | Huawei |
| R3-202092 | (TP for NR-IAB BL CR for TS 38.473):IP address update when IAB topology change | Huawei |
| R3-202093 | (TP for NR-IAB BL CR for TS 38.401) : IAB-node migration in EN-DC | Huawei |
| R3-202246 | (TP for NR-IAB BL CR for TS 38.473) IAB-node BAP Address reporting in F1 Setup | Google Inc. |
| R3-202306 | (TP for NR-IAB BL CR for TS 38.401): Cleanups | Ericsson |
| R3-202307 | (TP for NR-IAB BL CR for TS 38.473): IAB Topology Discovery | Ericsson |
| R3-202308 | (TP for NR-IAB BL CR for TS 38.473): Backhaul RLC Channel Mapping Configuration for IAB-nodes | Ericsson |
| R3-202309 | (TP for NR-IAB BL CR for TS 38.473): Cleanups | Ericsson |
| R3-202310 | (TP for NR-IAB BL CR for TS 38.473): IP Address Allocation for IAB-nodes | Ericsson |
| R3-202311 | [Draft] LS on RRC Message Design for IAB IP Address Allocation | Ericsson |
| R3-202430 | (TP for NR-IAB BL CR for TS 38.401) Misc corrections for TS38.401 | Nokia, Nokia Shanghai Bell |
| R3-202431 | (TP for NR-IAB BL CR for TS 38.473) Configure Donor-DU BAP Address in the Donor-DU | Nokia, Nokia Shanghai Bell |
| R3-202432 | (TP for NR-IAB BL CR for TS 38.473) BH RLC channel mapping configuration in Donor-DU and intermediate IAB nodes | Nokia, Nokia Shanghai Bell |
| R3-202433 | Discussion on remaining issues for IP address management | Nokia, Nokia Shanghai Bell |
| R3-202434 | (TP for NR-IAB BL CR for TS 36.423) Fix the remaining issues for F1-C Traffic transfer over X2 interface | Nokia, Nokia Shanghai Bell |
| R3-202442 | Discussion on F1 removal and BAP entity release | CATT |
| R3-202460 | Response to R3-202065 | Nokia, Nokia Shanghai Bell |
| R3-202472 | CB: # 0_Email_IAB_BLs | Qualcomm - Moderator |
| R3-202473 | CB: # 1_Email_IAB_TopolDisc | Ericsson - Moderator |
| R3-202474 | CB: # 2_Email_IAB_st2_cleanups | Samsung - Moderator |
| R3-202475 | CB: # 3_Email_IAB_X2_st2 | Samsung - Moderator |
| R3-202476 | CB: # 4_Email_IAB_BAPrelease | CATT - Moderator |
| R3-202477 | CB: # 5_Email_IAB_BAPconfig_of_donor_DU | Nokia - Moderator |
| R3-202478 | CB: # 6_Email_IAB_bearer_mapping | Nokia - Moderator |
| R3-202479 | CB: # 7_Email_IAB_F1AP_cleanups | Huawei - Moderator |
| R3-202480 | CB: # 8_Email_IAB_IPaddr_mgmt | Ericsson - Moderator |
| R3-202481 | CB: # 9_Email_IAB_PHY_param | ZTE - Moderator |
| R3-202482 | CB: # 10_Email_IAB_DC_operation | Nokia - Moderator |
| R3-202483 | CB: # 11_Email_IAB_node_migration | ZTE - Moderator |
| R3-202642 | (TP for NR-IAB BL CR for 38.401) Stage 2 clean-up for IAB integration procedure | Samsung |
| R3-202643 | BL CR to 36.420: Support for IAB | Samsung |
| R3-202647 | BL CR to 38.425: Support for IAB | Samsung |
| R3-202648 | (TP for NR_IAB BL CR for TS 38.473): F1AP Cleanups | Huawei |
| R3-202664 | (TP for NR-IAB BL CR for 38.401) Stage 2 clean-up for IAB integration procedure | Samsung |
| R3-202682 | CB: # 7_Email_IAB_F1AP_cleanups | Huawei - Moderator |
| R3-202719 | (TP for NR-IAB BL CR for TS 38.401): IAB IP Address allocation | Ericsson |
| R3-202720 | (TP for NR-IAB BL CR for TS 38.473): IP Address Allocation for IAB-nodes | Ericsson |
| R3-202721 | [Draft] LS on RRC Message Design for IAB IP Address Allocation | Ericsson |
| R3-202724 | CB: # 0_Email_IAB_BLs | Qualcomm - Moderator |
| R3-202747 | (TP for NR-IAB BL CR for TS 38.473) Configure Donor-DU BAP Address in the Donor-DU | Nokia, Nokia Shanghai Bell |
| R3-202750 | (TP for NR_IAB BL CR for TS 38.473): PHY Layer parameters configuration | ZTE, Sanechips |
| R3-202751 | (TP for NR_IAB BL CR for TS 38.401): Intra IAB donor-CU Topology Adaptation Procedure | ZTE,Sanechips |
| R3-202752 | (Draft) LS on UL F1-C traffic mapping for intra-CU migration scenario | ZTE |
| R3-202757 | (TP for NR-IAB BL CR for TS 38.470): Correction | Ericsson |
| R3-202758 | (TP for NR-IAB BL CR for TS 38.300): Correction | Ericsson |
| R3-202759 | (TP for NR-IAB BL CR for TS 38.473): IAB Topology Discovery | Ericsson |
| R3-202788 | (TP for NR-IAB BL CR for TS 38.473) BH RLC channel mapping configuration in Donor-DU and intermediate IAB nodes | Nokia, Nokia Shanghai Bell |
| R3-202789 | (TP for NR-IAB BL CR for TS 36.423) TP to clarify the F1-C traffic over LTE leg | Nokia, Nokia Shanghai Bell |
| R3-202836 | (TP for NR-IAB BL CR for 38.401) Stage 2 clean-up for IAB integration procedure | Samsung |
| R3-202841 | (TP for NR-IAB BL CR for TS 38.401) : IAB-node migration in EN-DC | Huawei |
| R3-202842 | (TP for NR-IAB BL CR for TS 38.401): IAB IP Address allocation | Ericsson |
| R3-202843 | (TP for NR-IAB BL CR for TS 38.473): IP Address Allocation for IAB-nodes | Ericsson |
| R3-202844 | [Draft] LS on RRC Message Design for IAB IP Address Allocation | Ericsson |
| R3-202850 | (TP for NR_IAB BL CR for TS 38.473): PHY Layer parameters configuration | ZTE, Sanechips |
| R3-202851 | LS on UL F1-C traffic mapping for intra-CU migration scenario | ZTE |
| R3-202852 | LS on cell-specific signals/channel configurations | ZTE |
| R3-202857 | (TP for NR-IAB BL CR for TS 38.473): IP Address Allocation for IAB-nodes | Ericsson |
| R3-202858 | RRC Message Design for IAB IP Address Allocation | Ericsson |
| R3-202859 | LS on cell-specific signals/channel configurations | ZTE |
| R3-202860 | (TP for NR-IAB BL CR for TS 38.401) : IAB-node migration in EN-DC | Huawei |
| R3-202895 | BL CR to 36.423: Support for IAB | Samsung |
| R3-202896 | BL CR to 38.401 Support for IAB | Huawei |
| R3-202897 | BL CR to 38.470: Support for IAB | Ericsson |
| R3-202898 | BL CR to 38.473: Support for IAB | Ericsson |
| R3-202926 | RRC Message Design for IAB IP Address Allocation | Ericsson |
| R3-202929 | draftCR TS 38.300 Mapping of Uplink Traffic to Backhaul RLC Channels | Ericsson |

RAN3 #108-e

| TDoc | Title | Source |
| --- | --- | --- |
| R3-203002 | BL CR to 38.423: Support for IAB | Samsung |
| R3-203003 | BL CR to 38.463: Support for IAB | Huawei |
| R3-203004 | BL CR to 36.413: Support for IAB | Huawei |
| R3-203005 | BL CR to 38.413: Support for IAB | Nokia, Nokia Shanghai Bell |
| R3-203018 | Support for IAB | Nokia, Nokia Shanghai Bell |
| R3-203036 | BL CR to 36.420: Support for IAB | Samsung |
| R3-203037 | BL CR to 38.425: Support for IAB | Samsung |
| R3-203066 | BL CR to 36.423: Support for IAB | Samsung |
| R3-203067 | BL CR to 38.401 Support for IAB | Huawei |
| R3-203068 | BL CR to 38.470: Support for IAB | Ericsson |
| R3-203069 | BL CR to 38.473: Support for IAB | Ericsson |
| R3-203099 | draftCR TS 38.300 Mapping of Uplink Traffic to Backhaul RLC Channels | Ericsson |
| R3-203150 | Considerations on IAB-DU features | ZTE, Sanechips |
| R3-203151 | (TP for NR_IAB BL CR for TS 38.473): Mapping configuration in donor DU and intermediate node | ZTE, Sanechips |
| R3-203152 | Discussion on mapping configuration in donor DU and intermediate node | ZTE, Sanechips |
| R3-203153 | (TP for NR_IAB BL CR for TS 38.473): PHY Layer parameters configuration | ZTE, Sanechips |
| R3-203154 | Discussion on IP address assignment for F1-C over LTE leg | ZTE, Sanechips |
| R3-203155 | (TP for NR_IAB BL CR for TS 38.401): Intra-CU migration | ZTE, Sanechips |
| R3-203156 | (TP for NR_IAB BL CR for TS 38.473): UP TNL Information update in intra-CU migration scenario | ZTE, Sanechips |
| R3-203343 | (TP for NR_IAB BL CR TS 38473) IAB DL traffic mapping and BH RLC channel mapping configuration | Qualcomm Incorporated |
| R3-203344 | (TP for NR_IAB BL CR TS 38300) IAB stage-2 clean up | Qualcomm Incorporated |
| R3-203345 | IP address allocation procedure for F1-C over LTE/X2 in IAB | Qualcomm Incorporated |
| R3-203346 | (TP for NR_IAB BL CR TS 38.401) IAB topology adaptation and redundancy clean up | Qualcomm Incorporated |
| R3-203525 | (TP for NR_IAB BL CR for TS38.463) DL UP TNL update | CATT |
| R3-203612 | (TP for NR-IAB BL CR for 38.473) Signaling design for bearer mapping configuration | Samsung |
| R3-203613 | (TP for NR-IAB BL CR for 38.473) Remaining issues for F1AP | Samsung |
| R3-203614 | (TP for NR-IAB BL CR for 38.401) Remaining issues on IP address management | Samsung |
| R3-203615 | (TP for NR-IAB BL CR for 38.473) Remaining issues on IP address management | Samsung |
| R3-203616 | (TP for NR-IAB BL CR for 38.401) Discussion on IP address allocation for LTE leg | Samsung |
| R3-203617 | (TP for NR-IAB BL CR for 38.473) Discussion on the UP information update during migration | Samsung |
| R3-203618 | (TP for NR-IAB BL CR for 38.463) Discussion on the UP information update during migration | Samsung |
| R3-203702 | IAB IP address management in NSA | Nokia, Nokia Shanghai Bell |
| R3-203703 | Discussion on F1-C over LTE leg or NR leg | Nokia, Nokia Shanghai Bell |
| R3-203704 | (TP for NR-IAB BL CR for TS 38.473) BH RLC channel mapping configuration in Donor-DU and intermediate IAB nodes | Nokia, Nokia Shanghai Bell, Huawei |
| R3-203762 | Discussion on IAB-DU Layer-1 Features | AT&T |
| R3-203778 | (TP for NR-IAB BL CR for 38.401) Correction on intra-CU topological redundancy procedure | Intel Corporation |
| R3-203813 | (TP for NR-IAB BL CR for TS 36.413): Correction to BL CR | Ericsson |
| R3-203814 | (TP for NR-IAB BL CR for TS 38.413): Correction to BL CR | Ericsson |
| R3-203815 | (TP for NR-IAB BL CR for TS 36.423): Correction to BL CR | Ericsson |
| R3-203816 | (TP for NR-IAB BL CR for TS 38.423): Correction to BL CR | Ericsson |
| R3-203817 | CR 38.472 Handling of SCTP Transport for IAB-MTs in INACTIVE State | Ericsson |
| R3-203818 | (TP for NR-IAB BL CR for TS 38.473): Correction of BH RLC CH ID | Ericsson |
| R3-203837 | IAB support in NPN | Huawei |
| R3-203838 | Draft Reply LS on IAB supporting in NPN deployment | Huawei |
| R3-203839 | (TP for NR-IAB BL CR for TS 38.401) : Miscellaneous stage 2 change for IAB BL CR to 38.401 | Huawei |
| R3-203840 | (TP for NR-IAB BL CR for TS 38.473):Remianing issue for IAB resource configuration | Huawei |
| R3-203841 | Remaining issue of F1AP over LTE | Huawei |
| R3-203842 | Discussion on IAB inactive | Huawei |
| R3-203843 | (TP for NR-IAB BL CR for TS 38.473): Miscellaneous correction for F1AP | Huawei |
| R3-203844 | (TP for NR_IAB BL CR for TS38.463) TNL information update when IAB topology update | Huawei, Nokia, Nokia Shanghai Bell |
| R3-203845 | (TP for NR-IAB BL CR for TS 38.473):BAP configuration and TNL information update when IAB topology change | Huawei |
| R3-203916 | (TP for NR-IAB BL CR for TS 38.473) BH RLC channel mapping configuration in Donor-DU and intermediate IAB nodes | Nokia, Nokia Shanghai Bell, Huawei |
| R3-203923 | (TP for NR-IAB BL CR for 38.473) Discussion on IAB barred Cell | Samsung Electronics Nordic AB |
| R3-203946 | Response to: Response to R3-203525, R3-203844-5, R3-203617-8, R3-203155-6, R3-203916, R3-203343, R3-203612, R3-203151-2 | Ericsson Japan K.K. |

RAN4 #94bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2003079 | Update on section 4 and section 5 based on latest agreement | Samsung |
| R4-2004801 | Email disccusion for updating IAB TS spec to capture RAN4 94b agreements | Qualcomm |
| R4-2004802 | Updated IAB TS spec based on RAN4 94 agreements | Qualcomm |
| R4-2003020 | Discussion on IAB class | CMCC |
| R4-2003313 | Discussion of IAB-MT class and Tx power | CATT |
| R4-2003774 | IAB-Node classification and related output power requirements | Nokia, Nokia Shanghai Bell |
| R4-2004148 | Discussion on IAB MT class | ZTE Corporation |
| R4-2004150 | Further discussion on IAB-MT power requirement | ZTE Corporation |
| R4-2004166 | IAB class definition | Ericsson |
| R4-2004545 | IAB class definitions | Huawei |
| R4-2004546 | TP to TS38.174 IAB class definitions | Huawei |
| R4-2004646 | On IAB-MT classes | Qualcomm Incorporated |
| R4-2003080 | Discussion on IAB-MT feature list | Samsung |
| R4-2003315 | Discussion of IAB-MT feature list | CATT |
| R4-2003316 | LS on RAN4 IAB-MT feature list agreement | CATT |
| R4-2003608 | IAB-MT Feature List | Qualcomm Incorporated |
| R4-2004149 | Discussion on IAB MT feature list | ZTE Corporation |
| R4-2004169 | IAB-MT madatory feature | Ericsson |
| R4-2004487 | Discussion on R16 IAB MT RF features | Huawei, HiSilicon |
| R4-2003081 | IAB-MT transmitter RF requirement | Samsung |
| R4-2003609 | IAB-MT Tx Requirements | Qualcomm Incorporated |
| R4-2003775 | IAB-MT ACLR and ACS in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2004158 | Further discussion on FR2 IAB-MT ACLR and ACS requirement | ZTE Corporation |
| R4-2004163 | IAB MT unwanted emission for FR1 | Ericsson |
| R4-2004164 | IAB MT unwanted emission for FR2 | Ericsson |
| R4-2004165 | IAB MT TX dynamic range | Ericsson |
| R4-2004167 | IAB MT Frequency error | Ericsson |
| R4-2004168 | IAB MT power control requirement | Ericsson |
| R4-2004549 | IAB-MT FR1 Rx sensitivity requirement. | Huawei |
| R4-2004644 | Definition of IAB-MT ACLR requirement in FR2 | Qualcomm Incorporated |
| R4-2004647 | Definition of IAB-MT dynamic range | Qualcomm Incorporated |
| R4-2003082 | IAB-MT receiver RF requirement | Samsung |
| R4-2003757 | Further considerations regarding radiated IAB-Node reference sensitivity requirement | Ericsson |
| R4-2003776 | IAB-MT blocking requirements in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2003777 | IAB-MT receiver sensitivity declaration range in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2004170 | IAB MT inband blocking and ACS for FR1 | Ericsson |
| R4-2004171 | IAB MT inband blocking and ACS for FR2 | Ericsson |
| R4-2004550 | IAB-MT ACS wanted signal level. | Huawei |
| R4-2004551 | IAB-MT in band blocking | Huawei |
| R4-2004645 | Definition of IAB-MT in-band selectivity requirements in FR2 | Qualcomm Incorporated |
| R4-2002359 | RRC connection control for IAB MTs | ZTE Corporation |
| R4-2004246 | Discussion on RRC connection mobility control requirement for IAB-MT | Huawei, Hisilicon |
| R4-2004252 | TP to TS 38.174 on random access requirement for IAB-MT | Huawei, Hisilicon |
| R4-2004408 | Analysis of RRC re-establishment and RRC release requirements for IAB MT | Ericsson |
| R4-2004409 | TP to TS 38.174 v0.0.1: RRC re-establishment requirements for IAB MT | Ericsson |
| R4-2004410 | TP to TS 38.174 v0.0.1: RRC re-direction requirements for IAB MT | Ericsson |
| R4-2004807 | RRC Connection Mobility Control in IAB Networks | Qualcomm |
| R4-2002355 | on RLM requirements for IAB MT | ZTE Corporation |
| R4-2002356 | TP for IAB RLM | ZTE Corporation |
| R4-2003532 | Discussion on RLM requirement for IAB-MT | Samsung |
| R4-2004248 | Discussion on RLM requirement for IAB-MT | Huawei, Hisilicon |
| R4-2004806 | IAB-MT RLM requirements | Qualcomm |
| R4-2002357 | on BFD and BFR requirements for IAB MT | ZTE Corporation |
| R4-2003533 | Discussion on BFR requirement for IAB-MT | Samsung |
| R4-2003534 | TP to TS 38.174 v0.0.1: Beam Candidate Detection Requirements for IAB MT | Samsung |
| R4-2004249 | Discussion on BFR requirement for IAB-MT | Huawei, Hisilicon |
| R4-2004803 | IAB-MT Link Recovery requirements | Qualcomm |
| R4-2004090 | [IAB EMC]further discussion on IAB EMC emission requirement | ZTE Corporation |
| R4-2004091 | [IAB EMC]further discussion on IAB EMC immunity requirement | ZTE Corporation |
| R4-2004092 | [IAB EMC]on how to handle IAB EMC | ZTE Corporation |
| R4-2004093 | [IAB EMC]TP to TR IAB EMC emission requirements | ZTE Corporation |
| R4-2004094 | [IAB EMC]TP to TR IAB EMC General part | ZTE Corporation |
| R4-2004095 | [IAB EMC]TP to TR IAB EMC immunity requirements | ZTE Corporation |

RAN4 #95e

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2006378 | TR38.809 V0.2.0 | Samsung |
| R4-2007467 | Email discussion for updating IAB TS spec to capture RAN4 95 agreements | Qualcomm |
| R4-2007577 | TP to TS 38.174: system parameter | Ericsson |
| R4-2007399 | Discussion on IAB MT class | ZTE Corporation |
| R4-2007401 | Further discussion on IAB-MT power requirement | ZTE Corporation |
| R4-2007903 | IAB-MT class definitions | Huawei |
| R4-2006279 | Discussion on IAB-MT Feature list | CATT |
| R4-2006657 | Support of Rel-15 UE features by IAB-MTs | AT&T |
| R4-2006797 | Further discussion on IAB-MT feature | Samsung |
| R4-2006798 | Clarificaiton on IAB-MT channel bandwidth | Samsung |
| R4-2006803 | On IAB feature list | CMCC |
| R4-2007119 | IAB-MT features | Nokia, Nokia Shanghai Bell |
| R4-2007131 | IAB-MT Tx Features | Qualcomm Incorporated |
| R4-2007317 | Further discussion on R16 IAB MT RF features | Huawei, HiSilicon |
| R4-2007400 | Discussion on IAB MT feature list | ZTE Corporation |
| R4-2007571 | IAB-MT madatory feature | Ericsson |
| R4-2006799 | RAN4 implication due to Sync from multiple parenets for “Case 1” OTA timing | Samsung |
| R4-2006276 | Discussion on IAB-MT modulation quality requirements | CATT |
| R4-2006282 | Discussion on IAB-MT power related issues | CATT |
| R4-2007402 | frequency error requirement for IAB | ZTE Corporation |
| R4-2007403 | Further discussion on FR1 IAB-MT ACLR and ACS requirement | ZTE Corporation |
| R4-2007543 | [IAB RF] further discussion on IAB TX IMD | ZTE Corporation |
| R4-2007573 | IAB-MT Frequency error | Ericsson |
| R4-2006280 | Discussion on IAB-MT REFSENS | CATT |
| R4-2007408 | In-band blocking for IAB MT | ZTE Corporation |
| R4-2006272 | TP for TR 38.809: Transmit ON/OFF power | CATT |
| R4-2006273 | TP for TS 38.174: Transmit ON/OFF power | CATT |
| R4-2006274 | TP for TR 38.809: IAB-DU Transmitted signal quality | CATT |
| R4-2006275 | TP for TS 38.174: IAB-DU Transmitted signal quality | CATT |
| R4-2006277 | TP for TR 38.809: IAB-MT Transmitted signal quality | CATT |
| R4-2006278 | TP for TS 38.174: IAB-MT Transmitted signal quality | CATT |
| R4-2007404 | TP to TR : IAB RX IM requirement (section 8.7 and 10.8) | ZTE Corporation |
| R4-2007405 | TP to TS 38.174: IAB RX IM requirement (section 7.7 and 10.8) | ZTE Corporation |
| R4-2007406 | TP to TR: IAB ICS requirement (section 8.8 and 10.9) | ZTE Corporation |
| R4-2007407 | TP to TS 38.174: IAB ICS requirement (section 7.8 and 10.9) | ZTE Corporation |
| R4-2007544 | [IAB RF] TP to TR 38.809 IAB TX IMD | ZTE Corporation |
| R4-2007545 | [IAB RF] TP to TS 38.174 IAB TX IMD | ZTE Corporation |
| R4-2007579 | TP to TR 38.809: Conducted RX spurious | Ericsson |
| R4-2007585 | TP to TS 38.174: Conducted RX spurious | Ericsson |
| R4-2006760 | Discussion on IAB-MT dynamic range | CMCC |
| R4-2006800 | Further discussion on IAB-MT transmitter requriement | Samsung |
| R4-2006931 | IAB-MT Tx power dynamic range and power control requirements | Nokia, Nokia Shanghai Bell |
| R4-2007120 | IAB-MT emission requirements | Nokia, Nokia Shanghai Bell |
| R4-2007121 | TP to TS 38.174: Output power requirements | Nokia, Nokia Shanghai Bell |
| R4-2007122 | TP to TR 38.809: Emission requirements | Nokia, Nokia Shanghai Bell |
| R4-2007130 | IAB-MT Tx Requirements | Qualcomm Incorporated |
| R4-2007574 | IAB-MT TX dynamic range and power control | Ericsson |
| R4-2007575 | IAB-MT maximum output power | Ericsson |
| R4-2007576 | IAB-MT unwanted emission for FR2 &FR1 | Ericsson |
| R4-2007904 | IAB-MT min number of TRX and emissions scaling | Huawei |
| R4-2007909 | IAB-MT TX ACLR and dynamic range | Huawei |
| R4-2006801 | Further discussion on REFSENS for IAB-MT | Samsung |
| R4-2006932 | IAB-MT blocking and selectivity requirements | Nokia, Nokia Shanghai Bell |
| R4-2007572 | IAB-MT REFSENS | Ericsson |
| R4-2007900 | IAB-MT LA FR2 Rx sensitivity requirement. | Huawei |
| R4-2007901 | FR1 IAB-MT sensitivity | Huawei |
| R4-2007902 | IAB-MT LA ACS and IBB | Huawei |
| R4-2007578 | TP to TR 38.809: OTA In-band blocking | Ericsson |
| R4-2007580 | TP to TR 38.809: radiated RX spurious | Ericsson |
| R4-2007581 | TP to TS 38.174: OTA ACS | Ericsson |
| R4-2007582 | TP to TS 38.174: OTA RX spurious | Ericsson |
| R4-2007583 | TP to TR 38.809: OTA ACS | Ericsson |
| R4-2007584 | TP to TS 38.174: OTA Inband blocking | Ericsson |
| R4-2007905 | TP to TS 38.174 -IAB-DU TX dynamic range | Huawei |
| R4-2007906 | TP to TS 38.174 -IAB-MT TX dynamic range | Huawei |
| R4-2007907 | TP to TS 38.174 -IAB-DU RX sensitivity | Huawei |
| R4-2007908 | TP to TS 38.174 -IAB-DU Rx dynamic range | Huawei |
| R4-2007054 | Discussion on IAB EMC requirements | Ericsson |
| R4-2007055 | TP to TR 38.809 on IAB EMC Emissions | Ericsson |
| R4-2007056 | TP to TR 38.809 on IAB EMC Immunity | Ericsson |
| R4-2007057 | TP to TR 38.809 on IAB EMC General | Ericsson |
| R4-2007538 | [IAB EMC]further discussion on IAB EMC emission requirement | ZTE Corporation |
| R4-2007539 | [IAB EMC]further discussion on IAB EMC radiated immunity requirement | ZTE Corporation |
| R4-2007540 | [IAB EMC]on how to handle IAB EMC | ZTE Corporation |
| R4-2007541 | [IAB EMC]TP to TR IAB EMC emission requirements | ZTE Corporation |
| R4-2007542 | [IAB EMC]TP to TR IAB EMC immunity requirements | ZTE Corporation |
| R4-2007269 | RRM requirements in IAB TR and TS | ZTE Corporation |
| R4-2007991 | TP to TS 38.174 v0.0.1: Adding references related to IAB | Ericsson |
| R4-2008196 | CR on 38174 RRM IAB TS | Nokia, Nokia Shanghai Bell |
| R4-2008238 | CR on 38174 RRM IAB TS | Nokia, Nokia Shanghai Bell |
| R4-2007189 | Pending issues on RRC mobility control for IAB-MT | ZTE Corporation |
| R4-2007488 | Discussion regarding RRC Connection Mobility Control in IAB Networks | Qualcomm |
| R4-2007489 | TP for RRC Connection Mobility Control in IAB Networks | Qualcomm |
| R4-2007992 | Handling 4 SMTC in RRC re-establishment and redirection for IAB MT | Ericsson |
| R4-2007993 | TP to TS 38.174 v0.0.1: Updates to RRC re-establishment requirements for IAB MT | Ericsson |
| R4-2007994 | TP to TS 38.174 v0.0.1: Updates to RRC re-direction requirements for IAB MT | Ericsson |
| R4-2008197 | discussion on Tansmit Timing requirement for IAB-MT | Nokia, Nokia Shanghai Bell |
| R4-2008198 | CR on 38174 Transmit Timing requirements for IAB-MT | Nokia, Nokia Shanghai Bell |
| R4-2008239 | CR on 38174 Transmit Timing requirements for IAB-MT | Nokia, Nokia Shanghai Bell |
| R4-2006016 | on RLM requirements for IAB MT | ZTE Corporation |
| R4-2006017 | TP for IAB RLM | ZTE Corporation |
| R4-2006433 | Radio Link Monitoring requirement for IAB-MT | Samsung |
| R4-2007490 | Discussion regarding RLM requirements of IAB-MTs | Qualcomm |
| R4-2007684 | Discussion on RLM requirement for IAB-MT | Huawei, Hisilicon |
| R4-2006015 | on BFD and BFR requirements for IAB MT | ZTE Corporation |
| R4-2006434 | Link Recovery requirement for IAB-MT | Samsung |
| R4-2006435 | TP to TS 38.174 v0.0.1: Beam Candidate Detection Requirements for IAB MT | Samsung |
| R4-2007486 | TP regarding BFD requirements of IAB-MTs | Qualcomm |
| R4-2007487 | Discussion regarding BFD and CBD requirements of IAB-MTs | Qualcomm |
| R4-2007683 | Discussion on BFD and CBD requirement for IAB-MT | Huawei, Hisilicon |

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template