3GPP TSG RAN Meeting #88e								 RP-200852

Electronic Meeting, June 29 - July 3, 2020

Agenda Item:	9.10.3

Source:	Apple Inc.

Title:	Views on NR Sidelink Enhancement

Document for:	Discussion/Decision

# Introduction

Starting from Rel-15, the initial NR design focused on the support and optimization of three major types of services, (1) enhanced Mobile Broadband (eMBB) (2) Ultra Low Latency High Reliability (URLLC) and (3) massive Machine Type Communications (mMTC). It was decided later in plenary to remove mMTC, a.k.a. Low Power Wide Area (LPWA) services, from NR scope and continue the LPWA service evolution in LTE as part of NB-IoT and eMTC enhancement.

In Rel-16, in addition to the enhancement of existing Rel-15 designs such as MIMO, URLLC, etc., several new verticals, or commercial use cases, have been designed and supported by NR. Among them, the one notable new vertical is NR V2X/Sidelink which can be viewed as the continuous support and evolution of LTE Sidelink into NR.

As part of approved Rel-17 WI/SI’s, significant amount of effort has been spent to ensure that new commercial use case can be supported or enhanced in NR, for example, in the approved Rel-17 sidelink enhancement [1], more verticals are considered including “commercial, V2X, and Critical Communication usage of sidelink”. We believe that it is very crucial to support and exploit more commercial use case in order to ensure that NR is commercially successful as a new RAT.

Sidelink, in general, has been designed and optimized in 3GPP since LTE Rel-12 in 2012. For nearly 8 years, sidelink design has gone through LTE D2D, LTE V2X, and NR V2X and will continue into Rel-17, and very likely, further beyond. We view sidelink design in 3GPP as a feature that has great commercial potential that has not been adequately realized in the field. With more emerging use cases for device to device commutations, we believe NR sidelink design has great potential not only limited to vehicular to other device communications, but also in the generic device to device personal communications as well as the new emerging market of wearable devices.

However, with the great potential of NR sidelink technologies, we do not think the current NR sidelink design adequately address the true deployment challenges especially in the area of coverage. In this contribution, we share our view on the NR sidelink enhancement, especially in the area of coverage enhancement which we believe is crucial for the commercial success of NR sidelink solutions.

# Overview of NR Sidelink Coverage

## Coverage Requirement

For NR V2X, the coverage requirement is up to 1 km as stated in TS 22.186 [2]. The targeted spectrum for NR V2X deployment is 5.9GHz ITS band, and the sidelink pathloss model is provided in TR 37.885 [3]. In this section, we evaluate the sidelink path loss that a device would experience at 1 km, in 5.9GHz frequency band, based on the path loss model specified in [3].

Figure 1 Pathloss for NR V2X at 1000 meters and 5.9GHz frequency band

Figure 1 shows the pathloss distribution at 1000 meter distance and 5.9GHz frequency band, based on the path loss model specified in [3]. Note that the path loss model also contains shadow fading with log normal distribution with standard deviation of 3dB for LOS and 4dB for NLOS, which is the reason why we showed the pathloss distribution.

Based on Figure 1, we have the following observations

- For NLOS, even at 90%, the pathloss can be more than 145 dB

- Compared to the Uu channel design, we also need to account for the receiver performance loss especially when the receiver is a normal cellular phone. The receiver performance loss compared to gNB can easily be over 20 dB

  - Antenna gain loss: 12-21 dB difference

    - gNB antenna gain: 8/17 dBi; UE antenna gain: -4/0 dBi

  - Noise figure loss: 4 dB difference

    - gNB noise figure: 5dB; UE noise figure: 9 dB

- Furthermore, for wearable devices, due to the form factor limitation, it is very likely that there is additional antenna gain loss of 5dB or even higher.

Note that, even though 1 km coverage requirement can be understood as the requirement for vehicular to vehicular communication, we think similar requirement is also applicable to handheld devices and wearable devices. It is important for us to consider not only the pedestrian UE, such as smartphones or wearable with slow mobility patterns, but also consider the other VRUs which has form factor constraints with less sophisticated receiver and limited antenna, such as bikes and motorcycles. For those VRUs, the UE can move in medium and high speed. Thus, 1 km range for V2X communication is still desired.

Given the above analysis, we believe that in order to meet the service requirement, NR sidelink needs to design based on coverage requirement for up to 160 dB MCL (Maximum Coupling Loss) or even higher.

Observation1: In order to meet the NR V2X service requirement, NR sidelink design needs to support coverage requirement for up to 160dB MCL (Maximum Coupling Loss) or even higher.

## Existing NR Sidelink PHY Channel Designs

As we discussed in the previous section, NR sidelink design faces very stringent coverage requirement. It is important to note that, for Uu design, the design requirement is approximately 140dBm MCL. In this section, we will compare the existing NR sidelink design with Uu design in order to shed light on the deficiency of the current NR design in terms of the coverage that it supports.

Table 1 Comparison of NR Uu and NR Sidelink RAN1 Design

| Design Aspect | NR Uu as of Rel-16 | NR Sidelink as of Rel-16 | Comments |
| --- | --- | --- | --- |
| Waveform & Modulation | Uplink supports DFT-s-OFDM with Pi/2 BPSK modulation | Only CP-OFDM is supported | NR Sidelink suffers additional coverage loss due to increased PAPR |
| SYNC Signal PSS/SSS for Uu S-PSS/SSS for Sidelink | 1 symbol PSS and 1 symbol SSS per SSB Multiple SSBs per transmission Typically 20ms periodicity | 2 symbol S-PSS and 2 symbol S-SSS per SSB Multiple SSBs per transmission 160ms periodicity | NR Sidelink may suffer additional coverage loss due to the longer periodicity |
| Broadcasting Channel PBCH for Uu PSBCH for Sidelink | 3 symbol PBCH per SSB Multiple SSBs per transmission Typically 20ms periodicity | 7(ECP)/9(NCP) symbol PBCH per SSB Multiple SSBs per transmission 160ms periodicity | NR Sidelink may suffer additional coverage loss due to the longer periodicity |
| Data Channel PUSCH/PDSCH for Uu PSSCH for Sidelink | Slot aggregation is allowed for up to 16 times | Slot aggregation is not allowed | NR Sidelink suffers additional coverage loss due to the lack of repetition |
| Feedback Channel PUCCH for Uu PSFCH for Sidelink | Both long format (1, 3, 4) and short format (0, 2) are supported PUCCH repetition up to 8 times | Only short format (like PUCCH format 0) is supported No repetition | NR Sidelink suffers additional coverage loss due to the lack of support of long format and repetition |
| Control Channel PDCCH for Uu PSCCH for Sidelink | Up to 3 symbols CORESET | Up to 3 symbols PSCCH Stage 1 SCI | - |

In table 1, we compare the major PHY channel design between NR sidelink and NR Uu interference. There are two major observations we can have.

- Compared to Uu design, NR sidelink design is clearly less capable in terms of the coverage support for almost all the PHY channels. Many good Uu designs, such as long PUCCH format, repetition, etc., have yet to be adopted by NR sidelink

- In addition, the current Uu design may not be deemed to be enough for all the coverage scenarios which is the reason why we have Rel-17 “Study on NR coverage enhancements”. As we discussed in the previous section, NR sidelink coverage requirement can be more stringent due to the high band and the reduced receiver/transmitter capability compared to Uu case.

With all the above discussion, we have the following observation

Observation 2: NR sidelink coverage can be the bottleneck for the actual commercial deployment especially for certain commercial use cases such as device to device communications and wearable devices. The coverage enhancement is greatly needed

# NR Sidelink Coverage Enhancement

Given the above discussion, it is nature to conclude, and we also firmly believe, that NR sidelink coverage enhancement is very crucial and, is still missing in the current scope. There are the following options if we would like to address this limitation in a timely manner in Rel-17.

- Rel-17, NR Sidelink enhancement [RAN1 WI: NR_SL_enh] [4]: The current scope of NR sidelink enhancement in RAN1 is very limited. Within the current scope, only limited sensing or resource allocation optimization, and, limited mode 2 enhancement will be considered. If time or scope permits, we think the coverage enhancement should also be considered as part of Rel-17 NR sidelink enhancement.

- Rel-17, Study on NR coverage enhancements [RAN1 SI: FS_NR_cov_enh] [5]: This is another agenda that can potentially cover the NR sidelink coverage enhancement. However, the current SID implicitly or explicitly limits the scope to be within Uu, potentially NRU as well. However, once coverage enhancement solutions are designed in this agenda, it can also be considered for NR sidelink as well.

- Rel-17, Study on support of reduced capability NR devices [RAN1 SI: FS_NR_redcap] [6]: This can be another agenda for coverage enhancement, especially to cover the wearable devices. Note that due to the form factor limitation of wearable devices, it suffers from two major constrains (1) The coverage loss due to the antenna gain loss (2) The power consumption concern due to the battery size. To mitigate those constrains, it is very important to allow device to device communication and to improve the coverage, both of which, can fit into NR sidelink design

On the other side, if coverage enhancement for NR sidelink cannot be addressed in Rel-17, we believe coverage enhancement should be one of the most important topics, if not the only most important topic, that should be addressed in the future release of NR sidelink enhancement. Without coverage enhancement, we believe the commercial use case of sidelink is significantly limited

Proposal 1: NR sidelink enhancement should consider and focus on coverage enhancement, preferably in Rel-17

# Conclusion

For the continuous evolution of NR and, in order to ensure NR being a commercially successful new RAT, we strongly believe that supporting new verticals, and new commercials use cases, is crucial for the current and the future NR work. Among which, we believe device to device communication and wearable devices are of high importance, for which, NR sidelink is one of the major enablers for such commercial use cases. We evaluated the existing NR sidelink design, and we believe that one of the critical enhancement that is missing is the coverage enhancement. As results, we have the following observations and proposal

Observation1: In order to meet the NR V2X service requirement, NR sidelink design needs to support coverage requirement for up to 160dB MCL (Maximum Coupling Loss) or even higher.

Observation 2: NR sidelink coverage can be the bottleneck for the actual commercial deployment especially for certain commercial use cases such as device to device communications and wearable devices. The coverage enhancement is greatly needed

Proposal 1: NR sidelink enhancement should consider and focus on coverage enhancement, preferably in Rel-17

# References

- RP-193231, “New WID on NR sidelink enhancement”, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- 3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

- 3GPP TR 37.885 V15.3.0, "Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR"

- RP-193231, "New WID on NR sidelink enhancement", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- RP-193240, "New SID on NR coverage enhancement", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- RP-193238, "New SID on support of reduced capability NR devices", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019