3GPP TSG RAN meeting #88e										RP-200901

Electronic Meeting, June 29 - July 3, 2020

## Status Report to TSG

Agenda item:			9.9.2

| WI / SI Name | Study on supporting NR from 52.6 GHz to 71 GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_52_to_71GHz |  |  |  |  |
| Unique ID | 860037 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-193259 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2020 (Updated from 06/2020) | Core part: mm/yyyy | Performance part: mm/yyyy | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: 20 % | Core part: xx% | Performance Part: xx% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Lee, Daewon (RAN1); Vintola, Ville (RAN4) |
|  | Company | Intel Corporation; Qualcomm |
|  | Email | daewon (dot) lee (at) intel (dot) com; vvintola (at) qti (dot) qualcomm (dot) com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Originally had planned for the following time budget.

Q1’ 2020

- R1 - #100: 1 TU

- R4 - #94: 1 TU

Q2’ 2020

- R1 - #100bis: 1 TU

- R4 - #94bis: 1 TU

- R1 - #101: 3 TU

- R4 - #95: 1 TU

However, due to the COVID-19 crisis, all physical meetings have been cancelled. RAN1 has started the SI in R1#101-e meeting, but RAN4 has not started the meeting in the same timeframe. If the completion date is pushed back by 1 quarter (to end of Q3’ 2020), RAN4 will only have 1 meeting to complete the SI, which might not be realistic. Therefore, the suggestion to move the SI completion to Dec 2020. Impact to the overall Rel-17 plans can be mitigated by shortening the WI appropriately.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

### 2.1	RAN1

#### 2.1.1	Agreements

##### Agreed Link Level Evaluation Assumptions and Parameters:

| Parameter Set 1 | Evaluation Objectives | Carrier Frequency [GHz] | Subcarrier Spacing [kHz] | Bandwidth [MHz] | Number of RB | Waveform |
| --- | --- | --- | --- | --- | --- | --- |
| Description | Primary Objective: - Evaluation of PDSCH/PUSCH performance including study of phase noise impairment impact for various numerology (i.e. subcarrier spacing, CP length) and possibly for various carrier frequencies. Evaluation KPI(s) include BLER. Secondary Objective: - Evaluation of SSB/PRACH performance including study of phase noise impairment impact for various numerology (i.e. subcarrier spacing, CP length) and possibly for various carrier frequencies. Evaluation KPI(s) include miss-detection, false alarm. | 60 GHz Optional: 70 GHz | PDSCH/PUSCH: - {120, 240, 480, 960} kHz - FFS: 1920 kHz Optional: - if evaluated companies are asked to provide information on other channels/signals and subcarrier spacing | PDSCH/PUSCH: - {400, 2000} MHz Optional: - Companies are asked to provide information if other bandwidths are evaluated Note: Evaluation of listed channel bandwidth does not mean RAN1 has agreed to support such channel bandwidth and are only for evaluation purposes to obtain useful insights. | For 400 MHz: - 256 (120 kHz), - 128 (240 kHz), - 64 (480 kHz), - 32 (960 kHz), - N/A (1920 kHz) For 2000 MHz: - N/A (120 kHz), - N/A (240 kHz), - FFS (480 kHz), - 160 (960 kHz), - 80 (1920 kHz), For other channel bandwidths: - Companies are asked to provide information. Companies are encouraged to utilize linearly scaled PRB sizes for a given bandwidth based on above. | For PDSCH: CP-OFDM For PUSCH: CP-OFDM and DFT-s-OFDM |

| Parameter Set 2 | CP Type | Channel Model | Antenna Configuration (Mg,Ng,M,N,P) | Mobility |
| --- | --- | --- | --- | --- |
| Description | Normal CP Extended CP Note: ECP is not expected to be applicable in all SCS and channel conditions, and companies providing results for ECP are encouraged to provide evaluation results with motivation/justification of simulated ECP cases | TDL model as defined in of TR38.901 Section 7.7.2: - TDL-A (5ns, 10ns, DS) - optional DS for consideration: 20ns, 40ns, 60ns DS CDL model as defined in of TR38.901 Section 7.7.1: - CDL-B (20ns, 50ns DS) - CDL-D (20ns, 30ns DS) with K-factor = 10 dB - optional DS for consideration: 100ns DS FFS: modification CDL-B/D model (a) Indoor Office NLOS: CDL-B (20 ns DS), and Indoor Office LOS: CDL-D (20 ns DS) - Use mean angular spread values from Table 7.5.6-Part2 (for ASD, ASA, and ZSA) and Table 7.5-10 (for ZSD) - Use mean angles of CDL-B/D for desired mean angles as baseline (no angle translation) - Note that the angular spread values in the table are quoted in log units - Mean K-factor for CDL-D from Table 7.5.6-Part2 (9 dB) (b) UMi – Street Canyon NLOS: CDL-B (50 ns DS), and UMi – Street Canyon LOS: CDL-D (30 ns) - Use mean angular spread values from Table 7.5.6-Part1 (for ASD, ASA, and ZSA) and Table 7.5-8 (for ZSD). - Use mean angles of CDL-B/D for desired mean angles as baseline (no angle translation) - Note that the angular spread values in the table are quoted in log units - Use mean K-factor for CDL-D from Table 7.5.6-Part1 (7 dB) Note: Mean angular spread values are used as desired AS value to scale the ray angles as described in TR38.901 section 7.7.5.1. As baseline, the ray angles are not translated, meaning (TR38.901 section 7.7.5.1). If companies perform translation of the ray angles they are encouraged to report the details. The mean K-factor is used to scale the tap powers as described in TR38.901 section 7.7.6. Note: for TDL/CDL model, the delay spread (DS) value mentioned is the delay spread scaling value (i.e. corresponding to normalized delay of 1.0). Note2: Other models (either TDL or CDL) with DS values not listed are optional. Note3: Companies are encouraged to provide evaluation results with motivation/justification of simulated DS values. | For TDL model: - 2x2 - 1x2 (optional) For CDL model: Configuration 1: - (Mg,Ng,M,N,P) = (1,1,8,16,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,4,4,2) UE with (0.5 dv, 0.5 dH) Configuration 2: - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) | 3 km/hr |

| Parameter Set 3 | PA Model | gNB TRP PN Model | UE PN Model | Pre-loaded Tx EVM | Additive Rx EVM | I-Q Imbalance | Frequency Offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Description | Optional: - Companies to provide modeling (in lieu of pre-loaded Tx EVM) | 3GPP TR38.803 example 2 BS PN profile Optional: - If other PN profile is used, companies to provide information on the modeling used Note: companies to provide information about the LO distribution model assumed in the simulations. | 3GPP TR38.803 example 2 UE PN profile Optional: - If other PN profile is used, companies to provide information on the modeling used Note: companies to provide information about the LO distribution model assumed in the simulations. | Optional: - 3% at Tx (In lieu of PA model), - If other values are used companies are asked to provide information on the values selected for simulation. | Optional: - 5% at Rx, - If other values are used companies are asked to provide information on the values selected for simulation. | Optional: - (-26dBc), - (-31dBc), - If other values are used companies are asked to provide information on the values selected for simulation. | Optional: - 0.1 ppm (for PDSCH/PUSCH) - 5, 10, 20 ppm (for initial access) |

| Parameter Set 4 | Channel Estimation | Transmission Rank | PDSCH SLIV | DMRS Configuration | PTRS Configuration | MCS/TBS |
| --- | --- | --- | --- | --- | --- | --- |
| Description | Realistic channel estimation | Rank 1 Note: companies are asked to provide information the precoding scheme (including granularity) used in the evaluations. | (S=2, L=12) Optional:(S=0, L=14) Note: Starting symbol, S, (indexed from 0) and length, L. | 1 DMRS symbol (front loaded), or 2 DMRS symbols at (2,11) symbol index Note: no data multiplexing is assumed in DMRS symbols | For CP-OFDM: (K = 4, L = 1) or (K = 2, L = 1) Note: PTRS per K number of PRBs, and PTRS every L number of OFDM symbols For DFT-s-OFDM: (Ng = 2, Ns = 2, L = 1) (Ng = 2, Ns = 4, L = 1) (Ng = 4, Ns = 2, L = 1) (Ng = 4, Ns = 4, L = 1) (Ng = 8, Ns = 4, L = 1) Note: Ng number of PT-RS groups, Ns number of samples per PT-RS group, and PTRS every L number of DFT-s-OFDM symbols Note2: companies are asked to provide the PT-RS configuration used for DFT-s-OFDM simulation among the listed above, where the selection of the PT-RS is chosen such that it provides similar overhead as the chosen PT-RS configuration for PUSCH CP-OFDM (if simulated). | From MCS Table 1 (TS38.214): - MCS 7 (QPSK), - MCS 16 (16QAM), - MCS 22 (64QAM), From MCS Table 2 (TS38.214): - MCS 27 (256QAM) (optional) Note: If normal CP and extended CP are to be compared, companies are asked to provide information on the MCS values used that provide similar payload sizes for the comparison. |

##### Agreed System Level Evaluation Assumptions and Parameters:

| Parameter Set 1 | Evaluation Objectives | Carrier Frequency [GHz] | Subcarrier Spacing [kHz] | Bandwidth [MHz] | Number of RB |
| --- | --- | --- | --- | --- | --- |
| Description | Primary objective: - Evaluation of single operator and multi-operator deployments including study of interference impact and coexistence between nodes. Evaluation KPIs include user throughput, latency, average buffer occupancy, ratio of mean served throughput and offered cell throughput, and resource utilization. Secondary objective: - obtain delay spread profiles (and inter-symbol interference statistics) for deployment scenarios of interest (note: performance impact from delay spread should be conducted in LLS, the SLS would be used to supplement findings) | 60 GHz Optional: 70 GHz | For 2000MHz BW: 960 kHz FFS: 120, 240, 480 kHz For 400MHz BW: 120 kHz FFS: 240, 480, 960 kHz Note: Other than value above, companies are encouraged to evaluating using subcarrier spacing values determined to be feasible from LLS study. Values for the subcarrier spacing may be revisited after further investigation from LLS study. | 2000 MHz 400 MHz (FFS: optional) Note: Channel bandwidth evaluated may be revisited after further investigation. | For 2000 MHz: - N/A (120 kHz), - N/A (240 kHz), - FFS (480 kHz), - 160 (960 kHz), - 80 (1920 kHz), For 400 MHz: - 256 (120 kHz), - 128 (240 kHz), - 64 (480 kHz), - 32 (960 kHz), - N/A (1920 kHz) For other channel bandwidths: - Companies are asked to provide information. Companies are encouraged to utilize linearly scaled PRB sizes for a given bandwidth based on above. |

| Parameter Set 2 | Deployment Scenario | UE distribution | Channel Model |
| --- | --- | --- | --- |
| Description | Primary scenarios: - Scenario indoor-A or C (FFS: which scenario is primary) Secondary scenarios: - Scenario indoor-C or A (FFS: which scenario is secondary) - Scenario outdoor-B Optional: - other scenarios listed below Indoor Office: Scenario Indoor-A) InH open office model: Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, ISD = 20m, BS randomly deployed within 10m x 10m virtual box FFS: if the office box can be reduced down to 50m x 50m FFS: minimum distance between BS Scenario Indoor-B) small InH open office model: Office box 20m x 20 m, 1 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS randomly deployed within 10m x 10m virtual box FFS: minimum distance between BS Scenario Indoor-C) InH open office model: Office box 120m x 50 m, 12 BS per operator, 1 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, ISD = 20m FFS: if the office box scenario can be reduced down to 50m x 50m Scenario Indoor-D) InH open office model: Office box 120m x 50 m, 6 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, ISD = 20m FFS: if the office box scenario can be reduced down to 50m x 50m Scenario Indoor-E) InH open office model: Office box 120m x 80 m, 3 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, a=20m, b=40m, c=20m, and d=40m Dense Urban: Scenario Outdoor-A) Dense Urban with 1 layer Hexagonal grid, single layer, 3 sectors per site, 7 sites locations, BS height 10m, UE height 1.5m, ISD = 150m FFS: whether ISD needs to be smaller FFS: Reducing deployment size from 7 sites to 1 site for performance evaluations with both single and two operator scenarios. Scenario Outdoor-B) Dense Urban with 2 layers Macro layer (sub 7GHz – not necessarily need to be simulated for the 60GHz evaluation): Hexagonal grid, single layer, 3 sectors per site, 7 sites locations BS height 25m, UE height 1.5m, ISD = 100m, fixed BS position Micro layer (above 52.6 GHz): BS height 10m, UE height 1.5m, 2 operator, 2 BS per hexgrid per operator, random position within macro hexagonal grid per operator, minimum distance between TRP and UE: 10m FFS: Reducing deployment size from 7 sites to 1 site for performance evaluations with both single and two operator scenarios. Scenario Outdoor-C) Dense Urban with 1 layer Hexagonal grid, single layer, 3 sectors per site, 3 sites locations, BS height 10m, UE height 1.5m, ISD = 150m Indoor Factory Hall: Scenario Factory-A) Indoor factory with Dense cluster & low BS (InF-DL) Grid, 300m x 150m x 10m factor hall ISD 50m, BS height 1.5m, UE height 1.5m, Typical clutter size 2m, Clutter height 6m, Clutter density 60% Scenario Factory-B) Indoor factory with sparse clutter & High BS (InF-SH) Grid, 300m x 150m x 10m factor hall ISD 50m, BS height 8m, UE height 1.5m, Typical clutter size 10m, Clutter height 2m, Clutter density 20% | Average of 5 or 10 UE per BS UE are either 100% indoor or 100% outdoor depending on deployment scenario. | InH open office: - gNB-to-gNB and gNB-to-UE links: InH – office channel & PL model from TR38.901 - UE-to-UE links: [InH – office channel & PL model from TR38.901] Dense Urban: - gNB-to-gNB and gNB-to-UE links: UMi street canyon channel & PL model from TR38.901 - UE-to-UE links: [D2D channel & PL model from TR36.843 Section A.2.1.2] Indoor factor: - gNB-to-gNB and gNB-to-UE links: InF channel & PL model from TR38.901 - UE-to-UE links: [InF channel & PL model from TR38.901] Note: 3D distance between an gNB and a UE is applied. 3D distance is also used for LOS probability and break point distance. Note: channel models in brackets, [ ], are working assumption and may be revisited. |

| Parameter Set 3 | Mobility | BS Antenna Configuration (Mg,Ng,M,N,P) | BS Antenna Pattern | BS Antenna element gain | UE Antenna Configuration (Mg,Ng,M,N,P) | UE Antenna Pattern | UE Antenna element gain |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Description | 3 km/hr | For outdoor macro/sectorized scenarios: (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH) For outdoor micro-layer scenarios: (Mg,Ng,M,N,P) = (1,3,8,16,2) with (0.5 dv, 0.5 dH) Note: 3 Panel single sector gNB with {0,+120,-120} degree boresight orientations. The gNB will only utilize 1 panel at given moment. For indoor scenarios: (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH) | For outdoor scenarios: - Antenna power pattern given in Table 7.3-1 of TR38.901 (with exception of antenna element gain) For indoor/factory scenarios: - Antenna power pattern given in Table A.2.1-7 of TR38.802 for ceiling mount (with exception of antenna element gain) | 5 dBi | Configuration 1: (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) Configuration 2 (optional): (Mg,Ng,M,N,P) = (1,2,4,4,2) with (0.5 dv, 0.5 dH) Note: In both configurations, the 2 panels are back-to-back with panel selection done the at receiver. The UE will only utilize 1 panel at a given moment. | Antenna power pattern given in Table A.2.1-8 of TR38.802 Note: Companies to provide information about boresight orientation (e.g. random orientation, vertical to ground, parallel to ground, etc) | 5 dBi |

| Parameter Set 4 | BS Power Limitation | UE Power Limitation | BS NF | UE NF | Transmission Rank |
| --- | --- | --- | --- | --- | --- |
| Description | 40 dBm EIRP Optional: 60 dBm EIRP Maximum TxP adjusted to meet EIRP limits | 25 dBm EIRP with 21 dBm max TxP Optional: 40dBm EIRP with 21 dBm max TxP | 7 dB | 10 dB Optional: 13dB | Rank adaptative transmission between Rank 1 and 2 |

| Parameter Set 5 | PDCCH Overhead | DMRS Overhead | CSI-RS Overhead | SRS Overhead | Other Overhead | Data Processing Latency |
| --- | --- | --- | --- | --- | --- | --- |
| Description | 2 symbol per slot | 1 symbol per slot | Companies to provide information | Companies to provide information | Companies to provide information | UE processing timeline in microseconds are assumed to be same as 120 kHz SCS PDSCH/PUSCH processing latency Optional: UE processing timeline in microseconds are assumed to be half of 120 kHz SCS PDSCH/PUSCH processing latency |

| Parameter Set 6 | TDD DL/UL Ratio | CSI feedback | Additive Rx EVM | Traffic Model | UE Receiver | Cell selection criteria | DL/UL Traffic Ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Description | Companies to provide information (if applicable) | Ideal feedback | Note: additive Rx EVM values may be revisited after LLS study | FTP Model 3 (27Mbyte file) Optional: - Full buffer, - FTP Model 1 (27 Mbyte file), - FTP Model 3 (0.5, 2, 16 Mbyte file) | MMSE-IRC | Random select from strongest RSRP with 1 dB HO Margin Note: UE with RSRP below a P_threshold are not considered in simulation and counted toward UE distribution count FFS: value of P_threshold. (including the possibility of negative Inf) | 50% DL, 50% UL Optional: 100% DL, 0% UL, 80% DL, 20% UL 0% DL, 100% UL |

| Parameter Set 7 | Channel access modeling | Synchronization Assumption |
| --- | --- | --- |
| Description | Companies to report details of LBT procedure and parameters (e.g. ED, CWmax, COT, etc.) if LBT procedure is used in the evaluations. | Companies are asked to provide information on the synchronization assumption made between operators for 2 operator deployment scenarios. |

##### Agreement:

- Draft LS agreed in R1-2005195, and Final LS in R1-2005196.

##### Conclusion:

- RAN1 will continue the study on the objectives of the SI and not stop the study until RAN4 response for the “LS to RAN4 on Phase noise and other RF Impairment modelling”. If RAN4 can provide the information requested with sufficient time to consider the information provided, RAN1 will consider the input from RAN4 as part of the on-going study.

##### Conclusion:

- Companies are encouraged to provide inputs and considerations for the following identified physical layer aspects:

  - Candidate numerology (SCS, and CP length) to be supported by RAN1 specification.

    - Discussions may include how RAN1 should conclude on determination of the candidate numerologies

    - Discussion may also include identification of any coupling with other system parameters, such as bandwidth (number of PRB), FFT size, etc

  - Candidate bandwidths (or range of bandwidth) to be supported by RAN1 specification and related considerations (e.g. maximum FFT size)

    - Discussions may include how RAN1 should conclude on determination of the candidate bandwidths

  - Identification of potential impacts to PHY due to the candidate numerology and bandwidths

    - Discussion may include how to address the impacts to PHY channels and procedures, such as initial access, UL/DL signal/channel, scheduling/HARQ

  - Identification of regulatory aspects to consider in channel access (and interference mitigation techniques) for 60GHz unlicensed NR operation

    - Note: some examples of consideration aspects could be CCA sensitivity levels, time unit for measurement and back-off counters, access categories, channel bandwidth occupancy, LBT bandwidth, maximum output power, ED threshold, etc.

  - Supported channel access and interference mitigation techniques

    - Discussion may include how RAN1 should conclude on channel access schemes and/or interference mitigation techniques (e.g. omni-directional LBT, directional LBT, receiver-aided LBT, no-LBT, ATPC, etc) and identification of various consideration aspects (in the decision-making process)

    - Discussions may also include whether to always mandate LBT operations or not

- In addition to the above considerations, the following physical layer aspects have been additionally mentioned (but not limited to) in RAN1#101-e and can be further studied:

  - Initial access signals/channels

    - Investigation of transmissions of SS/PBCH blocks (including beam switching time)

    - SSB and CORESET#0 multiplexing

    - PRACH sequence lengths to achieve max allowed EIRP

    - non-consecutive RO within RACH slot to provide LBT gap

  - Other DL/UL signals/channels

    - Performance verification of existing and improved RS, e.g., DMRS & PTRS

    - Coverage requirements for IAB and for short physical channels

    - Handling of control/data channel coverage by OFDM symbol shortening

    - Investigation of UL interlace transmissions

  - Beam management

    - Beam determination/refinement during initial access

    - Beam failure detection issues

    - DL/UL beam correspondence in licensed/unlicensed spectrum

  - Required processing timelines and scheduling

    - UE minimum processing timelines and PDCCH monitoring capabilities (BD/CCE limits) for high SCS and their potential impact on scheduling and HARQ functionality of NR

    - CSI processing timeline and CSI processing unit availability for different SCS

    - Handling of beam switching time for control/data channel transmission

    - Scheduling operation, including the T/F scheduling granularity and PDCCH monitoring unit for high SCSs

  - Channel access

    - OCB constraints and related specification impact

    - PSD constraints and related specification impact

    - FBE operations

    - LBT procedure with respect to {carrier BW, RB set, maximum power, ED threshold}

    - Shared COT mechanisms

    - Potential enhancements to increase the channel access opportunities

  - Others

    - Maintaining cell coverage/link budget for high SCSs

    - Supporting rank-2 SU-MIMO for DFT-s-OFDM

    - Multi-carrier based operation for multi-RAT coexistence in unlicensed band

#### 2.1.2	Remaining Open issues

Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

- Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

- Identify potential critical problems to physical signal/channels, if any [RAN1].

Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

### 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

### 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

### 2.4	RAN4

#### 2.4.1	Agreements

None

#### 2.4.2	Remaining Open issues

Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

- Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

### 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

### 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2003286	Channelization in unlicensed spectrum above 52.6GHz and below 71GHz	Futurewei

- R1-2003287	Considerations on channel access in the unlicensed spectrum above 52.6GHz and below 71GHz	Futurewei

- R1-2003293	Link level evaluation methodology and considerations for PHY design in 52.6-71 GHz using NR waveform	Huawei, HiSilicon

- R1-2003294	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

- R1-2003304	Discussion on potential physical layer impacts for NR beyond 52.6 GHz	Lenovo, Motorola Mobility

- R1-2003305	Discussion on channel access for NR beyond 52.6 GHz	Lenovo, Motorola Mobility

- R1-2003424	Discussion on requried changes to NR using existing DL/UL NR waveform	vivo

- R1-2003425	Discussion on channel access mechanism	vivo

- R1-2003426	Evaluation on different numerologies for NR using existing DL/UL NR waveform	vivo

- R1-2003461	Discussion on the required changes to NR for above 52.6GHz	ZTE, Sanechips

- R1-2003462	Discussion on the channel access mechanism for above 52.6GHz	ZTE, Sanechips

- R1-2003463	Discussion on the evaluation assumptions for above 52.6GHz	ZTE, Sanechips

- R1-2003636	System Analysis and evaluation methodology of NR opration up to 71 GHz	CATT

- R1-2003637	Channel Access Mechanism in support	CATT

- R1-2003679	On Required changes to NR using existing DL/UL NR waveform for supporting NR from 52.6GHz to 71 GHz	MediaTek Inc.

- R1-2003680	On channel access mechanism from 52.6GHz to 71GHz	MediaTek Inc.

- R1-2003764	Discussion on Required Changes to NR in 52.6 – 71 GHz	Intel Corporation

- R1-2003765	Channel Access Procedure for NR in 52.6 - 71 GHz	Intel Corporation

- R1-2003766	Considerations on performance evaluation for NR in 52.6-71GHz	Intel Corporation

- R1-2003777	Waveform Enhancements for 60GHz operation	Charter Communications, Inc

- R1-2003780	Channel access mechanisms for 60 GHz NR-U	Charter Communications, Inc

- R1-2003811	Required changes to NR using existing DL/UL NR waveform	Nokia, Nokia Shanghai Bell

- R1-2003812	NR coexistence mechanisms for 60 GHz unlicensed band	Nokia, Nokia Shanghai Bell

- R1-2003813	Performance of existing DL NR waveform beyond 52.6 GHz	Nokia, Nokia Shanghai Bell

- R1-2003849	On NR operations in 52.6 to 71 GHz	Ericsson

- R1-2003850	Channel Access Mechanism	Ericsson

- R1-2003851	Enhanced phase noise modeling	Ericsson

- R1-2003903	Design aspects for extending NR to up to 71 GHz	Samsung

- R1-2003904	Channel access mechanism for 60 GHz unlicensed spectrum	Samsung

- R1-2003905	Evaluaton assumptions for SLS and LLS	Samsung

- R1-2003921	Discussion on channel access mechanism for supporting NR from 52.6GHz to 71 GHz	Beijing Xiaomi Mobile Software

- R1-2003945	Discussions on Changes to NR Using Existing DL/UL NR Waveform	Quectel

- R1-2003961	Discussion on required changes to NR for above 52.6GHz	CMCC

- R1-2003962	Discussion on channel access for above 52.6GHz	CMCC

- R1-2004004	Discussion on required changes to NR using existing NR waveform	Spreadtrum Communications

- R1-2004005	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

- R1-2004038	Consideration on required physical layer changes to support NR above 52.6 GHz	LG Electronics

- R1-2004039	Considerations on channel access mechanism to support NR above 52.6 GHz	LG Electronics

- R1-2004040	Consideration on multi-carrier operation for NR above 52.6 GHz	LG Electronics

- R1-2004096	Discussion on the waveform for DL/UL at above 52.6 GHz	OPPO

- R1-2004097	Discussion on channel access mechanism for above 52.6GHz	OPPO

- R1-2004098	Discussion on deployment scenarios for carrier frequency above 52.6GHz	OPPO

- R1-2004174	Channel access mechanism	TCL Communication Ltd.

- R1-2004188	Considerations on bandwidth and subcarrier spacing for above 52.6 GHz	Sony

- R1-2004189	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

- R1-2004247	On Required changes to NR using existing DL/UL NR Waveform	Apple

- R1-2004248	On Unlicensed Channel Access Mechanisms for > 52.6GHz	Apple

- R1-2004288	Channel access mechanisms for NR from 52.6-71GHz	AT&T

- R1-2004299	Consideration on supporting above 52.6GHz in NR	InterDigital, Inc.

- R1-2004300	On Channel Access Mechanisms	InterDigital, Inc.

- R1-2004301	Consideration on required changes to NR using existing NR waveform	Fujitsu

- R1-2004303	Discussions on channel access mechanism for NR from 52.6GHz to 71 GHz	Quectel

- R1-2004312	Study on the required changes to NR using existing DL/UL NR waveform	NEC

- R1-2004313	Study on the channel access mechanism	NEC

- R1-2004333	Initial views of required changes of NR DL/UL waveform	Sharp

- R1-2004334	Channel access mechanism	Sharp

- R1-2004340	Discussion on required changes to NR using existing DL/UL waveform	ITRI

- R1-2004341	Discussion on channel access mechanism	ITRI

- R1-2004417	Evaluation Methodology and Required Changes on NR on 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2004418	Channel Access Mechanism for NR in 60 GHz unlicensed spectrum	NTT DOCOMO, INC.

- R1-2004419	Potential Enhancements for NR on 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2004489	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2004500	NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz	Qualcomm Incorporated

- R1-2004519	Discussions on required changes on supporting NR from 52.6GHz to 71 GHz	CAICT

- R1-2004520	Discussions on channel access mechanism on supporting NR from 52.6GHz to 71 GHz	CAICT

- R1-2004558	Discussion on channel access for unlicensed spectrum from 52.6GHz to 71 GHz	Potevio

- R1-2004593	On NR operation between 52.6 GHz and 71 GHz	Convida Wireless

- R1-2004594	Considerations on channel access mechanism for NR supporting from 52.6 to 71 GHz	Convida Wireless

- R1-2004606	System level evaluation methodology and results for 60 GHz unlicensed operation	Huawei, HiSilicon

- R1-2004703	Summary of discussions on supporting NR from 52.6 GHz to 71 GHz	Moderator (Intel Corporation)

- R1-2004751	Skeleton TR 38.808-001: Study on supporting NR from 52.6 GHz to 71 GHz	Intel Corporation

- R1-2004754	Summary of email discussions for [101-e-NR-52_71_GHz]	Moderator (Intel Corporation)

- R1-2005003	Summary#2 of email discussions for [101-e-NR-52_71_GHz]	Moderator (Intel Corporation)

- R1-2005094	Session notes for 8.1 (Study on supporting NR from 52.6 GHz to 71 GHz)	Ad-Hoc Chair (Ericsson)

- R1-2005185	Summary of email discussions for [101-e-Post-NR-52_71_GHz]	Moderator (Intel Corporation)

- R1-2005186	Summary#2 of email discussions for [101-e-Post-NR-52_71_GHz]	Moderator (Intel Corporation)

- R1-2005191	[Draft] LS to RAN4 on Phase noise and other RF Impairment modelling	Ericsson

- R1-2005193	Summary#3 of email discussions for [101-e-Post-NR-52_71_GHz]	Moderator (Intel Corporation)

- R1-2005195	[Draft] LS to RAN4 on Phase noise and other RF Impairment modelling	Ericsson

- R1-2005196	LS to RAN4 on Phase noise and other RF Impairment modelling	RAN1

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template