# 3GPP TSG-RAN WG4 Meeting #95-e															R4-2008868

# Electronic meeting, 25 May – 5 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.107 | CR | 0002 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.107 with correction to referencies to TS 37.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eLAA |  |  |  |  |  | Date: |  |  | 2020-06-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces corrrection of referencies to TS 37.213 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
|  |  | This CR update references to RAN1 specification TS 37.213 instead of TS 36.213. The reason is that RAN1 updated TS 36.213 with reference to physical layer procedure for shared spectrum channel access. This part is not available anymore in TS 36.213 and is moved to TS 37.213 specification. Thus, references are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference to RAN1 specification will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1, 6.1.4.2, 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of R4-2007474 |  |  |  |  |  |  |  |  |

<Start of changes>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing".

[3]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[4]	Void

[5]	3GPP TS 37.213: “Physical layer procedures for shared spectrum channel access”.

<Next change section>

# 5	Channel access procedures (core part)

## 5.1	Downlink channel access procedure

For downlink operation in Band 46 and Band 49, a channel access procedure for PDSCH transmission as described in TS 37.213 [5], Clause 4.1.1 is specified.

### 5.1.1	Channel access parameters

Channel access related parameters for PDSCH are listed in Table 5.1.1-1.

Table 5.1.1-1: Channel access parameters for PDSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| LBT measurement bandwidth | MHz | 10, 20 |
| Energy detection threshold | dBm/20MHz dBm/10MHz | -72 -75 |
| Maximum channel occupancy time | ms | 8 |

### 5.1.2	Minimum requirement

The Base Station shall be able to assess whether the medium is busy or idle with at least 90% probability, using a channel access procedure with the parameters in Table 5.1.1-1.

<Next change section>

#### 6.1.4.2	Procedure

MCOT and minimum idle time

1)	Set the base station to transmit a signal according to E-TM 1.1 at manufacturer’s declared rated output power with corresponding channel bandwidth (i.e. 10 MHz or 20 MHz). Channel Access Priority Class 3 parameters are selected to be tested based on Table 4.1.1-1 in TS 37.213.

2)	Measure the transmitter ON period during the continuous transmission (after the first channel access).

3)	Measure the transmitter OFF period between two consecutive transmitter ON periods.

4)	Verify minimum idle time as follows:

The transmitter OFF period between two consecutive transmitter ON periods shall not be less than 25 µs.

5)	Verify maximum channel occupancy time (MCOT) as follows:

a)	The duration of each transmitter ON period continuous transmission shall not exceed the maximum channel occupancy time (MCOT) requirement specified in clause 6.1.5.

Energy detection accuracy

6)	Generate the interfering signal of AWGN with corresponding channel bandwidth (i.e. 10 MHz or 20 MHz) at the same centre frequency as the tested channel. The interfering signal shall be at a level of (-72dBm+ 4dB)/20MHz or (-75dBm+4dB)/10MHz for 20 MHz and 10 MHz channel bandwidth, respectively.: The base station shall stop transmission on the current operating channel and will not resume normal transmissions as long as the interference signal is present.

7)	The step 6) is repeated multiple times considering the following sub-steps:

-	Interferer ON: if the interfering signal is present, the interfering signal should be present for 10ms.

-	Interferer OFF: if the interfering signal is removed, the interfering signal should be absent for 10ms.

-	The total number of interferer ON duration is assumed to be N and the total number of interferer OFF duration is assumed to be M. The value N, M and the sequence of interferer ON/OFF pattern shall be generated randomly for the test.

8)	In the test, a counter is maintained with initial value set to 0 when the test starts.

9)	For every 10ms Interferer ON period, the counter is increased by 1 if there is either an ON/OFF transition or no transmission by the DUT. To pass the test, the counter shall not be less than N*0.9.

### 6.1.5	Test Requirements

In normal conditions, the measurement result shall meet channel access related test requirements for PDSCH as listed in Table 6.1.5-1.

Table 6.1.5-1: Channel access test requirements for PDSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| LBT measurement bandwidth | MHz | 10, 20 |
| Maximum energy detection threshold | dBm/20MHz dBm/10MHz | -72 + 4dB -75 + 4dB |
| Maximum channel occupancy time | ms | 8 |

The Base Station shall be able to assess whether the medium is busy or idle with at least 90% probability, using a channel access procedure with the parameters in Table 6.1.5-1.

<End of changes>