3GPP TSG RAN Meeting #88e	RP-201066

Electronic Meeting, 29 June – 3 July, 2020

Agenda Item:	9.3.8

Source:	Huawei, HiSilicon

Title:	Discussion on critical leftovers for Rel-16 NR V2X sidelink and timeline of Rel-17 sidelink enhancements

Document for:	Discussion and decision

# Introduction

At RAN#86, a Rel-17 WI for NR sidelink enhancement has been approved [2], with a note captured as follows [3]:

- NR Sidelink enhancements: Power efficiency, Resource allocation and potential other critical leftovers (TBD in March)

The intention of this was to allow in March 2020 a discussion of what change to the scope could be appropriate, aided by the knowledge of the first cycle of Rel-16 maintenance in RAN1, and the target core part completion date for RAN2, 3, and 4 [1]. The Rel-16 maintenance phase was extended through 202-Q2 due to the conversion to e-meetings. Now that Rel-17 is expected to begin in earnest in all RAN WGs, we provide our views on what should be considered for critical leftovers in Rel-17, and timeline of Rel-17 sidelink enhancements WI.

# Completion of Rel-16 NR V2X

RAN1 has conducted substantial amounts of maintenance in 2020-H1, and any remaining discussions are on details which are well-understood. The email workload expected in 2020-Q3 is therefore substantially lower. RAN2, 3, and 4 have completed their core part work, with normal post-completion stabilization via maintenance expected.

Observation 1: Rel-16 NR V2X has reached an appropriate level of stability and completion that allows Rel-17 sidelink enhancements work to commence in Q3 in RAN1, and Q4 in RAN2,4.

The only Rel-16 feature not standardized is “Support for simultaneous configuration of Mode and Mode 2 for a UE”, which was removed from the WI in RAN#87e. It can be discussed if this is a critical leftover warranting Rel-17 work. Apart from this, the critical leftovers for Rel-17 should consider features not standardized at all in Rel-16 and of demonstrable commercial interest.

Observation 2: Critical leftovers for Rel-17 sidelink enhancement should consider features not standardized at all in Rel-16.

# Critical leftovers of Rel-16 NR V2X

## FR2 operation

The following agreements were made in RAN1#96:

Agreements:

- RAN1 concludes the following regarding beam management:

  - Beam management is beneficial

  - RAN1 has conducted limited study on the beam management.

  - In FR1, it is feasible to support V2X use cases without beam management.

  - In FR2, it is feasible to support some V2X use cases without beam management in some scenarios.

    - Panel selection is necessary to improve the communication range in FR2.

RAN1 has studied and agreed that beam management is beneficial. However, due to limited time, there is no support of beam management in FR1 nor FR2. According to the Rel-16 WID [4], NR sidelink in FR2 is supported by applying the design for FR1 and PT-RS to the numerologies agreed for FR2. However, the remaining FR2 design except numerologies (60 kHz and 120 kHz SCS) and PT-RS is not defined. In this case, a 5G-V2X UE cannot properly operate in FR2. For example, the following agreements were made in RAN1#98:

Agreements:

- 160ms is supported as the S-SSB periodicity for all SCS.

- The number of S-SSB transmissions within one S-SSB period is (pre)configurable

  - For FR2:

    - For 60kHz SCS, {1, 2, 4, 8, 16, 32}

    - For 120kHz SCS, {1, 2, 4, 8, 16, 32, 64}

Here FR2 is considered for SSB design, but no further description will be provided relating to beam reporting/indication or other FR2 aspects in Rel-16.

Based on TS 22.186 [5], NR V2X is required to support 700-1000 Mbps data rate for higher degree of automation for extended sensors (see table in the Annex). However, given the 70 MHz bandwidth in 
band n47 and the 50 MHz bandwidth in band n38, such requirements are hard or impossible to fulfil in Rel-16. Moreover, NR has shown much interest in commercial use cases such as XR scenarios. VR-based video transmission requires Gbps-level user-experienced data rate according to TS 22.261 [6] (see table in the Annex). It is practical that XR services are also supported by NR SL to allow offloading of processing from devices with more limited processing capability and battery power, and offloading heavy data transfers to/from the cellular network. As illustrated in Figure 1, the Gbps-level video data is transmitted from a UE, e.g., cellphone, to a XR headset via NR sidelink. The large bandwidth available in FR2 is essential to support such high-resolution video streaming.

Figure 1: XR service based on NR SL in FR2

Observation 3: Sidelink FR2 development is needed to fulfill the data rate requirements for commercial sidelink use cases.

In the NR Uu interface, support of FR2 operation is based on UE capability. For those UEs that are capable of FR2 communication, it is straightforward  to use their already-mounted FR2 RF hardware with little UE complexity introduced.

Observation 4: Support of FR2 sidelink in Rel-17 can reuse already-present FR2 RF hardware, and introduces little additional UE complexity.

In NR Uu, FR2 options include both single-TRP scenarios and multi-TRP scenarios, but multi-TRP scenarios do not apply to sidelink. Also, the number of beams in sidelink scenarios can probably be limited compared with Rel-15/16 Uu design. Thus, the design of sidelink FR2 can identify which parts of the Uu FR2 design are needed, and re-use them as far as appropriate with necessary adjustments for sidelink.

Proposal 1: Support for FR2 sidelink is a critical leftover from Rel-16. It should be included in the Rel-17 NR sidelink enhancements WID, with a starting point of making necessary adaptations to those parts of the  Rel-15/16 Uu FR2 design which are relevant to sidelink.

## Support of multiple numerologies

During RAN#86, a number of automotive companies [7] showed great interest to support having multiple subcarrier spacings configurable in one sidelink carrier, and this is important for future NR V2X commercial successes to address concerns from automotive sector. One particular use-case given is to allow different SCS on different but neighboring roads to be adapted to the different vehicle speeds along each, e.g. in the region around a junction between a slow approach road and a fast freeway. This can be easily achieved by defining multiple sidelink BWPs in a sidelink carrier. A recent LS from 5GAA also suggests 3GPP to support sidelink multi-carrier operations in Rel-17 [8].

The sidelink BWP is defined in Rel-16, allowing configuration to use 15 kHz, 30 kHz or 60 kHz SCS in FR1, and 60 kHz or 120 kHz SCS in FR2. However, taking into account the time allocation in Rel-16, only one SL-BWP is configured for all the UEs in a given sidelink carrier. Although it may be suitable for NR V2X deployments whose configurations are common over a wide area, this significantly reduces the flexibility to support diverse services with different numerologies for NR sidelink compared to that for NR Uu where multiple BWP configuration is supported.

In addition, support for multiple SL-BWPs to be configured to a UE, including narrow SL-BWPs, can benefit power saving for UEs in public safety and commercial use cases where power consumption needs to be minimized.

Proposal 2: Support for multiple sidelink BWPs to be configured per sidelink carrier is a critical leftover from Rel-16, and should be included in the Rel-17 NR sidelink enhancements WID.

# Timeline of Rel-17 sidelink enhancements WI

Taken into account the substantial progress made in RAN1, RAN2, RAN3 and RAN4 Q1 & Q2 e-meetings, the critical open issues of Rel-16 features have been addressed. Thus, the Rel-17 sidelink enhancements work item can begin in RAN1 in June (RAN#88e), and RAN2,4 in September (RAN#89e).

There are some specific timings included in the WID which need revising to reflect the 1Q postponement to the WI start:

- Objective 1 (sidelink evaluation methodology update): RAN#88 → RAN#89.

- Objective 2 (study phase of mode 2 enhancements): RAN#89 → RAN#90.

  - Final decision on study phase scope: RAN#88 → RAN#89.

Proposal 3: The Rel-17 sidelink enhancements WI starts from June in RAN1, and September in RAN2,4.

Proposal 4: Adjust the per-objective target dates in the WID to reflect the 1Q postponement:

- Objective 1 (sidelink evaluation methodology update): RAN#88 → RAN#89.

- Objective 2 (study phase of mode 2 enhancements): RAN#89 → RAN#90.

  - Final decision on study phase scope: RAN#88 → RAN#89.

# Conclusions

We have discussed what limited content is the most important for consideration as critical leftovers to include in the Rel-17 sidelink enhancements WID, and the timeline of the work. This should include support for FR2 in sidelink, and configuration of multiple numerologies / SL-BWPs in a sidelink carrier. In terms of Rel-17 WI commencement, RAN1 can start work from June, and RAN2,4 from September.

Proposal 1: Support for FR2 sidelink is a critical leftover from Rel-16. It should be included in the Rel-17 NR sidelink enhancements WID, with a starting point of making necessary adaptations to those parts of the  Rel-15/16 Uu FR2 design which are relevant to sidelink.

Proposal 2: Support for multiple sidelink BWPs to be configured per sidelink carrier is a critical leftover from Rel-16, and should be included in the Rel-17 NR sidelink enhancements WID.

Proposal 3: The Rel-17 sidelink enhancements WI starts from June in RAN1, and September in RAN2,4.

Proposal 4: Adjust the per-objective target dates in the WID to reflect the 1Q postponement:

- Objective 1 (sidelink evaluation methodology update): RAN#88 → RAN#89.

- Objective 2 (study phase of mode 2 enhancements): RAN#89 → RAN#90.

  - Final decision on study phase scope: RAN#88 → RAN#89.

Observation 1: Rel-16 NR V2X has reached an appropriate level of stability and completion that allows Rel-17 sidelink enhancements work to commence in Q3 in RAN1, and Q4 in RAN2,4.

Observation 2: Critical leftovers for Rel-17 sidelink enhancement should consider features not standardized at all in Rel-16.

Observation 3: Sidelink FR2 development is needed to fulfill the data rate requirements for commercial sidelink use cases.

Observation 4: Support of FR2 sidelink in Rel-17 can reuse already-present FR2 RF hardware, and introduces little additional UE complexity.

# References

- RP-193257 “Task list for 5G V2X in RAN1#100”, LG Electronics, RAN#86, Sitges, Spain, December 2019.

- RP-193257 “New WID on NR sidelink enhancement”, LG Electronics, RAN#86, Sitges, Spain, December 2019.

- RP-193216 “Release 17 package for RAN”, RAN chairman, RAN1 chairman, RAN2 chairman, and RAN3 chairman, RAN#86, Sitges, Spain, December 2019.

- RP-200129, “Revised WID on 5G V2X with NR sidelink”, LG Electronics, RAN#87e, E-meeting, March 2020.

- 3GPP TS 22.186, “Enhancement of 3GPP support for V2X scenarios; Stage 1”.

- 3GPP TS 22.261, “Service requirements for the 5G system; Stage 1”.

- RP-193084 “Rel-17 sidelink for V2X - Automotive Perspective Challenges”, GM ATCI, Volkswagen AG, Toyota ITC, Hyundai Motors, Daimler AG, Harman GmbH, Volvo Car Corporation, RAN#86, Sitges, Spain, December 2019.

- RP-200546, “LS on the 3GPP work on the NR sidelink”, from: 5GAA WG4.

# Annex

TS 22.186, Table 5.4-1 Performance requirements for extended sensors

| Communication scenario description |  | Req # | Payload (Bytes) | Tx rate (Message /Sec) | Max end-to-end latency (ms) | Reliability (%) | Data rate (Mbps) | Min required communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Degree |  |  |  |  |  |  |  |
| Sensor information sharing between UEs supporting V2X application | Lower degree of automation | [R.5.4-001] | 1600 | 10 | 100 | 99 |  | 1000 |
|  | Higher degree of automation | [R.5.4-002] |  |  | 10 | 95 | 25 (NOTE 1) |  |
|  |  | [R.5.4-003] |  |  | 3 | 99.999 | 50 | 200 |
|  |  | [R.5.4-004] |  |  | 10 | 99.99 | 25 | 500 |
|  |  | [R.5.4-005] |  |  | 50 | 99 | 10 | 1000 |
|  |  | [R.5.4-006] (NOTE 2) |  |  | 10 | 99.99 | 1000 | 50 |
| Video sharing between UEs supporting V2X application | Lower degree of automation | [R.5.4-007] |  |  | 50 | 90 | 10 | 100 |
|  | Higher degree of automation | [R.5.4-008] |  |  | 10 | 99.99 | 700 | 200 |
|  |  | [R.5.4-009] |  |  | 10 | 99.99 | 90 | 400 |
| NOTE 1: This is peak data rate. NOTE 2: This is for imminent collision scenario. |  |  |  |  |  |  |  |  |

TS 22.261, Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max Allowed End-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0.1-[1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% in uplink and 99.9% in downlink (note 4) | - | Stationary or Pedestrian | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0.1-[1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% (note 4) | ≤ [10] | Stationary or Pedestrian | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consume VR content via tethered VR headset (note 6) | [5 -10] ms (note 5) | 0.1-[10] Gbit/s (note 5) | [99,99%] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and may be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. |  |  |  |  |  |  |