3GPP TSG-RAN WG3 Meeting #107bis-e	R3-202165

# Online, 20th – 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1479 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Wrong ASN.1 IE-Id for the New eNB UE X2AP ID Extension IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Ericsson) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The ASN.1 definition of the RETRIEVE UE CONTEXT REQUEST message contains the wrong IE-Id for the New eNB UE X2AP ID Extension IE, i.e. the id-SeNB-UE-X2AP-ID-Extension instead of the id-New-eNB-UE-X2AP-ID-Extension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As the type definition of the SeNB UE X2AP ID Extension IE is equivalent to the one of the New eNB UE X2AP ID Extension IE the ASN.1 definition was kept unchanged, but NOTEs were added to the tabular definition and the ASN.1 definition of the RETRIEVE UE CONTEXT REQUEST message to clarify how the receiving node shall interpret the received SeNB UE X2AP ID Extension IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it introduces corrects the semantics of the SeNB UE X2AP ID Extension IE contained in the ASN.1 definition of the RETRIEVE UE CONTEXT message only. The impact can be considered isolated because the change affects only the Retrieve UE Context function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASN.1 and tabular definition of the RETRIEVE UE CONTEXT REQUEST message would stay unaligned without proper explanation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.28, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.28	RETRIEVE UE CONTEXT REQUEST

This message is sent by the new eNB to request the old eNB to transfer the UE Context to the new eNB.

Direction: new eNB  old eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB See Note 1) | YES | reject |
| Resume ID | M |  | 9.2.91 |  | YES | reject |
| ShortMAC-I | M |  | BIT STRING (SIZE (16)) | RRC Resume: Corresponds to the ShortResumeMAC-I in the RRCConnection ResumeRequest message as defined in TS 36.331 [9] RRC Reestablishment: Corresponds to the ShortMAC-I in the RRCConnectionReestablishmentRequestmessage as defined in TS 36.331 [9]. | YES | reject |
| New E-UTRAN Cell Identifier | M |  | BIT STRING (SIZE (28)) | RRC Resume: Corresponds to the cellIdentity within the VarShortResumeMAC-Input as specified in TS 36.331 [9]. RRC Reestablishment: Corresponds to the cellIdentity within the VarShortMAC-Input as specified in TS 36.331 [9]. | YES | reject |
| C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRC Re-establishment Request message (TS 36.331 [9]). If this IE is present, the Resume ID IE is ignored | YES | reject |
| Failure cell PCI | O |  | INTEGER (0..503, …) | Physical Cell Identifier | YES | reject |

NOTE 1:	The ASN.1 definition of the RETRIEVE UE CONTEXT REQUEST message contains the a wrong IE-Id, which references the SeNB UE X2AP ID Extension IE instead of the New eNB UE X2AP ID Extension IE. The old eNB interprets the content of this IE as the Extended eNB UE X2AP ID, which, together with the New eNB UE X2AP ID IE represents the eNB UE X2AP ID allocated at the new eNB.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

-- Allocated at the new eNB. 
-- This IE contains an Extended eNB UE X2AP ID, which, together with the New eNB UE X2AP ID IE 
-- represents the eNB UE X2AP ID allocated at the new eNB.

{ ID id-resumeID							CRITICALITY reject	TYPE ResumeID				PRESENCE mandatory}|

{ ID id-ShortMAC-I							CRITICALITY reject	TYPE ShortMAC-I				PRESENCE mandatory}|

{ ID id-NewEUTRANCellIdentifier				CRITICALITY reject	TYPE EUTRANCellIdentifier	PRESENCE mandatory}|

{ID id-FailureCellCRNTI						CRITICALITY reject	TYPE CRNTI					PRESENCE optional}|

{ID id-FailureCellPCI						CRITICALITY reject	TYPE PCI					PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>