3GPP TSG RAN meeting #88e										RP-201273

Revision of RP-201036

Electronic Meeting, June 29th-July 3rd, 2020

Agenda item:			9.3.9

| WI / SI Name | NR Mobility Enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_Mob_enh |  |  |  |  |
| Unique ID | 800087 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-192534 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | 06/2020 | Performance part: 12/2020 (updated from 09/2020) | Testing part: N/A |  |
| Overall Completion level | Study Item: xx % | Core part: 100% | Performance Part: 10% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Yi Guo |
|  | Company | Intel Corporation |
|  | Email | yi.guo@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

It is proposed to extend core part completion date from March 2020 to June 2020 and add the TUs for related working groups as following:

- xxx

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

### 2.1.1	Agreements

#### 2.1.1.1	RAN1#100bis-e Agreements

Decision: As per email decision posted on May 4th,

Agreement:

- Adopt the following text proposal for Section 15 of TS 38.213

----------- Start text proposal---------------

If

- the UE does not provide UplinkPowerSharingDAPS-HO, and

- UE transmissions on the target cell and the source cell overlap

the UE transmits only on the target cell, and cancels the transmission to source cell after [the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0. ]

A UE does not expect to cancel a transmission on the source cell [in symbols from the set of symbols] that occur, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than the [ PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0]

------------ End text proposal

Decision (May 5th): As the primary changes are all in brackets, proposed TP is not captured it in the editor’s CR. Update (either simply removing the brackets, or refining the sentences) is necessary in next meeting.

Decision: As per email decision posted on May 1st,

Agreement:

- gNB can configure for the UE a specific power sharing mode for DAPS

  - It is assumed that gNB shall only enable a power sharing mode for DAPS among the power sharing1modes that the UE indicated support of.

- gNB can disable power sharing between target and source MCG

  - no power sharing between target and source MCG can be indicated by gNB not configuring UplinkPowerSharingDAPS-HO-mode.

No corresponding TP at this point. Revisit in next meeting.

Decision: As per email decision posted on May 1st,

Agreement:

Previous agreement (achieved on Apr.24th) is updated as follows:

- RAN1 assumes the UE is not required to use interference cancellation basedNo additional receiver performance requirements will be defined for a UE receivingto receive a PDCCH and/or a PDSCH from both a source and a target cell in resources that overlap in time and frequency. Whether and how to define requirement would be up to RAN4.

- Note: RAN1 assumes that it is up to gNB to correctly configure MCS and resources to combat for interference in receiving signals during DAPS HO.

Agreement:

- LS to RAN4 on Simultaneous reception of DL signals in intra-frequency DAPS HO is approved in R1-2003058.

Decision: As per email decision posted on Apr.25th, agreements made from conference calls:

Agreement: Replace the existing FG 21-1 with the following two new FGs 21-1a and 21-1b

| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | Per BC | No | N/A | N/A |  | [Optional with capability signalling] |

- Components, components candidate values and other details are FFS

Decision: As per email decision posted on May 1st,

Agreement:

| 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 | DAPS (Note: RAN2 feature) | Yes | N/A | FFS: The UE is only able to drop the transmission to the source. | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 | 21-2 | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing | 21-2 | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |

Decision: As per email decision posted on Apr.27th, agreements made from conference calls:

Agreement:

- Delete FG 21-3 from the Rel. 16 NR UE feature list

#### 2.1.1.2	RAN1#101-e Agreements

Decision: As per email decision posted on June 05th

Agreement:

- Adopt TP#8 in R1-2004748 for Clause 15 of TS 38.213.

If the UE indicates support for UplinkPowerSharingDAPS-HO = Semistatic-mode1 semi-static power sharing mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates support for UplinkPowerSharingDAPS-HO = Semistatic-mode2 semi-static power sharing mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates support forUplinkPowerSharingDAPS-HO = Dynamic dynamic power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

Decision: As per email decision posted on May 28th

Agreement:

- Adopt TP#2 and TP#4 in R1-2004749 for Clause 15 of TS 38.213

For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively. The UE can provide pdcch-BlindDetectionMCG1-UE to indicate a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellstarget downlink cells for the target MCG and pdcch-BlindDetectionMCG2-UE to indicate a capability monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellssource downlink cells for the source MCG. If the UE is provided search space sets on both the target MCG and the source MCG, the UE does not expect to have in any slot any USS set without allocated PDCCH candidates for monitoring on both the target MCG and the source MCG.

The UE can provide pdcch-BlindDetectionMCG1-UE to indicate a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellstarget downlink cells for the target MCG and pdcch-BlindDetectionMCG2-UE to indicate a capability monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellssource downlink cells for the source MCG. If the UE is provided search space sets on both the target MCG and the source MCG, the UE does not expect to have in any slot any USS set without allocated PDCCH candidates for monitoring on both the target MCG and the source MCG simultaneously.

Decision: As per email decision posted on June 05th

Agreement:

- Introduce the following new FG 21-2d for inter-frequency DAPS-HO and adopt UL transmission based cancellation.

- Update the intra-frequency DAPS HO to include UL transmission cancellation feature.

| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. 3) Support of cancelling UL transmission to the source cell for intra-frequency DAPS-HO | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-2d | UL transmission cancellation | Indicates support of cancelling UL transmission to the source cell for inter-frequency DAPS-HO | 21-1b | Yes | N/A | UE does not support scheduling of overlapping PUSCH/PUCCH/SRS transmissions to source and target cells for inter-frequency DAPS-HO | per band combination | No | N/A | N/A |  | Optional with capability signalling |

Agreement:

Adopt TP#1-16 of R1- R1-2004957 for Clause 15 of TS38.213

If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell, and cancels the transmission to source cell after the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0. A UE does not expect to cancel a transmission to the source cell in symbols that occur, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0.

Agreement:

Adopt TP#2-1 of R1-2004957 for Clause 15 of TS38.213

15 Dual active protocol stack based handover <---------------------------Other parts are omitted -------------------------------> For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlapping in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.

Agreement:

Adopt TP#2-6 of R1-2004957 for Clause 15 of TS38.213

For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlap in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. For DAPS operation in a same frequency band, a UE does not transmit PRACH to the in active UL BWP of source MCG in a same slot when the transmission would overlapping in time with a PUSCH/PUCCH/SRS transmission to the target MCG or when a gap between the first or last symbol of a PUSCH/PUCCH/SRS transmission in active UL BWP of to the target MCG in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PRACH transmission in active UL BWP of to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.

Decision: As per email decision posted on May 31st

Agreement:

| 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 | DAPS (Note: RAN2 feature) | Yes | N/A | FFS: The UE is only able to drop the transmission to the source. UE is not expected to simultaneously transmit PRACH/PUSCH/PUCCH/SRS to source and target cell that overlap in time domain | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Decision: As per email decision posted on June 02nd

Agreement:

The following text proposal is adopted

15 Dual active protocol stack based handover <unchanged text omitted> The UE expects to be provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2 only for synchronous DAPS HO operation [10, TS 38.133]. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

### 2.1.2	Remaining Open issues

## 2.2	RAN2

### 2.2.1	Agreements

#### 2.2.1.1 RAN2#109bis-e Agreements

In principle agreed CRs: R2-2003857 (TS38.300), R2-2003855 (TS38.321), R2-2003853 (TS38.323), R2-2003850 (TS38.331)

CHO:

More than one measurement object can’t be configured for a candidate cell when 2 triggering events (MeasId) are used for CHO execution condition (condExecutionCond). Field description of condExecutionCond shall be updated accordingly.

We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact.

Use ‘’Conditional reconfiguration’’ consistently within RRC.

Rely on existing Stage-2 text that UE stops evaluating execution condition and capture nothing additional in NR/LTE RRC specification about CHO execution conditions after the CHO condition is met and CHO execution is started.

RRC procedure is not extended with a condition checking if T304 is running before the UE executes conditional reconfiguration.

RRC procedure in 5.3.5.13.4 is not extended with additional subclauses to make sure the condition is not fulfilled or leaving condition is not checked when unnecessary.

Update section 9.2.3.4.2 of TS 38.300 with a NOTE: ‘’source gNB can reconfigure the UE even after providing CHO configuration for candidate target cells’’ .

UE does not release otherConfig including delayBudgetReportingConfig and overheatingAssistanceConfig and does not release MCG SCells if the UE was configured with conditionalReconfiguration and the selected cell during reestablishment is a CHO candidate cell.

Section 5.3.10.3 (Detection of radio link failure) of TS 38.331 is updated with the subclause “the UE stops condition reconfiguration evaluation when MCG Failure Information is submitted”.

DAPS:

S3.9: RACH is allowed to source after RACH towards target is successful but it is up to RAN1 whether something is specified for the source RA + target UL collisions or left up to UE implementation. (No more RAN2 discussion on this until RAN1 decides.)

S2.1-1: All the functions in Figure 4.2.2-1 will be supported by the source and target MAC entity in DAPS HO.

S2.1-2: UE switches the UL PDCP data transmission upon successful RACH procedure (i.e. Msg.B for 2-step RACH).

S3.7-2: Forbid data transmission of non-DAPS DRBs in MSG3 for CBRA.

S2.6-2: Keep original agreement that RLC UM (UL/DL) with PDCP SN number continuity is supported for DAPS.

S2.2-1-1: The PDCP status report (to avoid packet duplication) for DL UM DRBs is needed for DAPS HO.

S3.2: PDCP status report for UM is mandatory to support for DAPS capable UE.

S3.2b: Secondary PDCP status report for AM is mandatory to support for DAPS capable UE.

S2.2-2-1: The second PDCP status report for DL UM DRBs is not needed for DAPS HO.

S2.6-5-1: Reordering_PDCP_RX_COUNT used for AM DRB reordering is needed for DAPS AM DRB.

S2.6-5-2: Last_Submitted_PDCP_RX_SN and Reordering_PDCP_RX_COUNT used for AM DRB reordering are needed for DAPS UM DRB.

S2.6-5-4: Last_Submitted_PDCP_RX_SN is set to [(Next_PDCP_RX_SN-1) modulo (Maximum_PDCP_SN+1)] upon PDCP reconfiguration for LTE UM DRB from normal PDCP to DAPS PDCP.

S2.6-5-5: For the change from DAPS PDCP to the normal PDCP upon the source release, the reordering function is still maintained.

S2.6-3: Do not introduce discard indication in source from PDCP to RLC upon UL switching.

S3.7-3: Discard timer is maintained during DAPS HO:

S2.3-5-2: For DAPS HO, capture PDCP handling for SRB in PDCP specification, the detailed text can be further discussed when capture it in PDCP specification.

S2.6-1: Leave the issue on uplink duplicated PDCP SDUs to RAN3.

S2.6-4: Leave the disucssion on PDCP anchor relocation in DAPS to RAN3.

RAN2 to progress solution to avoid that same key stream is applied to retransmitted SDUs with different ROHC compression headers. (Companies should bring contributions to next meeting)

1	Do not introduce special handling on PDCP status report to support DAPS HO.

2	Reordering_PDCP_RX_COUNT is set to the COUNT value associated to RX_HFN and Next_PDCP_RX_SN upon PDCP reconfiguration for LTE UM DRB and LTE AM DRB without reordering from normal PDCP to DAPS PDCP.

3	Do not support PHR reporting in another node

4	How to handle compressed PDCP SDUs stored in the reception buffer at PDCP re-establishment is not discussed in Rel-16 anymore

T312 in source is stopped upon executing a reconfiguration with sync even if DAPS is configured; No specificiation impact.

Do not introduce bye message from UE to the source upon UL switching

LTE DAPS+ LTE RACH-less is not allowed

We stick to legacy that source only provides one configuration to target in Rel-16.

RRC S2.2-1: Condition for statusReportRequired should be changed to Rlc-AM-UM “For RLC AM or RLC UM ( if dapsConfig is configured for this bearer), the field is optionally present, need R. Otherwise, the field is absent.”.

RRC S2.3-2: moreThanoneRLC is not applied for DAPS HO, remove the EN “FFS on moreThanonRLC in pdcp-Config” and clarify in the field description “This field is not present if dapsConfig is configured for this bearer.”

RRC S2.3-5-3: For DAPS HO, reestablishPDCP is not needed for SRB, no matter whether key is changed or not.

RRC S2.3-1: Do not capture in specification “stop RLM in source after RACH successful to target PCell”, and remove the EN “TBC on how/whether to capture stop RLM in source after RACH successful to target PCell”.

RRC S3.3: Agree below RRC changes:

3> consider radio link failure to be detected for the source MCG i.e. source RLF;

43> suspend all DRBs in the source;

43> release the source connection.

RRC S2.3-3: Agree below principle on the terminoligy and to be confirmed in ASN.1 review, e.g. whether to change source/target to source/target MCG;

Case 1 L1 configuration: “source or target" should be used since it is cell specific configuration;

Case 2 MAC/RLC/PDCP (Key, security/ROHC)/SDAP configuration: “source or target" could be used since they are common for all cells of source or target;

Case 3 C-RNTI, timers (e.g. T301, T310, T311) and constants (e.g. N310, N311): “source/target SpCell” should be used since it is PCell configuration;

Case 4 BCCH/MIB (5.3.5.5.2): “source/target SpCell” should be used since it is PCell configuration;

Case 5 RLF, and “revert back to the configuration used in source PCell”: “source/target SpCell” should be used since we only RLF in PCell instead of SCells;

Case 6 “revert back to the configuration used in source PCell”: “source PCell” could be used as legacy;

Case 7 SRB/DRB, RRM: “source or target" could be used since they are common for all cells of source or target;

RRC S2.5-1: To capture RAN1 parameters p-DAPS-FR1, p-DAPS-FR2 and 	UplinkPowerSharingDAPS-HO-mode and name them as “p-DAPS-Source, p-DAPS-Target and UplinkPowerSharingDAPS-HO-mode”

RRC S2.5-2: powerControlMode in HO preparation message ischanged to ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

RRC S3.4-1: Do not add 2> If dapsConfig is configured for any DRB when capturing UL switching indication in RRC;

RRC S3.4-2: To discuss whether to UL switching indication in RRC as

3> for each DRB configured with dapsConfig, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

RRC S2.3-8-1: When resume SRB upon DAPS HO failure, the old stored RRC message if any, (i.e.. the PDCP PDUs for SRB) shall be discarded;

RRC S3.5: Do not try to align the handling of SRB and non-DAPS DRB upon receiving DAPS HO command and upon fallback;

RRC S3.7-1: For non-DAPS DRB handling, do not agree that PDCP only reestablishment when RACH is successfully completed in target:

1 After DAPS handover, the network shall release the source first before triggering the new handover procedure.

2  Reuse existing bit statusReportRequired to control second PDCP status report for DAPS AM DRB.

3  Capture RRC S3.6 as the changes made in R2-2003372, i.e. without changing agreed SRB handling“establish PDCP for target and no PDCP reestablishment”.

4  Align the terminology of “DAPS” between PDCP and RRC as shown in R2-2002860:

- Remove “DAPS PDCP entity” and “normal PDCP entity” from the specification.

- Use “DAPS bearer” in the specification.

- Remove the text regarding association between PDCP entity and the RLC entity.

5  revise the text “if dapsConfig is configured for any DRB:” to “if any DAPS bearer is configured”.

7  maxSCH-TB-BitsDL/ maxSCH-TB-BitsUL are not needed for NR since for NR the supported max DL/UL data rate for each CC can be derived from the L1 parameters included in the FeatureSet (according to the calculation defined in 38.306 4.1), i.e. do not introduce maxSCH-TB-BitsDL/ maxSCH-TB-BitsUL for NR.

8  Agree below changes if no objections from companies:

2>	if the sdap-Config is included and when indication of successful completion of random access towards target cell is received from lower layers as specified in [3]:

CPC:

1	The UE does not inform the MN when CPC execution condition is fulfilled and the UE starts executing CPC, when CPC configuration is provided over SRB3.

2	A threshold parameter is not introduced to determine PCell quality for execution of CPC.

3 	Upon transmission of SCG failure information to the network, the UE stops evaluating the CPC execution criteria according to the current CPC configuration until a response is received from the network.

4 	Whether the UE continue measurements for candidate PSCells configured for execution condition upon CPC failure is left to the UE implementation.

5	The content of FailureReportSCG for CPC procedure failure should include failureType, measResultFreqList and measuResultSCG-Failure. These parameters are set according to the exiting SCGFailureInformation procedure. (same as legacy)

7	Use ULInformationTransferMRDC instead of RRCReconfigurationComplete message to inform the network of CPC execution when no SRB3 is configured and the MN informs the SN, i.e. ULInformationTransferMRDC message to MN includes an embedded RRCReconfigurationComplete message to the SN. This applies to both NR MN and LTE MN. (change of previous agreement).

1   If CPC configuration is not released by network, the UE autonomously releases the stored CPC configuration upon the SCG release.

2	measID and reportConfig associated with CPC config, and measObject(s) only associated to CPC shall be autonomously removed by UE when SCG is released.

4	Support of CPC configuration (CPC condition + CPC reconfiguration) in legacy HO command or CPC configuration in CPC configuration should not be considered in Rel-16.

ASN.1 issues:

Z251: Change status to ConcAgree with below changes in 5.3.5.1:

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit L2 indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

-	reconfiguration with sync for DAPS and security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: refresh of security and re-establishment of RLC and PDCP triggered by explicit L2 indicators;

- 	for DAPS bearer: establishment of target RLC, refresh of security and re-configure PDCP to add the ciphering function and the integrity protection function of target;

- 	for SRB: refresh of security and establishment of target RLC and target PDCP;

-	reconfiguration with sync for DAPS but without security key refresh, involving RA to the target PCell, establishment of target MAC, and:

-	for non-DAPS bearer: RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

- 	for DAPS bearer: establishment of target RLC, re-configure PDCP to add the ciphering function and the integrity protection function of target;

- 	for SRB: establishment of target RLC and target PDCP;

H058: Change status to ConcReject.

Z253: Change status to ConcAgree with below changes in 5.3.5.3:

2>	discard the keys used in source (the KgNB key, the S-KgNB key, the S-KeNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

S350: Change status to ConcAgree for P3 with below changes in 5.3.5.5.2:

2>	for each DRB without dapsHO-Config:

3>	associate the RLC entity, and the associated logical channel, to the target PCell;

Z256: Change status to ConcNoAction (covered by S350).

Z257: Change status to ConcAgree with the changes in 5.3.5.5.2 as below:

2>	for each SRB:

3>	establish an RLC entity or entities for the target, with the same configurations as for the source;

Z258: Change status to ConcAgree with the changes in 5.3.5.5.4 as below:

1>	if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity within the same cell group:

2>	if dapsConfig is configured for this bearer the RLC bearer is associated with an DAPS bearer:

H223: Change status to ConcReject with the changes in 6.3.2 to tag-Config as below:

tag-Config

The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured

H224: Change status to ConcReject.

Z259: Change status to ConcReject.

Z260: Change status to ConcAgree with the changes in 5.3.5.6.5 as below:

►	4>	configure the ciphering function of target for the DAPS PDCP entity with the ciphering algorithm according to securityConfig and apply the KUPenc key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received from target and sent to target by the UE;

Z261: Change status to ConcReject.

Z262: Change status to ConcAgree Z262 with the changes in 5.3.5.6.5 as below:

►	4>	configure the integrity protection function of target for the DAPS PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

C001: Change status to ConcAgree with the changes in 5.3.5.8.3 as below:

3>	revert back to the UE RRMmeasurement configuration used in the source;

Z263: Change status to ConcAgree with the changes in 5.3.10.3 as below:

4>	suspend the transmission of all DRBs in the source;

N007: Change status to ConcAgree with the changes in 6.2.2 as below:

daps-SourceRelease

Indicates the UE to release the source to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

H060: Change status to ConcAgree with the changes in 6.3.2 as below:

drb-ContinueROHC

Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if any DAPS bearer is configured.

H061: Change status to ConcReject.

G004: Change status to ConcAgree with the changes in 7.1.1 as below:

For T304 of MCG, in case of the handover from NR or intra-NR handover, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source, initiate the failure information procedure.

For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3.

Proposal-Z252: Change status to ConcAgree with the changes in 5.3.5.2 as below:

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended;

- the conditionalReconfiguration for CPC is included only when at least one RLC bearer is setup in SCG;

-	the conditionalReconfiguration for CHO is included only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.

Proposal-S302: Change status to ConcAgree S302 with the changes in 5.3.5.13.4 as below:

2> if trigger conditions for all event(s) associated to all measId(s) within condTriggerConfig are fulfilled for all associated measId(s) in condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

Proposal-S301: Change status to ConcAgree S301 with the changes in 5.3.7.2, 5.3.7.3 as below:

1>	release the MCG SCell(s), if configured;

1>	if UE is not configured with conditionalReconfiguration:

2>	release spCellConfig, if configured;

2>	suspend all RBs, except SRB0;

Section 5.3.7.3

2> if UE is configured with conditionalReconfiguration:

3> release spCellConfig, if configured;

3> suspend all RBs, except SRB0;

3> release the MCG SCell(s), if configured;;

C002: Change status to ConcAgree with the changes in 5.5.3.1 as below:

For conditional configuration execution triggering quantities, the network can configure up to 2 quantities, both using same RS type.

Z264: Change status to ConcReject.

Z272: Change status to ConcAgree with the changes in 6.3.2 as below:

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell.

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI.

Event I1:	Interference becomes higher than absolute threshold.

Z254: Change status to ConcNoAction since it has been solved in R2-2003799.

Z269: Change status to ConcAgree with the changes in 7.1.1 as below:

If T312 is configured  in MCG: Upon triggering a measurement report for a measurement identity for which T312 has been configured and useT312 has been set to true, while T310 in PCell is running.

If T312 is configured in SCG: Upon triggering a measurement report for a measurement identity for which T312 has been configured and useT312 has been set to true, while T310 in PSCell is running.

C003: Change status to ConcAgree with the changes in 7.1.1 as below:

Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon initiating the connection re-establishment procedure, upon the reconfiguration of rlf-TimersAndConstant, and upon the expiry of T310 in corresponding SpCell.

Upon SCG release, if the T312 is kept in SCG.

Z270: Change status to ConcAgree with the changes in 7.1.1 as below:

If the T312 is kept in MCG: If security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure.

If the T312 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure.as specified in 5.7.3.

#### 2.2.1.2 RAN2#110-e Agreements

From RAN2 perspective, the NR mobility WI is considered completed (UE capabilities may require corrections in the next meeting).

Agreed CRs: R2-2004518 (TS38.300), R2-2005758 (TS38.323), R2-2005755 (TS38.331), R2-2006379 (TS37.340), R2-2006435 (TS38.300),

R2-2005760 (TS38.321)	.

Capabilities:

R2-2005762	UE Capability for Rel-16 NR mobility enhancement

	Endorsed, To be merged to the capability mega-CR

R2-2005763	UE Capability for Rel-16 NR mobility enhancement

	Endorsed, To be merged to the capability mega-CR

R2-2005786	UE capabilities for RAN1 feature list

	The CR is endorsed for mobility capabilities

R2-2005785	UE capabilities for RAN1 feature list

	The CR is endorsed for mobility capabilities

No LS for now (assume RAN1/4 will get the general LS on RAN2 decisions on UE capabilities). Companies can raise issues in the respective WGs if needed.

1	singleUL-TransmissionDAPS can be supported by default (i.e. no bit needed)

2 	syncDAPS can be supported by default (i.e. no bit needed)

3	Introduce IOT bit for intraFreqTwoTAGs-DAPS.

Retain intra-requency DAPS capabilities in the CR.

1	the CHO capable UE must support maximum 8 candidate cells;

4	the CPC capable UE must support maximum 8 candidate cells;

2	For CHO, introduce additional IOT bit (i.e. mandatory with capability) on the support of 2 trigger events for same execution condition. This feature is mandatory for UEs supporting CHO (as per definition of IOT bits).

5	For CPC, introduce additional IOT bit (i.e. mandatory with capability) on the support of 2 trigger events for same execution condition. This feature is mandatory for UEs supporting CPC (as per definition of IOT bits).

3	Introduce capability bit (e.g. cpc-r16) to indicate the support of CPC;

6	For CHO/CPC, introduce separate capabilities FDD-to-TDD (and vice versa) CHO/CPC and FR1-to-FR2 (and vice versa) CHO/CPC;

12a introduce separate capabilities for intraFreq and interFreq as below:

Per Band/per BC (for intraFreq capabilities), I.e. put under BandParameters-v16xy:

intraFreqDiffSCS-DAPS-r16;

intraFreqAsyncDAPS-r16

intraFreqMultiUL-TransmissionDAPS-r16

Per BC (for interFreq capabilities), i.e. put under CA-ParametersNR-v16xy:

interFreqDiffSCS-DAPS-r16

interFreqAsyncDAPS-r16

interFreqMultiUL-TransmissionDAPS-r16.

12b	All UEs supporting DAPS support these capabilities (can discuss signalling details and naming):

SyncDAPS-r16

SingleUL-TransmissionDAPS-r16

intraFreqTwoTAGs-DAPS-r16  (with 2 TAGs)

(for interFreq since RAN2 agreed to “Reuse CA capability “supportedNumberTAG” for DAPS handover.)

8a	Remove UplinkPowerSharingDAPS-HO

8b	Add separate capabilities for 21-2, 21-2a, 21-2b as semiStaticPowerSharingDAPS-Mode1, semiStaticPowerSharingDAPS-Mode2 and dynamicPowersharingDAPS.

8c	RAN2 thinks that these apply only for multiple UL supporting UEs,

10	Remove pdcch-BlindDetectionSource and pdcch-BlindDetectionTarget from RAN2 agreed capabilities.

11	Add syncDAPS and simultaneous UL transmission based on RAN4 latest capability table.

13	Introduce separate capabilities for intraFreq and interFreq for power sharing capabilities.

CHO/CPC:

Fast MCG recovery and CHO coexistence is not optimized any further during Rel-16 (i.e. it is not supported and we take no actions in the CR anymore).

3	Changes related to SN release upon CHO execution are not pursued in Rel-16.

1	Change the CHO-related text in TS 38.300 (section 9.2.3.4.1) and say the evaluation is stopped when ‘handover is triggered’, not when ‘the execution condition is met’ (as proposed in  R2-2005344). Can discuss the exact wording.

4	TS 37.340 (section 10.6.1, for conditional PSCell change) is modified by stating the UE stops evaluating the execution conditions once ‘PSCell change is triggered’, instead of once ‘the execution condition is met.

5	Modify conditionalReconfiguration field description by adding a restriction CPC configuration cannot be provided in the legacy PSCell change command.

7	In case of SRB3, the UE does not send a CPC (RRC Reconfiguration) complete message to the source PSCell (SN) upon CPC execution.

DAPS:

At DAPS handover failure, upon fallback to source cell, for each SRB, the UE discards any PDCP SDUs along with the PDCP data PDUs.

2	At DAPS handover failure, upon fallback to source cell the UE performs RLC re-establishment for each SRB.

3	Inter-RAT handover during DAPS handover before release of source cell is not allowed in Rel-16.

4	Releasing UE to INACTIVE via RRCRelease during DAPS handover before release of source cell is not allowed in Rel-16.

5	When UE triggers RRC connection re-establishment, it releases the previous source cell resources, when applicable.

Do not specify any special handling for RoHC when security key is not updated in DAPS handover in Rel-16. (This means that changing security key ensures no problems occur, but it’s up to network implementation.) Add a NOTE to RRC/PDCP specification on this.

For downlink, the ROHC protocol of the target cell should maintain the IR state if operating in U-mode and O-mode during DAPS handover. This can be captured in a NOTE in PDCP specification.

ASN.1 review:

I103: Agree I103 to add “2> release source PCell configuration;”in 5.3.5.3.

I107: Agree I107, to combine the conditions “If any DAPS bearer is configured:” and “2>	for each SRB:” together in 5.3.5.6.3:

I109: Agree I109, to remove “the S-KgNB key, the S-KeNB key,” from 5.3.5.8.3:

I111: Agree I111 to add the field description for configRestrictInfoDAPS: “Includes fields for which souce cell explictly indicates the restriction to be observed by target cell during DAPS handover.”

I101: Agree I101 to remove the note “This step is performed so the UE only performs conditional reconfiguration execution while timer T311 is running once for a given failure detection .” from 5.3.5.3.

I105: Agree I105, to move the NOTE3 together with Note 1, 2  in 5.3.5.5.2:

I100: Change to ConcReject.

I104:  Change to ConcReject.

S303: Change to ConcAgree: Use Cond PCell for field attemptCondReconfig-r16 with

“The field is optional present, need N, if conditionalReconfiguration is added for CHO. Otherwise the field is not present.”

Z274: ConcAgree. To capture the changes based on Z274.

E231: ConcAgree.

M201: ConcAgree ; follow RRC Rapporteur’s view

Z275: ConcAgree. To capture the changes based on Z275.

H458:ConcReject.

Z276: ConcAgree. But double check the proposed changes.

G103: ConcReject.

J031: ConcReject.

B105: ConcReject.

E038: ConcReject.

B107: Not related to MOB WI.

H455: ConcReject.

Z278: CPC cannot be configure in PSCell change command.

C210: leave the discussion to SON/MDT WI.

S309: ConcAgree.

S305: ConcReject.

E234: ConcAgree.

O201: ConcReject.

H462: ConcAgree.

X007: ConcReject.

S308: ConcReject.

J030: Change to PropAgree

G104: ConcConcReject

I113: Remove the below EN, and keep current CR as it is.

Editor's note: It is FFS if the whole handling on release of spcellConfig, MCG SCells, etc shall be moved to under 1>	else: in 5.3.7.3, i.e. release when reestablishment is triggered.

S304: ConcReject

Z277: ConcAgree based on Alt. 1: The UE stops conditional reconfiguration evaluation for CPC upon submission of MCGFailureInformation, if configured.

Z273: Change to ConcReject : Do not introduce the definition of Non-DAPS bearer or changes to DAPS bearer definition.

RAN2 understanding is that DAPS HO is considered successful after RA completion. DAPS HO is considered complete after source release. Update conclusion of H460 accordingly.

J032: ConcReject.

S307: ConcAgree.

S306: ConcReject.

X004: ConcAgree: Intent is agreeable, but need to discuss how to clarify the wording (current condition may cover this)

X005: ConcAgree (add MAC reset)

S350: ConcReject (no change to specification; This doesn’t preclude UEs from having different implementations as long as they follows the specification)

UE autonomously deletes CPC configurations upon PCell change in Rel-16 (i.e. no change to CR)

I113, Since both solutions are seen equivalent, we keep existing text in the CR.

1	Complete the specification text for DAPS HO in 38.331, where only “source” or “target” is used, with the entities that are referred to.

2	Include the Text Proposal in Annex A to TS 38.331 (can modify to be consistent with terminology)

R2-2005997, Partly covered by ASN.1 summary discussion (on Z273); Parts not covered can be considered in the next meeting

R2-2004669	Stop condition on T310 (C003)

Endorsed

To be merged to running NR RRC CR.

1	The condition for inclusion of the statusReportRequired field for RLC-UM radio bearers should be changed so that it is not restricted to when the bearer is configured for DAPS. It should instead be restricted to when the UE supports DAPS.

2	I114: ConcAgree.

1	The UE does not perform MR-DC release and does not release UE configuration if the UE was configured with conditionalReconfiguration and the selected cell during re-establishment is a CHO candidate cell.

2	Apply the same criteria as in 1 for also other configurations in LTE to ensure delta signalling works. This can be done during CR implementation.

MAC issues:

1	Consider whether “target MAC entity” is a good way to indicate and use it consistently.

2	Discuss in the MAC CR email discussion [213] how to capture this.

### 2.2.2	Remaining Open issues

## 2.3	RAN3

### 2.3.1	Agreements

#### 2.3.1.1 RAN3#107bis-e Agreements

Following CRs have been endorsed as baseline:

-	38.300 (R3-202714, Intel Corporation)

-	37.340 (R3-202575, ZTE, CATT, NTT DOCOMO, Google, China Telecom)

-	38.401 (R3-201587, Intel Corporation, CATT, ZTE, Huawei)

-	38.410 (R3-202452, CATT, CMCC)

-	38.413 (R3-202676, CATT, Nokia, Nokia Shanghai Bell)

-	38.420 (R3-201545, Intel Corporation)

-	38.423 (R3-201588, Nokia, Nokia Shanghai Bell, Intel Corporation)

-	38.470 (R3-202574, ZTE, China Telecom, China Unicom)

-	38.473 (R3-202796, Ericsson)

Clean-up on BL CRs:

RAN3 agreed:

-	TP (R3-202578, ZTE) for TS 38.473 : Refine the Usage of UE CONTEXT MODIFICATION for Candidate Cell Modify and Cancel

-	TP (R3-202579, ZTE) for TS 38.473 : Further Clean-ups for Abnormal Conditions etc

-	TP (R3-202734, Nokia) for TS 38.423 : Clean-up of the BL CR to XnAP

HO interruption reduction:

RAN3 agreed:

-	DAPS handling over X2/Xn/E1:

Do not introduce one shot DAPS HO proposal from source. DAPS HO proposal is per E-RAB/DRB

TP (R3-202716, Intel Corporation) for TS 38.423 : DAPS HO handling during preparation

-	DAPS handling over S1/NG:

Align with Xn/X2 on whether DAPS Response Information per DRB or one shot and related TP may be provided in next meeting

Define 2 new class-2 messages for COUNT value transfer

Reuse the HO Notify and add new indicator for target node to AMF/MME

Liaise SA2/CT4 about S1/NG DAPS HO

FFS on whether the COUNT of DL Discarding is included in the S1/NG Early Status Transfer

TP (R3-202660, CATT) for TS 38.413 : Remaining Issues for DAPS HO

LS on S1/NG DAPS handover (R3-202845, To SA2/CT4)

-	PDCP SN continuity for RLC-UM:

The reset of PDCP SN/HFN for RLC-UM bearers at DAPS HO shall be performed for UL and DL. No TP nor LS is needed

Handover robustness improvements:

RAN3 agreed:

-	Conditional PSCell Change:

Use DL RRC MESSAGE TRANSFER to carry CHO command to the source gNB-DU, instead of using the UE Context Modification procedure.

For intra-NR, add a note that the gNB-CU’s UE Context Modification procedure (steps 14-15) could happen earlier than carrying the RRCReconfigurationComplete message (steps 12-13). Apply in the same way for EN-DC scenario.

Remove FFS and use SgNB Reconfiguration Complete.

TP (R3-202296, Intel Corporation) for TS 38.423 : Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios

TP (R3-202871, Intel Corporation) for TS 38.401 : Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios

-	Avoiding Simultaneous CHO and Conditional PSCell Change:

Address RAN2’s request to avoid CHO and CPC configuration in the UE

TP (R3-202762, ZTE, China Telecom, China Unicom) for TS 37.340 : Further Discussion of Parallel CHO and CPC Simultaneous Operation

-	CHO preparation:

S1/NG CHO is not supported in rel-16

TP (R3-202614, ZTE, CATT, China Telecom) for TS 38.423 : Further Clarification of Maximum Number of CHO Preparations

TP (R3-202780, Ericsson) for TS 37.340 : CHO in MR-DC operation

-	CHO modification:

TP (R3-202753, Nokia) for TS 38.423 : Enabling keeping 2 configs at the target

-	CHO cancellation:

TP (R3-202672, Vodafone, Nokia, British Telecom, Ericsson) for TS 38.423 : Implementation of the arrival probability

-	E1 aspects:

CU-UP resources are cell agnostic, i.e. establishing only one Bearer Context for all the candidate target cells per UP entity.

Data Forwarding specific:

RAN3 agreed:

-	CHO specific:

It is re-confirmed that the source decides when to start forwarding. The target shall always try processing early forwarded packets as much as it can.

FFS whether it is necessary to have a mechanism to indicate the target’s buffer/load situation for the source to take into account for early forwarding, with respect to the following:

- What is different to late forwarded packets is that early forwarded packets (with PDCP SNs assigned) don’t get sent to the UE when received. They are buffered until the UE accessed. Even if dropped due to buffer maxed out, the source neither knows whether dropped or not nor re-forwards them. FFS whether such SN gap could be critical when the UE accesses later.

- It is true that the target’s buffer can be relieved by the Discarding DL COUNT sent from the source. But currently there is no way for the source to know the situation, whether it needs to slow down forwarding or send Discarding DL COUNT more frequently to help relieve the target’s buffer.

TP (R3-202718, Intel Corporation) for TS 38.401 : ): Early Forwarding Transfer needs CU-UP level distinction

-	E1 aspects:

FFS whether autonomous report for Discarding DL COUNT from the source CU-UP is needed

#### 2.3.1.2 RAN3#108-e Agreements

Following CRs have been endorsed as baseline:

-	38.300 (R3-203058, Intel Corporation)

-	37.340 (R3-203078, ZTE, CATT, NTT DOCOMO, Google, China Telecom)

-	38.401 (R3-203079, Intel Corporation, CATT, ZTE, Huawei)

-	38.410 (R3-203033, CATT, CMCC)

-	38.413 (R3-203100, CATT, Nokia, Nokia Shanghai Bell)

-	38.420 (R3-203010, Intel Corporation)

-	38.423 (R3-203080, Nokia, Nokia Shanghai Bell, Intel Corporation)

-	38.470 (R3-203034, ZTE, China Telecom, China Unicom)

-	38.473 (R3-203081, Ericsson)

-	38.460 (R3-204283, ZTE, China Telecom, China Unicom)

-	38.463 (R3-204329, Ericsson, Intel Corporation)

Clean-up on BL CRs:

RAN3 agreed:

-	TP (R3-203763, Intel Corporation) for TS 38.401 : Clean-up from the last agreed R3-202823

-	TP (R3-204123, ZTE) for TS 38.423 : Refine XnAP Abnormal Conditions etc

HO interruption reduction:

RAN3 agreed:

-	DAPS handling over X2/Xn:

The DAPS response is per-DRB and an explicit code-point “DAPS HO not accepted” is used (fallback to MBB is not used)

The name of the Early Forwarding Transfer will be changed in X2/Xn to Early Status Transfer.

TP (R3-204301, Intel Corporation) for TS 38.423 : DAPS HO handling during preparation

TP (R3-204293, Nokia) for TS 38.423 : Change of the name of the Early Forwarding Transfer procedure

-	DAPS handling over S1/NG:

DAPS HO capability of the CN is detectable based on the criticality of the Message Type of the RAN Uplink Early Status Transfer.

The option for discarding will be removed in S1/NG.

HO Notify is confirmed to be used in S1/NG.

The location and the format of the DAPS HO response indication are to correspond to the way the same IE is used on Xn/X2 (CB # 34): the DAPS response is per-DRB and an explicit code-point “DAPS HO not accepted” is used (fallback to MBB is not used).

TP (R3-204297, Ericsson) for TS 38.413 : Support of NG DAPS HO

-	DAPS handling over E1:

The target CU-CP indicates “DAPS HO” per DRB, when requesting the target CU-UP to establish a DAPS requested DRB and the corresponding DL DRB forwarding tunnel.

There is no point in having “DAPS response” from the target CU-UP. The target CU-UP either accepts the DRB and establishes DL DRB forwarding tunnel as requested, or reject it.

TP (R3-204332, Intel Corporation) for TS 38.463 : DAPS HO handling during preparation

TP (R3-204227, Intel Corporation) for TS 38.401 : Correction for DAPS HO on inter-gNB-DU mobility scenarios

-	PDCP SN continuity for RLC-UM:

TP (R3-203548, Samsung, Ericsson) for TS 38.300 : Clarification of supporting PDCP SN continuity for RLC-UM bearer in DAPS HO

Handover robustness improvements:

RAN3 agreed:

-	Conditional PSCell Change:

TP (R3-204176, Google) for TS 38.401 : CPC Complete message correction scenarios

-	Avoiding Simultaneous CHO and Conditional PSCell Change:

No signalling solution specified in TS in Rel-16.

-	CHO MR-DC:

TP (R3-204148, Ericsson) for TS 38.423 : CHO in MR-DC operation

TP (R3-204149, Ericsson) for TS 37.340 : CHO in MR-DC operation

-	CHO modification:

A new cause value (Resources prepared for CHO or CPC are to be changed.) is used for CHO/CPC resource change notification on X2, Xn and F1.

TP (R3-204184, Huawei) for TS 38.423 : CHO Modification

TP (R3-204233, ZTE) for TS 38.473 : Introduction of New Cause Value for gNB-DU Initiated CHO Modify&Re-setup

-	CHO cancellation:

TP (R3-204163, NEC) for TS 38.423 : Conditional HO Cancel procedure and HO Success procedure

-	F1 aspects:

Introduce new class-2 procedure: “Access Success”  over F1

TP (R3-204228, Intel Corporation, ZTE, NEC) for TS 38.473 : F1 needs a new class-2 message like HO SUCCESS

TP (R3-204229, Intel Corporation, ZTE, NEC) for TS 38.401 : F1 needs a new class-2 message like HO SUCCESS

TP (R3-203218, ZTE, Intel Corporation) for TS 38.470 : Introduction of Access Success Procedure

TP (R3-204232, Intel Corporation, NEC) for TS 38.423 : HO SUCCESS alignment with F1 ACCESS SUCCESS

-	E1 aspects:

Agree to add CHO E1 impact stage 2 in 38.460.

Add Clarification for inactivity monitoring when CHO performed in stage 2 38.401

Adopt option2 one shot setup + option3 parallel setup for CHO E1 specification in 38.401 and 38.463.

TP (R3-204285, Intel Corporation) for TS 38.401 : CB: # 42_MobEnh_CHO_E1

TP (R3-204333, Intel Corporation, CATT, Ericsson, NEC) for TS 38.463 : CB: # 42_MobEnh_CHO_E1

Data Forwarding specific:

RAN3 agreed:

-	E1 aspects:

New IEs are introduced for Bearer Context Modification proc. to retrieve/provide DL COUNT values related for early data forwarding with the source/target CU-UPs. Old IEs are not re-used.

Autonomous reporting for Discarding DL COUNT from the source CU-UP is confirmed.

TP (R3-204334, Intel Corporation, ZTE, Google, CATT, Huawei, Samsung, NEC, LGE) for TS 38.463 : Early Forwarding support for DAPS/CHO over E1

### 2.3.2	Remaining Open issues

## 2.4	RAN4

### 2.4.1	Agreements

#### 2.4.1.1 RAN4#94bis-e Agreements

- Issue 1-1: Interruption in intra-frequency DAPS HO D1 for 60KHz

Agreement: 4 slots

- Issue 1-2: interruption when UL Tx overlap in time

Agreement: no need to capture this in RAN4 specification since corresponding UE behaviour has already been captured in RAN1 specification.

- Issue 1-3: different interruption during Delay (1) and (2)

Agreement: different interruption during Delay (1) and (2) has already been considered and reflected in existing specification. RAN4 will not revisit previous agreement

- Issue 1-4: Power imbalance in side condition for interruption requirements

Agreement: RAN4 will not capture side condition of power imbalance in core requirement. The issue can be discussed in performance part.

- Issue 1-8: RRM measurement during DAPS handover:

Agreement: it is up to UE implementation whether to perform RRM measurement on the frequencies of source cell and target cell during DAPS handover procedure.

- Side condition for sync DAPS HO regarding MRTD/MTTD at UE side is to be introduced. If side condition is not met, async DAPS HO is assumed.

- The followings will be decided in RAN4#-95-e

  - Whether the existing interruption requirements for intra-frequency DAPS HO are only applied for sync scenario.

  - Whether the existing interruption requirements for intra-band inter-frequency DAPS HO are only applied for sync scenario.

- Side condition for intra-frequency sync DAPS HO:

  - UE which only support sync DAPS HO shall meet existing DAPS HO requirement as long as:

    - Option 1: it is assumed that source and target cells are co-located.

      - 3 us MRTD and 5.21 us MTTD between source and target cells is not exceeded: 
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

    - Option 2: it is assumed that source and target cells can be non-co-located.

      - 33 us MRTD and 34.6 us MTTD between source and target cells is not exceeded
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

    - Option 3: specify “Tight Sync” and “Loose Sync” for sync DAPS handover capabilities.

      - Tight Sync : 3 us MRTD and 5.21 us MTTD between source and target cells is not exceeded: 
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

      - Loose Sync : 33 us MRTD and 34.6 us MTTD between source and target cells is not exceeded
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

- Side condition for intra-band inter-frequency sync DAPS HO:

  - UE which only support sync DAPS HO shall meet existing DAPS HO requirement as long as:

    - Option 1: it is assumed that source and target cells are co-located.

      - 3 us MRTD and 5.21 us MTTD between source and target cells is not exceeded:
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

    - Option 2: it is assumed that source and target cells can be non-co-located.

      - 33 us MRTD and 34.6 us MTTD between source and target cells is not exceeded
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

    - Option 3: specify “Tight Sync” and “Loose Sync” for sync DAPS handover capabilities.

      - Tight Sync : 3 us MRTD and 5.21 us MTTD between source and target cells is not exceeded: 
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

      - Loose Sync : 33 us MRTD and 34.6 us MTTD between source and target cells is not exceeded
Note: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

- Side condition for FR1-FR1 inter-band sync DAPS HO:

  - UE which only support sync DAPS HO shall meet existing DAPS HO requirement as long as:

    - maximum of 33 us MRTD and 34.6 us MTTD between source and target cells is not exceeded.

    - Side condition for FR1-FR2 inter-band sync DAPS HO:

  - UE which only support sync DAPS HO shall meet existing DAPS HO requirement as long as:

    - maximum of 25 us MRTD and 26.1 us MTTD between source and target cells is not exceeded.

- Other conditions for intra-frequency DAPS HO:

Intra-frequency DAPS HO requirements apply only if

  - the centre frequency of the SSB of the source cell and the centre frequency of the SSB of the target cell are the same, and

  - the subcarrier spacing of the two SSBs are also the same

  - the active BWP of source cell contains the initial BWP of source cell.

  - the active BWP of target cell contains the initial BWP of target cell.

  - active DL/UP BWPs of target cell is confined within the active DL/UL BWPs of source cell

#### 2.4.1.1 RAN4#95-e Agreements

- Issue 1-4 Definition of asynchronous DAPS HO for intra-frequency or intra-band scenarios shall be based on:

- Agreement: If side condition for sync is not met, async DAPS HO is assumed. (agreement in RNA4#94-e-bis)

- Issue 1-5 Additional note for sync condition:

- Agreement: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. (agreement in RNA4#94-e-bis)

- Issue 1-6 Response to RAN1 LS (R1-2003058):

- Agreement: RAN4 doesn’t need to reply RAN1 LS

- Issue 1-7 LS to RAN2 on misalignment between RAN2 and RAN4 on the following capabilities:

- Agreement: Since RAN2 is already aware of this, it is not necessary to send this LS.

- Issue 1-1: interruption requirements for async intra-frequency DAPS HO.

- Agreement: one more slot of interruption is allowed compared to sync scenario

- Issue 1-2: interruption requirements for async intra-band inter-frequency DAPS HO.

- Agreement: one more slot of interruption is allowed compared to sync scenario

- Issue 1-3: definition of synchronous DAPS HO for intra-frequency or intra-band scenarios shall be based on: 6 us MRTD and 7.6 us MTTD between source and target cells.

Note 1: If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.

Note 2: A UE is not expected to transmit in the uplink NRx-Tx earlier than after the end of the last received downlink symbol in the same or different cell where NRx-Tx is given by Table below.

Note 3: A UE is not expected to receive in the downlink NTx-Rx earlier than after the end of the last transmitted uplink symbol in the same or different cell where NTx-Rx is given by Table below.

| Transition time | FR1 |
| --- | --- |
| NTx-Rx | 25600 |
| NRx-Tx | 25600 |

Test case list for DAPS HO and CHO

| Test case number | Test purpose | Description | Responsible company |
| --- | --- | --- | --- |
| 1 | Intra-frequency sync DAPS handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Qualcomm |
| 2 | Intra-frequency async DAPS handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Qualcomm |
| 3 | Intra-band Inter-frequency sync DAPS handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Intel |
| 4 | Intra-band Inter-frequency async DAPS handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Intel |
| 5 | Inter-band Inter-frequency sync DAPS handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Huawei |
| 6 | Inter-band Inter-frequency async DAPS handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Huawei |
| 7 | Conditional intrafrequency handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Ericsson |
| 8 | Conditional interfrequency handover test in SA for FR1 | See table in section 4.5 in R4-2009020 | Ericsson |
| 9 | Inter-frequency sync DAPS handover test in SA for FR1-FR2 | See table in section 4.5 in R4-2009020 | Huawei |
| 10 | Inter-frequency async DAPS handover test in SA for FR1-FR2 | See table in section 4.5 in R4-2009020 | Huawei |
| 11 | Conditional intrafrequency handover test in SA for FR2 | See table in section 4.5 in R4-2009020 | Ericsson |
| 12 | Conditional interfrequency handover test in SA for FR2 | See table in section 4.5 in R4-2009020 | Ericsson |

- Applicability rule: the UE only supporting sync DAPS HO is not required to test async cases.

### 2.4.2	Remaining Open issues

For core part:

-	No open issues

For performance part:

-	Test cases for DAPS HO and CHO

## 2.5	RAN5

### 2.5.1	Agreements

### 2.5.2	Remaining Open issues

### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

### 2.6.1	Agreements

### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

### 3.1.1	Agreements with cross-TSG impacts

### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2001530	Remaining issues on DAPS-HO	Huawei, HiSilicon

- R1-2001531	Remaining PHY aspects for CHO	Huawei, HiSilicon

- R1-2001624	Remaining issues on NR mobility enhancements in physical layer	ZTE

- R1-2001625	Discussion on FR2 mobility interruption enhancements	ZTE

- R1-2001633	Discussion on UE feature for NR mobility enhancements	ZTE

- R1-2001832	Views on Rel-16 UE features for NR mobility enhancements	MediaTek Inc.

- R1-2001870	Summary on UE features for NR mobility enhancements	Moderator (AT&T)

- R1-2002010	Issue Summary for NR Mobility Enhancements	Moderator (Intel Corporation)

- R1-2002011	Corrections to Physical layer aspects of NR mobility enhancement	Intel Corporation

- R1-2002023	UE feature for NR Mobility Enhancement	Intel Corporation

- R1-2002148	Remaining issues for NR Mobility Enhancement	Samsung

- R1-2002157	UE features for NR mobility enhancement	Samsung

- R1-2002221	Remaining physical layer aspects of dual active protocol stack based HO	Nokia, Nokia Shanghai Bell

- R1-2002344	On remaining issues on NR mobility enhancements	Apple

- R1-2002354	Views on NR mobility ehancement UE feature	Apple

- R1-2002478	On UE features for NR Mobility Enhancements	Nokia, Nokia Shanghai Bell

- R1-2002490	Correction to UL power sharing for DAPS HO	Ericsson

- R1-2002495	Mobility UE features	Ericsson

- R1-2002558	Maintenance for NR mobility enhancements	Qualcomm Incorporated

- R1-2002570	Discussion on Mobility Enhancements UE features	Qualcomm Incorporated

- R1-2002594	Rel-16 UE features for mobility enhancement	Huawei, HiSilicon

- R1-2002806	Summary of email discussions for [100b-e-NR-Mob-Enh-01]	Moderator (Intel Corporation)

- R1-2002807	Summary of email discussions for [100b-e-NR-Mob-Enh-02]	Moderator (Intel Corporation)

- R1-2002808	Summary of email discussions for [100b-e-NR-Mob-Enh-03]	Moderator (Intel Corporation)

- R1-2002809	[draft] LS on Simultaneous reception of DL signals in intra-frequency DAPS HO	Intel Corporation

- R1-2003058	LS on Simultaneous reception of DL signals in intra-frequency DAPS HO	RAN1, Intel Corporation

- R1-2003076	Draft CR on UL transmission cancellation for UL DAPS HO	Intel Corporation

- R1-2003080	Chairman's Notes of AI 7.2.11.9	Ad-Hoc Chair (AT&T)

- R1-2003104	Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-01]	Moderator (AT&T

- R1-2003105	Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-02]	Moderator (AT&T)

- R1-2003106	Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-03]	Moderator (AT&T)

- R1-2003107	Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-04]	Moderator (AT&T)

- R1-2003130	Maintenance of NR Mobility Enhancements	Ad-Hoc Chair (Ericsson)

- R1-2003137	Draft CR on UL transmission cancellation for UL DAPS HO	Intel CorporationDsf

- R1-2003330	Remaining issues on NR mobility enhancements in physical layer	ZTE

- R1-2003331	Discussion on FR2 mobility interruption enhancements	ZTE

- R1-2003334	Discussion on UE feature for NR mobility enhancements	ZTE

- R1-2003506	Remaining issues on DAPS-HO	Huawei, HiSilicon

- R1-2003676	Remaining issues on Physical Layer Aspects for  DAPS-HO	MediaTek Inc.

- R1-2003678	Views on Rel-16 UE features for NR mobility enhancements	MediaTek Inc.

- R1-2003747	Issue Summary for NR Mobility Enhancements	Moderator (Intel Corporation)

- R1-2003748	Corrections to Physical layer aspects of NR mobility enhancement	Intel Corporation

- R1-2003759	UE feature for NR Mobility Enhancement	Intel Corporation

- R1-2003890	Remaining issues for NR Mobility Enhancement	Samsung

- R1-2003900	UE features for NR mobility enhancement	Samsung

- R1-2004148	Remaining PHY aspects for CHO	Huawei, HiSilicon

- R1-2004149	Rel-16 UE features for mobility enhancement	Huawei, HiSilicon

- R1-2004202	Remaining issues on mobility enhancements	Ericsson

- R1-2004203	UE features for mobility enhancements	Ericsson

- R1-2004235	On remaining issues on NR mobility enhancements	Apple

- R1-2004245	Views on NR mobility ehancement UE feature	Apple

- R1-2004287	Summary of UE features for NR mobility enhancements	Moderator (AT&T)

- R1-2004484	Discussion on Mobility Enhancements UE features	Qualcomm Incorporated

- R1-2004567	On UE features for NR Mobility Enhancements	Nokia, Nokia Shanghai Bell

- R1-2004580	Remaining physical layer aspects of dual active protocol stack based HO	Nokia, Nokia Shanghai Bell

- R1-2004747	Summary of email discussions for [101-e-NR-Mob-Enh-01]	Moderator (Intel Corporation)

- R1-2004748	Summary of email discussions for [101-e-NR-Mob-Enh-02]	Moderator (Intel Corporation)

- R1-2004749	Summary of email discussions for [101-e-NR-Mob-Enh-03]	Moderator (Intel Corporation)

- R1-2004957	Summary#2 of email discussions for [101-e-NR-Mob-Enh-01]	Moderator (Intel Corporation)

- R1-2004989	Draft CR on correction on PDCCH monitoring for DAPS HO	Intel Corporation

- R1-2004990	Draft CR on correction on power sharing mode for DAPS HO	Intel Corporation

- R1-2004991	Draft CR on correction on overlapping uplink transmissions for DAPS HO	Intel Corporation

- R1-2005040	Summary of email discussion/approval [101-e-NR-Mob_enh-UEFeatures-01]	Moderator (AT&T)

- R1-2005041	Summary of email discussion/approval [101-e-NR-Mob_enh-UEFeatures-02]	Moderator (AT&T)

- R1-2005042	Summary of email discussion/approval [101-e-NR-Mob_enh-UEFeatures-03]	Moderator (AT&T)

- R1-2005043	Summary of email discussion/approval [101-e-NR-Mob_enh-UEFeatures-04]	Moderator (AT&T)

- R1-2005092	Session notes for 7.2.9 (Maintenance of NR Mobility Enhancements)	Ad-Hoc Chair (Ericsson)

- R2-2002744	Corrections to Mobility Enhancements	Nokia, Intel Corporation (Rapporteurs)

- R2-2003368	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

- R2-2003369	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

- R2-2003105	E-mail discussion report [Post109bis-e#12][MOB] Resolving open issues for CHO	Nokia, Nokia Shanghai Bell

- R2-2002748	On measurement and evaluation during CHO execution	Futurewei

- R2-2002900	T304 running issue when CHO Execution	LG Electronics Inc.

- R2-2002951	Discussion of some remaining issues for CHO	OPPO

- R2-2002996	Corrections to conditional reconfiguration evaluation	PANASONIC R&D Center Germany

- R2-2003035	CHO and MR-DC operation	Ericsson

- R2-2003106	MCG recovery versus recovery via CHO - Rel-16 impact	Nokia, Nokia Shanghai Bell

- R2-2003260	Further details of CHO configuration and execution	China Telecom

- R2-2003333	Clarification on source reconfigiration during CHO 	Samsung

- R2-2003422	Further consideration on conventional HO overriding a CHO command	ZTE Corporation, Sanechips

- R2-2003577	Discussion on leftovers for CHO	Huawei, HiSilicon

- R2-2003609	UE configuration release in RRC reestbalishment	SHARP

- R2-2003847	Report from [AT109bis-e][207][MOB] Open CHO issues	Nokia, Nokia Shanghai Bell

- R2-2003578	Discussion on T312 support	Huawei, HiSilicon

- R2-2002599	Discussions on VarRLF-Report Setting	Quectel

- R2-2002901	Failure handling of both CHO and MR-DC	LG Electronics Inc.

- R2-2003036	Failure handling interaction	Ericsson

- R2-2003848	Offline discussion 208: Finalization of T312 for Fast handover failure recovery	Samsung (Offline rapporteur)

- R2-2002902	Consideration on CHO capability	LG Electronics Inc.

- R2-2003037	UE capabilities for CHO	Ericsson

- R2-2003579	Discussion on UE capabilities for CHO and T312	Huawei, HiSilicon

- R2-2003440	Report of [post109bis-e@13][NR MOB] Resolving open issues for CPC	CATT

- R2-2003441	Draft CR for transmission of RRCReconfigurationComplete upon CPC execution	CATT

- R2-2003799	Draft CR for transmission of RRCReconfigurationComplete upon CPC execution	CATT

- R2-2003442	Draft CR for transmission of RRCReconfigurationComplete upon CPC execution	CATT

- R2-2002749	Clarifications on issues of CPC-intra-SN	Futurewei

- R2-2002800	CPC with SRB3 Configuration	Apple

- R2-2002903	Left Issues for CPC in R16	LG Electronics Inc.

- R2-2003038	Remaining issues for conditional PSCell change	Ericsson

- R2-2003100	Remaining issues for CPC	Lenovo, Motorola Mobility

- R2-2003107	On how to close the open issues for Conditional PSCell Change	Nokia, Nokia Shanghai Bell

- R2-2003327	Discussion on CPC configuration handling during SCG Release	Samsung

- R2-2003423	Remaining issues for CPC	ZTE Corporation, Sanechips

- R2-2003580	Discussion the transaction id issues for CPAC	Huawei, HiSilicon

- R2-2003849	Report of [AT109bis-e][209][NR MOB] Resolution to remaining open issues of CPC	CATT (offline discussion rapporteur)

- R2-2002904	Consideration on CPC capability	LG Electronics Inc.

- R2-2003039	UE capabilities for conditional PSCell change	Ericsson

- R2-2003581	Discussion on UE capabilities for CPC	Huawei, HiSilicon

- R2-2003326	[S350] Discussion on radio bearer handling in DAPS	Samsung

- R2-2003424	[Z255] Correction for Pcell change in case of CPC	ZTE Corporation, Sanechips

- R2-2003664	[H223] Correction on TAG configuration applied to target cell	Huawei, HiSilicon

- R2-2003844	Report of [AT109bis-e][210][MOB ASN1] ASN.1 discussion for LTE and NR mobility (Intel/Ericsson)	Intel Corporation, Ericsson

- R2-2003371	Report of 109b#11 open issues on DAPS	Intel Corporation

- R2-2002590	Open issues for user plane aspects of DAPS HO	Ericsson

- R2-2003330	On Remaining Issues for DAPS UP	Nokia, Nokia Shanghai Bell

- R2-2002874	Remaining user plane issues of DAPS	vivo

- R2-2003045	Discussion on transmitting ROHC IR packets in target during DAPS HO	Huawei, HiSilicon, Vivo, Oppo, NEC, Apple, NTT DOCOMO INC., China Telecom

- R2-2003665	RoHC handling for inter-gNB and intra-gNB DAPS handover	SHARP Corporation

- R2-2002589	RoHC handling during DAPS handover without key change	Ericsson

- R2-2002863	Discussion on consecutive ROHC failure	LG Electronics Inc.

- R2-2002864	Handling of compressed PDCP SDUs stored in reception buffer	LG Electronics Inc.

- R2-2002608	PDCP Status Reporting enhancements for DAPS DRBs 	Qualcomm India Pvt Ltd

- R2-2002737	PDCP Status Report for UM DRBs in DAPS HO	MediaTek Inc.

- R2-2002953	Discussion on PDCP status report for UM DRB	OPPO

- R2-2002997	Handling of security issue for DAPS without key change	NEC

- R2-2003042	Discussion on DAPS HO without key change	Huawei, HiSilicon

- R2-2002799	Non-DAPS DRB Handling when fallback to source	Apple

- R2-2003845	Report of [AT109bis-e][205][MOB] Flagging and discussion of DAPS UP open issues for PDCP/RLC/MAC (Huawei)		Huawei, HiSilicon

- R2-2002869	CR on 38.321 for NR mobility enhancement	vivo

- R2-2003855	CR on 38.321 for NR mobility enhancement	vivo

- R2-2003043	PDCP CR on correction and outcome of [Post109bis-e#11] for DAPS handover	Huawei, HiSilicon, Mediatek Inc.

- R2-2003853 	PDCP CR on correction and outcome of [Post109bis-e#11] for DAPS handover	Huawei, HiSilicon, Mediatek Inc.

- R2-2003371	Report of 109b#11 open issues on DAPS	Intel Corporation

- R2-2003372	38.331 CR on NR MOB	Intel Corporation

- R2-2002860	Clean up the terminology for RRC and PDCP	LG Electronics Inc, Nokia, Nokia Shanghai Bell

- R2-2002591	Subsequent RRC Procedures after DAPS handover	Ericsson

- R2-2002875	Remaining control plane issues of DAPS	vivo

- R2-2002952	Correction on DAPS HO	OPPO

- R2-2003046	Discussion on control plane aspects of DAPS HO	Huawei, HiSilicon

- R2-2003108	Remaining control plane issues for DAPS	Nokia, Nokia Shanghai Bell

- R2-2003502	Discussion on network coordination and PHR report for DAPS HO	CMCC.

- R2-2003530	Indication of DAPS Handover Execution to the Source	ETRI

- R2-2003530	Report of [AT109bis-e][206][MOB] Flagging and discussion of DAPS CP open issues for RRC (Intel)	Intel Corporation

- R2-2002592	Inter-node signalling for DAPS handover	Ericsson

- R2-2002905	Consideration on DAPS Capability	LG Electronics Inc.

- R2-2003367	Discussion on  capabilities for MOB	Intel Corporation

- R2-2004355	LS on Simultaneous reception of DL signals in intra-frequency DAPS HO (R1-2003058; contact: Intel)	RAN1

- R2-2004662	Corrections on NR mobility enhancements (109b-927)	Intel Corporation

- R2-2004518	Corrections to Mobility Enhancements	Nokia, Intel Corporation (Rapporteurs)

- R2-2004670	Corrections on NR mobility enhancements	Intel Corporation

- R2-2004914	Correction on CHO failure handling	OPPO

- R2-2005755	Corrections to Rel-16 NR mobility enhancement	Intel Corporation

- R2-2005344	On stopping evaluating execution condition once triggering the legacy HO	ZTE Corporation, Sanechips

- R2-2005682	CHO Evaluating Handling during Legacy HO	LG Electronics Inc.

- R2-2005681	Stage 2 CR for CHO Evaluating Handling during Legacy HO	LG Electronics Inc.

- R2-2005380	Discussion on leftovers for CHO	Huawei, HiSilicon

- R2-2005456	Further consideration on CHO in MR-DC operation	CMCC

- R2-2005754	Summary of discussion [209] on CHO/CPC	Nokia

- R2-2005071	Introduction of Conditional PSCell Change for intra-SN without MN involvement	CATT

- R2-2005345	Remaining issues for CPC	ZTE Corporation, Sanechips

- R2-2005381	Discussion on leftovers for CPC	Huawei, HiSilicon

- R2-2005279	Corrections on procedure for CPC complete	Futurewei

- R2-2005762	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

- R2-2005763	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

- R2-2005784	UE capabilities for RAN1 feature list	Intel Corporation

- R2-2005786	UE capabilities for RAN1 feature list	Intel

- R2-2005785	UE capabilities for RAN1 feature list	Intel

- R2-2005779	Summary of discussion][214][MOB] UE capability CRs for NR mobility (Intel)	Intel Corporation

- R2-2004663	[109b#930] UE capabilities for NR mobility	Intel Corporation

- R2-2005311	Report of email discussion [Post109bis-e][963][NR16] UE capabilities	Intel Corporation, NTT DoCoMo

- R2-2004664	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

- R2-2004665	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

- R2-2005061	Discussion on UE capabilities for NR DAPS	Huawei, HiSilicon

- R2-2005160	UE capabilities for Mobility Enhancements WI	Nokia, Nokia Shanghai Bell

- R2-2005457	Discussion on the maxinum CPC candidates	CMCC

- R2-2004917	Discussion on UE capability for CHO and CPC	OPPO

- R2-2005684	Consideration on Conditional mobility capability	LG Electronics Inc.

- R2-2005751	Summary of discussion [207] on NR mobility ASN.1 review	Intel

- R2-2004661	Phase 1 class 2 issues on MOB WI (I101, I103, I104, I105, I107, I109, I100, S303, I111)	Intel Corporation

- R2-2004672	Phase 2 MOB RIL issues	Intel Corporation

- R2-2005512	[J033] RoHC handling with and without key change at the UE	SHARP Corporation

- R2-2004427	Clarification on tag-Config for DAPS (subject to [H223])	Samsung

- R2-2004666	Phase 1 open issue on DAPS CP (S350, I112)	Intel Corporation

- R2-2005064	[I112] discussion on RLC re-establishment upon fallback	Huawei, HiSilicon

- R2-2005708	[S350] Discussion on radio bearer handling during DAPS	Samsung Electronics

- R2-2005062	[S350] Discussion on reconfiguration procedure in DAPS	Huawei, HiSilicon

- R2-2004668	Phase 1 Open issue on  CPC (Z255)	Intel Corporation

- R2-2005348	[Z255] Further discussion on the handling of stored CPC configuration	ZTE Corporation, Sanechips

- R2-2004620	Remaining issues for conditional PSCell change	Ericsson

- R2-2004667	Phase 1 open issue on CHO (I113)	Intel Corporation

- R2-2005065	[I113] Discussion on handling CHO candidate cells upon RRC re-establishment	Huawei, HiSilicon

- R2-2004619	Re-establishment initiation and CHO	Ericsson

- R2-2004915	[O201] Correction on dapsConfig	OPPO

- R2-2005349	Clarification on not supporting CHO+DAPS	ZTE Corporation, Sanechips

- R2-2004693	[E232] Source and target entities at DAPS HO	Ericsson

- R2-2005997	TP on DAPS terminology related ASN.1 review issues (ao Z258)	Samsung Telecommunications

- R2-2005668	[S304] Clarification on applicable cell in CHO	Samsung R&D Institute UK

- R2-2005382	[C003] T312 discussion	Huawei, HiSilicon

- R2-2004669	Stop condition on T310 (C003)  Intel

- R2-2005683	Draft CR for Clarification to release CPC when SCG Release	LG Electronics Inc.

- R2-2004649	Disabling multi-leg RB for DAPS	vivo

- R2-2005430	[J030, J031] UE DAPS configuration release upon RLF	SHARP

- R2-2005529	[G104] Clarification on DAPS handover failure while the T310 is running	Google Inc.

- R2-2005134	[B105] TP for DAPS handover with fast MCG link recovery	Lenovo, Motorola Mobility

- R2-2005383	[H458] Triggering quantity discussion	Huawei, HiSilicon

- R2-2005511	[G103] Clarification on CHO handling during RRC connection re-establishment procedure	Google Inc.

- R2-2005346	[Z276] Discussion on UE configuration release in RRC re-establishment	ZTE Corporation, Sanechips

- R2-2005347	[Z277] Discussion on stopping conditional reconfiguration evaluation during fast MCG recovery	ZTE Corporation, Sanechips

- R2-2005612	Draft CR on 38.321 for NR mobility enhancement	LG Electronics

- R2-2004644	CR on 36.321 for LTE feMob	vivo	CR	Rel-16	36.321	16.0.0	1474	-	B	LTE_feMob-Core

- R2-2004645	CR on 38.321 for NR mobility enhancement	vivo	CR	Rel-16	38.321	16.0.0	0744	-	B	NR_Mob_enh-Core

- R2-2005058	CR on 38.323 for NR mobility enhancement	Huawei, HiSilicon, Mediatek Inc., LG Electronics

- R2-2005758	CR on 38.323 for NR mobility enhancement	Huawei, HiSilicon, Mediatek Inc., LG Electronics

- R2-2005761	CR on 38.321 for NR mobility enhancement	vivo

- R2-2004699	Open issues for control plane aspects of DAPS handover	Ericsson

- R2-2004896	Discussion on old stored RRC message handling upon DAPS HO failure	OPPO

- R2-2005513	Remaining issues on fallback from DAPS handover failure	SHARP Corporation

- R2-2005060	Discussion on DAPS CP remaining issue	Huawei, HiSilicon

- R2-2004648	Handling of the source SRB at DAPS failure	vivo

- R2-2005497	Handling of RLC for SRBs	LG Electronics Inc.

- R2-2004697	RoHC feedback to source cell after UL transmission switch	Ericsson

- R2-2004878	Compromised solutions for ROHC related security issue	Samsung

- R2-2004563	ROHC Handling for DAPS Handover without Key Change	MediaTek Inc.

- R2-2004788	Solutions for security issue in case of DAPS without key change	NEC

- R2-2005500	ROHC handling for DAPS HO without security key change	LG Electronics Inc.

- R2-2004916	Discussion on ROHC handling in DAPS HO	OPPO

- R2-2004947	DAPS handover UP remaining key issues	Qualcomm India Pvt Ltd

- R2-2004698	RoHC handling during DAPS handover without key change	Ericsson

- R2-2005056	Discussion on ROHC handling in DAPS HO without key change	Huawei, HiSilicon

- R2-2005057	Discussion on transmitting ROHC IR packets in target during DAPS HO	Huawei, HiSilicon, Vivo, Oppo, Apple, China Telecom, Samsung, LG Electronics, CATT, CMCC, Mediatek Inc., LG Uplus

- R2-2005161	Target cell’s ROHC behaviour for DAPS handover	Nokia, Nokia Shanghai Bell, Ericsson, Intel Corporation, NEC

- R2-2005448	Discussion of remaining issues for DAPS HO	CMCC

- R2-2004787	Handling of expiry of DataInactivityTimer for DAPS	NEC

- R2-2005685	Consideration on DAPS Capability	LG Electronics Inc.

- R2-2004621	Updates for R16 LTE Mobility Enhancements and LTE updates for R16 NR Mobility Enhancements	Ericsson	CR	Rel-16	36.331	16.0.0	4290	1	F	LTE_feMob-Core	R2-2003852

- R2-2004695	[E928][I114] Condition for setting statusReportRequired for RLC UM	Ericsson, Intel Corporation

- R2-2005350	[Z263] Discussion on UE configuration release in RRC re-establishment	ZTE Corporation, Sanechips

- R3-201545	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation

- R3-201554	TS38.470 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-201565	TS37.340 Stage2 Introduction of Rel-16 Mobility Enhancement in MR-DC	ZTE, CATT, NTT DOCOMO, INC., Google, China Telecom

- R3-201572	Baseline CR for introducing Rel-16 NR mobility enhancement	CATT, Nokia, Nokia Shanghai Bell

- R3-201586	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation

- R3-201587	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation, CATT, ZTE, Huawei

- R3-201588	Baseline CR for introducing Rel-16 NR mobility enhancement	Nokia, Nokia Shanghai Bell, Intel Corporation

- R3-201589	Baseline CR for introducing Rel-16 NR mobility enhancement	Ericsson, CATT

- R3-201638	TP for NR_Mob_enh BL CR for TS 38.423 Correction of Mis-usage of SN Status Transfer Message	ZTE

- R3-201653	(TP for NR_Mob_enh BL CR for TS 38.423): Clean-up of the BL CR	Nokia, Nokia Shanghai Bell

- R3-201654	(TP for NR_Mob_enh BL CR for TS 38.423): Completion of the open points related to the modification of a CHO	Nokia, Nokia Shanghai Bell

- R3-201656	Avoiding simultaneous CHO and CPC	Nokia, Nokia Shanghai Bell

- R3-201657	(TP for NR_Mob_enh BL CR for TS 38.423): A solution to enable the MN to block CPC usage	Nokia, Nokia Shanghai Bell

- R3-201659	Response LS on avoiding simultaneous CHO and CPC	Nokia, Nokia Shanghai Bell

- R3-201660	Continuation of the work on CHO for NG- and S1-based handover (incl. TPs for NGAP and S1AP BL CRs)	Nokia, Nokia Shanghai Bell

- R3-201670	(TP for NR_Mob_enh BL CR for TS 38.401) Clarification of Multiple APID Usage in CHO and CPC	ZTE

- R3-201672	(TP for NR_Mob_enh BL CR for TS 38.423) Further Clarification of Maximum Number of CHO Preparations	ZTE

- R3-201710	(TP for NR_Mob_enh BL CR for TS 38.473) Introduction of New Cause Value for gNB-DU Initiated CHO Modify and Cancel	ZTE

- R3-201714	(TP for NR_Mob_enh BL CR for TS 38.473) Refine the Usage of UE CONTEXT MODIFICATION for Candidate Cell Modify and Cancel	ZTE

- R3-201715	(TP for NR_Mob_enh BL CR for TS 38.473) Further Clean-ups for Abnormal Conditions etc	ZTE

- R3-201717	(TP for NR_Mob_enh BL CR for TS 38.473) Introduction of NR CGI IE into UL RRC MESSAGE TRANSFER Message	ZTE, CATT, Google

- R3-201721	TS38.460 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-201732	(TP for NR_Mob_enh BL CR for TS 38.423) Resolving FFS Issues with DAPS Response Information	ZTE

- R3-201765	CHO in E1 interface	NEC

- R3-201766	Target initiated CHO Modification	NEC

- R3-201794	(TP for NR_Mob_enh BL CR for TS 38.423) Remaining issues in CHO modification	Qualcomm Incorporated

- R3-201795	(TP for NR_Mob_enh BL CR for TS 38.423) Remaining issues in DAPS handover over Xn	Qualcomm Incorporated

- R3-201796	(TP for NR_Mob_enh BL CR for TS 38.413) Handover Success procedure for NGAP based DAPS handover	Qualcomm Incorporated

- R3-201797	Remaining issues for DAPS handover over NG	Qualcomm Incorporated

- R3-201798	Avoiding simultaneous CHO and CPC	Qualcomm Incorporated

- R3-201873	(TP for NR_Mob_enh BL CR for TS 38.413): Left issues for DAPS handover	Huawei

- R3-201875	Left issues for DAPS handover	Huawei

- R3-201876	(TP for NR_Mob_enh BL CR for TS 38.423): Avoidance of simultaneous CHO and CPC configuration	Huawei

- R3-201878	(TP for NR_Mob_enh BL CR for TS 37.340): Avoidance of simultaneous CHO and CPC configuration	Huawei

- R3-201879	(TP for NR_Mob_enh BL CR for TS 38.401): RRCReconfigurationComplete Transfer in Conditional Pscell Change	Huawei

- R3-201880	(TP for NR_Mob_enh BL CR for TS 38.423): Target Node Initiated CHO Modification	Huawei

- R3-201890	(TP for NR_Mob_enh BL CR for TS 37.340) Further Discussion of Parallel CHO and CPC Simultaneous Operation	ZTE, China Telecom, China Unicom

- R3-201892	(TP for NR_Mob_enh BL CR for TS 38.423) Introduction of Maximum Number of Candidate PScells Allowed by MN	ZTE, China Telecom, China Unicom

- R3-201927	(TP for NR_Mob_enh BL CR for TS 38.423): Early Data Forwarding for CHO	Apple Inc., Intel Corporation

- R3-201940	Remaining Issues for DAPS HO	CATT

- R3-201942	(TP for [NR_Mob_enh] BL CR for TS 38413)Remaining Issues for DAPS HO	CATT

- R3-201944	(TP for [NR_Mob_enh] BL CR for TS 38423)Remaining Issues for DAPS HO	CATT

- R3-201945	CR to TS 38.463 for Remaining Issues for DAPS HO	CATT

- R3-201946	Issue of Avoiding Simultaneous CHO and CPC	CATT

- R3-201948	(TP for [NR_Mob_enh] BL CR for TS 38423)Issue of Avoiding Simultaneous CHO and CPC	CATT

- R3-201949	Discussion on E1 impact during Conditional Handover	CATT

- R3-201950	CR to TS 38.463 for E1 impact during Conditional Handover	CATT

- R3-201951	Discussion on Early Data Forwarding for CHO	CATT

- R3-202007	Clarification of supporting PDCP SN continuity for RLC-UM bearer in DAPS HO	Samsung

- R3-202010	(TP for NR_Mob_enh BL CR for TS 38.300) Clarification of supporting PDCP SN continuity for RLC-UM bearer in DAPS HO	Samsung

- R3-202011	(TP for NR_Mob_enh BL CR for TS 38.423) Clarification of supporting PDCP SN continuity for RLC-UM bearer in DAPS HO	Samsung

- R3-202012	Supporting the target node initiated CHO modification	Samsung

- R3-202014	(TP for NR_Mob_enh BL CR for TS 38.423) Supporting the target node initiated CHO modification	Samsung

- R3-202015	(TP for NR_Mob_enh BL CR for TS 38.473) Supporting the target node initiated CHO modification	Samsung

- R3-202076	Supporting Conditional PSCell Change using MCG SRB	Samsung

- R3-202077	(TP for NR_Mob_enh BL CR for TS 38.401) Supporting Conditional PSCell Change using MCG SRB	Samsung

- R3-202079	(TP for NR_Mob_enh BL CR for TS 38.423) Supporting Conditional PSCell Change using MCG SRB	Samsung

- R3-202094	Arrival probability to avoid overload due to CHO	Vodafone, Nokia, Nokia Shanghai Bell, British Telecom

- R3-202096	(TP for NR_Mob_enh BL CR for TS 38.423): Implementation of the arrival probability	Vodafone, Nokia, Nokia Shanghai Bell, British Telecom

- R3-202180	(TP for NR_Mob_enh BL CR for TS 38.423) Target-initiated CHO Modification	China Telecommunication

- R3-202182	(TP for NR_Mob_enh BL CR for TS 38.423) Discussion on DAPS indicator and introduce fallback information per DRB for NR	China Telecommunication

- R3-202243	Discussion on avoiding simultaneous CHO and CPC configuration	Google Inc.

- R3-202245	(TP for NR_Mob_enh BL CR for TS 38.423) RRC Reconfiguration Complete for CPC	Google Inc.

- R3-202252	(TP for NR_Mob_enh BL CR for TS 38.423): Consideration on the fallback indication	Nokia, Nokia Shanghai Bell

- R3-202288	DAPS HO handling during preparation	Intel Corporation

- R3-202290	(TP for NR_Mob_enh-Core BL CR for TS 38.423): DAPS HO handling during preparation	Intel Corporation

- R3-202291	(TP for NR_Mob_enh-Core BL CR for TS 38.463): DAPS HO handling during preparation	Intel Corporation

- R3-202292	(TP for NR_Mob_enh-Core BL CR for TS 38.401): DAPS HO handling during preparation	Intel Corporation

- R3-202293	Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios	Intel Corporation

- R3-202294	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios	Intel Corporation

- R3-202296	(TP for NR_Mob_enh-Core BL CR for TS 38.423): Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios	Intel Corporation

- R3-202297	Early Forwarding support for DAPS/CHO over E1	Intel Corporation, ZTE, Google, CATT

- R3-202298	(TP for NR_Mob_enh-Core BL CR for TS 38.463): Early Forwarding support for DAPS/CHO over E1	Intel Corporation, ZTE, Google, CATT

- R3-202299	Early Forwarding may need candidate cell distinction	Intel Corporation

- R3-202301	(TP for NR_Mob_enh-Core BL CR for TS 38.423): Early Forwarding may need candidate cell distinction	Intel Corporation

- R3-202302	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Early Forwarding may need candidate cell distinction	Intel Corporation

- R3-202362	(TP for NR Mob BL CR for TS 38.413): Support of NG DAPS HO	Ericsson

- R3-202363	(TP for NR Mob BL CR for TS 38.423): DAPS HO - per-DRB Handover Preparation	Ericsson

- R3-202365	(TP for NR Mob BL CR for TS 38.300): DAPS HO - PDCP SN continuity for RLC-UM	Ericsson

- R3-202367	(TP for NR Mob BL CR for TS 38.423) CHO in MR-DC operation	Ericsson

- R3-202368	(TP for NR Mob BL CR for TS 37.340) CHO in MR-DC operation	Ericsson

- R3-202369	CHO Bearer Context in CU-UP	Ericsson

- R3-202370	CHO - Modification from target node	Ericsson

- R3-202371	(TP for NR Mob BL CR for TS 38.423): CHO - Modification from target node	Ericsson

- R3-202373	Optimizing multiple SN STATUS TRANSFER for disaggregated gNB	Ericsson

- R3-202374	Early data forwarding and CU-UP DL transmission start/stop	Ericsson

- R3-202375	Early data forwarding and CU-UP DL transmission start/stop	Ericsson

- R3-202377	Optimizing multiple SN STATUS TRANSFER for disaggregated gNB	Ericsson

- R3-202416	(TP for NR_Mob_enh-Core BL CR for TS 38.463): Early Forwarding support over E1	Huawei

- R3-202424	(TP for NR_Mob_enh BL CR for TS 38.423): Left issues for DAPS handover	Huawei

- R3-202427	Consideration on FFSs for DAPS handover	LG Electronics

- R3-202452	CR for introducing Rel-16 NR mobility enhancement	CATT, CMCC

- R3-202496	CB: # 12_Email_MobEnh_BLs_cleanups	Huawei - Moderator

- R3-202497	CB: # 13_Email_MobEnh_DAPS_X2XnE1	Intel - Moderator

- R3-202498	CB: # 14_Email_MobEnh_Common_PDCP_SN_Continuity_RLC-UM	Ericsson - Moderator

- R3-202499	CB: # 16_Email_MobEnh_DAPS_S1NG	CATT - Moderator

- R3-202500	CB: # 17_Email_MobEnh_CHO_CondPSCell_General	Intel - Moderator

- R3-202501	CB: # 18_Email_MobEnh_Avoid_CHO_and_CondPSCellChg	Nokia - Moderator

- R3-202502	CB: # 19_Email_MobEnh_CHOprep	Ericsson - Moderator

- R3-202503	CB: # 20_Email_MobEnh_CHOmod	Nokia - Moderator

- R3-202504	CB: # 21_Email_MobEnh_Overload	Vodafone - Moderator

- R3-202505	CB: # 22_Email_MobEnh_CHO_NR	ZTE - Moderator

- R3-202506	CB: # 24_Email_MobEnh_Data_fwd_CHO	Intel - Moderator

- R3-202507	CB: # 25_Email_Mob_Enh_Data_fwd_E1	Intel - Moderator

- R3-202574	TS38.470 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-202575	TS37.340 Stage2 Introduction of Rel-16 Mobility Enhancement in MR-DC	ZTE, CATT, NTT DOCOMO, INC., Google, China Telecom

- R3-202577	(TP for NR_Mob_enh BL CR for TS 38.401) Clarification of Multiple APID Usage in CHO and CPC	ZTE

- R3-202578	(TP for NR_Mob_enh BL CR for TS 38.473) Refine the Usage of UE CONTEXT MODIFICATION for Candidate Cell Modify and Cancel	ZTE

- R3-202579	(TP for NR_Mob_enh BL CR for TS 38.473) Further Clean-ups for Abnormal Conditions etc	ZTE

- R3-202614	(TP for NR_Mob_enh BL CR for TS 38.423) Further Clarification of Maximum Number of CHO Preparations	ZTE

- R3-202660	(TP for [NR_Mob_enh] BL CR for TS 38413)Remaining Issues for DAPS HO	CATT

- R3-202672	(TP for NR_Mob_enh BL CR for TS 38.423): Implementation of the arrival probability	Vodafone, Nokia, Nokia Shanghai Bell, British Telecom

- R3-202676	Baseline CR for introducing Rel-16 NR mobility enhancement	CATT, Nokia, Nokia Shanghai Bell

- R3-202679	(TP for NR_Mob_enh BL CR for TS 38.300) Clarification of supporting PDCP SN continuity for RLC-UM bearer in DAPS HO	Samsung

- R3-202692	TS38.460 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-202709	LS on S1/NG DAPS handover	CATT

- R3-202714	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation

- R3-202716	(TP for NR_Mob_enh-Core BL CR for TS 38.423): DAPS HO handling during preparation	Intel Corporation

- R3-202717	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios	Intel Corporation

- R3-202718	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Early Forwarding may need candidate cell distinction	Intel Corporation

- R3-202734	(TP for NR_Mob_enh BL CR for TS 38.423): Clean-up of the BL CR	Nokia, Nokia Shanghai Bell

- R3-202753	(TP for NR_Mob_enh BL CR for TS 38.423): Completion of the open points related to the modification of a CHO	Nokia, Nokia Shanghai Bell

- R3-202762	(TP for NR_Mob_enh BL CR for TS 37.340) Further Discussion of Parallel CHO and CPC Simultaneous Operation	ZTE, China Telecom, China Unicom

- R3-202780	(TP for NR Mob BL CR for TS 37.340) CHO in MR-DC operation	Ericsson

- R3-202796	Baseline CR for introducing Rel-16 NR mobility enhancement	Ericsson, CATT

- R3-202835	LS on S1/NG DAPS handover	CATT

- R3-202845	LS on S1/NG DAPS handover	CATT

- R3-202855	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios	Intel Corporation

- R3-202871	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios	Intel Corporation

- R3-203010	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation

- R3-203033	CR for introducing Rel-16 NR mobility enhancement	CATT, CMCC

- R3-203034	TS38.470 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-203042	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation

- R3-203078	TS37.340 Stage2 Introduction of Rel-16 Mobility Enhancement in MR-DC	ZTE, CATT, NTT DOCOMO, INC., Google, China Telecom

- R3-203079	Baseline CR for introducing Rel-16 NR mobility enhancement	Intel Corporation, CATT, ZTE, Huawei

- R3-203080	Baseline CR for introducing Rel-16 NR mobility enhancement	Nokia, Nokia Shanghai Bell, Intel Corporation

- R3-203081	Baseline CR for introducing Rel-16 NR mobility enhancement	Ericsson, CATT

- R3-203100	Baseline CR for introducing Rel-16 NR mobility enhancement	CATT, Nokia, Nokia Shanghai Bell

- R3-203146	(TP for NR_Mob_enh BL CR for TS 38.423) Refine XnAP Abnormal Conditions etc	ZTE

- R3-203147	(TP for NR_Mob_enh BL CR for TS 37.340) Concluding Rel-16 DAPS and Conditional Mobility in MR-DC Scenarios	ZTE

- R3-203187	(TP for NR_Mob_enh BL CR for TS 38.423) Correction of DL Forwarding IE set to “DL forwarding proposed”for DAPS	ZTE

- R3-203191	(TP for NR_Mob_enh BL CR for TS 38.423) Leftover Issues with DAPS Fallback to Rel-14 MBB	ZTE

- R3-203199	Further Discussion of Coordination between CHO and CPC	ZTE

- R3-203200	(TP for NR_Mob_enh BL CR for TS 38.473) Introduction of New Cause Value for gNB-DU Initiated CHO Modify&Re-setup	ZTE

- R3-203201	(TP for NR_Mob_enh BL CR for TS 38.463) Introduction of New Cause Value for gNB-CU-UP Initiated CHO Modify&Re-setup	ZTE

- R3-203202	(TP for NR_Mob_enh BL CR for TS 38.473) Introduction of NR CGI IE into UL RRC MESSAGE TRANSFER Message	"ZTE, CATT, Google, Qualcomm Incorporated, China Telecom, China 	Unicom"

- R3-203218	(TP for NR_Mob_enh BL CR for TS 38.470) Introduction of Access Success Procedure	ZTE, Intel Corporation

- R3-203219	TS38.460 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-203270	Avoiding simultaneous CHO and CPC	Nokia, Nokia Shanghai Bell

- R3-203271	(TP for NR_Mob_enh BL CR for TS 38.423): A solution to enable the MN to block CPC usage	Nokia, Nokia Shanghai Bell

- R3-203273	Response LS on avoiding simultaneous CHO and CPC	Nokia, Nokia Shanghai Bell

- R3-203274	(TP for NR_Mob_enh BL CR for TS 38.423): Completion of the work on the modification of a CHO	Nokia, Nokia Shanghai Bell

- R3-203276	(TP for NR_Mob_enh BL CR for TS 38.423): Consideration on the fallback indication	Nokia, Nokia Shanghai Bell

- R3-203277	(TP for NR_Mob_enh BL CR for TS 38.413): Consideration on the fallback indication	Nokia, Nokia Shanghai Bell

- R3-203325	(TP for NR_Mob_enh-Core BL CR for TS 38.423) Remaining issues in CHO modification	Qualcomm Incorporated

- R3-203327	(TP for NR_Mob_enh-Core BL CR for TS 38.423) Remaining issues in DAPS handover over Xn	Qualcomm Incorporated

- R3-203329	(TP for NR_Mob_enh-Core BL CR for TS 38.300) PDCP SN continuity for RLC-UM bearer in DAPS HO	Qualcomm Incorporated

- R3-203330	Avoiding concurrent CHO and CPC	Qualcomm Incorporated

- R3-203331	(TP for NR_Mob_enh-Core BL CR for TS 38.413) AMF capability indication in NG based DAPS handover	Qualcomm Incorporated

- R3-203505	(TP for NR_Mob_enh BL CR for TS 38.423): Left issues for DAPS handover	Huawei

- R3-203507	Left issues for DAPS handover for E1	Huawei

- R3-203508	(TP for NR_Mob_enh BL CR for TS 38.413): Left issues for S1/NG DAPS handover	Huawei

- R3-203510	(TP for NR_Mob_enh BL CR for TS 37.340): Stage 2 correction for CPC	Huawei

- R3-203511	(TP for NR_Mob_enh BL CR for TS 38.423): Avoidance of simultaneous CHO and CPC configuration	Huawei

- R3-203513	(TP for NR_Mob_enh BL CR for TS 38.423): CHO Modification	Huawei

- R3-203515	(TP for NR_Mob_enh BL CR for TS 38.473): CHO Modification	Huawei

- R3-203529	(TP for NR_Mob_enh BL CR for TS 38.423) Introduce DAPS HO indicator and fallback mechanism for NR	China Telecom

- R3-203531	Discussion on remaining issues for CHO Modification	China Telecom

- R3-203533	(TP for NR_Mob_enh BL CR for TS 38.423) Target-initiated CHO Modification	China Telecom

- R3-203534	Discussion on early data forwarding for Conditional Handover	China Telecom

- R3-203536	(TP for NR_Mob_enh BL CR for TS 38.423) Introduce Early Forwarding Tranfer Request message for early data forwarding	China Telecom

- R3-203548	(TP for NR_Mob_enh BL CR for TS 38.300) Clarification of supporting PDCP SN continuity for RLC-UM bearer in DAPS HO	Samsung, Ericsson

- R3-203562	Correction of Baseline CR R3-203100 for introducing Rel-16 NR mobility enhancement	CATT

- R3-203565	(TP for [NR_Mob_enh] BL CR for TS 38413)Remaining NG Issues for DAPS HO	CATT

- R3-203566	Remaining X2/Xn/E1 Issues for DAPS HO	CATT

- R3-203568	(TP for [NR_Mob_enh] BL CR for TS 38423)Remaining Issues for DAPS HO	CATT

- R3-203569	CR to TS 38.463 for Remaining Issues for DAPS HO	CATT

- R3-203570	Issue of Avoiding Simultaneous CHO and CPC	CATT

- R3-203572	(TP for [NR_Mob_enh] BL CR for TS 38423)Issue of Avoiding Simultaneous CHO and CPC	CATT

- R3-203573	Discussion on Remaining Issues of CHO Modification	CATT

- R3-203575	(TP for [NR_Mob_enh] BL CR for TS 38423)Remaining Issues of CHO Modification	CATT

- R3-203588	Autonomous report for Discarding DL COUNT	NEC

- R3-203589	CHO in E1 interface	NEC

- R3-203590	CR to 38.463 for E1 impact on CHO	NEC

- R3-203591	(TP for NR_Mob_enh BL CR for TS 38.473) Introduce HO Success message in F1AP	NEC

- R3-203633	(TP for NR_Mob_enh BL CR for TS 38.401) CPC Complete message correction	Google Inc.

- R3-203634	(TP for NR_Mob_enh BL CR for TS 38.423) Applicable architecture for CPC	Google Inc.

- R3-203635	(TP for NR_Mob_enh BL CR for TS 38.401) Access success notification for Cell ID	Google Inc.

- R3-203636	(TP for NR_Mob_enh BL CR for TS 38.473) Access success notification for Cell ID	Google Inc.

- R3-203639	(TP for NR_Mob_enh BL CR for TS 38.300): Data forwarding leftovers	Huawei

- R3-203723	(TP for NR_Mob_enh BL CR for TS 38.423) Conditional HO Cancel procedure and HO Success procedure in 38.423	NEC

- R3-203759	(TP for NR_Mob_enh BL CR for TS 38.423): Early Data Forwarding for CHO	Apple Inc.

- R3-203763	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Clean-up on 8.9.4 from the last agreed R3-202823	Intel Corporation

- R3-203764	DAPS HO handling during preparation	Intel Corporation

- R3-203766	(TP for NR_Mob_enh-Core BL CR for TS 38.423): DAPS HO handling during preparation	Intel Corporation

- R3-203767	(TP for NR_Mob_enh-Core BL CR for TS 38.463): DAPS HO handling during preparation	Intel Corporation

- R3-203768	(TP for NR_Mob_enh-Core BL CR for TS 38.401): DAPS HO handling during preparation	Intel Corporation

- R3-203769	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Correction for DAPS HO on inter-gNB-DU mobility scenarios	Intel Corporation

- R3-203770	F1 needs a new class-2 message like HO SUCCESS	Intel Corporation, ZTE

- R3-203771	(TP for NR_Mob_enh-Core BL CR for TS 38.473): F1 needs a new class-2 message like HO SUCCESS	Intel Corporation, ZTE

- R3-203772	(TP for NR_Mob_enh-Core BL CR for TS 38.401): F1 needs a new class-2 message like HO SUCCESS	Intel Corporation, ZTE

- R3-203773	Full configuration CHO command indication	Intel Corporation

- R3-203775	(TP for NR_Mob_enh-Core BL CR for TS 38.423): Full configuration CHO command indication	Intel Corporation

- R3-203776	Early Forwarding support for DAPS/CHO over E1	Intel Corporation, ZTE, Google, CATT, Huawei, Samsung, NEC, LGE

- R3-203777	(TP for NR_Mob_enh-Core BL CR for TS 38.463): Early Forwarding support for DAPS/CHO over E1	Intel Corporation, ZTE, Google, CATT, Huawei, Samsung, NEC, LGE

- R3-203786	Support of S1 and NG DAPS HO	Ericsson

- R3-203788	(TP for NR Mob BL CR for TS 38.413): Support of NG DAPS HO	Ericsson

- R3-203789	DAPS HO - per-DRB Handover Preparation	Ericsson

- R3-203790	(TP for NR Mob BL CR for TS 38.423): DAPS HO - per-DRB Handover Preparation	Ericsson

- R3-203792	Simultaneous CHO and CPC restriction	Ericsson

- R3-203793	(TP for NR Mob BL CR for TS 38.423) CHO in MR-DC operation	Ericsson

- R3-203794	(TP for NR Mob BL CR for TS 37.340) CHO in MR-DC operation	Ericsson

- R3-203795	CHO - Modification from target node	Ericsson

- R3-203796	DU Connection Success for CHO and CPC	Ericsson

- R3-203797	(TP for NR Mob BL CR for TS 38.473) DU Connection Success for CHO and CPC	Ericsson

- R3-203798	Optimizing multiple SN STATUS TRANSFER for disaggregated gNB	Ericsson

- R3-203799	Optimizing multiple SN STATUS TRANSFER for disaggregated gNB	Ericsson

- R3-203800	Early data forwarding and CU-UP DL transmission start/stop	Ericsson

- R3-203801	Early data forwarding and CU-UP DL transmission start/stop	Ericsson

- R3-203892	Discussion on E1 impact during Conditional Handover	CATT,CMCC

- R3-203893	CR to TS 38.463 for E1 impact during Conditional Handover	CATT,CMCC

- R3-203895	(TP for [NR_Mob_enh] BL CR for TS 38423)Early Data Forwarding for CHO	CATT

- R3-203918	Remaining issues on DAPS handover	LG Electronics

- R3-203925	(TP for NR_Mob_enh BL CR for TS 38.413) DAPS Response Information per DRB for NG	Samsung

- R3-203927	(TP for NR_Mob_enh BL CR for TS 38.423) DAPS Response Information per DRB for Xn	Samsung

- R3-203929	Discussion on avoiding simultaneous CHO and CPC	Samsung

- R3-203945	Response to R3-203792	Nokia, Nokia Shanghai Bell

- R3-203982	CB: # 31_MobEnh_BLs - Summary of email discussion	Intel - moderator

- R3-203983	CB: # 33_MobEnh_DAPS_S1_NG - Summary of email discussion	Nokia - moderator

- R3-203984	CB: # 34_MobEnh_DAPS_X2_Xn - Summary of email discussion	Ericsson - moderator

- R3-203985	CB: # 32_MobEnh_DAPS_RLC-UM - Summary of email discussion	Samsung - moderator

- R3-203986	CB: # 35_MobEnh_DAPS_E1 - Summary of email discussion	Intel - moderator

- R3-203987	CB: # 36_MobEnh_CondPSCell_chg - Summary of email discussion	Samsung - moderator

- R3-203988	CB: # 37_MobEnh_CHOprep_MR-DC - Summary of email discussion	Ericsson - moderator

- R3-203989	CB: # 38_MobEnh_CHOmod - Summary of email discussion	Huawei - moderator

- R3-203990	CB: # 39_MobEnh_CHOcancel - Summary of email discussion	NEC - moderator

- R3-203991	CB: # 41_MobEnh_CHO - Summary of email discussion	ZTE - moderator

- R3-203992	CB: # 42_MobEnh_CHO_E1 - Summary of email discussion	CATT - moderator

- R3-203993	CB: # 44_MobEnh_EarlyDataFwd - Summary of email discussion	China Telecom - moderator

- R3-203994	CB: # 45_MobEnh_NR_EarlyDataFwd - Summary of email discussion	Intel - moderator

- R3-204087	Optimizing multiple SN STATUS TRANSFER for disaggregated gNB	Ericsson

- R3-204123	(TP for NR_Mob_enh BL CR for TS 38.423) Refine XnAP Abnormal Conditions etc	ZTE

- R3-204148	(TP for NR Mob BL CR for TS 38.423) CHO in MR-DC operation	Ericsson

- R3-204149	(TP for NR Mob BL CR for TS 37.340) CHO in MR-DC operation	Ericsson

- R3-204163	(TP for NR_Mob_enh BL CR for TS 38.423) Conditional HO Cancel procedure and HO Success procedure in 38.423	NEC

- R3-204176	(TP for NR_Mob_enh BL CR for TS 38.401) CPC Complete message correction	Google Inc.

- R3-204184	(TP for NR_Mob_enh BL CR for TS 38.423): CHO Modification	Huawei

- R3-204186	(TP for NR_Mob_enh BL CR for TS 38.473): CHO Modification	Huawei

- R3-204226	(TP for NR_Mob_enh-Core BL CR for TS 38.463): DAPS HO handling during preparation	Intel Corporation

- R3-204227	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Correction for DAPS HO on inter-gNB-DU mobility scenarios	Intel Corporation

- R3-204228	(TP for NR_Mob_enh-Core BL CR for TS 38.473): F1 needs a new class-2 message like HO SUCCESS	Intel Corporation, ZTE, NEC

- R3-204229	(TP for NR_Mob_enh-Core BL CR for TS 38.401): F1 needs a new class-2 message like HO SUCCESS	Intel Corporation, ZTE, NEC

- R3-204230	(TP for NR_Mob_enh-Core BL CR for TS 38.463): Early Forwarding support for DAPS/CHO over E1	Intel Corporation, ZTE, Google, CATT, Huawei, Samsung, NEC, LGE

- R3-204232	(TP for NR_Mob_enh-Core BL CR for TS 38.423): HO SUCCESS alignment with F1 ACCESS SUCCESS	Intel

- R3-204233	(TP for NR_Mob_enh BL CR for TS 38.473) Introduction of New Cause Value for gNB-DU Initiated CHO Modify&Re-setup	ZTE

- R3-204283	TS38.460 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-204284	TS38.460 E1 Stage2 Introduction of Mobility Enhancement Features	ZTE, China Telecom, China Unicom

- R3-204285	(TP for NR_Mob_enh-Core BL CR for TS 38.401): CB: # 42_MobEnh_CHO_E1	Intel

- R3-204286	(TP for NR_Mob_enh-Core BL CR for TS 38.463): CB: # 42_MobEnh_CHO_E1	Intel

- R3-204293	(TP for NR_Mob_enh BL CR for TS 38.423) Change of the name of the Early Forwarding Transfer procedure	Nokia

- R3-204297	(TP for NR Mob BL CR for TS 38.413): Support of NG DAPS HO	Ericsson

- R3-204301	(TP for NR_Mob_enh-Core BL CR for TS 38.423): DAPS HO handling during preparation	Intel Corporation

- R3-204329	Optimizing multiple SN STATUS TRANSFER for disaggregated gNB	Ericsson

- R3-204332	(TP for NR_Mob_enh-Core BL CR for TS 38.463): DAPS HO handling during preparation	Intel Corporation

- R3-204333	(TP for NR_Mob_enh-Core BL CR for TS 38.463): CB: # 42_MobEnh_CHO_E1	Intel

- R3-204334	(TP for NR_Mob_enh-Core BL CR for TS 38.463): Early Forwarding support for DAPS/CHO over E1	Intel Corporation, ZTE, Google, CATT, Huawei, Samsung, NEC, LGE

- R4-2003090	Considerations for NR DAPS handover	Ericsson

- R4-2003091	Correction to DAPS HO requirements in 38.133	Ericsson

- R4-2003092	Testcases for LTE and NR mobility enhancements	Ericsson

- R4-2003193	Discussion on remaining issues on DAPS handover	Intel Corporation

- R4-2003194	Draft CR to TS 38.133: DAPS handover RRM requirement	Intel Corporation

- R4-2003195	Draft CR to TS 38.133: CHO RRM requirement	Intel Corporation

- R4-2003196	Draft CR to TS 38.133: CPC RRM requirement	Intel Corporation

- R4-2003562	Remaining issues on NR DAPS HO	Qualcomm Incorporated

- R4-2003605	Discussion on dual active protocol stack handover	MediaTek inc.

- R4-2003959	Discussion on remaining open issues on DAPS handover	NEC

- R4-2004308	Further discussion on remaining issues on DAPS handover	Huawei, HiSilicon

- R4-2004309	DraftCR on DAPS handover requirements	Huawei, HiSilicon

- R4-2004310	Discussion on UE capabilities signalling on DAPS handover	Huawei, HiSilicon

- R4-2004311	Draft LS on UE capabilities on DAPS HO	Huawei, HiSilicon

- R4-2004312	DraftCR on conditional handover requirements	Huawei, HiSilicon

- R4-2004313	DraftCR on conditional PSCell change requirements	Huawei, HiSilicon

- R4-2004435	CR Introduction of handover delay requirements for conditional handover (section 6.1)	Nokia, Nokia Shanghai Bell

- R4-2004436	CR Introduction of handover delay requirements for conditional handover (section 6.1)	Nokia, Nokia Shanghai Bell

- R4-2004437	Correction CR for Conditional PSCell Change	Nokia, Nokia Shanghai Bell

- R4-2004438	CR correcting DAPS handover requirements	Nokia, Nokia Shanghai Bell

- R4-2005240	Email discussion summary for [94e Bis][108] NR_Mob_enh_RRM	Moderator (Intel Corporation)

- R4-2005305	WF on NR Mobility enhancement RRM requirements	Intel

- R4-2005306	Draft LS on UE capabilities on DAPS HO	Huawei, HiSilicon

- R4-2005307	Correction to DAPS HO requirements in 38.133	Ericsson

- R4-2005308	Draft CR to TS 38.133: CHO RRM requirement	Intel Corporation, Nokia, Nokia Shanghai Bell

- R4-2005309	DraftCR on conditional PSCell change requirements	Huawei, HiSilicon

- R4-2005392	Email discussion summary for [94e Bis][108] NR_Mob_enh_RRM	Moderator (Intel Corporation)

- R4-2006163	Remaining issues on NR DAPS HO	Qualcomm Incorporated

- R4-2006543	Discussion on remaining issues on DAPS handover	Intel Corporation

- R4-2006544	CR to TS 38.133: DAPS handover RRM requirement	Intel Corporation

- R4-2006545	Reply LS on simultaneous reception of DL signals in intra-frequency DAPS HO	Intel Corporation

- R4-2006546	CR to TS 38.133: CHO RRM requirement	Intel Corporation

- R4-2006887	Discussion on dual active protocol stack handover	MediaTek inc.

- R4-2006978	Sync side conditions for NR DAPS handover	Ericsson

- R4-2006979	Correction to DAPS HO requirements in 38.133	Ericsson

- R4-2006980	Testcases for LTE and NR mobility enhancements	Ericsson

- R4-2007287	Discussion on remaining open issues on DAPS handover	NEC

- R4-2007759	Further discussion on remaining issues on DAPS handover	Huawei, HiSilicon

- R4-2007760	CR on DAPS handover requirements	Huawei, HiSilicon

- R4-2007761	Draft LS on UE capabilities on DAPS HO	Huawei, HiSilicon

- R4-2007762	CR on conditional PSCell change requirements	Huawei, HiSilicon

- R4-2008194	CR on 38133 NR DAPS handover	Nokia, Nokia Shanghai Bell

- R4-2008497	Email discussion summary for [95e][208] NR_Mob_enh_RRM	Moderator (Intel Corporation)

- R4-2008585	WF on MR Mobility Enhancements	Intel

- R4-2008586	Correction to DAPS HO requirements in 38.133	Ericsson

- R4-2009020	Email discussion summary for [95e][208] NR_Mob_enh_RRM	Moderator (Intel Corporation)

- R4-2009123	CR to TS 38.133: CHO RRM requirement	Intel Corporation

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template