3GPP TSG-RAN WG4 Meeting #96-e	R4-2012148

# Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0992 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for introduction of pathloss reference signal switching delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-08-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MAC-CE based pathloss RS switching is introduced in Rel-16 eMIMO WI. Accordingly, applicable timing requirement on MAC-CE based pathloss RS needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce MAC-CE based pathloss RS switching requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MAC-CE based pathloss RS switching requirement is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

## 8.x	Pathloss reference signal switching delay

### 8.x.1	Introduction

The requirements in this clause apply for pathloss reference signal activated or updated on serving cell in MR-DC or standalone NR in clause 7.1.1 in TS 38.213 [3].

UE shall complete the switch of pathloss reference signal within the delay defined in this clause.

### 8.x.2	Known conditions for pathloss reference signal

The pathloss reference signal is known if the following conditions are met during the period  between the last transmission of the L1-RSRP measurement reporting for the target pathloss reference signal and the completion of pathloss reference signal switch.

-	Pathloss reference signal switch command is received within 1280 ms upon the last transmission of the target pathloss reference signal resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target pathloss reference signal before the pathloss reference signal switch command

-	The target pathloss reference signal remains detectable during the pathloss reference signal switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the pathloss reference signal switching period

-	SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

### 8.x.3	MAC-CE based pathloss reference signal switch delay

The requirements in this clause apply for a UE to update a pathloss reference signal by MAC-CE for PUCCH, PUSCH, semi-persistent SRS and aperiodic SRS.

If the target pathloss reference signal is known, upon receiving PDSCH carrying MAC-CE activation in slot n, UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot n + THARQ+ 3ms + 5*Ttarget_PL-RS + 2msNR slot lengthTHARQ3Nslotsubframe,µ5*Ttarget_PL-RSNR slot length Nslotsubframe,µ.  The UE shall be able to apply old pathloss reference signals until the slot n + THARQ+ 3Nslotsubframe,µ. Where

- THARQ is the timing between pathloss reference MAC-CE activation command and acknowledgement as specified in TS 38.321 [7].

- Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS.

Note: longer application time is expected if measurement sample is not available due to measurement gap, DRX or other UE activities.

Note: longer application time is expected if the pathloss reference signal is unknown.

### >>>>>End of Change 1<<<<<