3GPP TSG RAN meeting #89e										RP- 201514

Electronic Meeting, September 14-18, 2020

Status Report to TSG

Agenda item:			9.2.7

| WI / SI Name | 5G V2X with NR sidelink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | 5G_V2X_NRSL |  |  |  |  |
| Unique ID | 830078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191723 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: mm/yyyy | Performance part: 12/202003/2021 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 100% | Performance Part: 30% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Hanbyul Seo |
|  | Company | LG Electronics |
|  | Email | hanbyul.seo@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#96-e : RRM(Performance)

In RRM session, 10 contributions were submitted for RRM test cases(performance part), and e-mail discussion summary was submitted for information.

One WF(R4-2012104) on work scope for RRM performance part(test cases), baseline of test cases, test cases, test set-up, work split and skeleton of test specification was approved.

- Work Plan

  - RAN4#96-e meeting (’20.August)

    - Agree the list of RRM test cases

    - Do work-split of test cases for draft CRs

  - RAN4#96-ebis & RAN4#97-e meeting (’20.October)

    - Discuss the draft CRs with the detailed test configurations and related parameter

  - RAN4#98 meeting (’21.March)

    - Endorse the final draft CRs.

    - Agree the one Big CR for RRM tests based on the endorsed draft CRs.

- Baseline of test cases

  - LTE V2X RRM test cases can be the baseline to define NR V2X RRM test case.

    - Further check which LTE V2X RRM test cases can be reused

  - V2X SL {SCS & CBW} in all the RRM test

    - {30kHz, 20MHz}, {30kHz, 40MHz}

    - Add note:The UE is only required to be tested in one of the supported test configurations.

  - Synchronization reference sources in the related RRM test cases

    - GNSS, SyncRef UE, gNB

      - Apply application rule for gNB

  - gNB(FR1 NR Cell) configurations in all the related RRM test

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

- Test cases

  - Introduce Test for UE transmit timing for

    - GNSS, SyncRef UE and gNB as timing reference.

  - Introduce Test for initiation/Cease of SLSS Transmissions for

    - SyncRef UE and gNB as timing reference.

  - Introduce Test for Selection / Reselection of V2X Synchronization Reference Source

    - when GNSS/gNB is configured as the highest priority

  - Test for L1 SL-RSRP measurements

    - Introduce Test for V2X UE Autonomous Resource Selection/Reselection

    - Decide whether or not introduce Test for Resource Re-evaluation and Resource Pre-emption after conclusion on feasibility of tests

  - Introduce Test for Congestion Control Measurement

  - Test for interruptions

    - Introduce Test for interruptions to WAN due to V2X Sidelink Communication

    - Not introduce Test for interruptions due to sync. source change

    - Not introduce Test for interruptions to WAN due to UE switching between LTE SL and NR SL

  - Not introduce Test for Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink

- Test set-up

  - Agreed for following tests

    - Test for UE transmit timing

    - Test for initiation/Cease of SLSS Transmissions

    - Test for Selection / Reselection of V2X Synchronization Reference Source

      - gNB is configured as the highest priority

    - Test for Congestion Control measurements

    - Test for interruptions to WAN due to V2X Sidelink Communication

  - Further discussion

    - Test for Selection / Reselection of V2X Synchronization Reference Source

      - GNSS is configured as the highest priority

        - Option 1 : 3 SyncRef UEs

          - SyncRef UE1 (sync to gNB directly), SyncRef UE2 (sync to GNSS in-directly) and SyncRef UE3 (sync to GNSS directly).

        - Option 2 : 2 SyncRef UEs

          - SyncRef UE1 (sync to GNSS in-directly) and SyncRef UE2 (sync to GNSS directly).

    - Test for V2X UE Autonomous Resource Selection/Reselection

      - Revise option 1 regarding RAN1 spec., ‘at least X⋅M_"total"  to be included in set SA, and the value of X can be configured as {20%, 35%, 50%}.

      - Option 1

        - Active UE and subchannel allocation: there are 50 active UEs in the system, first 10 UEs occupies subchannel 0, the next 10 occupies subchannel 1, the next 10 occupies subchannel 2, following the allocation until all the 50 active UEs are allocated. Now the 5 subchannels configured for UE to be tested are each occupied by 10 UEs.

        - The 10 UEs in the same subchannel take turns to access the channel, same as LTE but LTE has 20 UEs in one subchannel.

        - UEs on subchannel 0/1/3/4 always transmit in high RSRP above the threshold (corresponding to 20dB SNR). UEs on subchannel 2 transmit with high RSRP in T1 and low RSRP in T2 (corresponding to 5dB SNR as PSCCH SNR in LTE).

        - Reuse RSRP threshold in LTE, with the SNR in the previous bullet, to derive SL-RSRP and S-RSSI.

      - Other options are not precluded

    - Pre-emption Test

      - Timeline to guarantee TE having enough time to send high priority reservation to trigger pre-emption:

        - Option 1 : Configure the slot bitmap as 10000000000000000000 so that each available slot if far part enough to guarantee that the first transmission and the next reservation are not too close to accommodate TE decoding time and pre-emption timeline defined in RAN1

      - How to prevent test UE to randomly select resource after sending SCI

        - Option 1: The general resource selection spec violation may not necessary be tested in pre-emption test, if it can not be conveniently embedded in pre-emption test

      - Other UE cheating behavior specific to resource pre-emption can be further discussed if companies come up with any other example

    - Re-evaluation Test

      - How to avoid the test UE to select the resource and send the initial transmission before sensing procedure?

- Work Splits for Test Cases(draft CRs)

- Spec. Skeleton for Test Cases

RAN4#96-e : Demodulation

In Demodulation session, 13 contributions were submitted to RAN4#96-e, and e-mail discussion summary was submitted for information.

One WF (R4-2012758) on work scope and general assumptions for NR V2X demodulation performance was approved with following contents.

- Spec. structure for NR V2X demodulation

  - Introduce new section for NR V2X performance requirements in TS38.101-4.

- Work scope

  - List of test cases to define demodulation performance requirements in Rel-16

    - PSSCH/PSCCH/PSBCH/PSFCH demodulation test

    - Power imbalance test

    - HARQ buffer soft combining

    - PSCCH/PSSCH decoding capability

    - PSFCH decoding capability

  - Do not define 2nd-stage SCI performance requirement explicitly

  - Do not define performance requirements for CSI reporting in Rel-16

- Reference receiver

  - LTE V2X reference receiver assumption is used as starting point

- General simulation assumption

  - Define performance requirements for FR1 only

  - 1T2R antenna configuration

  - Channel bandwidth and SCS

    - 20MHz CBW and 30kHz SCS as baseline for performance tests without maximum PSCCH/PSSCH allocation

    - FFS on CBW for PSCCH/PSSCH decoding capability test (options : 20MHz or 40MHz)

  - Define performance requirements based on Rank1

  - TDL-A 30ns as baseline for propagation condition

  - Relative velocity for test cases

    - 30km/h, 260km/h, 500km/h

  - The number of DMRS for PSSCH

    - Option 1: Depending on decision on relative velocity

    - Option 2: 3 DMRS symbols for all test cases

    - Other options are not precluded

  - TO and FO

    - Tx UE side (values with respect to sync source reference timing/frequency)

      - FO: 0.1 ppm for GNSS

      - TO: 12Ts for GNSS, 
       0 if Tx UE is used as SyncRefUE, otherwise 8Ts

    - Rx UE side (values with respect to Tx UE timing/frequency

      - FO: 0.2ppm for GNSS, 0.1ppm for SyncRefUE

      - TO: 24Ts for GNSS
       8Ts (30kHz SCS) for SyncRefUE

  - PSFCH feedback instead of AT commands can be used to test NR V2X UE’s performance

  - For PSFCH configuration for test cases, FFS on PSFCH are transmitted on every slot or every 4 slots

  - Number of S-SSB per SL period for 30kHz SCS

    - Option 1: 1

    - Option 2: 2 S-SSB with different contents

  - 256QAM reception capability

    - 256QAM reception is an optional UE capability

    - FFS on introduction of 256QAM demodulation test case

- Work split for draft CR

One document (R4-2012757) for simulation assumptions for NR V2X demodulation was approved with following scenarios.

- PSSCH demodulation test

- PSCCH demodulation test

- PSBCH demodulation test

- PSFCH demodulation test

- PSCCH and PSSCH decoding capability

PSFCH decoding capability

2.4.2	Remaining Open issues

RRM performance part

- Complete Test set-up for Test for V2X UE Autonomous Resource Selection/Reselection

- Decide whether or not introduce Test for Resource Re-evaluation and Resource Pre-emption after conclusion on feasibility of tests including test set-up

Demodulation performance part

- Finalize test cases and detail simulation assumptions

- Define performance requirements for agreed test cases

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#96-e

- R4-2012210	Email discussion summary for [96e][210] 5G_V2X_NRSL_RRM	Moderator (LGE)

- R4-2012104	WF on NR V2X RRM requirements	LG Electronics

- R4-2010038	Discussion on NR V2X RRM test case	MediaTek inc.

- R4-2010086	Work Plan and List of Test Cases for NR V2X RRM	LG Electronics Inc.

- R4-2011055	List of RRM test cases for NR V2X	Huawei, HiSilicon

- R4-2009768	Discussion on test cases for NR V2X RRM	Xiaomi

- R4-2010087	Discussion of Test Cases for NR V2X RRM	LG Electronics Inc.

- R4-2011056	Discussion on RRM test setup for NR V2X	Huawei, HiSilicon

- R4-2011382	CR: RRM test cases for NR V2X	Qualcomm, Inc.

- R4-2011383	NR V2X RRM test case discussion	Qualcomm, Inc.

- R4-2009830	On PSBCH demodulation performance requirement for NR V2X	CATT

- R4-2010010	Work plan for V2X demodulation performance	LG Electronics Inc.

- R4-2010011	Discussion on Spec. structure for V2X demodulation	LG Electronics Inc.

- R4-2010012	Discussion on test cases for NR V2X performance	LG Electronics Inc.

- R4-2010039	Discussion on NR V2X Demod test case	MediaTek inc.

- R4-2011022	Discusson on work scope of performance requirements for Rel-16 sidelink	Huawei, HiSilicon

- R4-2011023	Discusson on spec structure of performance requirements for Rel-16 sidelink	Huawei, HiSilicon

- R4-2011024	Discusson on test scenarios of performance requirements for Rel-16 sidelink	Huawei, HiSilicon

- R4-2011323	Discussion on NR V2X work scope	Intel Corporation

- R4-2011324	Discussion on NR V2X test scenarios	Intel Corporation

- R4-2011381	NR V2X Demod requirement	Qualcomm, Inc.

- R4-2012727	Email discussion summary for [96e][326] V2X_Demod	Moderator(LG Electronics)

- R4-2012757	Simulation assumptions for NR V2X demodulation	MediaTek

- R4-2012758	WF on work scope and general simulation assumptions for NR V2X demodulation performance 	LG Electronics

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template