3GPP TSG RAN meeting #89-e									       RP-201618

Online, 14th – 18th September 2020

# Source:	CMCC, Ericsson

# Title:	Status report for WI on enhancement of data collection for SON_MDT in NR and EN-DC

# Document for:	Approval

Agenda Item:	9.8.14

Status Report to TSG

Agenda item:			9.8.14

| WI / SI Name | # Enhancement of data collection for SONMDT in NR and EN-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |
| Unique ID | 860053 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-193255 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 09/2021 (changed) | Performance part: mm/yyyy | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx% | Core part: 10% | Performance Part: xx% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Liang LIU, Angelo Centonza |
|  | Company | CMCC, Ericsson |
|  | Email | liuliang@chinamobile.com, angelo.centonza@ericsson.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The change of the target completion date is due to conversion of face to face RAN WG meetings in Q1 2020 to online meetings and postponement of Stage-3 functional completion to June 2020.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2 #111-e meeting

The following agreements were made:

- SON

=>	RAN2 to consider the SON aspects of CHO and SON aspects of 2-step RA as part of the WI.

=>	RAN2 to consider the SON aspects of DAPS HO as part of the WI.

=>	The following scenarios are considered:

- 1) Successful CHO and HO (i.e. no failure happens). FFS consideration in RAN2/3

- 2) Unsuccessful CHO due to late CHO execution.

- 3) Unsuccessful CHO after CHO execution.

- 4) Successful or Unsuccessful  CHO after unsuccessful CHO or handover failure.

- Note: other scenarios are not ruled out…

=>	RAN2 should study what CHO failure information can be stored in RLF report.

=>	RAN 2 to discuss the method for distinguishing between different handover types in RLF report. FFS the details, e.g., explicitly way or not.

=>	RAN2 to agree studying the RLF report and/or FailureInformation message contents in the DAPS failure scenarios.

=>	New logged content for 2-step RA is introduced in:

- RA report

- RLF report

- CEF report

=>	Study the necessity of introducing new method for more precise identification of the DL coverage quality during the UL coverage outage.

- MDT

=>	The coexistence issue between IDC and MDT feature is identified and the legacy mechanism defined in LTE spec is the baseline. FFS on potential enhancements.

=>	Study the support of logged and Immediate MDT in MR-DC scenario. For M5/M6/M7, it is proposed to apply them for EN-DC/MR-DC cases with different bear types. FFS on details.

2.2.1	Remaining Open issues

SON Features

- Rel-16 Leftovers

- MRO for SN change failure (SCGFailureInformation related enhancements)

- UE history information in EN-DC

- Successful Handovers Reports:

- Rel-17 New SON Features

- UE reporting (e.g., RA report) associated to 2-step RACH

- Identify scenarios and associated UE reporting (e.g., RLF report, CEF report) for mobility enhancement optimization (CHO and DAPS)

- NR-U related SON/MDT optimization:

MDT Features

- Rel-16 Leftovers

- Immediate MDT measurements for MN terminated SCG/split bearer and SN terminated MCG/split bearer scenarios of EN-DC

- MDT for MR-DC

- Rel-17 New MDT Features

- Enhancement of logged and immediate MDT (including coexistence with IDC)

L2 measurements

- Potential L2 enhancements

2.3	RAN3

2.3.1	Agreements

RAN2 #109-e meeting

- The following BLCRs are endorsed,

BL CR to TS 38.300 in R3-205654,  Endorsed as BL CR unseen

- The following agreements were made,

PCI Selection

For centralized PCI assignment, in non-split gNB architecture, the OAM assigns a single PCI for each NR cell in the gNB, and the gNB selects this value as the PCI of the NR cell.

For distributed PCI assignment, in non-split gNB architecture, the OAM assigns a list of PCIs for each NR cell in the gNB. To resolve PCI conflict the gNB may select a PCI value from the list of PCIs.

For distributed PCI assignment, in split architecture case, PCI conflict detection and reassignment are located at gNB-CU. It is FFS whether the list of available PCIs is configured in CU or DU

R3-205306 rev in R3-205694 Agreed

Energy Efficiency

In split gNB architecture Energy Efficiency measurements are calculated based on RLC SDU Data Volume measurements; non-split architecture is FFS.

LS to SA5 in R3-205657 final Agreed

Successful Handover Report

WA: Access And Mobility Indication message is used to transmit Successful Report to the source NG-RAN node over Xn

Define “Successful HO Report” as RRC container in XnAP.

Agree to send LS to RAN2.

LS on Successful Handover Report (to: RAN2) (SS) in R3-205759 final Agreed

UE History Information in EN-DC

Enhancement of UE History Information for Secondary Node applies to all MR-DC scenario.

UE history information of secondary node includes: PSCell list, time UE stayed in the cell.

To-be-continued: Feasibility of correlating MN and SN UE history.

It is beneficial if the MR-DC based UHI and the legacy UHI are correlated when received. Whether this is feasible and the details of the solution are FFS.

Load Balancing Enhancements

Clarify the definition of TNL capacity Indicator IE.

The received TNL Capacity Indicator IE represents the lowest TNL capacity available for the cell.

Rel-16 Cat. F, WI code [R16_SON] in R3-205771 Agreed unseen

Rel-16 Cat. F, WI code [R16_SON]in R3-205772 Agreed unseen

Continue the discussion on how to clarify the definition of TNL capacity Indicator. Take the sentence “The received TNL Capacity Indicator IE represents the lowest TNL capacity available for the cell.” as the start point. Mainly focus on the following two open issues:

Whether we need to list the interface.

- If the interface is listed, whether E1 should be included. To be continued...

MRO for SN Change Failure

In case of a PSCell change failure, when the MN is responsible for SCG mobility, the MN corrects own configuration (no new signaling towards the SN is needed).

In case of a PSCell change failure, when the SN is responsible for SCG mobility, the MN forwards the SCGFailureInformation to the SN initiating the last PSCell change (or the last serving SN, in case of too late SN change).

In case of an SCG failure that is a result of an SN-initiated PSCell change, the SN initiating the last PSCell change (or the last serving SN, in case of too late SN change) is responsible to derive the needed correction for its SCG mobility configuration.

The definitions of SCG MRO failure events formulated in the TR 37.816 will be used, but it is FFS:

- if they shall apply to inter-SN change only or also to intra-SN PSCell change;

- If MN’s action is needed to declare SCG MRO failure event;

To support pre-Rel-17 UE, in case of SCG failure, the MN shall be able to identify if the last PSCell change was initiated by itself or an SN, and which SN it was. Further enhancements may be based on enhanced SCG failure information provided from the UE.

RACH Optimization Enhancements

Support of inter-en-gNB RACH coordination in Rel-17 is beneficial, feasibility to be further evaluated in light of the NG-RAN solution to be defined.

LS to RAN2 in R3-205662 final agreed unseen

Coverage and Capacity Optimization

E-UTRAN CCO function should be considered as baseline for NG-RAN CCO solution for dynamic coverage changes with an index-based solution for coverage switching among deployment options

In NG-RAN scenario, a NG-RAN node may send to a neighbor NG-RAN node a coverage modification list which includes deployment related information concerning the serving cells. Whether to include SSB beam information for NR cell (on top of cell info) is FFS.

Exchange at least NG-RAN CGI, Cell Coverage State, Cell Deployment Status Indicator, Cell Replacing Info in NG-RAN NODE CONFIGURATION UPDATE message over Xn for coverage modification

DU signals to CU coverage related configuration information. Whether to include SSB beam information (on top of cell info) is FFS.

CSI-RS based beam coverage tuning is an optimization and is not covered as part of NR CCO for Rel-17

Inter-System Inter-RAT Energy Saving

A cell state indication, triggered at change of cell status, should be sent from the NG-RAN node to the eNB to indicate the status of the concerned cell for energy saving purpose.

A cell activation request should be sent from eNB to NG-RAN node to request a previously switched-off cell/s to be re-activated.

A cell activation response should be sent from NG-RAN node to eNB to indicate that one or more cell(s) previously switched-off has (have) been activated.

Enhance Inter-System SON Information message on S1AP and NGAP to support inter-system Energy Savings.

To be continued on the basis of 5665,5666...

Inter-System Load Balancing

Introduce Inter System Load Balancing mechanisms on the basis of the solution available in E-UTRAN

Introduce Inter System Load Balancing by means of mechanisms that resemble or reuse the SON Configuration Transfer IE for the purpose of configuring load balancing metrics and reporting load balancing measurements

Use S1: eNB CONFIGURATION TRANSFER, S1: MME CONFIGURATION TRANSFER, NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER for the transfer of inter system load balancing via means of mechanisms that resemble or reuse the SON Configuration Transfer IEs. It is FFS whether further details on the signalling part need to be introduced.

Adopt signalling of the Composite Available Capacity (Cell Capacity Class value and Capacity Value) for inter system MLB. Adoption of further MLB metrics is FFS.

Event Based Reporting and Periodic Reporting (only in case specific conditions are met), are agreed to be supported for inter system MLB. The mechanism should avoid excessive signaling.

Two-Step RACH Optimization

PRACH parameters coordination for 2-step RA should be supported.

WA: reuse the existing NR PRACH Configuration structure for PRACH coordination for 2-step RA.

RAN3 to send LS to RAN2.

LS to RAN2 in R3-205797 final  Agreed unseen

Mobility Enhancement Optimization

Scope:

SON Enhancements for CHO (i.e MRO for CHO) will be supported.

SON Enhancements for DAPS handover will be supported.

Postpone SON Enhancements for CPC with waiting for the progress of R17 CPC enhancements and SON enhancements for CHO. It is FFS whether SON enhancements for conditional PSCell change should be supported.

Study resource optimization for CHO, based on contributions

Decide if the problem of data forwarding in case of a HO to wrong cell is part of the SON WI (SON for Mobility Enhancements) or is to be treated as TEI-17.

MRO for CHO:

FFS whether CHO specific failure types are needed. The existing definitions of too late handover /too early handover/ handover to wrong cell are the starting point for further study.

From RAN3 point of view, in order to support MRO for CHO, more information is needed from UE. (FFS on the details).

Study the contents of the RLF INDICATION or HANDOVER REPORT message to support MRO enhancements for CHO. In order to progress in this area it is necessary to converge on the CHO failure case definition.

SON Enhancements for DAPS handover:

Reporting of failure information of the source link from UE may be needed for DAPS handover (FFS: Need further discussion).

From RAN3 point of view, in order to support SON enhancements for DAPS handover, more information is needed from UE. (FFS on the details).

Study the contents of the RLF INDICATION or HANDOVER REPORT message for the failure scenarios in DAPS HO. In order to progress in this area it is necessary to converge on the DAPS failure case definition.

MDT enhancement

Beam related UE configurations (including rs type, number of beam to average, the absolute threshold for the consolidation of measurement results) are out of RAN3 scope

Send an LS to SA5 asking whether section 4.1.2.15.2 in TS32.422 implies signalling of a URI for streaming trace reporting to LTE as part of the MDT configuration

LS to SA5 in R3-205798 final Agreed unseen

MDT for MR-DC

Scenario clarification:

MDT enhancement in MR-DC in rel-17 should consider the following scenarios:

-	EN-DC (Rel-16 leftovers)

-	NGEN-DC

-	NE-DC

-	NR-DC

Immediate MDT:

For management based immediate MDT in NR-DC, OAM provides the MDT configuration to MN and SN independently.

For MDT in NGEN-DC and NE-DC, the SN receiving the management based immediate MDT and the signalling based immediate MDT in EN-DC is taken as baseline.

M1/M2/M8/M9 can be supported by immediate MDT without further coordination between MN and SN in all MR-DC cases.

Support of M4-M7 are pending RAN2 progress.

Logged MDT:

Whether log MDT can be configured either from MN or SN is pending to RAN2 progress.

Issue 4, MDT data Anonymization in MR-DC, propose to agree on:

The MDT anonymization process in EN-DC can be applied to all MR-DC use cases.

2.3.2	Remaining Open issues

SON Features

- Rel-16 Leftovers

- PCI selection

- Energy efficiency (OAM requirements):

- Successful Handovers Reports

- UE history information in EN-DC

- Load balancing enhancement

- MRO for SN change failure

- RACH Optimisation enhancements

- Rel-17 New SON Features

- CCO

- Inter-system inter-RAT energy saving

- Inter-system load balancing

- 2-step RACH optimization

- Mobility enhancement optimization

  - CHO optimsation

  - DAPS optimisation

- NR-U related SON/MDT optimization

MDT Features

- MDT enhancement:

- MDT for MR-DC

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

RAN2 #111e

R2-2006651	Clarification for SON Scope and Use Cases	CATT

R2-2006652	Clarification for MDT Scope and Use Cases	CATT

R2-2006678	Discussion on rel-17 Radio Link Failure Report	NTT DOCOMO INC.

R2-2006746	Mobility support in SON/MDT for Rel17	Intel Corporation

R2-2006747	Scope of MDT for Rel17	Intel Corporation

R2-2007070	Open Issues in MDT	QUALCOMM Europe Inc. - Spain

R2-2007071	Open Issues in SON	QUALCOMM Incorporated

R2-2007072	Open Issues in MDT	Qualcomm Incorporated

R2-2007155	Enhancements and scope of R17 SON	OPPO

R2-2007156	Enhancements and scope of R17 MDT	OPPO

R2-2007196	SON for Rel-16 mobility enhancement	NEC

R2-2007233	R17 MDT scope for MR-DC and early measurments	Samsung Telecommunications

R2-2007300	Discussion on MDT enhancements	vivo

R2-2007301	Discussion on SON enhancements	vivo

R2-2007392	Optimization for Rel-16 Features	Samsung

R2-2007434	MDT Enhancement for 2-step RA	CMCC

R2-2007435	SON Consideration for R16 Mobility Enhancement	CMCC

R2-2007464	MRO for Inter-RAT handover	Lenovo, Motorola Mobility

R2-2007465	MRO for CHO and DAPS Handover	Lenovo, Motorola Mobility

R2-2007515	Rel-17 MDT enhancements scope	Nokia, Nokia Shanghai Bell

R2-2007516	Rel-17 SON enhancements scope	Nokia, Nokia Shanghai Bell

R2-2007661	SON Scope and Requirements for Rel.17	Ericsson

R2-2007662	SON Summary	Ericsson

R2-2007667	MDT scope and requirements	Ericsson

R2-2007769	Discussion for RAN2 SON scope and requirements	Huawei, HiSilicon

R2-2007770	Discussion for MDT	Huawei, HiSilicon

R2-2007771	Summary on 8.13.3 MDT	Huawei, HiSilicon

R2-2007782	Consideration on SON enhancements	ZTE Corporation, Sanechips

R2-2007783	Consideration on MDT enhancements	ZTE Corporation, Sanechips

R2-2007996	Work plan for enhancement of data collection for SON_MDT in NR and EN-DC WI	CMCC, Ericsson

R2-2008116	Work plan for enhancement of data collection for SON_MDT in NR and EN-DC WI	CMCC, Ericsson

R2-2008288	SON Summary	Ericsson

RAN3 #109e

R3-204618	Reply LS on energy efficiency	3GPP SA5

R3-204633	Discussion on the PRACH Coordination in Spectrum Shared between LTE and NR	China Telecom,ZTE,Huawei

R3-204634	(TP for [NR_SON_MDT] BL CR for TS 38.423) Addition of LTE PRACH Coordination in XnAP	China Telecommunications

R3-204756	PCI selection for NR cells	Huawei

R3-204757	PCI selection for NR cells	Huawei

R3-204758	PRACH configuration conflict detection	Huawei

R3-204759	PRACH configuration conflict detection	Huawei

R3-204760	RACH optimisation in 2 step RACH	Huawei

R3-204761	Beam related MDT configuration	Huawei, Deutsche Telekom, BT

R3-204762	Beam related MDT configuration	Huawei, Deutsche Telekom, BT

R3-204763	Beam related MDT configuration	Huawei, Deutsche Telekom, BT

R3-204764	Inter-system inter-RAT energy saving	Huawei

R3-204765	Inter-system inter-RAT energy saving	Huawei

R3-204766	Inter-system inter-RAT energy saving	Huawei

R3-204767	Energy efficiency	Huawei

R3-204768	[Draft] Reply LS on reply LS on energy efficiency	Huawei

R3-204801	Inter-system Mobility Load Balancing	Qualcomm Incorporated

R3-204802	Inter-system Energy savings	Qualcomm Incorporated

R3-204803	Coverage and Capacity Optimization	Qualcomm Incorporated

R3-204804	Enhancements to Mobility Settings Change Procedure	Qualcomm Incorporated

R3-204852	Detection of PCI conflict	Nokia, Nokia Shanghai Bell

R3-204878	PCI Selection consideration	NEC

R3-204896	Support on MRO for SN Change Failure	China Telecommunication

R3-204897	Discussion on support of Successful Handover report	China Telecommunication

R3-204898	Discussion on support of UE history information	China Telecommunication

R3-204899	Discussion on Mobility Enhancement Optimization	China Telecommunication

R3-204906	MDT in MR-DC	Huawei

R3-204907	MDT in MR-DC	Huawei

R3-204908	MDT in MR-DC	Huawei

R3-204918	RLF report for CHO MRO	Lenovo, Motorola Mobility,ZTE

R3-204919	RLF report for DAPS handover	Lenovo, Motorola Mobility, ZTE

R3-204935	Successful Handovers Reports	Huawei

R3-204936	UE history information in EN-DC	Huawei

R3-204937	Load balancing enhancement	Huawei

R3-204938	Load balancing enhancement	Huawei

R3-204939	MRO for SN change failure	Huawei

R3-204940	CCO	Huawei

R3-204941	CCO	Huawei

R3-204942	inter-system Load balancing	Huawei

R3-204943	inter-system Load balancing	Huawei

R3-204944	Impacts of mobility enhancements	Huawei

R3-204977	Detection of wrong SN change	Nokia, Nokia Shanghai Bell

R3-204978	Clarification of the use of the TNL load information	Nokia, Nokia Shanghai Bell

R3-204979	Clarification of the TNL Capacity Indicator	Nokia, Nokia Shanghai Bell

R3-204980	Clarification of the TNL Capacity Indicator	Nokia, Nokia Shanghai Bell

R3-204981	Load metric based on PRB occupancy	Nokia, Nokia Shanghai Bell

R3-204982	Load information per slice	Nokia, Nokia Shanghai Bell

R3-204983	Per-slice load status	Nokia, Nokia Shanghai Bell

R3-204984	Per-slice load status	Nokia, Nokia Shanghai Bell

R3-204985	Data forwarding in case of a HO to wrong cell	Nokia, Nokia Shanghai Bell

R3-204986	Enabling fetching DL data in case of a HO to wrong cell	Nokia, Nokia Shanghai Bell

R3-205005	Energy Efficiency calculations in NG RAN	Ericsson

R3-205006	Reply LS on energy efficiency	Ericsson

R3-205007	Successful Handover Report CR 38.423	Ericsson

R3-205008	Successful Handover Report CR 38.473	Ericsson

R3-205009	Successful Handover Report	Ericsson

R3-205010	Discussion on further enhancements for MLB in Rel-17	Ericsson

R3-205011	TP for RACH report availability indication on F1 interface	Ericsson

R3-205012	Solution for RACH Conflict Detection and Resolution at gNB-DU	Ericsson

R3-205013	PRACH Configuration of neighbouring cells for EN DC scenario	Ericsson

R3-205014	NR CCO – Use cases and tools	Ericsson

R3-205015	NR CCO – Corrective actions	Ericsson

R3-205016	XnAP NR CCO Support	Ericsson

R3-205017	F1AP NR CCO additions for triggered actions	Ericsson

R3-205018	X2AP NR CCO additions for triggered actions	Ericsson

R3-205019	Addition of NR CCO as part of NR SON functions	Ericsson

R3-205020	Discussion on inter-system MLB for NG-RAN Rel-17	Ericsson

R3-205021	2-step RACH optimization for SON	Ericsson

R3-205022	Resource optimization for Conditional Handover	Ericsson

R3-205023	Mobility Robustness Optimization for Conditional Handover	Ericsson

R3-205024	Mobility Robustness Optimization for Conditional Handover CR 38.300	Ericsson

R3-205025	Enabling URI configuration within Trace Activation over S1AP and X2AP	Ericsson

R3-205026	Enabling URI configuration within Trace activation over S1AP	Ericsson

R3-205027	Enabling URI configuration within Trace activation over X2AP	Ericsson

R3-205028	Beam measurement inclusion in M1 measureurement of immediate MDT	Ericsson

R3-205029	Beam measurement inclusion in M1 measureurement of immediate MDT for NGAP	Ericsson

R3-205065	CCO	Huawei

R3-205105	Discussion on PSCell MLB and SUL PRB usage	CATT

R3-205106	Support of PSCell MLB and SUL PRB usage	CATT

R3-205107	CR on 38.423 for SUL PRB usage	CATT

R3-205108	CR on 38.473 for SUL PRB usage	CATT

R3-205109	CR on 36.423 for PSCell MLB and SUL PRB usage	CATT

R3-205111	Discussion on Rel-16 leftover issues for PRACH coordination	CATT

R3-205112	CR on PRACH coordination for F1AP	CATT

R3-205113	CR on PRACH coordination for X2AP	CATT

R3-205114	Discussion on RACH report for SgNB	CATT

R3-205115	LS to RAN2 on RACH report for SgNB	CATT

R3-205116	Discussion on RACH optimisation for 2-step RACH	CATT

R3-205117	LS to RAN2 on RACH report for 2-step RACH	CATT

R3-205125	Discussion on MRO for CHO mobility enhance	CATT

R3-205126	Discussion on MRO for DAPS mobility enhance	CATT

R3-205129	Consideration on support of PCI selection	CATT

R3-205130	Consideration on support of SN change failure in case of MR-DC	CATT

R3-205131	Enhancement of UE history information in EN-DC scenario	CATT

R3-205204	2-step RACH Configuration Exchange	Nokia, Nokia Shanghai Bell

R3-205205	RACH Conflict Resolution	Nokia, Nokia Shanghai Bell

R3-205206	Enhancement of RACH Conflict Resolution	Nokia, Nokia Shanghai Bell

R3-205207	Enhancement of RACH Conflict Resolution	Nokia, Nokia Shanghai Bell

R3-205250	Load balancing issue via Xn and F1	LG Electronics

R3-205259	Discussion on inter-system inter-RAT energy saving	CATT

R3-205260	Support of MDT for MR-DC	CATT

R3-205305	Discussion on PCI Selection in NR	ZTE

R3-205306	CR for TS38.300 on PCI Selection	ZTE

R3-205307	Consideration on RAN energy efficiency	ZTE, China Unicom ,China Telecom

R3-205308	CR for TS38.300 on energy efficiency	ZTE, China Unicom ,China Telecom

R3-205309	draft reply LS to SA5 for energy efficiency	ZTE, China Unicom ,China Telecom

R3-205310	UE History Information in MR-DC	ZTE, Lenovo, Motorola Mobility

R3-205311	37340 CR for UHI of MR-DC	ZTE

R3-205312	CR for UHI of MR-DC in TS36.423	ZTE

R3-205313	CR for UHI of MR-DC in TS38.413	ZTE

R3-205314	CR for UHI of MR-DC in TS38.423	ZTE

R3-205315	Discussion on Load Balancing Enhancements	ZTE

R3-205316	CR for TS 36.423 on Load Balancing Enhancements	ZTE

R3-205317	CR for TS 38.423 on Load Balancing Enhancements	ZTE

R3-205318	CR for TS 38.463 on Load Balancing Enhancements	ZTE

R3-205319	CR for TS 38.473 on Load Balancing Enhancements	ZTE

R3-205320	SN change failure	ZTE

R3-205321	CR for TS 38.300 MRO for SN change failure	ZTE

R3-205322	CR for TS 38.423 MRO for SN change failure	ZTE

R3-205323	Left issue for Rel-16 RACH Optimization	ZTE, China Telecom, China Unicom

R3-205324	Discussion on Coverage and Capacity Optimization	ZTE, China Unicom ,China Telecom

R3-205325	CR for TS38.300 on Coverage and Capacity Optimization	ZTE, China Unicom ,China Telecom

R3-205326	CR for TS38.423 on Coverage and Capacity Optimization	ZTE, China Unicom ,China Telecom

R3-205327	CR for TS38.473 on Coverage and Capacity Optimization	ZTE, China Unicom ,China Telecom

R3-205328	Consideration on inter-system inter-RAT energy saving	ZTE, China Telecom, China Unicom

R3-205329	CR for TS36.300 on Inter-System Inter-RAT Energy Saving	ZTE, China Telecom, China Unicom

R3-205330	CR for TS38.300 on Inter-System Inter-RAT Energy Saving	ZTE, China Telecom, China Unicom

R3-205331	CR for TS38.413 on Inter-System Inter-RAT Energy Saving	ZTE, China Telecom, China Unicom

R3-205332	Discussion on Inter-System Load Balancing	ZTE

R3-205333	CR for TS 38.300 on Inter-System Load Balancing	ZTE

R3-205334	CR for TS 38.413 on Inter-System Load Balancing	ZTE

R3-205335	Initial consideration on Two-Step RACH Optimization for SON	ZTE, China Telecom, China Unicom

R3-205336	Initial consideration on Mobility Enhancement Optimization for SON	ZTE, Lenovo, Motorola Mobility

R3-205386	MRO for SN Change Failure	Samsung

R3-205387	Support of SN change failure (option 1)	Samsung

R3-205388	Support of SN change failure (option 2)	Samsung

R3-205389	LS on information needed for MRO in SCG Failure Report	Samsung

R3-205390	CCO baseline solution for NG-RAN	Samsung

R3-205391	Support of CCO function	Samsung

R3-205392	Support of CCO function	Samsung

R3-205410	Discussion on mobility enhancement optimization on DAPS	Samsung

R3-205419	NGAP impacts for Inter-System Inter-RAT Energy Saving	Samsung

R3-205420	Discussion for Inter-System Inter-RAT Energy Saving	Samsung

R3-205421	NGAP impacts for Successful Handover Report	Samsung

R3-205422	XnAP impacts for Successful Handover Report	Samsung

R3-205423	Discussion for Successful Handover Report	Samsung

R3-205429	Inter-system load balancing	CMCC

R3-205430	Load balancing enhancements	CMCC

R3-205431	CR to 36.300 for inter-system load balancing	CMCC

R3-205432	CR to 36.300 for inter-system inter-RAT energy saving	CMCC

R3-205433	CR to 38.300 for inter-system load balancing	CMCC

R3-205434	CR to 38.300 for inter-system inter-RAT energy saving	CMCC

R3-205435	Support of inter-system inter-RAT energy saving	CMCC

R3-205436	Work plan for enhancement of data collection for SON_MDT in NR and EN-DC WI	CMCC, Ericsson

R3-205437	2-step RACH optimisation	CMCC

R3-205445	BLCR to 38.300_Addition of SON features enhancement	CMCC

R3-205446	Mobility optimization enhancement	CMCC

R3-205453	MDT Enhancements coexistence with IDC	ZTE

R3-205454	MDT coexistence with IDC CR for 38.401	ZTE

R3-205455	MDT coexistence with IDC CR for E1AP	ZTE

R3-205456	MDT coexistence with IDC CR for F1AP	ZTE

R3-205457	R3-20xxxx L2 measurement	ZTE

R3-205458	Discussion for UE History Information in EN-DC	BEIJING SAMSUNG TELECOM R&D

R3-205459	X2AP impacts for UE history information in EN-DC	BEIJING SAMSUNG TELECOM R&D

R3-205507	CB: # 1000_SONMDT_BLCRs - Summary of email discussion	CMCC - moderator

R3-205508	CB: # 1001_SONMDT_PCISelect - Summary of email discussion	Huawei - moderator

R3-205509	CB: # 1002_SONMDT_EnergyEff - Summary of email discussion	Ericsson - moderator

R3-205510	CB: # 1003_SONMDT_SuccessHO - Summary of email discussion	Samsung - moderator

R3-205511	CB: # 1004_SONMDT_UEHist - Summary of email discussion	ZTE - moderator

R3-205512	CB: # 1005_SONMDT_LoadBalance - Summary of email discussion	CATT - moderator

R3-205513	CB: # 1006_SONMDT_SNChangeFail - Summary of email discussion	Nokia - moderator

R3-205514	CB: # 1007_SONMDT_RACH - Summary of email discussion	Nokia - moderator

R3-205515	CB: # 1008_SONMDT_CCO - Summary of email discussion	Qualcomm - moderator

R3-205516	CB: # 1009_SONMDT_InterSystemEnergy - Summary of email discussion	CMCC - moderator

R3-205517	CB: # 1010_SONMDT_InterSystemLoad - Summary of email discussion	Ericsson - moderator

R3-205518	CB: # 1011_SONMDT_2StepRACH - Summary of email discussion	CATT - moderator

R3-205519	CB: # 1012_SONMDT_MobEnh - Summary of email discussion	Lenovo - moderato

R3-205520	CB: # 1013_SONMDT_MDTEnh - Summary of email discussion	Ericsson - moderator

R3-205521	CB: # 1014_SONMDT_MDTMR-DC - Summary of email discussion	Huawei - moderator

R3-205534	XnAP NR CCO Support	Ericsson

R3-205535	Discussion and solution on PCI Reconfiguration	Ericsson, Verizon Wireless

R3-205536	Discussion and Solution for Distributed PCI Selection	Ericsson, Verizon Wireless

R3-205537	Discussion and Solution for Centralised PCI Selection	Ericsson, Verizon Wireless

R3-205538	Energy Efficiency calculations in NG RAN	Ericsson

R3-205627	LS on Successful Handover Report	Samsung

R3-205647	LS to RAN2 on RACH report for SgNB	CATT

R3-205649	CR to 36.300 for inter-system inter-RAT energy saving	CMCC

R3-205650	CR to 38.300 for inter-system inter-RAT energy saving	CMCC

R3-205651	BLCR to 38.300_Addition of SON features enhancement	CMCC

R3-205654	BLCR to 38.300_Addition of SON features enhancement	CMCC

R3-205655	CB: # 1009_SONMDT_InterSystemEnergy - Summary of email discussion	CMCC - moderator

R3-205656	CB: # 1001_SONMDT_PCISelect - Summary of email discussion	Huawei - moderator

R3-205657	Reply LS on energy efficiency	Ericsson

R3-205658	CB: # 1002_SONMDT_EnergyEff - Summary of email discussion	Ericsson - moderator

R3-205659	CB: # 1004_SONMDT_UEHist - Summary of email discussion	ZTE - moderator

R3-205660	CB: # 1005_SONMDT_LoadBalance - Summary of email discussion	CATT - moderator

R3-205661	CB: # 1006_SONMDT_SNChangeFail - Summary of email discussion	Nokia - moderator

R3-205662	LS to RAN2 on RACH report for SgNB	CATT

R3-205663	CB: # 1007_SONMDT_RACH - Summary of email discussion	Nokia - moderator

R3-205664	CB: # 1008_SONMDT_CCO - Summary of email discussion	Qualcomm - moderator

R3-205665	CR to 36.300 for inter-system inter-RAT energy saving	CMCC

R3-205666	CR to 38.300 for inter-system inter-RAT energy saving	CMCC

R3-205667	CB: # 1010_SONMDT_InterSystemLoad - Summary of email discussion	Ericsson - moderator

R3-205677	LS on Successful Handover Report	Samsung

R3-205694	CR for TS38.300 on PCI Selection	ZTE

R3-205741	CB: # 1014_SONMDT_MDTMR-DC - Summary of email discussion	Huawei - moderator

R3-205743	MDT in MR-DC	Huawei

R3-205755	Clarification of the TNL Capacity Indicator	Nokia, Nokia Shanghai Bell

R3-205756	Clarification of the TNL Capacity Indicator	Nokia, Nokia Shanghai Bell

R3-205759	LS on Successful Handover Report	Samsung

R3-205761	LS to RAN2 on RACH report for 2-step RACH	CATT

R3-205762	CB: # 1011_SONMDT_2StepRACH - Summary of email discussion	CATT - moderator

R3-205771	Clarification of the TNL Capacity Indicator	Nokia, Nokia Shanghai Bell

R3-205772	Clarification of the TNL Capacity Indicator	Nokia, Nokia Shanghai Bell

R3-205793	[draft] LS on URI for streaming trace reporting in LTE	Ericsson

R3-205797	LS to RAN2 on RACH report for 2-step RACH	CATT

R3-205798	LS on URI for streaming trace reporting in LTE	Ericsson