3GPP TSG-RAN Meeting #89e                	  RP-201697

Electronic Meeting, September 14 - 18, 2020

Agenda Item:	9.1.2

Source:	MediaTek Inc.

Title:	Motivation to introduce new R17 WI on further RRM enhancement

Document for:		Discussion

# Introduction

The baseline NR RRM requirements have been introduced in Rel-15. Due to the limited time and RAN4 RRM session capacity, RAN4 accepted the worst case requirement without considering further possibility of enhancement and also listed some leftovers that were not discussed in Rel-15 time frame. Some leftover requirements are later captured in a Rel-16 RRM enhancement WI [1], but some are not. In addition, RAN2 in Rel-16 TEI introduced NeedforGap reporting, but the corresponding RAN4 requirement is still missing. In order to catch up those unaddressed RRM requirements as well as further optimize some RRM functions, it is suggested to consider some further enhancement on RRM requirements in Rel-17 time frame.

In this contribution, we first provide the justification on the proposed new WI for further RRM enhancement, followed by the new WI scope. This contribution is a revision from [8].

# Justifications

## 2.1 Active TCI-state switch for CSI reporting via CSI-RS reconfiguration

The whole Rel-15 TCI-state mechanism was designed in RAN1, covering different physical channels and reference signals. In Rel-15, RAN4 has specified the TCI-state switch requirements for PDCCH and PDSCH, triggered by either RRC, MAC or DCI. However, the active TCI-state switch requirements for CSI reporting via CSI-RS reconfiguration was not considered neither in Rel-15 time frame nor even in Rel-16 RRM enhancement WI [1].

The lack of the requirement would create some problem in real network deployment, as illustrated in Figure 1. UE was under the coverage of Tx beam #0 of gNB and configured with TCI-state #0 for PDCCH, PDSCH and CSI-RS. Once UE moves to the coverage of Tx beam #1, network has to trigger TCI-state switch to TCI-state #1 for PDCCH, PDSCH and CSI-RS. The requirements on the preparation time for TCI-state switch for PDCCH and PDSCH are now ready in Section 8.10 of TS38.133. Therefore, network can expect the time that UE starts to receive PDCCH and PDSCH based on new TCI-state. However, network has no expectation on how long UE should be able to report CSI based on new TCI-state. Without a clear expectation, network has no idea whether to trust the CSI report from UE and may tend to adopt conservative MCS in scheduling, leading to throughput loss.

Figure 1. Potential throughput loss due to no requirements for TCI-state switch for CSI-RS for CSI reporting.

The simplest way to avoid this throughput loss is to introduce the requirement about the preparation time of TCI-state switch for CSI-RS for CSI reporting. Then network can have the expectation on UE’s preparation time and even try to optimize the triggering timing for the TCI-sate switch for PDSCH, PDCCH and CSI-RS. The active TCI state switch may consider either configuring a TCI state for the same CSI-RS for CQI or configuring	 new CSI-RS with new TCI state to replace the previous CSI-RS.

Another issue that may require some further study is about the TCI-state mis-match between PDSCH and CSI reporting. As we know the purpose of CSI reporting is to provide the highest usable CQI/PMI/RI for network to schedule PDSCH. If the TCI-state of PDSCH is already changed but the TCI-state of CSI remains the old one, UE’s CSI reporting becomes useless to network. RAN4 can further discuss how to address this mis-match issue.

Scope 1: Introduce UE requirements for TCI state switch for CSI reporting via CSI-RS reconfiguration.

## 2.2 Interruption due to SRS antenna switch

The mechanism of SRS antenna switch was introduced in Rel-15 by RAN1. Detail procedure was specified in TS38.214, as captured in Appendix. The main purpose for this antenna switch is to enable the DL CSI acquisition for different number of Tx and Rx antenna pair, e.g., '1T2R', '2T4R', '1T4R', '1T4R/2T4R', or 'T=R'.

In TS38.331, the corresponding UE capabilities are specified. UE can report in a band combination whether it supports SRS antenna switch. Furthermore, UE can also inform network whether the SRS antenna switch will impact the reception on other band(s) through indicating txSwitchImpactToRx and the transmission on the other band(s) through indicating TxSwitchWithAnotherBand.

The detail descriptions for txSwitchImpactToRx and TxSwitchWithAnotherBand can be found in TS38.306.

srs-TxSwitch Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE, which is mandatory with capability signaling. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas. supportedSRS-TxPortSwitch-r16, which is optional to report, indicates downgrading configuration of SRS Tx port switching pattern. If the UE indicates the support of downgrading configuration of SRS Tx port switching pattern using supportedSRS-TxPortSwitch-r16, the UE shall report the values for this as below, based on what is reported in supportedSRS-TxPortSwitch. supportedSRS-TxPortSwitch; supportedSRS-TxPortSwitch-r16. t1r2; t1r1-t1r2. t1r4; t1r1-t1r2-t1r4. t2r4; t1r1-t1r2-t2r2-t2r4. t2r2; t1r1-t2r2. t4r4; t1r1-t2r2-t4r4. t1r4-t2r4; t1r1-t1r2-t2r2-t1r4-t2r4. - txSwitchImpactToRx indicates the entry number of the first-listed band with UL in the band combination that affects this DL, which is mandatory with capability signaling; - txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL in the band combination that switches together with this UL, which is mandatory with capability signaling. For txSwitchImpactToRx and txSwitchWithAnotherBand, value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities.

From UE performance perspective, impacting Rx or Tx on another band(s) implies that the interruption can be expected on other band(s) when performing SRS antenna switch. However, in current RAN4 RRM spec the corresponding interruption requirements due to SRS antenna switch are missing. This inconsistency between RAN2 and RAN4 should be avoid. Furthermore, it would be beneficial to network to know accurately the starting time and the length of the interruption in order to minimize the degradation to outer loop link adaption (OLLA). Therefore, it is desired to introduce the interruption requirements due to SRS antenna switch.

Scope 2: Introduce the interruption requirements due to SRS antenna switch.

## 2.3 Measurement requirements regarding NeedforGap.

In RAN2#110e meeting, a new NeedforGap mechanism was introduced in Rel-16 TEI [2][3][4][5]. UE can report needForGapsInfoNR-r16 in the messages RRCReconfigurationComplete and RRCResumeComplete. Through needForGapsInfoNR-r16, UE reports whether gap is needed for SSB based measurements on an intra-frequency layer or on a band belonging with a target inter-frequency layer. This information is important for both network and UE to have a common understanding on which condition measurement gap is not needed. By avoid configuring gap in some scenarios, network and UE throughputs can be increased correspondingly.

#### – NeedForGapsInfoNR The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured. NeedForGapsInfoNR information element -- ASN1START -- TAG-NeedForGapsInfoNR-START NeedForGapsInfoNR-r16 ::= SEQUENCE { intraFreq-needForGap-r16 NeedForGapsIntraFreqlist-r16, interFreq-needForGap-r16 NeedForGapsBandlistNR-r16 } NeedForGapsIntraFreqlist-r16 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 NeedForGapsBandlistNR-r16 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 NeedForGapsIntraFreq-r16 ::= SEQUENCE { servCellId-r16 ServCellIndex, gapIndicationIntra-r16 ENUMERATED {gap, no-gap} } NeedForGapsNR-r16 ::= SEQUENCE { bandNR-r16 FreqBandIndicatorNR, gapIndication-r16 ENUMERATED {gap, no-gap} } -- TAG-NeedForGapsInfoNR-STOP -- ASN1STOP NeedForGapsInfoNR field descriptions. intraFreq-needForGap Indicates the measurement gap requirement information for NR intra-frequency measurement.. interFreq-needForGap Indicates the measurement gap requirement information for NR inter-frequency measurement.. NeedForGapsIntraFreq field descriptions. servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.. gapIndicationIntra Indicates whether measurement gap is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value no-gap indicates a measurement gap is not needed to measure the SSB associated to the initial DL BWP for all configured BWPs, no matter the SSB is within the configured BWP or not.. NeedForGapsNR field descriptions. bandNR Indicates the NR target band to be measured.. gapIndication Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed..

However, this new NeedforGAP mechanism does not have its corresponding RAN4 requirements captured in TS36.133 and TS38.133. Without UE requirements, there are some ambiguity for network to leverage this newly-introduced UE capability. For example,

- The current CSSF requirements do not consider this additional information of NeedforGap yet. If a certain inter-frequency layer can be measured gaplessly, it should be counted in CSSF outside gap rather than CSSF within gap. Similarly, some updates can be considered also for the requirement of an intra-frequency layer with gap in Rel-15 but reported with ‘no-gap’ in Rel-16.

- Whether and when to consider interruptions due to RF re-tuning (e.g., Network control small gap, NCSG) is another issue that needs some further discussion. If each inter-frequency layer is considered separately, UE may be able to measure every layer gaplessly together with its serving CCs. However, when all these inter-frequency layers are configured at the same time to UE, UE may not have sufficient separate sets of RF resources to receive signals all inter-frequency layers simultaneous. With limited and insufficient RF resources, UE still needs to perform RF re-tuning to jump from one inter-frequency layer to another. Interruption requirements may needs to be considered due to RF re-tuning.

Scope 3: Introduce the measurement and interruption requirements for the new NeedforGap mechanism in NR.

## 2.4 Fast DCI-based BWP switch in FR2

In Rel-15, the requirement for BWP switch delay was introduced in Table 8.6.2-1 of Section 8.6 in TS38.133 for DCI-based, timer-based and RRC-based triggering methods. The current DCI and timer-based requirements depend on a per-UE capability (Type 1 or Type 2). The Type 1 delay requirements are in general within 1ms for both FR1 and FR2. The Type 2 delay requirements for FR2 are 9 slots and 18 slots for both 60KHz and 120KHz SCS, respectively, although they are the same in terms of ms.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots [ms]) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 [1] | 3 [3] |
| 1 | 0.5 | 2 [1] | 5 [2.5] |
| 2 | 0.25 | 3 [0.75] | 9 [2.25] |
| 3 | 0.125 | 6 [0.75] | 18 [2.25] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

According to current requirements, UE is not required to receive or transmit signals during the switch delay on a carrier which BWP switch occurs. That means for 60KHz and 120KHz SCS, there are going be 8 and 17 slots service suspended during BWP switch, respectively. These intervals of zero throughput on carriers which BWP switch occurs is illustrated in Figure 2. These intervals of zero throughput are obvious not desirable for the system. It is worthwhile to study the possibility of some enhancement to reduce the throughput loss by having shorter switch delay.

In Rel-16, the delay for BWP switch has been extended for multiple purposes (e.g., Dormancy) and under different scenarios (cross-CC and multiple CC). The current BWP switch delay for above new use cases are already very complicated with different rules and UE capability to be considered. Fast BWP switch is possible under above use cases via UE capability reporting, e.g., extend 200us for each additional CC that UE has to perform BWP switch at the same time. However, for the BWP switch on single CC (Rel-15 baseline), there is no mechanism to further shorten the BWP switch delay. So we think RAN4 can simply target to shorten the BWP switch delay on single CC. Depending on RAN4 conclusion, it can be adding a new UE capability (e.g., Type 3) or a directly tightening on Type 2 delay.

Scope 4: Study the feasibility to introduce a shorter DCI-based BWP switch delay for FR2. Introduce the corresponding requirement, if concluded feasible.

Figure 2. Intervals of zero throughput during BWP switch on FR2 carriers with 60KHz and 120KHz SCS

## 2.5 Enhanced utilization of UL slots before and after measurement gap

In Rel-15, RAN4 introduced the requirement of measurement gap interruption to create a common understanding between UE and network on whether the slots cannot be used for data transmission and reception. The reference timing of measurement gap has been agreed to follow the DL subframe timing. (E.g., Note 4 in Table 9.1.2-3 of TS38.133). However, for TDD carrier with timing advance, UL slot boundary would be advanced and may not align with DL slot. According to Section 4.2 of TS38.213, the timing advance can be up to 2 slots. Therefore, the UL slot(s) right after the measurement gap may be overlapped by gap. To deal with this issue the following requirements were introduced in Section 9.1.2 of TS38.133.

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s), - when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is other than 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if for the UL transmission is less than the length of one slot; L=2 otherwise.

In principle, it is up to UE implementation on whether to conduct UL transmission on those slot(s). Therefore, network actually has no confidence on whether UE can transmit or not on those slot(s). In a conservative way network will never try to schedule UL grant on those slots. In an aggressive way network can anyway schedule the UL grants regardless of UE’s real situation. Anyhow, there is either some throughput loss or some waste of the radio resources. Note that the same issue may also happens on those slots right before measurement gap, as illustrated in Figure 3. It would be better to study a mechanism to efficiently utilize the UL slots right before and after measurement gap.

Figure 3 UL slots partially overlapped by measurement gap

Current NR PUSCH/PUCCH scheduling is very flexible. It can occupy only a subset of the OFDM symbols in slots. But even so, network still faces the same risk. The fundamental reason leading to this issue is that network has no idea about the absolute timing difference between UE’s DL slots and UL slots. It makes network difficult to determine how many OFDM symbols are still available for UL transmission. To resolve this issue for better network efficiency, a mechanism can be studied to let network know the timing difference between UE’s DL slots and UL slots.

In RAN4#95e meeting, the interruption requirements for Tx switching between two uplink carriers was introduced [6][7]. The interruption length is specified in terms of # of OFDM symbols. However, there is only a high-level description for the starting OFDM symbol of the interruption.

The DL interruption starts from the first OFDM symbol which fully or partially overlap with the UL switching period in NR carrier.

If the starting OFDM symbol is not well known by the network, it is very difficult for network to schedule the proper subset of DL OFDM symbols in a slot to avoid interruption. If there is always some uncertainty of 1 or 2 OFDM symbol about the starting point, network may needs to further reduce the number of schedule DL OFDM symbols to ensure UE can receive the signals.

Figure 4 Starting OFDM symbol for interruption due to Tx switching between two uplink carriers

Scope 5: Study the method to enhance the utilization of UL slots or symbols before and after measurement gap.

# Scope of further RRM enhancement WI

The scope of this further RRM enhancement WI are summarized as following,

Work phase:

Introduce the following requirements:

- Active TCI-state switch for CSI reporting via CSI-RS reconfiguration

  - Introduce the delay requirements for active TCI-state switch for CSI reporting via CSI-RS, including

    - New TCI state is configured for the same CSI-RS for CQI

    - New CSI-RS configuration with new TCI state is configured to replace the previous CSI-RS configuration

  - Specify UE behavior when the TCI-state for PDSCH is different to the TCI-state for CSI-RS for CSI reporting

- SRS antenna switching

  - Introduce the interruption requirements due to SRS antenna switch in NR SA and MR-DC for the following cases

    - NR SRS antenna switching impacting LTE CC

    - NR SRS antenna switching impacting NR CC

- Gapless measurement

  - Introduce the measurement requirements for new NeedforGap mechanism

    - Measurement delay

    - Carrier-specific scaling factor

    - Interruption due to RF re-tuning.

Study phase:

Study the feasibility and the necessary to introduce the requirement for the following aspects:

- DCI-based BWP switch in FR2

  - Study the feasibility to introduce shortened delay for the DCI-based BWP switch in FR2

    - Target the scenario for BWP switch on single CC.

    - According to the conclusions, corresponding RRC signaling or UE capability support may be needed.

- Enhanced utilization of UL slots before and after measurement gap

  - Study the mechanism to allow UE to report the timing difference between gap boundary and UL slots boundary to network for the purpose of enhancing the utilization of (partial) UL slots before and after measurement gap

    - The study should consider at least different measurement gap configurations, e.g., per-UE gap or per-FR gap, with or without measurement gap timing advance, and different timing advance groups.

    - The study should consider the frequency, accuracy and granularity for UE to reporting this timing difference.

    - According to the conclusions, corresponding RRC signaling, MAC mechanism or UE capability support may be needed.

# Conclusions

In this contribution, we provide the justification of the proposed new Rel-17 WI of further RRM enhancement with the following scope.

Scope 1: Introduce UE requirements for TCI state switch for CSI reporting via CSI-RS reconfiguration.

Scope 2: Introduce the interruption requirements due to SRS antenna switch.

Scope 3: Introduce the measurement and interruption requirements for the new NeedforGap mechanism in NR.

Scope 4: Study the feasibility to introduce a shorter DCI-based BWP switch delay for FR2. Introduce the corresponding requirement, if concluded feasible.

Scope 5: Study the method to enhance the utilization of UL slots or symbols before and after measurement gap.

# Reference

[1] RP-193211, “Revised WID: NR RRM enhancement in R16”, Intel Corporation

[2] R2-2004806, “Introduction of NeedForGap capability for NR measurement”, MediaTek inc.

[3] R2-2004808, “Introduction of NeedForGap capability for NR measurement”, MediaTek inc.

[4] R2-2006353, “Introduction of NeedForGap capability for NR measurement”, MediaTek inc.

[5] R2-2006354, “Introduction of NeedForGap capability for NR measurement”, MediaTek inc.

[6] R4-2008624, “CR on DL interruption on LTE carriers at Tx switching between two uplink carriers”, Huawei, HiSilicon

[7] R4-2008625, “CR on DL interruption Tx switching between two uplink carriers”, Huawei, HiSilicon

[8] R4-2010316, “Motivation to introduce new R17 WI on further RRM enhancement”, MediaTek inc.

# Appendix

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with one of the following configurations depending on the indicated UE capability ('1T2R', '2T4R', '1T4R', '1T4R/2T4R', or 'T=R'): - up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or - up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, or - zero or one SRS resource set configured with higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' with four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and - zero or two SRS resource sets each configured with higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic' and with a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. The UE shall expect that the two sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value(s) of the higher layer parameter aperiodicSRS-ResourceTrigger in each SRS-ResourceSet are the same, and the value of the higher layer parameter slottOffset in each SRS-ResourceSet is different. Or, - up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4. The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. If the indicated UE capability is '1T4R/2T4R,' the UE shall expect to be configured with the same number of SRS ports, either one or two, for all SRS resources in the SRS resource set(s). If the indicated UE capability is '1T2R', '2T4R', '1T4R', '1T4R/2T4R', the UE shall not expect to be configured or triggered with more than one SRS resource set in the same slot. If the indicated UE capability is 'T=R,' the UE shall not expect to be configured or triggered with more than one SRS resource set in the same symbol. The value of Y is defined by Table 6.2.1.2-1. Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching Y [symbol]. 0; 15; 1. 1; 30; 1. 2; 60; 1. 3; 120; 2.