3GPP TSG-RAN Meeting #89e	RP-201873

# Electronic Meeting,  September 14 – 18, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0261 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of fallbacks for FR2 CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | - |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple, Ericsson, US Cellular, T-Mobile US, Verizon, Bell Mobility, AT&T, Turkcell, SK Telecom, Softbank, LG Uplus, Telstra, Reliance Jio, Telecom Italia, British Telecom, Orange, Deutsche Telekom, CHTTL |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-09-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Mandating additional requirements for all possible fallback band combinations for a band combination consisting of mixed contiguous and non-contiguous intra-band CA in FR2 results in an excessive number of fallback combinations. That requires that requirements for an excessive amount of fallback band combinations must be specified in 3GPP and also complied to by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that a terminal which supports a higher order CA or DC configuration, which includes FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, shall conform to the RF performance requirements in this specification of the higher order combination and support the fallbacks without additional RF performance requirements for the fallbacks with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination. Requirements in other specifications are not affected. The result of this is that a fallback band combination which: - consists of multiple sub-blocks, and - has at least one sub-block comprising a contiguous CA combination (i.e. a letter B, C, D, …) is exempt from the requirements. Impact analysis Impacted functionality: Requirements for band combinations. Inter-operability: No inter-operability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This specification is not aligned with other specifications (38.331 and 38.306) with regards to which fallback band combinations are supported and requirements for excessive number of fallback band combinations must be complied to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigurationDedicated in the PCell and SCells for NR SA.

For FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, there are no RF performance requirements for the fallbacks with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination. Requirements in other specifications are not affected.

NOTE: The above is an exception with regards to compliance to RF performance requirements. The exception applies to fallback band combinations consisting of multiple subblocks where at least one of the subblocks consists of a contiguous CA combination. The exception does not apply to other band combinations.

## <End of Changes>