# 3GPP TSG-RAN WG2 Meeting #111 electronic									R2-2008566

# Elbonia, August 17th – 28th 2020

| CR--Form--v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1779 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on LTE MOB capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation, China Telecom, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1 Based on RAN1 LS R1-2007420, NOTE: Support for UL cancellation is not introduced in Rel-16”, UL cancellation is not supported for LTE in Rel-16, and therefore the capability ul-TransCancellationDAPS-r16 shall be removed;2 multipleTimingAdvance It is mandatory for UEs to support 2 TAGs for DAPS handover. However, the UE may only support intraFreqDAPS, and does not need to support multipleTimingAdvance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Delete ul-TransCancellationDAPS-r16; 2 clarify, multipleTimingAdvance is mandatory for interFreqDAPS capble UE. Impact Analysis: Impacted functionality: Mobility Capabilities Inter-operability: - If the network is implemented according to the CR and the UE is not, there will be ASN.1 decoding problem in network due to use of an incompatible version of IE UE-EUTRA-Capability by the UE. - If the UE is implemented according to the CR and the network is not, there will be ASN.1 decoding problem in network due to use of an incompatible version of IE UE-EUTRA-Capability by the network. This CR is considered mandatory to support the reporting of UE Rel-16 mobility capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | “ul-TransCancellationDAPS-r16” cannot be supported in rel-16, but still in the specification. The intraFreqDAPS only UE has to indicate the support of multipleTimingAdvance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.219, 4.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS36.331 CR4362 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2006933 |  |  |  |  |  |  |  |  |

#### 4.3.4.219	Void

/*** Next change***/

#### 4.3.5.3	multipleTimingAdvance

This field defines whether multiple timing advances are supported for each band combination supported by the UE. It is mandatory for UEs of this release of the specification to support this capability for band combinations having an UL on multiple FDD bands as specified in TS 36.101 [6]. If the band combination comprised of more than one band entry (i.e., inter-band or intra-band non-contiguous band combination), the field indicates that different timing advances on different band entries are supported. If the band combination comprised of one band entry (i.e., intra-band contiguous band combination), the field indicates that different timing advances across component carriers of the band entry are supported. It is mandatory for UEs to support 2 TAGs for inter frequency DAPS handover.