3GPP TSG-RAN2 Meeting # 111-e	R2-2008666>

# Online, ,  17th August - 28th August

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4443 | rev | - | Current version: | 14.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of SI scheduling for extended SIBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_D2D_Prox-Core, LTE_SC_PTM-Core, LTE_SL_V2V-Core, LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2020-09-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following scenario was discovered from the field experience, when SIB19 and onwards are broadcast over the E-UTRAN network: Case: When an eNB broadcasts SIB1 which includes scheduling information of SI messages including SIB19 and onwards, some legacy UEs that do not handle the SIB-Type IE correctly are unable to acquire SIB1 and consider the cell as barred. - It happens no matter whether SIB19 and onwards are scheduled separately from the other legacy SIBs (SIB2 to SIB18) via a different SI message, or SIB19 and onwards are scheduled together with the legacy SIBs in the same SI message. The problematic UEs are known to include different types of UE such as those used for IoT service, and smartphones. Since the problem is stemming from a UE behaviour that is not compliant with the 3GPP standard, it could be fixed if the UE software were upgraded to the correct UE behaviour defined in the standard. Nevertheless, there were substantial opinions from operators at RAN2 #111-e meeting that not all of the defective UEs can be upgraded, and so there will remain UE(s) in the network which cannot handle the uncomprehended field in SIB1 properly. Suppose that UEs not handling SIB1 properly remain in the network, the problem can instead be addressed by amending the standard such that additional scheduling information for SIB19 and onwards is introduced into SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional scheduling information is introduced into SIB1 so that SIB19 and onwards can be scheduled via the additional field, and thereby hidden from legacy UEs that would mistakenly consider the cell as being barred. SIB19 and onwards can be scheduled together with the legacy SIBs (up to SIB18) into the same SI message, or SIB19 and onwards can be scheduled separately from the legacy SIBs via the different SI message. The existing scheduling information remains in the standard such that the network can choose to schedulde SIB19 and onwards using the existing scheduling information (e.g. if the problematic UEs are known not to exist in this network) or using the newly introduced additional scheduling information (but not in both). Impact analysis: Impacted functionality: SIB broadcast related to SIB19+ Impacted architecture option: LTE SA Inter-operability: - If the network is implemented according to the CR and if it chooses scheduling SIB19 and onwards via the additional field and if the UE is not implemented acccording to the CR, the UE cannot receive SIBs that the network schedules in the extended SIB scheduling information (i.e. schedulingInfoList-v12xy and/or schedulingInfoListExt-r12). As a consequence the UE would not be able to access services and features related to those SIBs. - If the UE is implemented according to the CR and the network is not, the UE can receive SIBs properly, since all of SIBs are scheduled via the legacy scheduling information. No inter-operability issue is foreseen. - If both UE and network are implemented according to the CR and if the network chooses to schedule SIB19 and onwards via the additional field and if the network starts broadcasting ETWS/CMAS within a modification period, then for the remainder of that modification period, the UE may be unable to acquire SIB19 and onwards that are scheduled by the additional field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legacy UEs in the field that implement the ENUMERATED extension incorrectly or are unable to receive SIB19+ due to incorrect implementation may bar a cell broadcasting SIB19 and onwards. If the network was to mainatin service to the incorrectly implemented legacy UEs by not broadcasting SIB19 and onwards then the other UE(s) would be unable to receive SIB19+, and as a consequence they would not be able to access services and features related to those SIBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3, 5.2.3a, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is not backward-compatible for a UE, that is currently capable of receiving SIB19+, in a network that is implemented to use the additional scheduling information introduced by this CR will no longer be able to receive SIB19+. Hence, these UEs would also need to be implementedd according to this CR in order to continue to receive SIB19+. However, the CR, if implemented in the network, addresses the issue for those problematic legacy UEs that cannot be software-updated. In a network without any problematic legacy UEs (e.g. since the eNB operates on a band which those UEs do not support), the network may choose to schedule SIB19 and onwards using the legacy field. It thereby avoids the above mentioned problems with UEs that handle the legacy scheduling list correctly but that do not implement this CR. In such a network, no inter-operability issue is foreseen. However, to enable this possibility, it is vital that UEs requiring SIB19 and onwards are able to acquire them using the legacy scheduling list, as well as the new scheduling list introduced by this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.2.3	Acquisition of an SI message

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList and schedulingInfoListExt if present) in SystemInformationBlockType1;

2>	determine the integer value x = (n – 1)*w, where w is the si-WindowLength;

2>	the SI-window starts at the subframe #a, where a = x mod 10, in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

NOTE:	E-UTRAN should configure an SI-window of 1 ms only if all SIs are scheduled before subframe #5 in radio frames for which SFN mod 2 = 0.

1>	receive DL-SCH using the SI-RNTI from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received, excluding the following subframes:

2>	subframe #5 in radio frames for which SFN mod 2 = 0;

2>	any MBSFN subframes;

2>	any uplink subframes in TDD;

1>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message;

5.2.3a	Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE

When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList and schedulingInfoListExt (if present) in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT);

2>	determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT);

2>	if the UE is a NB-IoT UE:

3>	the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset);

2>	else:

3>	the SI-window starts at the subframe #0 in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

1>	if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB and SystemInformationBlockType1-NB. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

1>	else:

2>	receive and accumulate SI message transmissions on DL-SCH on narrowband provided by si-Narrowband, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength-BR, only in radio frames as provided in si-RepetitionPattern and subframes as provided in fdd-DownlinkOrTddSubframeBitmapBR in bandwidthReducedAccessRelatedInfo, or until successful decoding of the accumulated SI message transmissions;

1>	if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message;

Next change

6.2.2	Message definitions

<< skip unchanged part >>

–	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10xy-IEs		OPTIONAL

}

SystemInformationBlockType1-v10xy-IEs ::= SEQUENCE {

-- This field is only for late non-critical extensions from Rel-10 or Rel-11 onwards

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v12xy-IEs		OPTIONAL

}

SystemInformationBlockType1-v12xy-IEs ::= SEQUENCE {

schedulingInfoList-v12xy			SchedulingInfoList-v12xy	OPTIONAL,	-- Need OR

schedulingInfoListExt-r12			SchedulingInfoListExt-r12	OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL,	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10))		OPTIONAL,	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL,	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v1450-IEs				OPTIONAL

}

SystemInformationBlockType1-v1450-IEs ::=	SEQUENCE {

tdd-Config-v1450						TDD-Config-v1450		OPTIONAL,	-- Cond TDD-OR

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfoList-v12xy ::=	SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-v12xy

SchedulingInfoListExt-r12 ::=	SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfoExt-r12

SchedulingInfo ::=	SEQUENCE {

si-Periodicity

SI-Periodicity-r12,

sib-MappingInfo						SIB-MappingInfo

}

SchedulingInfo-v12xy ::=	SEQUENCE {

sib-MappingInfo-v12xy		SIB-MappingInfo-v12xy				OPTIONAL	-- Need OR

}

SchedulingInfoExt-r12 ::=	SEQUENCE {

si-Periodicity-r12			SI-Periodicity-r12,

sib-MappingInfo-r12			SIB-MappingInfo-v12xy

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13				INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13						ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712,

b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-MappingInfo-v12xy ::=	SEQUENCE (SIZE (1..maxSIB-1)) OF SIB-Type-v12xy

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430}

SIB-Type-v12xy ::=			ENUMERATED {

sibType19-v1250, sibType20-v1310, sibType21-v1430, spare13,

spare12, spare11, spare10, spare9, spare8, spare7, spare6,

spare5, spare4, spare3, spare2, spare1, ...}

SI-Periodicity-r12 ::=		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12						Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

-- ASN1STOP

| SystemInformationBlockType1 field descriptions |
| --- |
| bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3. |
| category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell. |
| cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4. |
| cellBarred barred means the cell is barred, as defined in TS 36.304 [4]. |
| cellIdentity Indicates the cell identity. NOTE 2. |
| cellReservedForOperatorUse As defined in TS 36.304 [4]. |
| cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. If absent, coverage enhancement S criteria is not applicable. NOTE 3. |
| cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3. |  |
| csg-Identity Identity of the Closed Subscriber Group the cell belongs to. |
| csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4]. |
| eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not supported by the network in the cell. NOTE 2. |
| eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present. |
| fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is present, SystemInformationBlockType1-BR-r13 is transmitted in RRCConnectionReconfiguration, and if RRCConnectionReconfiguration does not include systemInformationBlockType2Dedicated, UE may assume the valid subframes in fdd-DownlinkOrTddSubframeBitmapBR are not indicated as MBSFN subframes. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping. |
| hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. |
| ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2. |
| multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42, table 5.5-1] that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. NOTE 2. |
| p-Max Value applicable for the cell. If absent the UE applies the maximum power according to its capability as specified in TS 36.101 [42, 6.2.2]. NOTE 2. |
| q-QualMin Parameter "Qqualmin" in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinOffset Parameter "Qqualminoffset" in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell. |
| schedulingInfoList Indicates scheduling information of SI messages. The schedulingInfoList-v12xy (if present) provides additional SIBs mapped into the SI message scheduled via schedulingInfoList (without suffix). If E-UTRAN includes schedulingInfoList-v12xy, it includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| schedulingInfoListExt Indicates scheduling information of additional SI messages. The UE concatenates the entries of schedulingInfoListExt to the entries in schedulingInfoList, according to the general concatenation principles for list extension as defined in 5.1.2. If the schedulingInfoListExt is present, E-UTRAN ensures that the total number of entries of this field plus schedulingInfoList (without suffix) shall not exceed the value of maxSI-Message. |
| sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList (without suffix) list. If present, sib-MappingInfo-v12xy indicates one or more additional SIBs mapped to the concerned SI message listed in the schedulingInfoList (without suffix) list. If schedulingInfoList-v12xy or schedulingInfoListExt-r12 is present, E-UTRAN shall not include any value indicating SIB of type 19 or higher in sib-MappingInfo (without suffix). If schedulingInfoList-v12xy is present, E-UTRAN ensures that the total number of entries of this field plus sib-MappingInfo (without suffix) shall not exceed the value of maxSIB-1. |
| si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging. |
| si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21, 6.4.1] and TS 36.213 [23, 7.1.6]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21]. |
| si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window. |
| si-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. |
| si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23, Table 7.1.7.2.1-1] for a 6 PRB bandwidth and a QPSK modulation. |
| schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h. |
| si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13. |
| startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. |
| systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| systemInfoValueTagSI SI message specific value tag as specified in clause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. |
| systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12 and SIB14. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message. |
| tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2. |
| trackingAreaCode A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5. |

NOTE 1:	The value the UE applies for parameter "Qqualmin" in TS 36.304 [4] depends on the q-QualMin fields signalled by E-UTRAN and supported by the UE. In case multiple candidate options are available, the UE shall select the highest priority candidate option according to the priority order indicated by the following table (top row is highest priority).

| q-QualMinRSRQ-OnAllSymbols | q-QualMinWB | Value of parameter "Qqualmin" in TS 36.304 [4] |
| --- | --- | --- |
| Included | Included | q-QualMinRSRQ-OnAllSymbols – (q-QualMin – q-QualMinWB) |
| Included | Not included | q-QualMinRSRQ-OnAllSymbols |
| Not included | Included | q-QualMinWB |
| Not included | Not included | q-QualMin |

NOTE 2:	E-UTRAN sets this field to the same value for all instances of SIB1 message that are broadcasted within the same cell.

NOTE 3:	E-UTRAN configures this field only in the BR version of SIB1 message.

NOTE 4:	E-UTRAN configures at most 6 PLMNs in total (i.e. across all the PLMN lists in SIB1).

NOTE 5:	E-UTRAN configures only one value for this parameter per PLMN.

| Conditional presence | Explanation |
| --- | --- |
| BW-reduced | The field is optional present, Need OR, if schedulingInfoSIB1-BR in MIB is set to a value greater than 0. Otherwise the field is not present. |
| FBI-max | The field is mandatory present if freqBandIndicator (i.e. without suffix) is set to maxFBI. Otherwise the field is not present. |
| mFBI | The field is optional present, Need OR, if multiBandInfoList is present. Otherwise the field is not present. |
| mFBI-max | The field is mandatory present if one or more entries in multiBandInfoList (i.e. without suffix, introduced in -v8h0) is set to maxFBI. Otherwise the field is not present. |
| RSRQ | The field is mandatory present if SIB3 is being broadcast and threshServingLowQ is present in SIB3; otherwise optionally present, Need OP. |
| RSRQ2 | The field is mandatory present if q-QualMinRSRQ-OnAllSymbols is present in SIB3; otherwise it is not present and the UE shall delete any existing value for this field. |
| Hopping | The field is mandatory present if si-HoppingConfigCommon field is broadcasted and set to on. Otherwise the field is optionally present, need OP. |
| QrxlevminCE1 | The field is optionally present, Need OR, if q-RxLevMinCE1-r13 is set below -140 dBm. Otherwise the field is not present. |
| TDD | This field is mandatory present for TDD; it is not present for FDD and the UE shall delete any existing value for this field. |
| TDD-OR | The field is optional present for TDD, need OR; it is not present for FDD. |
| WB-RSRQ | The field is optionally present, need OP if the measurement bandwidth indicated by allowedMeasBandwidth in systemInformationBlockType3 is 50 resource blocks or larger; otherwise it is not present. |
| SI-BR | The field is mandatory present if schedulingInfoSIB1-BR is included in MIB with a value greater than 0. Otherwise the field is not present. |

End of changes