3GPP TSG RAN meeting #90e										RP- 202251

Electronic Meeting, December 7-11, 2020

Status Report to TSG

Agenda item:			9.2.7

| WI / SI Name | 5G V2X with NR sidelink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | 5G_V2X_NRSL |  |  |  |  |
| Unique ID | 830078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191723 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: mm/yyyy | Performance part: 03/2021 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 100% | Performance Part: 65% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Hanbyul Seo |
|  | Company | LG Electronics |
|  | Email | hanbyul.seo@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#97-e : RRM(Performance)

In RRM session, 24 contributions were submitted.

Following 2 CRs, 8 draft CRs and 1 draft big CR were endorsed for performance requirements.

| Draft big CR(10.4, A3.19, A.9, B.4) Introduction of Rel-16 NR V2X RRM performance requirements (TS 38.133) | Draft Big CR | R4-2017105 |
| --- | --- | --- |
| 10.4/10.4.1/10.4.2 Intra-frequency PSBCH-RSRP accuracy requirements for FR1 | Draft CR | R4-2017104 |
| 10.4.3 Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 10.4.4 Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 | CR | R4-2017102 |
| A.3.19 V2X sidelink communication A.3.19.1 Introduction A.3.19.2 Reference resource pool configurations for V2X Sidelink Communication A.3.19.3 Reference measurement channels for V2X Sidelink Communication | Draft CR | R4-2017109 |
| A.9 V2X Tests A.9.1 V2X Tests in FR1 A.9.1.1 Test for V2X UE Transmit Timing | Draft CR | R4-2017106 |
| A.9.1.2 Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication | Draft CR | R4-2017107 |
| A.9.1.3 Test for V2X Synchronization Reference Selection/Reselection | Draft CR | R4-2017108 |
| A.9.1.4 Test for L1 SL-RSRP Measurement A.9.1.4.1 Test for V2X UE Autonomous Resource Selection/Reselection A.9.1.4.2 Test for V2X UE Resource Pre-emption | Draft CR | R4-2017109 |
| A.9.1.4.3 Test for V2X UE Resource Re-evaluation | Draft CR | R4-2017110 |
| A.9.1.5 Test for Congestion Control Measurement | Draft CR | R4-2017347 |
| A.9.1.6 Test for Interruption | Draft CR | R4-2017112 |
| B.4 Conditions for V2X | CR | R4-2017103 |

One WF on performance requirements was agreed (R4-2017100).

- Agreements of L1 SL-RSRP measurement

  - Define ±4.5dB as absolute measurement accuracy of L1 SL-RSRP at SNR=0dB when measured at UE antenna connector

- Agreements of NR V2X operating bands

  - Introduce NR_TDD_FR1_B for n38 and NR_TDD_FR1_J for n47 for NR V2X operating bands

- Agreements of Minimum NR V2X RPs related to side condition

  - For NR_TDD_FR1_B

    - -126.5 dBm/15kHz , -123.5dBm/30kHz, -120.5dBm/60kHz @ Es/Iot  -6 dB

    - -120.5 dBm/15kHz , -117.5dBm/30kHz, -114.5dBm/60kHz @ Es/Iot  0 dB

  - For NR_TDD_FR1_J

    - -122.5 dBm/15kHz , -119.5dBm/30kHz, -116.5dBm/60kHz @ Es/Iot  -6 dB

    - -116.5 dBm/15kHz , -113.5dBm/30kHz, -110.5dBm/60kHz @ Es/Iot  0 dB

- Agreements of Test Cases

  - Common resource pool configuration

    - Configure one Rx resource pool and one normal Tx resource pool

    - Configure subchannel size with 10RB

    - Configure Sensing window with 100ms

  - PSCCH Reference Measurement Channel(RMC)

    - # of PSCCH symbols per slot = 2 symbols

    - # of PSCCH PRB = 10 RB

    - Information Bit Payload(w/o CRC) = 26 bits

    - TBS = 360 bits

  - PSSCH Reference Measurement Channel(RMC)

    - # of PSSCH symbols per slot = 10 symbols

    - # of PSSCH PRB = 10 RB

    - QPSK & 1/3 code rate

    - Information Bit Payload(w/o CRC) = 672 bits

    - TBS = 2160 bits

    - Note 2 : 2nd state SCI and PSFCH are not allocated per slot.

  - Introduce Test for Resource Pre-emption

  - Introduce Test for Resource Re-evaluation (a separated test case from pre-emption test)

  - For Test for Selection / Reselection of V2X Synchronization Reference Source for GNSS as higher priority

    - Set up with 3 SyncRef UEs

      - SyncRef UE1 (sync to gNB directly), SyncRef UE2 (sync to GNSS in-directly) and SyncRef UE3 (sync to GNSS directly). Work Plan

RAN4#97-e : Demodulation

In Demodulation session, 28 contributions were submitted to RAN4#97-e, and e-mail discussion summary was submitted for information.

Two WFs (R4-2017687, R4-2017686) on single link and multiple link tests for NR V2X demodulation performance were approved with following contents.

- Common assumptions for reference receiver

  - CFO estimation: Cross-DMRS symbol CFO estimation.

  - CE channel estimation:

    - Frequency domain interpolation: MMSE interpolation

    - Time domain interpolation: Linear interpolation

  - RX window: CP/2 from GNSS

- PSSCH demodulation

  - Test 1: QPSK for 500km/h relative velocity (already agreed the test)

    - 20 PRB for PSSCH resource allocation but FFS for sub-channel size (10 or 20 PRB)

    - FFS for PSFCH periodicity (1 periodicity or 4 periodicity)

    - DMRS pattern

      - {3,4} DMRS symbols if PSFCH periodicity is 4

      - 3 DMRS symbols if PSFCH periodicity is 1

    - Propagation condition

      - TDLA30 is baseline and companies are encouraged to provide analysis for TDLB100 or allocation 3symbols for PSCCH with error floor and margin to 10% BLER

    - Beta value for 2nd stage SCI

      - Beta value = 3.5 and if technical issue will be observed, beta value can be revised in the next meeting.

  - FFS whether to introduce additional following test

    - Option 1: [Test 2] 16QAM for 260km/h relative velocity

    - Option 2: [Test 3] 64QAM for 30km/h relative velocity

    - Option 3: both option 1 and option 2

    - Keep FFS for above options and make decision in RAN4#98e

  - Test 2: 16QAM for 260km/h relative velocity

    - If the test is introduced, following parameters are considered

    - FFS for PRB for PSSCH resource allocation (10 PRB or 20 PRB with n sub-channel size)

    - PSFCH periodicity : 4 periodicity

    - DMRS pattern

      - Option 1: {3,4} DMRS symbols if PSFCH periodicity is 4

      - Option 2: {2,3} DMRS symbols if PSFCH periodicity is 4

    - Beta value for 2nd stage SCI

      - Beta value = 5 and if technical issue will be observed, beta value can be revised in the next meeting.

  - Test 3: 64QAM for 30km/h relative velocity

    - If the test is introduced, following parameters are considered

    - FFS for PRB for PSSCH resource allocation (Align with Test 1 or 10PRB)

    - FFS for PSFCH periodicity (1 periodicity or 4 periodicity)

    - DMRS pattern : 2 DMRS symbols when PSFCH periodicity is 1 or 4

    - Beta value for 2nd stage SCI

      - Beta value = 5 and if technical issue will be observed, beta value can be revised in the next meeting.

  - FFS whether to introduce tests for 256QAM modulation order and gNB based sync.

- PSCCH demodulation

  - FFS for payload size (24 or 26bit) depending on decision of sub-channel size

  - Relative velocity : 260km/h (TDLA30-1400)

- PSBCH demodulation

  - The number of S-SSB per one period : 1 S-SSB

  - Relative velocity : 30km/h (TDLA30-180)

- PSFCH demodulation

  - Feedback mode : NACK only

  - Test metric : Pr(NACK miss)<1% and Pr(DTX to NACK)<1%

  - PSFCH periodicity: 1 periodicity

  - Number of cyclic shift pair : 1 cyclic shift pair

  - Relative velocity : 30km/h (TDLA30-180)

- Power imbalance requirements

  - Ruse the methodology from LTE V2X power imbalance test

  - FFS for ICS level (-27dBc or higher than -27dBc)

  - FFS for distance between two links (10 or 30 PRB)

  - MCS for PSSCH : 4

  - SCI format 2-A configuration : 35bit payload, alpha=1, and beta_offset=3.5

    - If no technical issues will be observed in the next RAN4 meeting, this configuration will be used for power imbalance requirements.

  - Test metric: PSSCH BLER 10% of sidelink UE 2

- HARQ buffer soft combining test

  - The main test purpose is to verify HARQ buffer combining. Configuring of high MCS and long interval between transmission and retransmission is not precluded.

  - Test design

    - 20MHz CBW and FFS for sub-channel size (10 or 20 PRB)

    - For verification of n16 and n24, 16 or 24 UEs transmit signal one by one circularly for every slot and in the first subchannel.

    - For verification of n32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The 32nd UE transmit signal in the same slot as the first UE but in the second subchannel.

    - For verification of n64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal in the same slot as the first two UEs but in the third subchannel.

  - FFS for MCS level (13 or 28 in MCS index Table 1)

  - AWGN channel model for propagation condition

  - PSCCH resource allocation : 2 symbols and 10 RBs

  - PSFCH configuration: PSFCH with periodicity 1

  - SCI stage 2 configuration : 35bit payload and beta_offset=3.5

    - If no technical issues will be observed in the next RAN4 meeting, this configuration will be used for HARQ buffer soft combining requirements.

  - Test metric : 5% of PSSCH BLER

- PSCCH/PSSCH decoding capability test

  - FFS for CBW/SCS

    - Need to further check on the supporting CHBW sets on V2X band, mandatory or optional  and typical deployment scenarios, RAN4 planned to make decision among 40MHz and 20MHz in RAN4#98e

  - FFS for PSFCH configuration (no PSFCH or PSFCH with periodicity 1)

  - DMRS configuration : 2 DMRS symbols

  - MCS level : 4

  - Propagation conditions: Static propagation condition without external noise

  - Test metric: Probability of missed PSCCH 1%

  - Other issues

    - Further discuss how to take into account psfch-TxNumber capability if needed

    - Further discuss whether to verify 2*floor(NRB /10 RBs) capability

- PSFCH decoding capability test

  - Test design

    - N UEs transmit PSFCH one by one following procedure from Section 16.3 of TS 38.1213

    - N is {5, 15, 25, 32, 35, 45, 50, 64} depending on UE capability

    - Just for information. Example for 20 MHz, 1 sub-channel and PSFCH periodicity 1

      - When N is not larger than 51, all N UEs should transmit one by one from RB 0 to A-1 with CS pair index 0.  When N=64, UE i,  0≤i≤50 should transmit PSFCH one by one from RB 0 to 50 with CS pair index 0, the remaining 13 UEs (i.e. UEi, 51≤i≤63) should transmit PSFCHs from RB 0 to RB 12 with CS pair index 3.

  - FFS for CBW/SCS (20MHz/30kHz or 40MHz/30kHz)

  - FFS for number of sub-channels : 1 or maximum possible for considered scenario (i.e. 5 for 20 MHz and 10 for 40 MHz)

  - PSFCH periodicity 1

  - Feedback mode : ACK/NACK

  - Propagation conditions: Static propagation condition without external noise

  - Test metric : Pr(DTX to ACK)<1% and Pr(ACK miss)<1%

- FFS whether to introduce SDR with active sidelink test

Two documents (R4-2017470 and R4-2017472) for single and multiple link simulation assumptions for NR V2X demodulation based on above agreements were approved with following scenarios.

- PSSCH/PSCCH/PSBCH/PSFCH demodulation test

- Power imbalance test / HARQ buffer soft combining test / PSCCH and PSSCH decoding capability / PSFCH decoding capability

2.4.2	Remaining Open issues

RRM performance part

- Finalize performance requirements with Big CR based on the endorsed Draft Big CR

Demodulation performance part

- Finalize test cases and detail simulation assumptions

- Define performance requirements for agreed test cases

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#97-e (RRM)

- R4-2017278	Email discussion summary for [97e][208] 5G_V2X_NRSL_RRM	Moderator (LG Electronics)

- R4-2017100	WF on NR V2X RRM requirements	LG Electronics

- R4-2014213	On interruption requirement on LTE SL due to changing of NR SL sync source	ZTE Corporation

- R4-2014294	Discussion of maintenance issues for NR V2X	 LG Electronics Inc.

- R4-2017101	CR of NR V2X operating band group	LG Electronics Inc.

- R4-2014634	NR V2X RRM core and performance requirement remaining issues	Qualcomm, Inc.

- R4-2017345	CR: Interruption requirement for NR V2X synchronization source chang	Qualcomm, Inc.

- R4-2014767	Remaining issues on NR V2X RRM requirement	MediaTek inc.

- R4-2017105	Draft Big CR: Introduction of Rel-16 NR V2X RRM performance requirements (TS 38.133)	LG Electronics

- R4-2017102	CR of NR V2X measurement accuracy requirements(SL-RSSI and L1 SL-RSRP)	LG Electronics Inc.

- R4-2017103	CR of Annex.B for NR V2X side conditions	LG Electronics Inc.

- R4-2014768	Discussion on L1 SL-RSRP measurement test case	MediaTek inc.

- R4-2017104	DraftCR on PSBCH-RSRP accuracy requirements	Huawei, HiSilicon

- R4-2014640	NR V2X RRM test case discussion	Qualcomm, Inc.

- R4-2017106	DraftCR on UE Transmission Timing Accuracy Tests for NR V2X	Huawei, HiSilicon

- R4-2017107	draft CR of Test for initiation and cease  of SLSS Transmission with V2X Sidelink Communication	LG Electronics Inc.

- R4-2017108	RRM test cases for NR V2X Synchronization Reference Selection/Reselection	Xiaomi

- R4-2014214	Selection or Reselection of V2X Synchronization Reference Source	ZTE Corporation

- R4-2014212	On L1 SL-RSRP accuracy for NR V2X	ZTE Corporation

- R4-2017109	CR: RRM autonomous resource selection test cases for NR V2X	Qualcomm, Inc.

- R4-2017110	CR on V2X UE Resource Selection Tests for Re-evaluation	MediaTek inc.

- R4-2015468	Discussion on UE Autonomous Resource Selection/Reselection Test for NR V2X	Huawei, HiSilicon

- R4-2017347	CR on V2X UE Congestion Control Measurement Test	MediaTek inc.

- R4-2017112	DraftCR on Interruption Tests for NR V2X	Huawei, HiSilicon

RAN4#97-e (Demodulation)

- R4-2017612	Email discussion summary for [97e][317] V2X_Demod_Part1	Moderator (LGE)

- R4-2017687	WF on single link tests for NR V2X demodulation performance	LGE

- R4-2017470	Simulation assumptions for NR V2X single link test case	Huawei

- R4-2017613	Email discussion summary for [97e][318] V2X_Demod_Part2	Moderator (Intel)

- R4-2017686	WF on multiple link tests for NR V2X demodulation performance	Intel

- R4-2017472	Simulation assumptions for NR V2X multiple link test case	Huawei

- R4-2014419	Simulation results of NR V2X demodulation test	CATT

- R4-2014537	Discussion on V2X work scope and general simulation assumptions	Intel Corporation

- R4-2014779	Discussion on V2X Demod test case	MediaTek inc.

- R4-2014417	Discussion on single link demodulation test for NR V2X	CATT

- R4-2014420	CR for 38.101-1: Introduce PSBCH performance requirements for NR V2X	CATT

- R4-2014538	Discussion on Single Link V2X requirements	Intel Corporation

- R4-2014637	NR V2X Demod single link requirement	Qualcomm, Inc.

- R4-2014652	Discussion on NR V2X single link test cases	LG Electronics Inc.

- R4-2014668	Initial simulation results for NR V2X single link test cases	LG Electronics Inc.

- R4-2014780	CR on NR V2X PSFCH demodulation requirements	MediaTek inc.

- R4-2015640	Discussion on performance requirements for NR V2X single-link test	Huawei, HiSilicon

- R4-2015641	Simulation results for NR V2X single-link test	Huawei, HiSilicon

- R4-2014418	Discussion on multiple link demodulation test for NR V2X	CATT

- R4-2014539	Discussion on Multiple Link V2X requirements	Intel Corporation

- R4-2014636	NR V2X Demod multiple link requirement	Qualcomm, Inc.

- R4-2014638	CR: Demod HARQ buffer soft combining test cases for NR V2X	Qualcomm, Inc.

- R4-2014669	Discussion on NR V2X multiple link test cases	LG Electronics Inc.

- R4-2014670	Initial simulation results for NR V2X multiple link test cases	LG Electronics Inc.

- R4-2015642	Discussion on performance requirements for NR V2X multi-link test	Huawei, HiSilicon

- R4-2015643	Draft CR: Introduce power imbalance with two links test for NR sidelink	Huawei, HiSilicon

- R4-2015644	Draft CR: Introduce PSCCH/PSSCH decoding capability test for NR sidelink	Huawei, HiSilicon

- R4-2015645	Draft CR:  PSFCH decoding capability test for NR sidelink	Huawei, HiSilicon

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template