3GPP TSG RAN Meeting #90-e	RP-202305

e-meeting,  December 7th  - December 11th  2020

Source: 	OPPO

Title:	Discussion on WI Scope of NR Positioning Enhancements

TDOC Type:	discussion

Agenda Item:	9.7.3

Release:	Rel-17

Introduction

RAN1 part of the study item of NR positioning enhancements [1] has been accomplished in RAN1#103e meeting. Based on throughout evaluations and analysis, RAN1 has made conclusions or recommendations on identified important enhancement for Rel-17. Meanwhile, there is still one remaining RAN2 meeting to complete the RAN2 part of this study item. Accordingly, a new version of the corresponding technical report (TR) was approved which captures all of the existing outcome of the study [2].

Due to high completion rate of this SI and tight timeline of Rel-17 NR, it is mature for RAN1 to start work item of NR positioning enhancement from the next RAN1 meeting. Thus, in this contribution, we will discuss the potential enhancements for work item based on the outcomes of this study item.

Discussion

Outcome of the study item

The RAN1 outcome of this study item has been captured in the latest TR [2], which is summarized as below:

Based on the study, the following enhancements have been recommended for normative work - NR positioning for UEs in RRC_INACTIVE state - On-demand transmission and reception of DL PRS - The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or gNB Rx/Tx timing delays - The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy - The enhancements of signalling & procedures for reducing NR positioning latency, including DL and DL+UL positioning methods. The following enhancements are considered beneficial and are being recommended to be studied further and if needed, specified during normative work - Simultaneous transmission by the gNB and aggregated reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs - Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers - Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation - Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs - Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs - Enhancements of signalling & procedures for reducing NR positioning latency related to the request and response of positioning assistance data, the reception of DL PRS (e.g., priority rules for the reception of DL PRS) From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. This does not imply that measurements have to be reported in RRC_IDLE state. It is recommended to proceed with a normative work to support NR positioning enhancements.

Since RAN1 has completed the study and RAN2 still have one more meeting to finalize the study, we suggest to approve the enhancement mainly related to RAN1 for WI and to update the WI in the next RAN meeting to include the enhancement mainly related to RAN2.

Proposal 1: Approve RAN1-led enhancements for Rel-17 positioning WI in RAN#90e

- Update Rel-17 positioning WI to include RAN2-led enhancement in RAN#91e

Positioning enhancements for work item

Positioning in RRC_INACTIVE state

NR positioning for UEs in RRC_INACTIVE state is beneficial not only for reduction of UE power consumption, but also for positioning latency reduction. Thus, it is an important feature for Rel-17 positioning enhancement. RAN1 made the following conclusions for NR positioning in RRC_INACTIVE state:

NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions - Support of UE positioning measurements for UEs in RRC_inactive state - Options that can be considered include DL-PRS or DL-PRS and SSB - Support of gNB positioning measurements for UEs in RRC_inactive state The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects: - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting, which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

The key building block of this functionality is the design of UL transmission (i.e., positioning reporting) for RRC_INACTIVE state. RAN2 has a work item of small data transmission for RRC__INACTIVE state (SDT). However, the current scope of SDT is only focusing on User Plane (UP). The reporting of positioning measurement results is conveyed via the Control Plane (CP). Thus, there will be much more additional works for the design of positioning reporting in RRC_INACTIVE state.

Observation 1: The reporting of positioning measurements or calculated location information in RRC_INACTIVE state will need a lot of additional standardization effort, even if the work for RAN2 WI of SDT is reused as much as possible.

Compared to the DL based positioning method, UL or DL+UL based positioning methods will require more workload due to the transmission of uplink reference signal for positioning. Moreover, they also may lead to higher power consumption for UE since some uplink reference signal for positioning is targeted to neighboring cells that suffer larger pathloss. Thus, DL based positioning method should be with higher priority for the perspective of benefits and standardization efforts.

Observation 2: For NR positioning in RRC_INACTIVE state, compared to DL based positioning methods, UL and DL+UL based positioning methods has lower priority from the perspective of benefits and standardization efforts.

Based on the above discussions, we have the following proposal:

Proposal 2: NR positioning in RRC_INACTIVE state should be included in Rel-17 WI scope

- UL and DL+UL based positioning methods has lower priority than DL based method

On-demand transmission and reception of DL PRS

RAN1 made the following conclusion for on-demand DL PRS:

From a physical layer perspective, on-demand transmission and reception of DL PRS, which includes at least the following is recommended - UE-initiated request of on-demand DL PRS transmission - LMF (network)-initiated request of on-demand DL PRS transmission - Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

Rel-16 positioning only supports periodic DL PRS. Network usually has to configure DL PRS with relatively small periodicity in order to meet the highest requirement of all UEs, thereby leading larger overhead of DL PRS. By introducing on-demand DL PRS, network can transmit DL PRS to all UEs with longer periodicity so that the basis requirements of NR positioning are ensured. If some UE requires higher positioning accuracy, network can transmit additional PRS dedicated to the UE to improve the performance. In this case, the overhead of DL PRS in the whole system can be reduced since the additional PRS are only transmitted from the neighbouring TRPs of the given UE. On the other hand, on-demand DL PRS can also reduce the latency since it allows UE to carry out more measurements within a given period with more frequent DL PRS.

Observation 3: UE-initiated on-demand DL PRS is beneficial to reduce both DL PRS overhead and positioning latency.

Proposal 3: UE-initiated on-demand DL PRS should be included in Rel-17 WI scope.

Aperiodic / Semi-persistent DL PRS

In order to implement on-demand DL PRS, a likely solution is based on the scheme of aperiodic / semi-persistent DL PRS. Thus, aperiodic/semi-persistent DL PRS and on-demand DL PRS are coupled in some sense. It is beneficial to have a common framework for both features. Similar to on-demand DL PRS, aperiodic / semi-persistent DL PRS is also helpful to reduce both DL PRS overhead and positioning latency.

Observation 4: It is beneficial to have a common framework for on-demand DL PRS and aperiodic / semi-persistent DL PRS.

RAN1 made the following conclusion for on-demand DL PRS:

Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work. - Note: Aperiodic reception in the above corresponds to DCI-triggered reception Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work. - Note: Semi-persistent reception in the above corresponds to MAC-CE activated reception

Compared to aperiodic DL PRS, it may be more controversial for semi-persistent DL PRS according to RAN1 discussions. In NR, all of aperiodic/semi-persistent/periodic mechanisms are support for many features. Thus, it is natural for support semi-persistent DL PRS along with aperiodic and periodic DL PRS. On one hand, the requirements for UE and network are similar for aperiodic and semi-persistent DL PRS. On the other hand, semi-persistent DL PRS may reduce overhead of DCI triggering DL PRS in some use cases. Thus, we suggest to support both aperiodic and semi-persistent DL PRS.

Proposal 4: Aperiodic / semi-persistent reception of DL PRS should be included in Rel-17 WI scope.

Latency reduction

Latency reduction is one of the key targets for the study of potential positioning enhancement. Based on the “divide-and-conquer” approach, RAN1 evaluated the latency introduced by each step/component and made the following conclusions:

The following enhancements of signaling & procedures for reducing NR positioning latency are recommended for normative work, including DL and DL+UL positioning methods - The details of the solutions are left for further discussion in normative work, which may include the following aspects: - Latency reduction related to the measurement gap - Latency reduction related to the reporting and request of the measurement (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules) - Latency reduction related to measurements The following enhancements of signaling & procedures for reducing NR positioning latency can be studied and specified, if needed - Latency reduction related to the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure) - Latency reduction related to the reception of DL PRS (e.g., priority rules for the reception of DL PRS) No assumptions are made on whether the LCS architecture specified in TS 23.273 is enhanced or not.

Among the listed FFS aspects, we believe measurement gap is the key bottleneck for the positioning latency from RAN1 perspective.  In Rel-16 NR, only measurement gap (MG) based positioning is supported. Since the request of measurement gap is based on RRC signaling, a processing delay of about 10ms is inevitable, which is too large for the tight requirement of Rel-17 positioning latency.

Observation 5: The latency related to measurement gap is the key bottleneck for the latency reduction.

Proposal 5: Latency reduction related to measurement gap should be included in Rel-17 WI scope.

Controversial topics

Aggregation of DL PRS from multiple PFLs

The pertinent RAN1 agreement for the aggregation of DL PRS from multiple PFLs is as below:

Simultaneous transmission by the gNB and reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs can be studied further and if needed, specified during normative work - From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, bands and RF architectures, can be further studied

Theoretically, aggregation of multiple Positioning Frequency Layers (PFLs) may improve the positioning accuracy since it can potentially increase the equivalent bandwidth of DL PRS source and improve the resolution of timing-based measurements. However, its feasibility is doubtable due to the limitation of both UE and gNB. Aggregating DL PRS resources from different frequency layers has very tight requirement on the phase continuity and timing alignment between different Tx and Rx chains. During the study item, the negative impacts of those impairments on the performance were not fully studied.

To be more specific, from the perspective of UE implementation, there are two types of problems related to aggregation of multiple PFLs:

- Due to the limited IF spectrum coverage with baseband circuits, e.g., amplifier, filter etc., the typical implementation of commercial UEs needs multiple different receive signal paths to support CA when each CC has large bandwidth. Thus, it is very difficult, if not infeasible, to ensure the phase continuity and timing alignment among different CCs

- To support coherent aggregation of DL PRS resources from multiple positioning frequency layers, it would impose great complexity on UE transceiver design. For example, it would require the UE to use signal RF and IF signal path to receive the signals from multiple carriers and the it also requires the UE to implement much higher-speed ADC device.

Observation 6: The feasibility of multiple-PFL aggregation for UE has NOT been justified.

Observation 7: The feasibility of multiple-PFL aggregation will lead to huge complexity increase of UE implementation.

From the perspective of gNB implementation, the feasibility is still in doubt as well. TS 38.104 gives the requirement for timing alignment error as below

TS 38.104 #### 6.5.3.2 Minimum requirement for BS type 1-C and BS type 1-H For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

According to the current RAN4 transmission requirement of timing alignment error (TAE) for different component carriers, aggregation of multiple positioning layers cannot offer better positioning accuracy. Although RAN4 spec only specified the worst performance of gNB, there is no evident on the feasible performance of timing alignment error for practical gNB.

Observation 8: The feasibility of multiple-PFL aggregation for gNB has NOT been justified so far.

Considering the uncertainty about its feasibility for UE/gNB and the tight timeline of Rel-17, we suggest not to include this feature in WI and save time units for more important features.

Proposal 6: Aggregation of DL PRS from multiple PFLs should NOT be in Rel-17 WI scope.

Aggregation of SRS for positioning resources

The pertinent RAN1 agreement for the aggregation of SRS for positioning resources is as below:

Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers can be studied further and if needed, specified during normative work. - From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, particular bands and RF architectures, can be further studied.

The issues of the feasibility of aggregation of multiple Positioning Frequency Layers (PFLs) also apply to aggregation of SRS for positioning resources. Thus, we have similar observations and proposal as below:

Observation 9: The feasibility of SRS for positioning resources for UE has NOT been justified so far.

Observation 10: The feasibility of SRS for positioning resources for gNB has NOT been justified so far.

Proposal 7: Aggregation of SRS for positioning resources should NOT be in Rel-17 WI scope.

Enhancements for multipath/NLOS mitigation

The pertinent RAN1 agreement for Enhancements of multipath/NLOS mitigation is as below:

Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation can be studied further, and if needed, specified during normative work for improving positioning accuracy. - Note: The details of the enhancements of reporting are left for further discussion in normative work, which may include, but are not limited to the following information associated with multi-path, e.g., LOS/NLOS identification, time of arrival of the multi-path components, signal power and/or relative power, power delay profile, angle, and/or polarization information, coherence bandwidth, etc.

Enhancements for multipath/NLOS mitigation is another controversial issue. The system-level performance gain of spec-based method of multipath/NLOS mitigation is not clear since some sources showed that implementation-based method can achieve better performance than spec-based method.

On the other hand, for the sources showing performance gain of spec-based method, the detailed enhancement proposals are very diverged and the simulation results haven’t been double-checked so far. Even if we continue studying it, it is very difficult to converge to some solid scheme due to the diverged proposals.

Observation 11: The performance gain of spec-based multipath/NLOS mitigation is not been verified during the study item.

Proposal 8: Enhancements for multipath/NLOS mitigation should NOT be in Rel-17 WI scope.

- Implementation based scheme can offer satisfying performance

Positioning in IDLE state

Compared to positioning in RRC_INACTIVE state, positioning in IDLE state is more controversial due to the potential UL transmission/reporting in IDLE state. For technical perspective, positioning reporting in IDLE state has more challenges:

- IDLE UE doesn’t store the AS context. A reporting in IDLE state will have large impact on UE behavior and NW behavior (including gNB and core network), leading to lots of specification efforts.

- The power consumption of UE in IDLE state will increase significantly due to UL transmission.

- Compared to positioning in RRC_INACTIVE state, additional benefits of positioning in IDLE state is not well justified.

Observation 12: The benefit of positioning in IDLE state has not been well justified and it requires lots of standardization efforts.

After lengthy discussions, companies cannot achieve a consensus in RAN1 to support this feature. As a compromise, RAN1 made the following conclusion for positioning in IDLE state:

Agreement: Capture the following in the TR: From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. - Note: This does not imply that measurements have to be reported in RRC_IDLE state. Conclusion: It is up to RAN2 to decide whether to support the enhancements of NR positioning reporting of DL positioning measurements and/or positioning estimates for RRC_IDLE UEs.

According to the above conclusion, it is up to RAN2 to make decision whether to support positioning reporting in IDLE state in the following RAN2 meeting in 2021 Q1. Thus, we propose to postpone positioning in IDLE state to wait for RAN2 decision.

Proposal 9: Postpone the WI discussion on positioning in IDLE state in RAN#90e meeting.

WI updates in next RAN meeting

Integrity and reliability of assistance data and position information

Integrity and reliability of assistance data and position information are important topics for Rel-17 positioning enhancement. However, there are still under RAN2 discussion. Thus, we suggest defer them until RAN2 finished the related study.

Proposal 10: Revisit integrity and reliability of assistance data and position information Enhancements when RAN2 finishes the study item.

Conclusions

In this contribution, we discuss some identified enhancements for positioning based on the outcome of the SI. Based on the discussion, we have the following observations and proposal:

Observation 1: The reporting of positioning measurements or calculated location information in RRC_INACTIVE state will need a lot of additional standardization effort, even if the work for RAN2 WI of SDT is reused as much as possible.

Observation 2: For NR positioning in RRC_INACTIVE state, compared to DL based positioning methods, UL and DL+UL based positioning methods has lower priority from the perspective of benefits and standardization efforts.

Observation 3: UE-initiated on-demand DL PRS is beneficial to reduce both DL PRS overhead and positioning latency.

Observation 4: It is beneficial to have a common framework for on-demand DL PRS and aperiodic / semi-persistent DL PRS.

Observation 5: The latency related to measurement gap is the key bottleneck for the latency reduction.

Observation 6: The feasibility of multiple-PFL aggregation for UE has NOT been justified.

Observation 7: The feasibility of multiple-PFL aggregation will lead to huge complexity increase of UE implementation.

Observation 8: The feasibility of multiple-PFL aggregation for gNB has NOT been justified so far.

Observation 9: The feasibility of SRS for positioning resources for UE has NOT been justified so far.

Observation 10: The feasibility of SRS for positioning resources for gNB has NOT been justified so far.

Observation 11: The performance gain of spec-based multipath/NLOS mitigation is not been verified during the study item.

Observation 12: The benefit of positioning in IDLE state has not been well justified and it requires lots of standardization efforts.

Proposal 1: Approve RAN1-led enhancements for Rel-17 positioning WI in RAN#90e

- Update Rel-17 positioning WI to include RAN2-led enhancement in RAN#91e

Proposal 2: NR positioning in RRC_INACTIVE state should be included in Rel-17 WI scope

- UL and DL+UL based positioning methods has lower priority than DL based method

Proposal 3: UE-initiated on-demand DL PRS should be included in Rel-17 WI scope.

Proposal 4: Aperiodic / semi-persistent reception of DL PRS should be included in Rel-17 WI scope.

Proposal 5: Latency reduction related to measurement gap should be included in Rel-17 WI scope.

Proposal 6: Aggregation of DL PRS from multiple PFLs should NOT be in Rel-17 WI scope.

Proposal 7: Aggregation of SRS for positioning resources should NOT be in Rel-17 WI scope.

Proposal 8: Enhancements for multipath/NLOS mitigation should NOT be in Rel-17 WI scope.

- Implementation based scheme can offer satisfying performance

Proposal 9: Postpone the WI discussion on positioning in IDLE state in RAN#90e meeting.

Proposal 10: Revisit integrity and reliability of assistance data and position information Enhancements when RAN2 finishes the study item.

References

| [1] | RP-202094, Study on NR Positioning Enhancements, |
| --- | --- |
| [2] | R1-2009842, 3GPP TR 38.857 Study on NR Positioning Enhancements (V0.4.0), Ericsson |