3GPP TSG RAN meeting #90e										RP- 202308

Electronic Meeting, December 7-11, 2020

Status Report to TSG

Agenda item:		9.8.15

| WI / SI Name | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_OTA |  |  |  |  |
| Unique ID | 880078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201800 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2021 | Performance part: Sep/2021 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 25% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Zhu, Siting Xing, Jinqiang, |
|  | Company | CAICT, OPPO |
|  | Email | zhusiting@caict.ac.cn xingjinqiang@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #97-e (Nov 2020, Electronic Meeting)

•	General

- WF on NR MIMO OTA was approved [2]

- WF on FR2 MIMO OTA simulation assumption was approved [3]

- LS on FR1 MIMO OTA was approved [4]

- TS 38.151 v0.1.0 NR MIMO OTA requirements was approved [5]

- TP to TS 38.151 v0.0.1 on general part was approved [6]

- TP to TS 38.151 v0.0.1 on FR1 test system for requirements was approved [7]

- TP to TS 38.151 v0.0.1 on FR1 Channel model and RMC [8]

•	Testing methodologies

- System implementation of FR2 3D-MPAC system

    - The system layout of FR2 3D-MPAC is concluded, based on proposal 1 in [24].

- Channel model validation procedure

    - Theoretical values of channel model validation with Base Station antenna filtering effect shall be provided as reference.

    - Focus on high power clusters for channel model validation, consider 40dB threshold as a starting point and further discuss the final threshold.

- System validation limits

    - Reference figure for channel model validation

      - Simulated curve (channel model with BS filtering effect) with limited number of probes (16 probes for FR1 and 6 probes for FR2) is agreed as a reference, to be added into the TR to determine pass fail limits

      - Simulated curve (channel model with BS filtering effect) with infinite number of probes is optional to be added

      - Simulation results of the reference figure from CE vendors are encouraged next meeting

    - Channel model validation limit

      - Analysis of channel model implementation limits from CE vendors are encouraged

- Test Parameters for FR1 MIMO OTA performance requirements

    - CDL-A UMi model is not suitable for 4x4 4-layer performance testing. Channel models mapping for NR MIMO OTA test need to be reconsidered. The group will confirm the conclusion in RAN4#98e based on below options

      - Option1: CDL-C UMa for 4x4 and CDL-A UMi for 2x2 (is agreed baseline)

      - Option2: CDL-C UMi for 4x4 and CDL-C UMa for 2x2

      - Option3: CDL-C UMi for 4x4 and CDL-A UMa for 2x2

    - For band frequency <3GHz, the maximum downlink RS-ERPE should be -80dBm/15kHz for 10MHz gNB setting

      - Further study the maximum downlink RS-EPRE for frequency band >3GHz

      - Further study the maximum downlink RS-EPRE for 40MHz bandwidth for the above frequency bands

    - Only middle channel shall be verified for each band. Test Frequencies as specified in TS 38.508-1 subclause 4.3.1.

    - Minimum number of slots: 20k for 15kHz SCS; 40k for 30kHz SCS

- Test Parameters for FR2 MIMO OTA performance requirements

    - Adopt 16QAM RMC as the only RMC for FR2 MIMO OTA.

      - Feasibility and potential relaxed KPIs for n259 and n260 can be discussed

    - UMi CDL-C is selected as 1st priority for FR2 requirements

    - Keep the agreement of 36 evenly spaced test points for FR2 MIMO OTA test

      - Analysis on number of test points vs uncertainty of FR2 MIMO OTA requirements is encouraged

    - Only middle channel shall be verified for each band.

    - Minimum number of slots: 20k for FR2 UMi models; 75k for FR2 Indoor Office models;

•	Performance requirements

- Figure of Merit for FR1 MIMO OTA performance requirements

    - For 10MHz CHBW testing, the 11 of total 12 PMODE should reach 70%TP.

    - For 40MHz CHBW testing, the restriction of PMODE at 70%TP is FFS.

    - Group agreed to further check additional test metric with below options:

      - Option 1：TP@90% can pass 10 of total 12 rotations

      - Option 2：TP@95% can pass 10 of total 12 rotation

- Figure of Merit for FR2 MIMO OTA performance requirements

    - As a starting point, adopt 70% of maximum throughput value as outage point

      - Further study the final outage of throughput for FR2 MIMO OTA requirements

    - RAN4 agreed not to introduce “[50%] percentile of the CCDF curve” as another FoM.

    - Decision should be made on how to treat the orientations those can not reach target outage throughput in the future

- Framework on performance requirements

    - Views on Framework are shared in [9] and [10]

    - Lab alignment aspects

      - Labs/companies volunteer to participate in the performance requirement part shall complete the lab alignment measurements and system validation measurements, results should be submitted to RAN4 for review.

      - Lab alignment activities shall be divided in two independent parts. Part 1: lab alignment for FR1; Part 2: lab alignment for FR2 (if applicable). Simulation approach for FR2 requirement development is also considered.

      - Only aligned labs/companies can share measurement results into MIMO OTA data pool to define requirements.

      - Pass/fail limit for FR1/FR2 lab alignment is FFS. Pass/fail limit for system validation is FFS.

    - FR1 requirement aspects

      - For FR1, the TRMS requirements should be derived from measurement results of commercial devices, minimum number of 15 devices for defining requirements in each band.

      - For FR1, requirements for SA at Band n41, n77, n78 and n79, are the first priority.

      - The minimum number of supported NR bands for each FR1 sample is FFS

      - PC3 is the 1st priority

    - FR2 requirement aspects (including simulation framework)

      - Given the limited number of FR2 commercial smartphones, both simulation results and measurement results are permitted to define requirements.

      - Simulation parameters should be aligned with testing condition as much as possible, such as RMC, channel models, chamber environment, different UE form factors, etc.

      - The potential gap between simulation and measurement should be discussed.

      - For FR2, requirements for PC3 NSA at Band n257, n258, n260 and n261 are the first priority.

    - FR2 simulation work plan

      - The simulation work plan in [11] is agreed as baseline. The work plan will be updated based on new WI work plan according to potential new Rel-17 timeline in Dec. RAN plenary meeting.

    - FR2 MIMO OTA simulation assumptions

      - FR2 MIMO OTA simulation assumptions should take TR 38.827 as baseline, following items still need further alignment in detail for simulation:

        - BS antenna beamforming configuration

        - The direction of Strongest beam from BS

        - Clusters used in chamber for each Channel Model

        - PSP for simulation

        - UE antenna array

      - Companies are encouraged to provide views on above simulation assumptions

        - other aspects are not precluded if necessary

2.4.2	Remaining Open issues

- Channel model mapping for NR MIMO OTA requirements

- Detailed testing parameters for requirements (e.g. Maximum downlink power)

- Pass/fail limit and reference figure of channel model validation

- Further discuss the Figure of Merit for FR1 and FR2

- Further discuss FR2 MIMO OTA simulation assumptions

- Specify the FR1/FR2 MIMO OTA requirements

  - Measurement results of FR1 or FR2 UEs are encouraged for discussion

  - FR2 simulation results of UE performance are encouraged

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

N/A

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#97-e

- R4-2017629  Email discussion summary for [97e][330] NR_MIMO_OTA, Moderator(CAICT)

- R4-2017585	WF on NR MIMO OTA, vivo, CAICT, Spirent, Huawei, Hisilicon, Samsung

- R4-2017632	WF on FR2 MIMO OTA simulation assumption, Huawei

- R4-2017583	LS on FR1 MIMO OTA, vivo, CAICT

- R4-2016216	TS 38.151 v0.1.0 NR MIMO OTA requirements, vivo

- R4-2016218	TP to TS 38.151 v0.0.1 on general part, vivo, CAICT

- R4-2016222	TP to TS 38.151 v0.0.1 on FR1 test system for requirements, vivo, CAICT

- R4-2017584	TP to TS 38.151 v0.0.1 on FR1 Channel model and RMC, vivo, CAICT

- R4-2015311	Framework on NR MIMO OTA requirements development, CAICT, vivo

- R4-2016588	Discussion on MIMO OTA framework, Huawei, HiSilicon

- R4-2014829	Proposal of FR2 MIMO OTA simulation approach workplan, MediaTek Beijing Inc

- R4-2016209	On FR1 4x4 vs. 2x2 channel models, Keysight

- R4-2015352	Analysis on the impact of number of test points, OPPO

- R4-2016208	On FR2 MIMO OTA channel model down selection, Keysight

- R4-2016219	Discussions on FR2 MIMO OTA requirements, vivo, CAICT

- R4-2016235	Views on for FR2 MIMO OTA, Qualcomm

- R4-2016539	Simulation assumptions for NR FR2 MIMO OTA, Huawei, HiSilicon

- R4-2014688	Effect of White Box Approach on Simple-Sectored Multi-Probe Anechoic Chamber Design, BUPT

- R4-2015368	Discussion on MIMO OTA test methodologies, Huawei, HiSilicon

- R4-2014723	Discussion on FR1 and FR2 MIMO OTA, Samsung

- R4-2016589	Discussion on MIMO OTA open issues, Huawei, HiSilicon

- R4-2015258	on UE orientation clarification, Xiaomi

- R4-2015353	The rules for 3D-MPAC system implementation, OPPO

- R4-2016210	On Probe Configurations and Channel model vs. OTA test system coordinate systems for FR2 MIMO OTA, Keysight

- R4-2014536	Channel Model Assumptions, Spirent

- R4-2016561	FR1 MIMO OTA channel model validation results, CAICT, Keysight, vivo

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template