3GPP TSG-RAN WG4 Meeting #97-e	R4-2017099

# Online Meeting, 2 – 13 November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1393 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of intra-frequency sync and async DAPS HO test cases in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Perf |  |  |  |  |  | Date: |  |  | 2020-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per work split agreement in RAN4#95-e, intra-frequency sync and async DAPS HO test cases in FR1 are introduced in this CR. Per agreements in RAN4#96-e, the tests consist of 5 intervals and the last interval is used to verify the CSI reporting to source cell is stopped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Intra-frequency DAPS HO in FR1 in synchronous scenario - Intra-frequency DAPS HO in FR1 in asynchronous scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance part of NR mobility WI will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.1.7 (new), A.6.3.1.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2016555 |  |  |  |  |  |  |  |  |

# < Start of change >

#### A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario

##### A.6.3.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in synchronous scenario specified in clause 6.1.3.2.

##### A.6.3.1.7.2	Test Parameters

Supported test configurations are shown in Table A.6.3.1.7.2-1. Both handover delay and interruption length are tested by using the parameters in Table A.6.3.1.7.2-2, and A.6.3.1.7.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

Before the start of T1, the UE is connected to the cell1 and not aware of the cell2. During T1, the UE does not have any timing information of the cell2.

Starting T2, the cell2 becomes detectable. During T2, the UE performs cell detection and measurements on the cell2 and shall send event report to the network. After receiving the event report A3, the network sends a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing DAPS handover command is sent to the UE. During T3, UE shall be able to perform random access, DL reception or UL transmission in the cell2 while the DL scheduling and UL feedback in the cell1 shall be avoided. After successful RACH procedure of the cell2, UE is scheduled with PDSCH from cell1 and cell2 in alternative TTIs where both cell1 and cell2 belong to the same TAG. In the end the network sends a RRC message implying cell1 release to the UE. During T3, the handover delay Dhandover1 for target cell addition need to be verified.

The start of T4 is the instant when the last TTI containing cell1 release command is sent to the UE. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE stops sending the periodical CSI report to the cell1.

Table A.6.3.1.7.2-1: Intra-frequency DAPS handover in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.7.2-2: General test parameters synchronous Intra-frequency DAPS handover in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | Dhandover2 | DHandover2 is defined in clause 6.1.3.2.1 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.7.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency DAPS handover test case

| Parameter |  |  |  |  | Unit |  |  | Cell 1 |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | T1 | T2 |  |  | T3 | T4 |  | T5 |  | T1 | T2 | T3 | T4 |  | T5 |
| NR RF Channel Number |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |
| Duplex mode |  |  | Config 1 |  |  |  |  |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  |  |  |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1 |  | MHz |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  | Config 1 |  | MHz |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  |  | ms |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  | Config 1 |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  |  | Config 1,2 |  |  |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz |  |  |  |  | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 |  | kHz |  |  |  |  | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  |  |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  | dB |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/15kHz |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Config 1,2 |  |  | dBm/SCS |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  |  |  |  |  | -95 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | dB |  |  | 8 | -1.5 |  |  | -1.5 | -1.5 | -1.5 |  | -Infinity |  | 0.36 | 0.36 | 0.36 | 0.36 |  |
|  |  |  |  |  | dB |  |  | 8 | 8 |  |  | 8 | 8 | 8 |  | -Infinity |  | 9 | 9 | 9 | 9 |  |
| SSB_RP |  | Config 1,2 |  |  | dBm/SCS |  |  | -90 | -90 |  |  | -90 | -90 | -90 |  | -89 |  | -89 | -89 | -89 | -89 |  |
|  |  | Config 3 |  |  | dBm/SCS |  |  | -87 | -87 |  |  | -87 | -87 | -87 |  | -86 |  | -86 | -86 | -86 | -86 |  |
| IoNote3 |  | Config 1,2 |  |  | dBm/ 9.36MHz |  |  | -61.41 | -58.21 |  |  | -58.21 | -58.21 | -58.21 |  | -61.41 |  | -58.21 | -58.21 | -58.21 | -58.21 |  |
|  |  | Config 3 |  |  | dBm/ 38.16MHz |  |  | -55.31 | -52.11 |  |  | -52.11 | -52.11 | -52.11 |  | -55.31 |  | -52.11 | -52.11 | -52.11 | -52.11 |  |
| Propagation condition |  |  |  |  |  |  |  |  |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.7.3 Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The target cell add delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

If the target cell is known, then Tsearch = 0 ms

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release cell 1 less than Dhandover2 = (TRRC_procedure + Tinterrupt2) from the beginning of time period T4.

NOTE:   Dhandover2 is defined in clause 6.1.3.2.1.

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

UE shall not report CSI to cell 1 during T5.

#### A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario

##### A.6.3.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in asynchronous scenario specified in clause 6.1.3.2.

##### A.6.3.1.8.2	Test Parameters

Supported test configurations are shown in Table A.6.3.1.8.2-1. Both handover delay and interruption length are tested by using the parameters in Table A.6.3.1.8.2-2, and A.6.3.1.8.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

Before the start of T1, the UE is connected to the cell1 and not aware of the cell2. During T1, the UE does not have any timing information of the cell2.

Starting T2, the cell2 becomes detectable. During T2, the UE performs cell detection and measurements on the cell2 and shall send event report to the network. After receiving the event report A3, the network sends a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing DAPS handover command is sent to the UE. During T3, UE shall be able to perform random access, DL reception or UL transmission in the cell2 while the DL scheduling and UL feedback in the cell1 shall be avoided. After successful RACH procedure of the cell2, UE is scheduled with PDSCH from cell1 and cell2 in alternative TTIs where both cell1 and cell2 belong to the same TAG. In the end the network sends a RRC message implying cell1 release to the UE. During T3, the handover delay Dhandover1 for target cell addition needs to be verified.

The start of T4 is the instant when the last TTI containing cell1 release command is sent to the UE by cell2. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE stops sending the periodical CSI report to the cell1.

Table A.6.3.1.8.2-1: Intra-frequency DAPS handover in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.8.2-2: General test parameters Intra-frequency asynchronous DAPS handover in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 7 s | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | Dhandover2 | DHandover2 is defined in clause 6.1.3.2.1 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.8.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency DAPS handover test case

| Parameter |  |  |  |  | Unit |  | Cell 1 |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 |  | T2 | T3 | T4 |  | T5 |  | T1 | T2 |  |  | T3 |  | T4 |  | T5 |
| NR RF Channel Number |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
| Duplex mode |  |  | Config 1 |  |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1 |  | MHz |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  | Config 1 |  | MHz |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  |  | ms |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  | Config 1 |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  |  | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/15kHz |  | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS |  | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  |  | -95 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | dB |  | 8 |  | -1.5 | -1.5 | -1.5 | -1.5 |  | -Infinity |  |  |  | 0.36 | 0.36 | 0.36 |  | 0.36 |  |
|  |  |  |  | dB |  | 8 |  | 8 | 8 | 8 | 8 |  | -Infinity |  |  |  | 9 | 9 | 9 |  | 9 |  |
| SSB_RP |  | Config 1,2 |  | dBm/SCS |  | -90 |  | -90 | -90 | -90 | -90 |  | -89 |  |  |  | -89 | -89 | -89 |  | -89 |  |
|  |  | Config 3 |  | dBm/SCS |  | -87 |  | -87 | -87 | -87 | -87 |  | -86 |  |  |  | -86 | -86 | -86 |  | -86 |  |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz |  | -61.41 |  | -58.21 | -58.21 | -58.21 | -58.21 |  | -61.41 |  |  |  | -58.21 | -58.21 | -58.21 |  | -58.21 |  |
|  |  | Config 3 |  | dBm/ 38.16MHz |  | -55.31 |  | -52.11 | -52.11 | -52.11 | -52.11 |  | -55.31 |  |  |  | -52.11 | -52.11 | -52.11 |  | -52.11 |  |
| Propagation condition |  |  |  |  |  |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.8.3 Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The target cell add delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

If the target cell is known, then Tsearch = 0 ms

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release cell 1 less than Dhandover2 = (TRRC_procedure + Tinterrupt2) from the beginning of time period T4.

NOTE:   Dhandover2 is defined in clause 6.1.3.2.1.

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

UE shall not report CSI to cell 1 during T5.

# < End of change >