3GPP TSG-RAN WG1 Meeting #104-e	R1-2102070

# e-Meeting, January 25th – February 5th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0164 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to the sidelink resource reservation period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electroincs), Huawei, HiSilicon, Ericsson, Samsung, NTT DOCOMO, ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A resource indicated by the resource reservatoin field of an SCI may be located outside the resource pool in the current sidelink Mode 2 resource allocation procedure. Correction is necessary to ensure any resource indicated by an SCI remains within the resource pool. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resource reservation period is applied to the set of slots belonging to a resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In some cases an SCI reserves a resource outside the resource pool and the corresponding periodic resource reservation does not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4, 8.1.5, 8.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

## -	*** Unchanged text is omitted ***

The resource reservation interval, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

Notation:

t0SL,t1SL,t2SL,...

t'0SL, t'1SL, t'2SL,… denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

## *** Unchanged text is omitted ***

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSLtmSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SLtm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where t'n'SLtn'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SLt0SL,t1SL,...,tTmaxSL, otherwise slot t'n'SLtn'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SLt0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

## *** Unchanged text is omitted ***

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below.

## *** Unchanged text is omitted ***

If a set of sub-channels in slot t'mSLtmSL is determined as the time and frequency resource for PSSCH transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots t'm+j×Prsvp_TX'SLtm+j×Prsvp_TX'SL  are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,…t0SL,t1SL,t2SL,... is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

## *** Unchanged text is omitted ***

### 8.1.7	UE procedure for determining the number of logical slots for a reservation period

A given resource reservation period Prsvp in milliseconds is converted to a period Prsvp' in logical slots as:

Prsvp'=T'max N2010240 ms×Prsvp

where T'max is the number of slots that belong to a resource pool as defined in Clause 8.

## *** Unchanged text is omitted ***