3GPP TSG RAN WG1 Meeting # 104-e	R1-2101980

# e-Meeting, January 25th - February 5th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0358 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on clarification of TPC command for PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SC_PTM-Core |  |  |  |  |  | Date: |  |  | 2021-01-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Section 5.3.3.1.3, 2 bits TPC command for PUCCH can be used for the case when format 1A CRC is scrambled by SC-RNTI, or G-RNTI with current version. However, this field should not be used for this case because HARQ-ACK feedback is not supported for LTE MBMS. Some clarification is needed to fix this ambiguity issue. - TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3] - If the format 1A CRC is scrambled by RA-RNTI, P-RNTI, or SI-RNTI: - The most significant bit of the TPC command is reserved. - The least significant bit of the TPC command indicates column of the TBS table defined of [3]. - If least significant bit is 0 then = 2 else= 3. - Else - The two bits including the most significant bit indicates the TPC command. |
| Summary of change: |  | Add the missing clarification that TPC command for PUCCH in format 1A is reserved when the format 1A CRC is scrambled with SC-RNTI or G-RNTI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to use the indication of TPC command for PUCCH when the format 1A CRC is scrambled with SC-RNTI or G-RNTI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding, which has no impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the second version for this CR. The first version is R1-2100081 and it was discussed in [104-e-LTE-6.1CRs-01]. |  |  |  |  |  |  |  |  |

##### 5.3.3.1.3	Format 1A

DCI format 1A is used for the compact scheduling of one PDSCH codeword in one cell and random access procedure initiated by a PDCCH order. The DCI corresponding to a PDCCH order can be carried by PDCCH or EPDCCH.

The following information is transmitted by means of the DCI format 1A:

-	Carrier indicator – 0 or 3 bits. This field is present according to the definitions in [3].

-	Flag for format0/format1A differentiation or flag for format0A/format1A differentiation – 1 bit, where value 0 indicates format 0 or format 0A and value 1 indicates format 1A

Format 1A is used for random access procedure initiated by a PDCCH order only if format 1A CRC is scrambled with C-RNTI and all the remaining fields are set as follows:

- Localized/Distributed VRB assignment flag – 1 bit is set to '0'

- Resource block assignment – bits, where all bits shall be set to 1

- Preamble Index – 6 bits

- PRACH Mask Index – 4 bits, [5]

- All the remaining bits in format 1A for compact scheduling assignment of one PDSCH codeword are set to zero

Otherwise,

-	Localized/Distributed VRB assignment flag – 1 bit as defined in 7.1.6.3 of [3]

-	Resource block assignment – bits as defined in clause 7.1.6.3 of [3]:

- For localized VRB:

bits provide the resource allocation

- For distributed VRB:

- If  or if the format 1A CRC is scrambled by RA-RNTI, P-RNTI, SI-RNTI, SC-RNTI or G-RNTI:

- bits provide the resource allocation

- Else

- 1 bit, the MSB indicates the gap value, where value 0 indicates  and value 1 indicates

-  bits provide the resource allocation,

where  is defined in [2].

-	Modulation and coding scheme – 5bits as defined in clause 7.1.7 of [3]. The MSB is set to 0 when the UE is configured with blindSubframePDSCH-Repetitions set to TRUE and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3] and the repetition number is greater than 1 and the higher layer parameter mcs-restrictionSubframePDSCH-Repetitions is configured to 1.

-	HARQ process number – 4 bits if higher layer parameter dl-STTI-Length is configured for the cell and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3], otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC/NE-DC and higher layer parameter subframeAssignment-r15/subframeAssignment-r16, or for cases with EN-DC/NE-DC with FDD primary cell and higher layer parameter subframeAssignment-r15/subframeAssignment-r16 configured and the corresponding DCI is not mapped onto the UE specific search space given by the C-RNTI as defined in [3]), 4 bits (for cases with TDD primary cell, or for cases with EN-DC/NE-DC with FDD primary cell and higher layer parameter subframeAssignment-r15/subframeAssignment-r16 configured and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3])

-	New data indicator – 1 bit

-	If the format 1A CRC is scrambled by RA-RNTI, P-RNTI, SI-RNTI, SC-RNTI or G-RNTI:

-	If  and Localized/Distributed VRB assignment flag is set to 1

-	the new data indicator bit indicates the gap value, where value 0 indicates  and value 1 indicates .

-	Else the new data indicator bit is reserved.

-	Else

-	The new data indicator bit as defined in [5]

-	Redundancy version – 2 bits

-	TPC command for PUCCH – 2 bits as defined in clause 5.1.2.1 of [3]

-	If the format 1A CRC is scrambled by RA-RNTI, P-RNTI, or SI-RNTI:

-	The most significant bit of the TPC command is reserved.

-	The least significant bit of the TPC command indicates column of the TBS table defined of [3].

-	If least significant bit is 0 then = 2 else= 3.

-	Else if the format 1A CRC is scrambled by G-RNTI or SC-RNTI:

-	The two bits of the TPC command are reserved

-	Else

-	The two bits including the most significant bit indicates the TPC command

-	Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

-	SRS request – 0 or 1 bit. This field can only be present in DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI as defined in [3]. The interpretation of this field is provided in clause 8.2 of [3]. This field is not present when the DCI is used for scheduling PDSCH in a LAA SCell.

-	HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in clause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

-	SRS timing offset – 3 bits as defined in [3]. This field is present only when the DCI format is used for scheduling PDSCH in a LAA SCell and the UE is configured with uplink transmission on the LAA Scell.

-	Repetition number – 2 bits as defined in Table 5.3.3.1.17-1, where the number of transmissions for PDSCH is given by higher layer parameter maxNumber-SubframePDSCH-Repetitions for the value of '11'. This field is present only when the UE is configured with higher layer parameter blindSubframePDSCH-Repetitions set to TRUE and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3].

If the UE is not configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI, and the number of information bits in format 1A is less than that of format 0, zeros shall be appended to format 1A until the payload size equals that of format 0.

If the UE is configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1A mapped onto a given search space is less than that of format 0 for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 1A until the payload size equals that of format 0, except when format 1A assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

If the UE is configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1A mapped onto a given search space is less than that of format 0A for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 1A until the payload size equals that of format 0A, except when format 1A assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

If the UE is configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1A mapped onto a given search space is less than that of format 0C for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 1A until the payload size equals that of format 0C, except when format 1A assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

If the number of information bits in format 1A carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 1A.

When the format 1A CRC is scrambled with a RA-RNTI, P-RNTI, SI-RNTI, SC-RNTI or G-RNTI then the following fields among the fields above are reserved:

- HARQ process number

- Downlink Assignment Index (used for cases with TDD primary cell and either FDD operation or TDD operation, and is not present for cases with FDD primary cell and either FDD operation or TDD operation)