3GPP TSG-RAN4 Meeting #98-e	R4-2103572

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1673 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to relexed cell reselection requirements R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2021-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Some table numbers are wrong - It was already agreed in WF [R4-2009265] in RAN4#95e, no relaxation for the EMR carriers if T331 is running. For the carrier which is not configured as EMR carrier, T331 has no impact on it. The intention of EMR is for establishing CA/DC, thus EMR carrier can’t be configured on intra-frequency layer. Thus non-EMR carrier is removed from relaxed measurements requirements applicability since there is no intra-carrier EMR. - To correct several typos. - Format of title 4.2.2.11 is incorrect. It should be H4 - It is specified in TS 38.304 - For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency: - If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Otherwise, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].. So the relaxed measurement requirements applies when either condition Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is satisfied. Thus in TS38.133 core part, “and” shall be “or”. |
| Summary of change: |  | - Wrong table numbers are corrected. - Relaxed measurements requirements applicability is updated. - Typos are corrected. - Format of title 4.2.2.11 is corrected. - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the relaxed measurement requirements can be applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.9.2, 4.2.2.9.3, 4.2.2.9.4, 4.2.2.10.2, 4.2.2.11, 4.2.2.11.2, 4.2.2.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.2.2.8	Minimum requirement at transitions

When switching from low mobility scenario or not-at-cell-edge scenario to low mobility and not-at-cell-edge scenario during cell-reselection period, the UE shall fulfill the requirements corresponding to low mobility scenario or not-at-cell-edge scenario over measurement period (Trelaxed) and thereafter switch to requirements corresponding to low mobility and not-at-cell-edge scenario. The measurement period, Trelaxed, is any of:

-	Tmeasure,NR_Intra and Tevaluate,NR_Intra, defined in section 4.2.2.9 for intra-frequency measurements on NR cells,

-	Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in section 4.2.2.10 for inter-frequency measurements on NR cells and

-	Tmeasure,EUTRAN and Tevaluate,EUTRAN defined in sections 4.2.2.11 for inter-RAT E-UTRAN measurements.

When switching from low mobility and not-at-cell-edge scenario to low mobility scenario or not-at-cell-edge scenario during cell-reselection period, the UE shall fulfill the requirements corresponding to low mobility scenario or not-at-cell-edge scenario upon fulfilling the switching criteria.

When switching from normal mode to low mobility scenario or not-at-cell-edge scenario or low mobility and not-at-cell-edge scenario during cell-reselection period, the UE shall fulfill the requirements corresponding to normal mode over measurement period (Tnormal) and thereafter switch to requirements corresponding to low mobility scenario or not-at-cell-edge scenario or low mobility and not-at-cell-edge scenario. The measurement period, Tnormal, is any of:

-	Tmeasure,NR_Intra and Tevaluate,NR_Intra, defined in section 4.2.2.3 for intra-frequency measurements on NR cells,

-	Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in section 4.2.2.4 for inter-frequency measurements on NR cells and

-	Tmeasure,EUTRAN and Tevaluate,EUTRAN defined in sections 4.2.2.5 for inter-RAT E-UTRAN measurements.

When switching from low mobility scenario or not-at-cell-edge scenario or low mobility and not-at-cell-edge scenario to normal mode during cell-reselection period, the UE shall fulfill the requirements corresponding to normal mode upon fulfilling the switching criteria.

No requirement is defined for multiple transitions of scenarios within one measurement period.

<End of modified section 1>

<Start of modified section 2>

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with lowMobilityEvalutation [2] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and UE has fulfilled only the lowMobilityEvalutation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.2-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.2-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.2-1.

Table 4.2.2.9.2-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |  |  |

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with cellEdgeEvaluation [2] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criteria and combineRelaxedMeasCondition [2] not configured, and UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.3-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.3-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.3-1.

Table 4.2.2.9.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1.

<End of modified section 2>

<Start of modified section 3>

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with lowMobilityEvalutation [2] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvalutation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.2-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.2-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.10.2-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 | cycles) |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

<End of modified section 3>

<Start of modified section 4>

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

4.2.2.11.1	Introduction

This clause contains the requirements for measurements on inter-RAT E-UTRAN cells when the UE is configured with any of following relaxed measurement critera:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is configured with lowMobilityEvalutation [2] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.5 apply for this clause except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.11.2-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.11.2-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.11.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.2-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is configured with cellEdgeEvaluation [2] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the cellEdgeEvaluation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.5 apply for this clause except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.11.3-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.11.3-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.11.3-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-RAT E-UTRAN frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.3-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |

<End of modified section 4>