3GPP TSG-RAN4 Meeting #98-e	R4-2101008

# Online, 25 Jan. - 5 Feb., 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1539 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Scell activation delay maintenance (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT -Core |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE capability of “scellWithoutSSB” is missing in the FR2 SCell activation requirement. Similar as the discussion for intra-band FR1 Scell activation without SSB, the wording of current FR2 SCell activation shall be revised to: If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revision the wording for current FR2 SCell activation for the case when UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current FR2 SCell activation is not precise for the case when UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 8.3	SCell Activation and Deactivation Delay

### 8.3.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to activate a deactivated SCell and deactivate an activated SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC.

The requirements shall apply for EN-DC, standalone NR carrier aggregation, NE-DC, and NR-DC.

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ 5ms, if the SCell measurement cycle is equal to or smaller than 160ms.

-	TFirstSSB_MAX + Trs + 5ms, if the SCell measurement cycle is larger than 160ms.

If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3.

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

If the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

If the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

where,

TSMTC_MAX:

-	In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

The requirements for FR1 unknown SCell activation specified in this clause apply when one of the following conditions is met

-	‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes TFineTiming.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

End of Change