3GPP TSG-RAN5 Meeting #90-e						R5-210196

Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0084 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to SMS test case 9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In “Figure 8.8/3GPP TS 24.011: RP-Cause element layout” the extra occurrence of the “Cause value” has been removed by CT1 CR0068 (C1-207121). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In “Figure 8.8/3GPP TS 24.011: RP-Cause element layout” contained in clause 9.5.2, the extra occurrence of the “Cause value” was removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose will remain erroneous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of Modified Section >

## 9.5	Mobile Originating SMS / RP-ERROR / 5GS

### 9.5.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to send an SMS over IP }

then { UE sends a SIP MESSAGE request containing a short message }

}

(2)

with { UE having sent a SIP MESSAGE request containing a short message }

ensure that {

when { UE receives a 202 Accepted response, followed by a SIP MESSAGE request containing an RP-ERROR message }

then { UE sends a 200 OK response }

}

### 9.5.2	Conformance Requirements

[TS 24.341, clause 5.3.1.1]:

In addition to the procedures specified in subclause 5.3.1, the SM-over-IP sender shall support the procedures specified in 3GPP TS 24.229 [10] appropriate to the functional entity in which the SM-over-IP sender is implemented. The SM-over-IP sender shall build and populate RP-DATA message, containing all the information that a mobile station submitting an SM according to 3GPP TS 24.011 [8] would place, for successful delivery. The SM-over-IP sender shall parse and interpret RP- DATA, RP-ACK and RP-ERROR messages, containing all the information that a mobile station receiving an SM according to 3GPP TS 24.011 [8] would see, in a SM submission or status report.

NOTE 1:	If the SM-over-IP sender uses SMR entity timers as specified in 3GPP TS 24.011 [8], then TR1M is set to a value greater than timer F (see 3GPP TS 24.229 [10]).

NOTE 2:	If the SM-over-IP sender expects to receive a SM submit report will include the "+g.3gpp.smsip" parameter in the Contact header field when sending a REGISTER request.

[TS 24.341, clause 5.3.1.2]:

When an SM-over-IP sender wants to submit an SM over IP, the SM-over-IP sender shall send a SIP MESSAGE request with the following information:

a)	the Request-URI, which shall contain the PSI of the SC of the SM-over-IP sender;

NOTE 1:	The PSI of the SC can be SIP URI or tel URI based on operator policy. The PSI of the SC can be obtained using one of the following methods in the priority order listed below:

1)	provided by the user;

2)	if UICC is used, then:

-	if an ISIM is present, then the PSI of the SC is obtained from the EFPSISMSC in DF_TELECOM as per 3GPP TS 31.103 [18];

-	if an ISIM is not present, then the PSI of the SC is obtained from the EFPSISMSC in DF_TELECOM as per 3GPP TS 31.102 [19]; or

-	if the PSI of the SC is not available in EFPSISMSC in DF_TELECOM, then the PSI of the SC contains the TS-Service-Centre-Address stored in the EFSMSP in DF_TELECOM as per 3GPP TS 31.102 [19]. If the PSI of the SC is based on the E.164 number from the TS-Service-Centre-Address stored in the EFSMSP in DF_TELECOM then the URI constructed can be either a tel URI or a SIP URI (using the "user=phone" SIP URI  parameter format).

3)	if SIM is used instead of UICC, then the PSI of the SC contains the TS-Service Centre Address stored in the EFSMSP in DF_TELECOM as per 3GPP TS 51.011 [20]. If the PSI of the SC is based on the E.164 number from the TS-Service-Centre-Address stored in the EFSMSP in DF_TELECOM then the URI constructed can be either a tel URI or a SIP URI (using the "user=phone" SIP URI  parameter format); or

4)	if neither the UICC nor SIM is used, then how the PSI of the SC is configured and obtained is through means outside the scope of this specification.

b)	the From header, which shall contain a public user identity of the SM-over-IP sender;

NOTE 2:	The IP-SM-GW will have to use an address of the SM-over-IP sender that the SC can process (i.e. an E.164 number). This address will come from a tel URI in a P-Asserted-Identity header (as defined in RFC 3325 [13]) placed in the SIP MESSAGE request by the P-CSCF or S-CSCF.

NOTE 3:	The SM-over-IP sender has to store the Call-ID of the SIP MESSAGE request, so it can associate the appropriate SIP MESSAGE request including a submit report with it.

c)	the To header, which shall contain the PSI of the SC of the SM-over-IP sender;

d)	the Content-Type header, which shall contain "application/vnd.3gpp.sms"; and

e)	the body of the request shall contain an RP-DATA message as defined in 3GPP TS 24.011 [8], including the SMS headers and the SMS user information encoded as specified in 3GPP TS 23.040 [3].

NOTE 4:	The address of the SC is included in the RP-DATA message content. The address of the SC included in the RP-DATA message content is stored in the EFSMSP in DF_TELECOM of the (U)SIM of the SM-over-IP sender.

NOTE 5:	The SM-over-IP sender will use content transfer encoding of type "binary" for the encoding of the SM in the body of the SIP MESSAGE request.

NOTE 6:	Both the address of the SC and the PSI of the SC can be configured in the EFPSISMSC in DF_TELECOM of the USIM and ISIM respectively using the USAT as per 3GPP TS 31.111 [21].

The SM-over-IP sender may request the SC to return the status of the submitted message. The support of status report capabilities is optional for the SC.

When a SIP MESSAGE request including a submit report in the "vnd.3gpp.sms" payload is received, the SM-over-IP sender shall:

-	if SM-over-IP sender supports In-Reply-To header usage and the In-Reply-To header indicates that the request corresponds to a short message submitted by the SM-over-IP sender, generate a 200 (OK) SIP response according to RFC 3428 [14].

if SM-over-IP sender supports In-Reply-To header usage and the In-Reply-To header indicates that the request does not correspond to a short message submitted by the SM-over-IP sender, a 488 (Not Acceptable here) SIP response according to RFC 3428 [14].

-	if SM-over-IP sender does not support In-Reply-To header usage, generate a 200 (OK) SIP response according to RFC 3428 [14]; and extract the payload encoded according to 3GPP TS 24.011 [8] for RP-ACK or RP-ERROR.

[TS 24.341 clause 5.3.1.3]:

When a SIP MESSAGE request including a status report in the "vnd.3gpp.sms" payload is delivered, the SM-over-IP sender shall:

-	generate a SIP response according to RFC 3428 [14];

-	extract the payload encoded according to 3GPP TS 24.011 [8] for RP-DATA; and

-	create a delivery report for the status report as described in subclause 5.3.2.4. The content of the delivery report is defined in 3GPP TS 24.011 [8].

[TS 24.341 clause 5.3.2.4]:

When an SM-over-IP receiver wants to send an SM delivery report over IP, the SM-over-IP receiver shall send a SIP MESSAGE request with the following information:

a)	the Request-URI, which shall contain the IP-SM-GW;

NOTE 1:	The address of the IP-SM-GW is received in the P-Asserted-Identity header in the SIP MESSAGE request including the delivered short message.

b)	the From header, which shall contain a public user identity of the SM-over-IP receiver.

c)	the To header, which shall contain the IP-SM-GW;

d)	the In-Reply-To header which shall contain the Call-Id of the SIP MESSAGE request that was received in the received short message;

e)	the Content-Type header shall contain "application/vnd.3gpp.sms"; and

f)	the body of the request shall contain the RP-ACK or RP-ERROR message for the SM delivery report, as defined in 3GPP TS 24.011 [8].

NOTE 2:	The SM-over-IP sender will use content transfer encoding of type "binary" for the encoding of the SM in the body of the SIP MESSAGE request.

[TS 24.011 clause 8.2.5.4]:

This element is a variable length element always included in the RP-ERROR message, conveying a negative result of a RP-DATA message transfer attempt or RP-SMMA notification attempt. The element contains a cause value and optionally a diagnostic field giving further details of the error cause.

The coding of the cause value is given in table 8.4/3GPP TS 24.011. The mapping between error causes in 3GPP TS 24.011 and 3GPP TS 29.002 (MAP) is specified in 3GPP TS 23.040. Parameters included in the return error from MAP (e.g. System Failure) are mapped directly into the diagnostic field.

| 8 7 6 5 4 3 2 1 |  |  |
| --- | --- | --- |
| 0 | 1 0 0 0 0 1 0 RP-Cause IEI | 1 octet |
| Length indicator |  | 1 octet |
| 0 ext | Cause value | 1 octet |
| Diagnostic field |  | 1 octet * |

Figure 8.8/3GPP TS 24.011: RP-Cause element layout

Table 8.4/3GPP TS 24.011 (part 1): Cause values that may be contained in an RP-ERROR message
in a mobile originating SM-transfer attempt

| Cause value | Cause number | Cause |
| --- | --- | --- |
| 7 6 5 4 3 2 1 | # |  |
| 0 0 0 0 0 0 1 | 1 | Unassigned (unallocated) number |
| 0 0 0 1 0 0 0 | 8 | Operator determined barring |
| 0 0 0 1 0 1 0 | 10 | Call barred |
| 0 0 0 1 0 1 1 | 11 | Reserved |
| 0 0 1 0 1 0 1 | 21 | Short message transfer rejected |
| 0 0 1 1 0 1 1 | 27 | Destination out of order |
| 0 0 1 1 1 0 0 | 28 | Unidentified subscriber |
| 0 0 1 1 1 0 1 | 29 | Facility rejected |
| 0 0 1 1 1 1 0 | 30 | Unknown subscriber |
| 0 1 0 0 1 1 0 | 38 | Network out of order |
| 0 1 0 1 0 0 1 | 41 | Temporary failure |
| 0 1 0 1 0 1 0 | 42 | Congestion |
| 0 1 0 1 1 1 1 | 47 | Resources unavailable, unspecified |
| 0 1 1 0 0 1 0 | 50 | Requested facility not subscribed |
| 1 0 0 0 1 0 1 | 69 | Requested facility not implemented |
| 1 0 1 0 0 0 1 | 81 | Invalid short message transfer reference value |
| 1 0 1 1 1 1 1 | 95 | Semantically incorrect message |
| 1 1 0 0 0 0 0 | 96 | Invalid mandatory information |
| 1 1 0 0 0 0 1 | 97 | Message type non-existent or not implemented |
| 1 1 0 0 0 1 0 | 98 | Message not compatible with short message protocol state |
| 1 1 0 0 0 1 1 | 99 | Information element non-existent or not implemented |
| 1 1 0 1 1 1 1 | 111 | Protocol error, unspecified |
| 1 1 1 1 1 1 1 | 127 | Interworking, unspecified |
| Note: All other cause values shall be treated as cause number 41, "Temporary Failure" |  |  |

Reference(s)

3GPP TS 24.341[14], clauses 5.3.1.1, 5.3.1.2, 5.3.1.3 and 5.3.2.4, and TS 24.011 [25], clause 8.2.5.4.

### 9.5.3	Test description

#### 9.5.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	SMS over IP is enabled.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

#### 9.5.3.2	Test procedure sequence

Table 9.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to send an SMS over IP. |  |  |  |  |
| 1A-1F | Steps 2-7 of generic procedure specified in Table 4.9.19.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 2 | Check: Does the UE send a SIP MESSAGE request including a vnd.3gpp.sms payload that contains a short message? | --> | SIP MESSAGE request | 1 | P |
| 3 | SS responds with 202 Accepted. | <-- | 202 ACCEPTED |  |  |
| 4 | SS sends a SIP MESSAGE request including a vnd.3gpp.sms payload and RP-ERROR message. | <-- | SIP MESSAGE request |  |  |
| 5 | Check: Does the UE respond with 200 OK? | --> | 200 OK | 2 | P |

#### 9.5.3.3	Specific message contents

Table 9.5.3.3-1: Message for MO SMS (step 2, table 9.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in annex A.7.3, Condition A2, A5

Table 9.5.3.3-2: 202 ACCEPTED (step 3, table 9.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.3.3

Table 9.5.3.3-3: Short message submission report for MO SMS (step 4, table 9.5.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.7.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | RP-ERROR message with RP-Cause Data: Length: 2, Length indicator = 1 Extension: not extended Cause value: 38 (Network out of order) |  | TS 24.011 [25] TS 23.040 [24] |

Table 9.5.3.3-4: 200 OK for other requests than REGISTER or SUBSCRIBE (step 5, table 9.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, Condition A5, A22

# < End of Modified Section >