3GPP TSG-RAN5 Meeting #90-e	R5-210259

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0085 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to A.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Annex A.11 contains two Editor’s Notes that need to be addressed. EN1: TS 24.229 does not require the UE to follow RFC 6665 for mobile initiated de-registration. EN2: Annex A.10 suppresses further PUBLISH requests by sending 503 Service Unavailable. Therefore, UEs are not expected to send un-PUBLISH requests upon mobile initiated de-registration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed both Editor’s Notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

# A.11	Mobile Initiated De-Registration / 5GS

IMS de-registration is initiated on the UE. The SS waits for the UE to send a REGISTER request, in accordance with 3GPP TS 24.229 [7], clause 5.1.1.6.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 0A |  |  | SUBSCRIBE | Optional: The UE unsubscribes from one of its subscribed to event packages. |
| 0B |  |  | 200 OK | If the UE sent SUBSCRIBE, the SS responds to SUBSCRIBE with 200 OK. |
| 0C |  |  | NOTIFY | If the UE sent SUBSCRIBE, the SS sends a final NOTIFY |
| 0D |  |  | 200 OK | If the UE sent SUBSCRIBE, the UE responds to NOTIFY with 200 OK. |
| 1 |  |  | REGISTER | The UE sends a de-registration request for IMS services. |
| 2 |  |  | 200 OK | The SS responds to REGISTER with 200 OK. |
| Note 1: Steps 0A-0D may be repeated for any or all event packages subscribed to by the UE. It is the UE’s decision which unsubscriptions to perform. Note 2: The UE can send the 200 OK for NOTIFY (step 0D) after the REGISTER request (step 1) or even not send it at all. |  |  |  |  |

Specific message contents

SUBSCRIBE (step 0A)

Use the default message “SUBSCRIBE for reg-event package” in Annex A.1.4 of TS 34.229-1 [2] or “SUBSCRIBE for conference event package” in Annex A.5.1 of TS 34.229-1 [2] or “SUBSCRIBE for message-summary event package” in Annex A.6.1 of TS 34.229-1 [2], and with the following exceptions:

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| From |  |  |  |  |
| addr-spec |  | Same as in original SUBSCRIBE that set up the corresponding subscription |  |  |
| tag |  | Same as in original SUBSCRIBE that set up the corresponding subscription |  |  |
| To |  |  |  |  |
| addr-spec |  | As specified in TS 34.229-1 [2] Annex A.1.4/A.5.1/A.6.1 |  |  |
| tag |  | Same as in 200 OK for original SUBSCRIBE that set up the corresponding subscription |  |  |
| CSeq |  |  |  |  |
| value |  | value of the previous SUBSCRIBE sent by the UE for this dialog incremented by one |  |  |
| method |  | SUBSCRIBE |  |  |
| Expires |  |  |  |  |
| delta-seconds |  | 0 |  |  |

200 OK for SUBSCRIBE (step 0B)

Use the default message “200 OK for SUBSCRIBE” in Annex A.1.5, A.5.2 or A.6.3 of TS 34.229-1 [2], whatever appropriate, with the following exceptions:

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| To |  |  |  | RFC 3261 [6] |
| addr-spec |  | As specified in TS 34.229-1 [2] Annex A.1.4/A.5.1/A.6.1 |  |  |
| tag |  | Same as in step 0A |  |  |
| Expires |  |  |  | RFC 3261 [6] |
| delta-seconds |  | 0 |  |  |

NOTIFY (step 0C)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [6] |
| Method |  | NOTIFY |  |  |
| Request-URI |  | UE’s contact address in SIP URI form, as provided in the Contact header within the SUBSCRIBE creating the dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  | order of the parameters in this header must be like in this table |  | RFC 3261 [6] |
| via-param1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by |  | IP address and protected server port of SS |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| via-param2: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by |  | scscf.3gpp.org |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| From |  |  |  | RFC 3261 [6] |
| addr-spec |  | same URI as received in the To header of the corresponding SUBSCRIBE message |  |  |
| tag |  | same as to-tag in step 0A |  |  |
| To |  |  |  | RFC 3261 [6] |
| addr-spec |  | same URI as received in the From header of the corresponding SUBSCRIBE message |  |  |
| tag |  | same as from-tag in step 0A |  |  |
| Call-ID |  |  |  | RFC 3261 [6] |
| callid |  | same as value received in SUBSCRIBE message |  |  |
| CSeq |  |  |  | RFC 3261 [6] |
| value |  | 1 |  |  |
| method |  | NOTIFY |  |  |
| Contact |  |  |  | RFC 3261 [6] |
| addr-spec | A1 | <sip:scscf.3gpp.org> |  |  |
| addr-spec | A2 | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |
| addr-spec | A3 | <scscf.3gpp.org> |  |  |
| Event |  |  |  | RFC 6665 [28] RFC 3680 [18] |
| event-type | A1 | reg |  |  |
| event-type | A2 | conference |  |  |
| event-type | A3 | message-summary |  |  |
| Max-Forwards |  |  |  | RFC 3261 [6] |
| value |  | 69 |  |  |
| Subscription-State |  |  |  | RFC 6665 [28] |
| substate-value |  | terminated |  |  |
| Content-Length |  |  |  |  |
| value |  | 0 |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Final NOTIFY sent for reg-event |
| A2 | Final NOTIFY sent for conf-event |
| A3 | Final NOTIFY sent for message-summary |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

200 OK (step 0D)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2].

REGISTER (step 1)

Use the default message “REGISTER” in Annex A.1.1 of TS 34.229-1 [1] with conditions A2 and A17 "UE initiated IMS re-registration or de-registration" with the following exceptions:

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Contact |  |  |  | RFC 3261 [6] |
| addr-spec |  | SIP URI with IP address or FQDN and protected server port of the UE, AND, if the UE supports GRUU, the following parameter: +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>”, OR * |  |  |
| expires |  | 0 (if present) |  |  |
| Expires |  | (must be present if addr-spec is *) |  | RFC 3261 [6] |
| delta-seconds |  | 0 (if present) |  |  |
| Supported |  | header may be missing or it may contain any value |  |  |
| Authorization |  | value not checked |  |  |

NOTE:	In contrast to Annex A.1.1 of TS 34.229-1 [2], the Contact header does not have any further mandatory feature parameters.

200 OK (step 2)

Use the default message “200 OK for REGISTER” in Annex A.1.3 of TS 34.229-1 [2] with the following exceptions:

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Contact |  |  |  | RFC 3261 [6] |
| addr-spec |  | same value as in REGISTER request if "*" is not included in the Contact header field of the REGISTER request in step 1 same value as in the Contact header field of the "200 OK" response to the initial registration if "*" is included in the Contact header field of the REGISTER request in step 1 (NOTE) |  |  |
| expires |  | 0 |  |  |
| NOTE: According to 3GPP TS 24.229 [7] clause 5.4.1.4.1 when the S-CSCF gets a wild-carded contact address for de-registration it shall include all de-registered contact addresses in the contact header of the 200 OK response there is no “*” in DL |  |  |  |  |