3GPP TSG-RAN5 Meeting #90-e	R5-211435

# Electronic Meeting, 22nd February 2021 - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0100 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS over 5GS TC 7.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Anritsu, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Based on RFC 3261 cl. 17.2 the UE may send 100Trying before 420 Bad Extension answer: The server transaction MUST generate a 100 (Trying) response unless it knows that the TU will generate a provisional or final response within 200 ms, in which case it MAY generate a 100 (Trying) response. This provisional response is needed to quench request retransmissions rapidly in order to avoid network congestion. 2. Based on RFC 3281 cl. 13.1, the UAC needs to send SIP ACK for every final response it receives. The 420 Bad Extension in step 10 is considered as final response and as such the test body needs to be extended for step 11 SIP ACK. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added optional step 9a for SIP 100Trying 2. Added step 11 to acknowledge ACK the SIP 420 Bad Extension |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-210592 R5-210592r1 - Added ‘Table 7.2.3.3-3: ACK’ in Specific message contents. - Added ‘Table 7.2.3.3-2: 100 Trying’ in Specific message contents. R5-211435: This is the final version of R5-210592r1 |  |  |  |  |  |  |  |  |

## 7.11	MTSI MT Voice call without preconditions at terminating UE and originating UE requiring them / 5GS

### 7.11.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to not use preconditions }

ensure that {

when { UE receives INVITE for voice call where remote UE requires usage of preconditions }

then { UE rejects INVITE with 420 Bad Extension response }

}

### 7.11.2	Conformance Requirements

[TS 24.229, clause 5.1.4.1]

If an initial INVITE request is received the terminating UE shall check whether the terminating UE requires local resource reservation.

NOTE 1:	The terminating UE can decide if local resource reservation is required based on e.g. application requirements, current access network capabilities, local configuration, etc.

During the session initiation, if local resource reservation is required at the terminating UE and the terminating UE supports the precondition mechanism, and:

a)	the received INVITE request includes the "precondition" option-tag in the Supported header field or Require header field and the precondition mechanism is enabled as specified in subclause 5.1.5A, the terminating UE shall use the precondition mechanism and shall include a Require header field with the "precondition" option-tag:

-	in responses to that INVITE request if those responses include an SDP body;

-	in responses to subsequent requests received in-dialog that include an SDP body and include "precondition" option-tag in Supported header field or Require header field; and

-	in subsequent requests that include an SDP body, that it sends towards the originating UE during the session initiation;

b)	the received INVITE request includes the "precondition" option-tag in the Supported header field, and the precondition mechanism is disabled as specified in subclause 5.1.5A, the terminating UE shall not use the precondition mechanism:

c)	the received INVITE request includes the "precondition" option-tag in the Require header field, and the precondition mechanism is disabled as specified in subclause 5.1.5A, the terminating UE shall reject the INVITE request with a 420 (Bad Extension) response; and

d)	the received INVITE request does not include the "precondition" option-tag in the Supported header field or Require header field, the terminating UE shall not use the precondition mechanism.

### 7.11.3	Test description

#### 7.11.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	The precondition mechanism is disabled at the UE.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

#### 7.11.3.2	Test procedure sequence

Table 7.11.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 9 | SS sends an INVITE request with a Require header field containing the precondition option-tag. | <-- | INVITE |  |  |
| 9A | Optional step: UE may send a 100 Trying provisional response. | --> | (Optional) 100 Trying |  |  |
| 10 | UE sends a 420 Bad Extension response with an Unsupported header field containing the precondition option-tag. | --> | 420 Bad Extension | 1 | P |
| 11 | SS acknowledges the reception of 420 Bad Extension. | <-- | ACK |  |  |

#### 7.11.3.3	Specific message contents

Table 7.11.3.3-1: INVITE (step 9, table 7.11.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.9, Conditions A1, A3, and A4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  |  |  | RFC 3261 [6] |
| option-tag |  | precondition |  |  |

Table 7.2.3.3-2: 100 Trying (step 9A, Table 7.2.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.2 Condition A2

Table 7.11.3.3-3: 420 Bad Extension for INVITE (step 10, table 7.11.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.25 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Unsupported |  |  |  | RFC 3261 [6] |
| option-tag |  | precondition |  |  |

Table 7.2.3.3-4: ACK (step 11, Table 7.2.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.7 Conditions A2 and A4