3GPP TSG-RAN5 Meeting #90-e	R5-211437

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0103 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS A.9.2-Optional UPDATE after EPS fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause A.9.2 for EPS fallback requires the UE to send UPDATE after falling back to E-UTRA. From a core specification point of view, it is not entirely clear though if such UPDATE is mandatory when preconditions are not used. As RAN5 is planning for a corresponding LS to CT1 (see R5-211311), and will have to wait for a response, it is proposed to make UPDATE optional for the time being. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Made the UPDATE and consequencial 200 OK optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legit UEs might fail EPS fallback relevent TCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This document is the outcome of R5-210658r2. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

## A.9.2	EPS Fallback for Voice Call / steps after fallback / 5GS

Expected sequence

| Step | Direction |  | Message/Procedure | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 0A | <-- |  | 180 Ringing | - |
| 0B | - |  | SS starts a timer (5 seconds) to wait for optional UPDATE or REGISTER from the UE. | - |
| - | - |  | EXCEPTION: Step 1a1-1c1 describes behaviour that depends on UE implementation. The “lower case letter” identifies a step sequence that takes place if such implementation was applied. | - |
| 1a1 | --> |  | UPDATE | Optional: The UE sends an UPDATE request containing a second SDP offer. |
| 1a2 | <-- |  | 200 OK for UPDATE | If the UE sent UPDATE, the SS sends a 200 OK response for UPDATE containing an SDP answer. |
| 1b1 | --> |  | REGISTER | Optional: The UE sends a REGISTER request |
| 1b2 | <-- |  | 200 OK for REGISTER | If the UE sent REGISTER, the SS sends a 200 OK response for REGISTER containing an SDP answer. |
| 1c1 |  |  | The timer started in step 0B is expired. |  |
| 2 | - |  | -Void |  |
| 3 | <-- |  | Void |  |
| 4 | <-- |  | 200 OK | SS responds to INVITE with 200 OK. |
| 5 | --> |  | ACK | UE acknowledges. |

Specific message contents

180 Ringing (Step 0A)

Use the default message "180 Ringing for INVITE" in Annex A.2.6 of TS 34.229-1 [2] with conditions A1 and A13.

UPDATE (Step 1a1)

Use the default message "UPDATE" in Annex A.2.5 of TS 34.229-1 [2] with conditions A1 and A5 and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 2] [Note 4] - a=fmtp: (format) [Note 2, 3] Note 1: At least one "c=" field shall be present. Note 2: The value for fmt, payload type and format is not checked Note 3: Parameters for the AMR codec are not checked Note 4: The AMR channel number shall be “/1” or omitted. |

200 OK (Step 1a2)

Use the default message "200 OK for requests other than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] with conditions A1, A10 and A21 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 OK response copied from the received UPDATE and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - "o=" line identical to previous SDP sent by SS except that sess-version is incremented. |

REGISTER (Step 1b1)

Use the default message "REGISTER" in Annex A.1.1 of TS 34.229-1 [2] applying conditions A2 and A31.

200 OK (Step 1b2)

Use the default message "200 OK for REGISTER" in Annex A.1.3 of TS 34.229-1 [2] with condition A2.

200 OK (Step 4)

Use the default message "200 OK for requests other than REGISTER or SUBSRIBE" in Annex A.3.1 of TS 34.229-1 [2] with conditions A1, A10, A19, and A21.

ACK (Step 5)

Use the default message "ACK" in Annex A.2.7 of TS 34.229-1 [2] with condition A1.

<End of modified section 1>