3GPP TSG-RAN5 Meeting #90-e	R5-211446

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0113 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR IMS TC 8.36-MO explicit communication transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no test coverage to verfiy that an UE could correctly handle MO explicit communication transfer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 8.36 for MO explicit communication transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No testing of UE compliance for such use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.36 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0278 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This document is the outcome of R5-210668r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.36	MO Explicit Communication Transfer / Consultative Call Transfer / 5GS

8.36.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and having established a voice call with A (the transferee) }

ensure that {

when { UE is being made to attempt Consultative Call Transfer }

then { UE puts A on hold and sets up voice call with B (the transfer target) and puts B on hold and sends REFER to the transferee }

}

(2)

with { UE having initiated consultative call transfer }

ensure that {

when { UE receives NOTIFY }

then { UE sends 200 OK response for NOTIFY }

}

(3)

with { UE having processed the NOTIFY exchange }

ensure that {

when { UE receives instruction to be put on hold by A }

then { UE processes call hold instruction and responds to it }

}

(4)

with { UE having been put on hold by A }

ensure that {

when { UE receives BYE from B }

then { UE sends 200 OK for BYE }

}

(5)

with { Call with B having ended }

ensure that {

when { UE receives NOTIFY from A }

then { UE sends 200 OK for NOTIFY and may send BYE }

}

8.36.2	Conformance Requirements

[TS 24.629, clause 4.5.2.1]:

A UE that has initiated an emergency call, shall not perform any transfer operation involving the dialog associated with the emergency call.

A UE that initiates a transfer operation shall if the Contact address of the transferee is a GRUU:

-	issue a REFER outside an existing dialog as specified in RFC 3515 [2] as updated by IETF RFC 6665 [14] and IETF RFC 7647 [16], where:

a)	the request URI shall contain the SIP URI of the transferee as received in the Contact header field:

b)	the Refer-To header field shall indicate the public address of the transfer target;

c)	in case of Consultative transfer, the transferor UE has a consultation communication with the transfer target, a Replaces header field parameter shall be added to the Refer-To URI together with a Require=replaces header field parameter;

d)	the Referred-By header field can be used to indicate the identity of the transferor. When privacy was required in the original communications dialog and a Referred-By header field is included, the UE shall include a Privacy header field set to "user"; and

e)	the Target-Dialog header field identifies the dialog to be transferred;

otherwise the UE shall:

-	issue a REFER request in the original communications dialog as specified in RFC 3515 [2], where:

a)	the request URI shall contain the SIP URI of the transferee as received in the Contact header field;

b)	the Refer-To header field shall indicate the public address of the transfer target;

c)	in case of consultative transfer, the transferor UE has a consultation communication with the transfer target, a Replaces header field parameter shall be added to the Refer-To URI together with a Require=replaces header field parameter; and

d)	the Referred-By header field can be used to indicate the identity of the transferor. When privacy was required in the original communications dialog and a Referred-By header field is included, the UE shall include a Privacy header field set to "user".

If assured transfer is requested, the UE may include an Expires header field in the Refer-To URI of the REFER request.

NOTE 1:	The value of the Expires header field indicates the maximum duration of the transfer attempt. If the transfer does not succeed within this duration, the UE will receive a NOTIFY request indicating the transfer failure.

After the REFER request is accepted by the other end with a 2xx response, the transferor UE gets notifications of how the transferee's communication setup towards the transfer Target is progressing.

When a NOTIFY request is received on the REFER dialog that indicates that the transferee and the transfer Target have successfully setup a communication, the transferor UE may terminate the original communication with the transferee UE, by sending a BYE request on the original dialog.

If an assured transfer attempt is not completed (i.e. the UE has not received a NOTIFY request with a "message/sipfrag" body’s status line containing a final response code indicating the end of the transfer operation), the UE may request to terminate the transfer attempt by:

-	sending a REFER request in the same communications dialog as the previous REFER request as specified in RFC 3515 [2] as updated by IETF RFC 6665 [14] and IETF RFC 7647 [16], where:

a)	the request URI shall contain the SIP URI of the transferee as received in the Contact header field; and

b)	the Refer-To header field shall indicate the public address of the transfer target and shall contain the method parameter set to "CANCEL"; and

c)	if applicable include a Target-Dialog header field that identifies the dialog under transfer.

If the UE receives a NOTIFY request indicating that the assured transfer attempt failed,followed by a re-INVITE or an UPDATE request taking the UE off HOLD the UE may decide to retrieve the original communication by sending a re-INVITE request in the original SIP dialog.

NOTE 2:	If the user requests the retrieval of the original communication while the transfer attempt has not been completed, the UE needs to first request the termination of the transfer attempt before retrieving the original communication via a re-INVITE request.

Reference(s)

3GPP TS 24.629 [36], clause 4.5.2.1.

8.36.3	Test description

8.36.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE has registered to IMS and set up the MO call, by executing the generic test procedure in Annex A.2 up to the last step and thereafter executing the generic test procedure in A4.1.

-	The SS has accepted the UE’s MO call.

8.36.3.2	Test procedure sequence

Table 8.36.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is made to attempt Consultative Call Transfer | - | - | - | - |
| 2 | Check: Does the UE send INVITE or UPDATE with a SDP offer to hold the call with A? | --> | INVITE or UPDATE | 1 | P |
| 3 | Optional: The SS responds to the INVITE with a 100 Trying provisional response | <-- | 100 Trying | - | - |
| 4 | The SS responds to INVITE or UPDATE with 200 OK to indicate that the remote UE is no more sending any media (call hold) or resumes sending media (call resume) | <-- | 200 OK | - | - |
| 5 | Optional: If the UE sent INVITE in step 1 then UE acknowledges the receipt of 200 OK for INVITE | --> | ACK | - | - |
| 6 | The UE is made to attempt an IMS speech call with B | - | - | - | - |
| 7 | Steps 1 of A.4.1 happens Check: Does the initiates a voice call with B? |  | INVITE | 1 | P |
| 8-18 | Steps 2-12 of A.4.1 happen | - | - | - | - |
| 19 | The UE is made to hold the call | - | - |  |  |
| 20 | Check: Does the UE send INVITE or UPDATE with a SDP offer to hold the call with B? | --> | INVITE or UPDATE | 1 | P |
| 21 | Optional: The SS responds to the INVITE with a 100 Trying provisional response | <-- | 100 Trying | - | - |
| 22 | The SS responds to INVITE or UPDATE with 200 OK to indicate that the remote UE is no more sending any media (call hold) or resumes sending media (call resume) | <-- | 200 OK | - | - |
| 23 | Optional: If the UE sent INVITE in step 1 then UE acknowledges the receipt of 200 OK for INVITE | --> | ACK | - | - |
| 24 | Check: Does the UE send REFER to SS, simulating the transferee, referring to the transfer target | --> | REFER | 1 | P |
| 25 | The SS responds to REFER with 200 OK | <-- | 200 OK |  |  |
| 26 | The SS, simulating the transferee, sends initial NOTIFY for the implicit subscription created by the REFER request | <-- | NOTIFY |  |  |
| 27 | The UE responds to NOTIFY with 200 OK | --> | 200 OK | 2 | P |
| 28 | SS sends INVITE with a SDP offer to hold or resume the call |  | INVITE |  |  |
| 29 | Optional: The UE responds with a 100 Trying provisional response | --> | 100 Trying |  |  |
| 30 | The UE responds to INVITE with 200 OK to indicate that the UE is no more expecting to receive any media | --> | 200 OK | 3 | P |
| 31 | The SS acknowledges the receipt of 200 OK for INVITE | <-- | ACK | - | - |
| 32 | The SS, simulating the transfer target, releases the call between UE and the transfer target with BYE | <-- | BYE | - | - |
| 33 | The UE responds to BYE with 200 OK | --> | 200 OK | 4 | P |
| 34 | The SS, simulating the transferee, sends a NOTIFY to confirm that the call transfer has been completed | <-- | NOTIFY | - | - |
| 35 | The UE responds to NOTIFY with 200 OK | --> | 200 OK | - | - |
| 36 | Optional: UE may send BYE request to release call with the transferee | --> | BYE | - | - |
| 37 | If the UE has sent BYE in step 33 then SS sends 200 OK for BYE | --> | 200 OK | 5 | P |

8.36.3.3	Specific message contents

Table 8.36.3.3-1: INVITE (step 2/20, table 8.36.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.1, Conditions A1, A3 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | the same URI in the request line as the SS has sent in the Contact header of an earlier message within the same dialog |  |  |
| Supported Option-tag |  | precondition |  |  |
| Message-body |  | Header optional Contents if present: The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 2] b=AS: (bandwidth-value) Time description: t=0 0 Media description: The same media description with the initial session. Attributes for media: The same codec with the initial session. Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 2: At least one "c=" field shall be present. |  | TS 24.229 [7] |

Table 8.36.3.3-2: UPDATE (step 2/20, table 8.36.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.5, condition A1, A3 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | the same URI in the request line as the SS has sent in the Contact header of an earlier message within the same dialog |  |  |
| Supported Option-tag |  | precondition |  |  |
| Message-body |  | Header optional Contents if present: The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 2] b=AS: (bandwidth-value) Time description: t=0 0 Media description: The same media description with the initial session. Attributes for media: The same codec with the initial session. Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 2: At least one "c=" field shall be present. |  | TS 24.229 [7] |

Table 8.36.3.3-3: 100 Trying (step 3/21, table 8.36.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in annex A.2.2, Condition A1

Table 8.36.3.3-4: 200 OK (step 4/22, table 8.36.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in annex A.3.1, Condition A10 and A22

Table 8.36.3.3-5: ACK (step 5/23, table 8.36.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in annex A.2.6, Conditions A1 and A3

Table 8.36.3.3-6: INVITE (step 7, table 8.36.3.2-1)

Derivation Path: TS 34.229-5, Table of Step 1 in A.4.1

Table 8.36.3.3-7: REFER (step 24, table 8.36.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.10, Condition A5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | sip:final@conf-factory appended with px_IMS_HomeDomainName |  |  |
| Refer-To addr-spec |  | SIP URI or tel URI of the user invited to the conference. If an active session exists, the Replaces header in the header portion of the SIP URI shall be included (mandatory inclusion is stated in IR.92 ) and set to the dialog ID of the active session according to RFC 3891. In this case, if the user has been invited with a tel URI, the UE shall convert the tel URI to a SIP URI according to RFC 3261 [6] clause 19.1.6. (NOTE: the dialog ID is percent encoded according to RFC 3986). |  |  |
| To addr-spec tag |  | remote SIP URI as used in To header in step 12 remote tag of the dialog with the conference focus created in step 12 |  |  |
| Route Route-param |  | URIs of the Record-Route header of 183 response sent in step 4 (step 3 of A.4.1) |  |  |

Editor’s Note: other specific contents are for FFS.

<End of modified section 1>