3GPP TSG-RAN5 Meeting #90-e	R5-211835

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5272 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 7, 42, 43 and 65 for UE category M1 and NB1 to clause 6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R16_M1_NB1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Rel-16 NB-IoT and Cat M1 test frequencies are missing for band 7, 42, 43 and 65. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-16 NB-IoT and Cat M1 test frequencies added for band 7, 42, 43 and 65. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT and Cat M1 test frequencies for band 7, 42, 43 and 65 will be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2EA.3, 6.2.2EA.5, 6.2.2F.3, 6.2.2F.5, 6.2.2FA.3, 6.2.2FA.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-211237 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

#### 6.2.2EA.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration and UL-MIMO unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2EA-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 2 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 3 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 4 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 5 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 7 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 8 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 11 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 12 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 13 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 14 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 18 |  |  | 23 | ±25 | 20 | ±25 | 14 | ±2.5 |
| 19 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 20 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 21 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 26 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 27 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 28 |  |  | 23 | +2/-2.5 | 20 | +2/-2.5 | 14 | ±2.5 |
| 31 | 267 | ±2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| … |  |  |  |  |  |  |  |  |
| 39 |  |  | 23 | ±2 | 20 | ±2 |  |  |
| 41 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 42 |  |  | 23 | +2/-3 | 20 | +2/-3 | 14 | ±2.5 |
| 43 |  |  | 23 | +2/-3 | 20 | +2/-3 | 14 | ±2.5 |
| … |  |  |  |  |  |  |  |  |
| 71 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 72 | 267 | ±2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 73 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 74 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 85 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 87 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 88 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: Void NOTE 7: Applicable for category M1 and M2 HD-FDD UE. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.2.2E.

{Many clauses omitted}

#### 6.2.2EA.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2EA.5-1.

Table 6.2.2EA.5-1: UE Power Class test requirements

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 2 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 3 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 4 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 5 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 7 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 8 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 11 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 12 |  |  | 23 | ±2.7 | 20 | ±2.72 | 14 | ±3.2 |
| 13 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 14 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 18 |  |  | 23 | ±2.75 | 20 | ±2.75 | 14 | ±3.2 |
| 19 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 20 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 21 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 26 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 27 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 28 |  |  | 23 | +2.7/-3.2 | 20 | +2.7/-3.2 | 14 | ±3.2 |
| 31 | 26 | ±2.77 | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| … |  |  |  |  |  |  |  |  |
| 39 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 41 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 42 |  |  | 23 | +3.0/-4.0 | 20 | +3.0/-4.0 | 14 | ±3.5 |
| 43 |  |  | 23 | +3.0/-4.0 | 20 | +3.0/-4.0 | 14 | ±3.5 |
| … |  |  |  |  |  |  |  |  |
| 71 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 72 | 26 | ±2.77 | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 73 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 74 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 85 |  |  | 23 | ±2.7 | 20 | ±2.72 | 14 | ±3.2 |
| 87 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 88 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: Void NOTE 7: Applicable for category M1 and M2 HD-FDD UE |  |  |  |  |  |  |  |  |

{Many clauses omitted}

6.2.2F.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2.2F.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2.2F.3-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 1 | 23 | ±2 | 20 | ±2 |
| 2 | 23 | ±2 | 20 | ±2 |
| 3 | 23 | ±2 | 20 | ±2 |
| 4 | 23 | ±2 | 20 | ±2 |
| 5 | 23 | ±2 | 20 | ±2 |
| 7 | 23 | ±2 | 20 | ±2 |
| 8 | 23 | ±2 | 20 | ±2 |
| 11 | 23 | ±2 | 20 | ±2 |
| 12 | 23 | ±2 | 20 | ±2 |
| 13 | 23 | ±2 | 20 | ±2 |
| 14 | 23 | ±2 | 20 | ±2 |
| 17 | 23 | ±2 | 20 | ±2 |
| 18 | 23 | ±2 | 20 | ±2 |
| 19 | 23 | ±2 | 20 | ±2 |
| 20 | 23 | ±2 | 20 | ±2 |
| 21 | 23 | ±2 | 20 | ±2 |
| 25 | 23 | ±2 | 20 | ±2 |
| 26 | 23 | ±2 | 20 | ±2 |
| 28 | 23 | ±2 | 20 | ±2 |
| 31 | 23 | ±2 | 20 | ±2 |
| 41 | 23 | ±2 | 20 | ±2 |
| 42 | 23 | ±2 | 20 | ±2 |
| 43 | 23 | ±2 | 20 | ±2 |
| 65 | 23 | ±2 | 20 | ±2 |
| 66 | 23 | ±2 | 20 | ±2 |
| 70 | 23 | ±2 | 20 | ±2 |
| 71 | 23 | ±2 | 20 | ±2 |
| 72 | 23 | ±2 | 20 | ±2 |
| 73 | 23 | ±2 | 20 | ±2 |
| 74 | 23 | ±2 | 20 | ±2 |
| 85 | 23 | ±2 | 20 | ±2 |
| 87 | 23 | ±2 | 20 | ±2 |
| 88 | 23 | ±2 | 20 | ±2 |

The normative reference for this requirement is TS 36.101 clause 6.2.2F.

{Many clauses omitted}

#### 6.2.2F.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2F.5-1.

Table 6.2.2F.5-1: UE Power Class test requirement

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 1 | 23 | ±2.7 | 20 | ±2.7 |
| 2 | 23 | ±2.7 | 20 | ±2.7 |
| 3 | 23 | ±2.7 | 20 | ±2.7 |
| 4 | 23 | ±2.7 | 20 | ±2.7 |
| 5 | 23 | ±2.7 | 20 | ±2.7 |
| 7 | 23 | ±2.7 | 20 | ±2.7 |
| 8 | 23 | ±2.7 | 20 | ±2.7 |
| 11 | 23 | ±2.7 | 20 | ±2.7 |
| 12 | 23 | ±2.7 | 20 | ±2.7 |
| 13 | 23 | ±2.7 | 20 | ±2.7 |
| 14 | 23 | ±2.7 | 20 | ±2.7 |
| 17 | 23 | ±2.7 | 20 | ±2.7 |
| 18 | 23 | ±2.7 | 20 | ±2.7 |
| 19 | 23 | ±2.7 | 20 | ±2.7 |
| 20 | 23 | ±2.7 | 20 | ±2.7 |
| 21 | 23 | ±2.7 | 20 | ±2.7 |
| 25 | 23 | ±2.7 | 20 | ±2.7 |
| 26 | 23 | ±2.7 | 20 | ±2.7 |
| 28 | 23 | ±2.7 | 20 | ±2.7 |
| 31 | 23 | ±2.7 | 20 | ±2.7 |
| 41 | 23 | ±2.7 | 20 | ±2.7 |
| 42 | 23 | ±3.0 | 20 | ±3.0 |
| 43 | 23 | ±3.0 | 20 | ±3.0 |
| 65 | 23 | ±2.7 | 20 | ±2.7 |
| 66 | 23 | ±2.7 | 20 | ±2.7 |
| 70 | 23 | ±2.7 | 20 | ±2.7 |
| 71 | 23 | ±2.7 | 20 | ±2.7 |
| 72 | 23 | ±2.7 | 20 | ±2.7 |
| 73 | 23 | ±2.7 | 20 | ±2.7 |
| 74 | 23 | ±2.7 | 20 | ±2.7 |
| 85 | 23 | ±2.7 | 20 | ±2.7 |
| 87 | 23 | ±2.7 | 20 | ±2.7 |
| 88 | 23 | ±2.7 | 20 | ±2.7 |

6.2.2FA.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2.2FA.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2.2FA.3-1: UE Power Class 6

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NA | NA | NA | NA | 14 | ±2.5 |
| 2 | NA | NA | NA | NA | 14 | ±2.5 |
| 3 | NA | NA | NA | NA | 14 | ±2.5 |
| 4 | NA | NA | NA | NA | 14 | ±2.5 |
| 5 | NA | NA | NA | NA | 14 | ±2.5 |
| 7 | NA | NA | NA | NA | 14 | ±2.5 |
| 8 | NA | NA | NA | NA | 14 | ±2.5 |
| 11 | NA | NA | NA | NA | 14 | ±2.5 |
| 12 | NA | NA | NA | NA | 14 | ±2.5 |
| 13 | NA | NA | NA | NA | 14 | ±2.5 |
| 14 | NA | NA | NA | NA | 14 | ±2.5 |
| 17 | NA | NA | NA | NA | 14 | ±2.5 |
| 18 | NA | NA | NA | NA | 14 | ±2.5 |
| 19 | NA | NA | NA | NA | 14 | ±2.5 |
| 20 | NA | NA | NA | NA | 14 | ±2.5 |
| 21 | NA | NA | NA | NA | 14 | ±2.5 |
| 25 | NA | NA | NA | NA | 14 | ±2.5 |
| 26 | NA | NA | NA | NA | 14 | ±2.5 |
| 28 | NA | NA | NA | NA | 14 | ±2.5 |
| 31 | NA | NA | NA | NA | 14 | ±2.5 |
| 41 | NA | NA | NA | NA | 14 | ±2.5 |
| 42 | NA | NA | NA | NA | 14 | ±2.5 |
| 43 | NA | NA | NA | NA | 14 | ±2.5 |
| 65 | NA | NA | NA | NA | 14 | ±2.5 |
| 66 | NA | NA | NA | NA | 14 | ±2.5 |
| 70 | NA | NA | NA | NA | 14 | ±2.5 |
| 71 | NA | NA | NA | NA | 14 | ±2.5 |
| 72 | NA | NA | NA | NA | 14 | ±2.5 |
| 73 | NA | NA | NA | NA | 14 | ±2.5 |
| 74 | NA | NA | NA | NA | 14 | ±2.5 |
| 85 | NA | NA | NA | NA | 14 | ±2.5 |
| 87 | NA | NA | NA | NA | 14 | ±2.5 |
| 88 | NA | NA | NA | NA | 14 | ±2.5 |

The normative reference for this requirement is TS 36.101 clause 6.2.2F.

#### 6.2.2FA.4	Test description

Same test description as in clause 6.2.2F.4.

#### 6.2.2FA.5	Test requirements

Same test requirements as in clause 6.2.2F.5 with the following exceptions:

-	Instead of Table 6.2.2F.5-1  use Table 6.2.2FA.5-1.

Table 6.2.2FA.5-1: UE Power Class test requirement for Power Class 6

| EUTRA band | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 14 | ±3.2 |
| 2 | 14 | ±3.2 |
| 3 | 14 | ±3.2 |
| 4 | 14 | ±3.2 |
| 5 | 14 | ±3.2 |
| 7 | 14 | ±3.2 |
| 8 | 14 | ±3.2 |
| 11 | 14 | ±3.2 |
| 12 | 14 | ±3.2 |
| 13 | 14 | ±3.2 |
| 14 | 14 | ±3.2 |
| 17 | 14 | ±3.2 |
| 18 | 14 | ±3.2 |
| 19 | 14 | ±3.2 |
| 20 | 14 | ±3.2 |
| 21 | 14 | ±3.2 |
| 25 | 14 | ±3.2 |
| 26 | 14 | ±3.2 |
| 28 | 14 | ±3.2 |
| 31 | 14 | ±3.2 |
| 41 | 14 | ±3.2 |
| 42 | 14 | ±3.5 |
| 43 | 14 | ±3.5 |
| 65 | 14 | ±3.2 |
| 66 | 14 | ±3.2 |
| 70 | 14 | ±3.2 |
| 71 | 14 | ±3.2 |
| 72 | 14 | ±3.2 |
| 73 | 14 | ±3.2 |
| 74 | 14 | ±3.2 |
| 85 | 14 | ±3.2 |
| 87 | 14 | ±3.2 |
| 88 | 14 | ±3.2 |

[End of changes]