3GPP TSG-RAN5 Meeting #90-e	R5-210934

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1171 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating REFSENS for SUL for new R17 configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The minimum requirement of REFSENS for SUL is updated in R17 core spec 38.101-1. In 38.521-1 test case 7.3C need to be updated to align with the core spec. And test case REFSENS for SUL (3CC) need to be added into clause 7.3C in TS 38.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating 7.3C to align with latest core spec. Adding new clause 7.3C.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum requirement of 7.3C is incorrect. REFSENS can’t be tested for band combinations with UL SUL and DL CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C, 7.3C.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper is in R5-210935 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.3C	Reference sensitivity for SUL

7.3C.0	Minimum conformance requirements

7.3C.0.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3C.0.2	Minimum conformance requirements for Reference sensitivity power level

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in Table 7.3.2.3-1 and 7.3.2.3-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-1 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.0.2-1 with   reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this section of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this section as subset.

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3C.0.2-1: Supplementary uplink configuration for reference sensitivity

| DL band | UL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHZ | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n78 | n80 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |
| n78 | n81 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |
| n78 | n82 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |
| n78 | n83 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |
| n78 | n84 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 |  | 100 |  |  |  |
| n78 | n86 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |
| n79 | n80 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |
| n79 | n81 | 15 | 25 | 50 | 75 | 100 |  |  | 100 | 100 |  |  |  |  |  |

For the UE that supports any of the SUL operation given in Table 7.3C.0.2-2, exceptions to the requirements specified in Table7.3.2.3-1 are allowed when the uplink is active in a lower frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band as noted in Table 7.3C.0.2-2. For these exceptions, the UE shall meet the requirements specified in Table 7.3C.0.2-2 and Supplementary Uplink configuration (exceptions due to harmonic issue given in Table 7.3C.0.2-3.

<Unchanged Text Skipped>

7.3C.0.3	ΔRIB,c for SUL

7.3C.0.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

7.3C.0.3.2	SUL band combination

For the UE which supports SUL band combination, the minimum requirement for reference sensitivity in subclause 7.3C.0 shall be increased by the amount given in ΔRIB,c defined in subclause 7.3C.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14] for the applicable operating bands.

7.3C.0.3.2.1	ΔRIB,c  for two bands

Table 7.3C.0.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n78-n80 | n78 | 0.5 |
| SUL_n78-n81 | n78 | 0.5 |
| SUL_n78-n82 | n78 | 0.5 |
| SUL_n78-n83 | n78 | 0.5 |
| SUL_n78-n84 | n78 | 0.5 |
| SUL_n78-n86 | n78 | 0.5 |

7.3C.0.3.2.2	ΔRIB,c for three bands

Table 7.3C.0.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1_SUL_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n28_SUL_n41-n83 | n28 | 0.2 |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3C.2 and 7.3C.3.

<End of modified section 1>

<Start of modified section 2>

7.3C.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with parameters specified in Tables 7.3.2.3-1 and Tables 7.3.2.3-2.

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands n78 and n79 specified in Table 7.3.2.5-1 and 7.3.2.5-2 (for 4 Rx ports) shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.4.1-3 and 7.3.2.5-2 (for 4 Rx ports) with exceptions listed in clause 7.3C.2.5.1.

7.3C.2.5.1	Reference sensitivity exceptions due to harmonic issue

For SUL operation with DL band listed in Table 7.3C.2.3-2 with supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2.3-1, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in Table 7.3C.2.5.1-1 due to harmonic exceptions.

Table 7.3C.2.5.1-1: Reference sensitivity for SUL operation (exceptions due to harmonic issue)

| SUL band | DL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kHz | dBm | dBm | dBm | dBm | dBm | dBm | dBm | dBm | dBm | dBm | dBm | dBm |
| n80 | n781,2 | 15 |  | -70.9 +TT | -70.9 +TT | -70.8 +TT |  |  | -70.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -71.2 +TT | -71.0 +TT | -70.9 +TT |  |  | -70.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -71.6 +TT | -71.3 +TT | -71.2 +TT |  |  | -71.0 +TT |  |  |  |  |  |
| n80 | n783 | 15 |  | -93.7 +TT | -92.2 +TT | -91.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.0 +TT | -92.3 +TT | -91.5 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.4 +TT | -92.6 +TT | -91.8 |  |  |  |  |  |  |  |  |
| n82 | n783 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -83.6 +TT |  |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0 +TT | -84.9 +TT |  |  | -83.7 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.7 +TT | -85.3 +TT | -85.1 +TT |  |  | -83.9 +TT |  |  |  |  |  |
| n81 | n784 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -84.5 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0 +TT | -84.9 +TT |  |  | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT |  | -84.2 +TT |
|  |  | 60 |  | -85.7 +TT | -85.3 +TT | -85.1 +TT |  |  | -84.8 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT |  | -84.3 +TT |
| n81 | n785 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT |  |  | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| n86 | n786 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT |  |  |  |  |  |
| n86 | n787 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| n81 | n798 | 15 |  |  |  |  |  |  | -82.8 +TT | -82.4 +TT |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT |  | -84.9 +TT |
|  |  | 60 |  |  |  |  |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT |  | -85.0 +TT |
| NOTE 1: 1 indicates requirement for test configuration specified by Table [TBD] NOTE 2: 2 indicates requirement for test configuration specified by Table [TBD] NOTE 3: 3 indicates requirement for test configuration specified by Table [TBD] NOTE 4: 4 indicates requirement for test configuration specified by Table [TBD] NOTE 5: 5 indicates requirement for test configuration specified by Table [TBD] NOTE 6: 6 indicates requirement for test configuration specified by Table [TBD] NOTE 7: 7 indicates requirement for test configuration specified by Table [TBD] NOTE 8: 8 indicates requirement for test configuration specified by Table [TBD] NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3C.2.5.1-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2.5.1-2: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

For the UE which supports SUL band combination, the minimum requirement for reference sensitivity in Table 7.3C.2.3-1 shall be increased by the amount given in ΔRIB,c defined in subclause 7.3C.0.3.

7.3C.3	Reference sensitivity power level for SUL (3CC)

Editor’s Note:

- No test points defined for Reference sensitivity power level testing for SUL with DL CA. This test case is covered by 7.3.2 and 7.3C.2.

7.3C.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL and 2 DL CA operation and conditions of low signal level, ideal propagation and no added noise.

7.3C.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports SUL operation on the SUL bands with downlink 2CA.

7.3C.3.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 7.3C.0.

7.3C.3.4	Test description

NOTE:	No testing neet to be performed since the testing has been covered in test case 7.3.2 and 7.3C.2.

For band combination CA_nX_SUL_nY-nZ, test the REFSENS of SUL configuration or NR band as listed in table 7.3C.3.4-1.

Table 7.3C.3.4-1 Test band combinations and configuration

| Band configuration | Verifying REFSENS of SUL configrations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1_SUL_n78-n84 | SUL_n78-n84 | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n28_SUL_n41-n83 | SUL_n41-n83 | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n28 | 7.3.2 | Table 7.3.2.4.1-1 |

7.3C.3.5	Test requirement

Same test requirement as clause 7.3.2 and 7.3C.2 for each band or band combinations listed in table 7.3C.3.4-1

<End of modified section 2>