#3GPP TSG RAN Meeting #91-e						RP-210280

Electronic Meeting, March 16 – 26, 2021

______________________________________________________________________Agenda item: 9.7.3

Source: LG Electronics

Title: Discussion on the WI scope of NR sidelink enhancement

Document for: Discussion

- Introduction

NR sidelink enhancement WI has an objective for the latency and reliability enhancement for mode 2 with a study phase by this RAN meeting [1]:

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89.

RAN1 has reached the following conclusion which is included in the LS [2] together with detailed observations made during the study phase:

Conclusion: - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature. - The detailed observations can be found in the attachment of the LS

This contribution discusses how the WGs need to proceed for this objective.

- Discussion

As RAN1 concluded that the inter-UE coordination in Mode 2 is feasible and beneficial, the condition to enter the normative work (i.e., the part “specify the identified solution(s) if deemed feasible and beneficial”) has been met. So RAN1 and RAN2 need to start specifying the inter-UE coordination based on the conclusion and observation WGs have drawn so far. For example, RAN1 concluded that there are three categories for the inter-UE coordination as mentioned in [2], and WGs can further work to specify their details, not precluding the possibility of supporting all the categories or down-selecting some of them.

During the WG discussions, it was commented that there can be some solutions that can improve the reliability and latency in mode 2 but not belong to the inter-UE coordination. WGs did not reach consensus on the feasibility and benefit of such solutions yet and the study phase of the concerned objective is now finished. Considering solutions other than the inter-UE coordination would require an additional extension of the study phase, which is undesirable in view of the WI completion, it is recommended focus on the solutions included in the RAN1 conclusion, i.e., the inter-UE coordination, during the remaining Rel-17 timeframe.

- Proposal

In handing the objective for the solutions in mode 2 for enhanced reliability and reduced latency, it is proposed that WGs focus on specifying the inter-UE coordination based on the WG conclusions made so far.

References

- RP-202846, “WID revision: NR sidelink enhancement,” LG Electronics.

- RP-210032, “LS on Mode 2 enhancements in NR sidelink,” RAN WG1.