3GPP TSG-RAN Meeting #91-e		                                                        RP-210439

Electronic Meeting, March 22–26 2021

# Title:	Ka-band proposed as exemplary band for NR-NTN in Rel-17

# Source:	Hughes, Inmarsat, Thales, Panasonic, Fraunhofer, ESA, Intelsat

# Type:	Discussion

# Document for:	Decision

# Agenda Item:	9.7.6

# Release	Rel-17

# Introduction

As identified during the study phase, satellite networks operating in the so called Ka band (20, 30 GHz) allocated to satellite services will benefit from the use of the New Radio protocol to provide 5G eMBB and backhaul services via “VSAT” type terminals including roof top and moving platform mounted (ESIM) devices and to support seamless integration with the 5G system.

As in the WF R4-2103877 [2] and as recommendation in session chair note – this discussion need to be handled in RAN#91-e. The GTW Agreements “It’s FFS whether Ka bands can be introduced in the Rel-17 NTN WD as exemplary band with FR2 usage assumption pending on RAN-P decision.

In this paper, we propose that satellite Ka band to be considered as an exemplary for FR2 and FR2-like band for the NR-NTN-solutions Work Item in line with the study phase outcome reflected in TR 38.821 and propose a way on how to handle this band that falls across FR2 lower boundary.

# Discussion

## 2.1 Regulatory aspect

The following bands are commonly referred to as “Ka band” and identified by ITU Radio Regulation for use by Geo-stationary Orbit (GEO) and Non-Geostationary Orbit (NGSO) based satellite services on a worldwide basis, historically denominated as “FSS” and “MSS”. The duplex mode for all satellite bands is FDD and therefore the bands are distinctly assigned to UL and DL ranges. These are allocated by ITU and are subject to the provisions of ITU Radio Regulation Article 5 of the ITU Radio Regulation. These frequencies will henceforth be collectively referred to as “satellite Ka band” throughout the rest of this document.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space-earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth-space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

Table 2.1-1: “Ka band” spectrum for GEO based Satellite services

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space-earth) | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth-space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

Table 2.1-2: “Ka band” spectrum for NGSO Satellite services

Observation 1: For many years, the entirety of the “satellite Ka band” frequencies in both FSS and MSS denominated spectrum have been widely used to provide both fixed and mobile connectivity services to land-, sea- and air-based platforms (ESIMs), according to ITU-R and regional regulations.

The provision of mobility services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum and regulated by ITU-R regulations and recently reinforced by WRC Resolutions 156 [10], 169 [11] and 173 [12].

Proposal 1: For NTN coexistence study purpose, RAN should consider all the bands collectively referred to as “satellite Ka band”, in both “FSS” and “MSS” denomination, since the provision of mobility services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum and regulated by ITU-R regulations and recently reinforced by WRC Resolutions 156 [10], 169 [11] and 173 [12].

Based on the current 3GPP definition of FR1 and FR2 range, the UL portion of Ka band is well within the FR2 range, whereas the DL portion falls outside of the FR1 and FR2 ranges, as shown in the diagram below.

Figure 1. Illustration of NTN Ka frequency band

It should however be noted that the current 3GPP FR1 and FR2 range definitions are exclusively based on the frequency bands currently defined in 3GPP for TN services.

Observation 2: For satellite Ka band with FDD mode, the uplink falls entirely in FR2 while the downlink falls in between FR1 & FR2.

Observation 3: The FR1 and FR2 boundaries as currently defined in 3GPP are based on frequency band allocations that are currently valid for Terrestrial Networks.

Proposal 2: When treating Non-Terrestrial Networks, the FR1/FR2 boundaries may need to be adapted in line with the frequency band allocations that are valid for satellite communications.

Prior studies have been conducted within 3GPP on frequency bands outside of the FR1 and FR2 defined ranges, such as in the 7-24 GHz range [7].  For analysis purpose, the study further splits the 7-24 GHz range provisionally into 3 sub-ranges.  Satellite Ka band DL would fall entirely in the “sub-range 3”, which encompasses the 16-24 GHz subset. The study report observes that “sub-range 3” exhibits RF characteristics closer to FR2, and that UE transceiver architectures tend to benefit more from the RF+IF architecture. Current state-of-the-art VSAT and ESIM UEs used in the satellite Ka band range largely implement a similar architecture.

Observation 4: “3GPP TR 38.820: NR; 7-24 GHz frequency range V16.0.0” observes that frequencies above 16 GHz exhibit characteristics similar to FR2 range frequencies.

Proposal 3: In line with early analysis from “3GPP TR 38.820: NR; 7-24 GHz frequency range”, The DL portion of Ka frequency bands allocated to satellite services can be treated as FR2 band, in the scope of RAN specification work, particularly for the identification of RF requirements.

Proposal 4: RAN to consider the entirety of the Ka band frequency range allocated to satellite services as FR2 when defining an exemplary FR2 band for NR-NTN solutions.

NOTE: By addressing Ka band as exemplary band, other millimeter bands (e.g. Ku band, Q/V band) allocated to satellite services could leverage the RAN work with minimal incremental impact.

Proposal 13: Other bands allocated to satellite communications above 7 GHz (i.e. Ku band, Q/V band) should benefit from similar treatment and considered as FR2, similar to Ka band, when proposed for dedicated Work Item.

## 2.2 Rational Ka-Band fixed and mobile Market

Ka band has been used for many years to provide both fixed and mobile broadband services to VSAT type UEs.  As highlighted in TR 38.821 [4] in Table 6.1.1.1-3, VSAT refers to “fixed or mounted” devices with external antenna fixed or on-board moving platform.

Using these types of UEs, satellites services in Ka-band provide connectivity services to end users in areas beyond terrestrial network coverage. Today, broadband satellite operators provide broadband connectivity to end users at their homes and businesses.

In addition, machine-type broadband connectivity over satellite (such as ATMs, solar farms, etc.) has been provided for years by several satellite communications operators also in the higher frequency bands such as Ka band.

There is a thriving and growing market for satellite broadband connectivity to mobile platforms, also known as ESOMP (Earth Station on Moving Platform) or ESIMs (Earth Stations In Motion), mounted on ships, aircraft and land vehicles. For example, despite the impact of the COVID-19 pandemic, the commercial aviation connectivity market alone is projected to grow from USD 2.5 billion in 2020 to at least USD 10.5 billion in 2027, and the number of connected commercial airliners will grow to more than 23,100 by 2025 [13].  This is not accounting for private jets and other aviation verticals.

Similarly, the connected ship market, which largely relies on satellite connectivity, is projected to grow to USD 9.1 billion by 2027, in the large cargo and defense vessels segments alone [14].

The “VSAT” type terminals density for these kinds of networks is expected to be of the order of 1 “VSAT” type terminal per-km2 in “Rural”, “Urban Macro” and “Dense Urban” areas.

In addition, there are emerging and future use cases where fixed and mobile 5G NTN services in Ka-band can extend and complement the current 5G terrestrial services, as well as future 5G NTN in lower bands. Examples of 5G fixed and mobile use cases that are well served via satellite broadband in Ka band include:

- 5G broadband connectivity direct to the home, business or government;

- Expanded use of machine type communication via satellite for applications ranging from smart agriculture to environmental protection and transportation and connectivity to local area IoT services;

- Mobile broadband services onto moving platforms such as small airplanes, UAVs, small boats, RVs, trucks, cars and fast trains;

- Multi-connectivity for fixed and mobile, hybrid satellite-terrestrial connectivity, network resilience, edge network delivery, trunking, direct-to-node broadcast, local area public safety.

Most of the above services have been studied as part of TR 38.811 [15].  Moreover, the specification of NR-NTN solutions for VSAT and ESIM services in FR2 bands is part of the key mission of the NR-NTN WI, and in line with the outcome of TR 38.821 [4].

Observation 5: A significant portion of the NR-NTN use cases identified in TR 38.811 [15] are relevant to satellite Ka band and other FR2 and FR2-like frequency bands. Addressing satellite Ka band as exemplary band in NR-NTN-solutions WI during Rel-17 will support the seamless integration of high capacity satellite networks for connectivity of fixed and mobile services.

Proposal 5: For its adjacent channel coexistence studies, satellite Ka band should consider a density of fixed and mobile “VSAT” type UEs (i.e. including ESIMs) of maximum 1 UE per-km2 in “Rural”, “Urban Macro” and “Urban Dense” areas.

## 2.3 Target UE Types

In the revised WID “Solutions for NR to support non-terrestrial networks (NTN)” in RP-202908 [3], the following principle has been added:

“VSAT” devices with external antenna (including fixed and moving platform mounted devices) at least in FR2 are supported for the RAN1-3 specifications. “VSAT” characteristics in TR 38.821 can be assumed for the RAN1-3 specifications.

As described in RP-202732 [6], it shall be understood that moving platform mounted “VSAT” devices refer to ESOMP (Earth Station On Moving Platform) or ESIM (Earth Station In Motion) [5]. A moving platform mounted UE has been referred to as an Earth Station on Moving Platform (ESOMP); however, recently ITU has changed the definition to Earth Station in Motion (ESIM). These devices are typically mounted on vessels, airplanes, trains or fast vehicles. The UE can also act as a relay to other UEs on board moving platforms.

ESIM (Earth stations in motion) [5] address a complex challenge – how to provide reliable and high-bandwidth connectivity to what are literally moving targets. They provide broadband communications, including Internet connectivity, to a variety of platforms in motion. As per ITU definition there are currently three types of ESIM: ESIM on aircraft (aeronautical ESIM), ESIM on ships (maritime ESIM), and ESIM on land vehicles (land ESIM). ESIM on land vehicles was also referred to as to Vehicle-Mounted Earth Stations (VMES). The VMES may transmit and receive data when the vehicle is in motion and when the vehicle is stationary.

In order to define the RF requirements for “VSAT” devices operating in Ka band, 3GPP RAN may consider ETSI harmonized standard as in [8] and [9].

Observation 6: For Ka band, both FSS and MSS denominated spectrum allocations are used and regulated to provide mobility services to ESIM (Earth Station in Motion) UEs mounted on moving platforms (Vessels, vehicles, Trains, airplanes, etc.).

Proposal 6: For Ka band, “VSAT” UE including fixed/moving platform mounted terminals are considered as baseline. The RF characteristics of “VSAT” UE in Table 6.1.1.1-3 in 3GPP TR 38.821 shall be assumed in the Rel-17 WI NR-NTN-solutions.

## 2.4 Co-existence scenarios

#### 2.4.1	General

Three high level coexistence scenarios relevant for FR2 are envisaged as:

- TN to NTN

- NTN to TN

- NTN - NTN

However, NTN-NTN coexistence scenarios are largely well-documented by ITU-R radio regulations.  Furthermore, NTN-NTN coexistence scenarios involve almost exclusively co-channel coexistence, so they should be deprioritised.

Coexistence scenarios TN to NTN and NTN to TN in adjacent bands are further represented in Figure 2.

Figure 2. Different interference scenarios in Ka adjacent bands between NTN 5G NR and TN 5G NR

As illustrated in Figure 2, the interference scenarios in adjacent bands are identified as follows:

- i1: DL TN 5G NR to UL NTN 5G NR

- i2: UL NTN 5G NR to DL TN 5G NR

- i3: UL NTN 5G NR to UL TN 5G NR

- (i4): UL TN 5G NR to UL NTN 5G NR

Please also note that interference types i1 and i2 in adjacent bands are UL-DL (since NTN networks are FDD while TN networks are TDD), while interference types i3 and i4 in adjacent bands are UL-UL. Moreover, interference type i4 can be further down-scoped from coexistence cases.

Observation 7:  It can be noted that for FR2, coexistence scenarios will be much simpler than for FR1, because the NTN-TN scenarios for FR2 are limited only to i1 (DL TN in UL NTN), i2 (UL NTN in DL TN), i3 (UL NTN in UL TN).

Observation 8: It should be also noted that in FR2, all NTN UEs and satellites use highly directional antennas, further simplifying all coexistence scenarios, and that NTN-NTN coexistence scenarios are almost exclusively based on co-channel coexistence and spatial separation.

Proposal 7: FR2 coexistence studies could be further down-scoped by deprioritizing NTN-NTN coexistence scenarios.

#### 2.4.2 Ka-band NTN UL coexistence with terrestrial UL and DL in adjacent frequency bands

As represented in both Figure 2 and Figure 3, the Ka-band NTN coexistence involves coexistence with TN n258 (24.25 – 27.5 GHz), as follows:

- NTN to TN: (NTN UE UL to TN BS DL) & (NTN UE UL to TN UE UL)

- TN to NTN: (TN BS DL to NTN UE UL)

Figure 3. Coexistence in adjacent bands of NTN Ka UL with TN n258

Observation 9: TN Deployments in FR2 are currently limited to Urban scenarios with small cells, furthermore, Rural scenario is usually not considered for FR2.

Proposal 8: Rural scenario can be removed in FR2 NTN-TN coexistence analysis

Observation 10: Indoor TN deployments in FR2 are unlikely to be affected by NTN deployment due to path loss and building attenuation.

Proposal 9: Indoor scenario can be removed or at least deprioritised in FR2 NTN-TN coexistence analysis.

Observation 11:  RAN WF from 98e selected Rural, Urban Macro, Dense Urban and Indoor scenarios for NTN-TN coexistence in FR1.  Since in FR2 Rural and Indoor become not relevant, this allows down-selection to Urban Macro and Dense Urban for FR2 NTN-TN coexistence analysis.

Proposal 10: RAN can limit scope to scenarios with Urban Macro and Dense Urban TN deployment for NTN-TN coexistence analysis in FR2.

#### 2.4.2 Ka-band NTN DL coexistence with TN UL and DL in adjacent frequency bands

The satellite Ka band DL (17.3 – 20.2 GHz) range does not overlap, nor is adjacent to any terrestrial band. As such it is unnecessary to perform coexistence analysis between NTN DL and TN.

Observation 12:  There are no 3GPP TN bands specified in or adjacent to the satellite Ka DL (17.3 – 20.2 GHz) range, and thus no 3GPP TN deployments.

ETSI define RF specifications for satellite transmissions in line with ITU-R regulations and appropriate regional resolutions.  These specifications can be used as a starting point by RAN when defining NTN RF requirements.

Proposal 11: For satellite Ka band NTN DL (17.3 – 20.2 GHz), RAN does not need to undertake any coexistence study with TN FR2.

Proposal 12: For the purpose of specifying NTN BS (i.e. satellite TX) RF requirements in satellite Ka band DL (17.3 – 20.2 GHz), RAN can use ITU-R, ETSI and appropriate regional regulatory sources as reference.

# Conclusions

In this contribution we are seeking RAN to consider the following proposals:

Proposal 1: For NTN coexistence study purpose, RAN should consider all the bands collectively referred to as “satellite Ka band”, in both “FSS” and “MSS” denomination, since the provision of mobility services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum and regulated by ITU-R regulations and recently reinforced by WRC Resolutions 156, 169 and 173.

Proposal 2: When treating Non-Terrestrial Networks, the FR1/FR2 boundaries may need to be adapted in line with the frequency band allocations that are valid for satellite communications.

Proposal 3: In line with early analysis from “3GPP TR 38.820: NR; 7-24 GHz frequency range”, The DL portion of Ka frequency bands allocated to satellite services can be treated as FR2 band, in the scope of RAN specification work, particularly for the identification of RF requirements.

Proposal 4: RAN to consider the entirety of the Ka band frequency range allocated to satellite services as FR2 when defining an exemplary FR2 band for NR-NTN solutions.

Proposal 5: For its adjacent channel coexistence studies, satellite Ka band should consider a density of fixed and mobile “VSAT” type UEs (i.e. including ESIMs) of the order of 1 UE per-km2 in “Rural”, “Urban Macro” and “Urban Dense” areas.

Proposal 6: For Ka band, “VSAT” UE including fixed/moving platform mounted terminals are considered as baseline. The RF characteristics of “VSAT” UE in Table 6.1.1.1-3 in 3GPP TR 38.821 shall be assumed in the Rel-17 WI NR-NTN-solutions.

Proposal 7: FR2 coexistence studies could be further down-scoped by deprioritizing NTN-NTN coexistence scenarios.

Proposal 8: Rural scenario can be removed in FR2 NTN-TN coexistence analysis

Proposal 9: Indoor scenario can be removed or at least deprioritised in FR2 NTN-TN coexistence analysis.

Proposal 10: RAN can limit scope to scenarios with Urban Macro and Dense Urban TN deployment for NTN-TN coexistence analysis in FR2.

Proposal 11: For satellite Ka band NTN DL (17.3 – 20.2 GHz), RAN doesn’t need to undertake any coexistence study with TN FR2.

Proposal 12: For the purpose of specifying NTN BS (i.e. satellite TX) RF requirements in satellite Ka band DL (17.3 – 20.2 GHz), RAN can use ITU-R, ETSI and appropriate regional regulatory sources as reference.

Proposal 13: Other bands allocated to satellite communications above 7 GHz (i.e. Ku band, Q/V band) should benefit from similar treatment and considered as FR2, similar to Ka band, when proposed for dedicated Work Item.

# Reference

- R4-2006894, “Possible Bands for NR Satellite Networks,” Thales, Hughes Network Systems ltd., Intelsat, WG4 Meeting #95-e, June 2020

- R4-2103877 – WF RAN 98-e

- RP-202908, “Solutions for NR to support non-terrestrial networks (NTN)” WID Revised

- 3GPP TR 38.821 “Solutions for NR to support non-terrestrial networks (NTN), (Release 16)”

- https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

- RP-202732, “About fixed and moving platform mounted device for NTN”

- 3GPP TR 38.820, “7 - 24 GHz frequency range”, V16.0.0, June 2020

- ETSI EN 303 978 “Satellite Earth Stations and Systems (SES); Harmonized EN for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the R&TTE Directive”

- ETSI EN 303 979 “Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”

- WRC Resolution 156 (WRC-15); https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0047PDFE.pdf

- WRC Resolution 169 (WRC-19); https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0056PDFE.pdf

- WRC Resolution 173 (WRC-19); https://www.itu.int/dms_pub/itu-r/oth/0c/0a/R0C0A00000D0016PDFE.pdf

- https://www.fortunebusinessinsights.com/industry-reports/connected-aircraft-market-101954

- https://www.prnewswire.com/news-releases/connected-ships-market-size-worth-9-1-billion-by-2027--cagr-5-4-grand-view-research-inc-301047337.html

- TR 38.811

-END-