3GPP TSG RAN meeting #91e										RP- 210515

Electronic Meeting, March 16 - 26, 2021

Status Report to TSG

Agenda item:		9.7.15

| WI / SI Name | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_OTA |  |  |  |  |
| Unique ID | 880078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201800 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Mar/2022 | Performance part: Sep/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 50% | Performance Part: 10% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Zhu, Siting Xing, Jinqiang, |
|  | Company | CAICT, OPPO |
|  | Email | zhusiting@caict.ac.cn xingjinqiang@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #98-e (Jan 2021, Electronic Meeting)

- General

- WF on NR MIMO OTA was approved [2]

- WF on TP work split and Work Plan was approved [3]

- WF on FR2 MIMO OTA simulation assumptions was approved [4]

- TS 38.151 v0.2.0 NR MIMO OTA requirement was agreed [5]

- TP to TS 38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements was approved [6]

- TP to TS 38.151 v0.1.0 on FR2 test system for requirements was approved [7]

- TP to TS 38.151 v0.1.0 on FR2 Channel model and RMC was approved [8]

•	Testing methodologies

- System implementation of FR2 3D-MPAC system

    - Keep the probe locations the same among system implementations at this time, but enhanced implementation or solution can be considered by RAN4 in future.

    - Further study how to address the FR2 blocking issue

- Conclude the agreed 36 test points for FR2 MIMO OTA testing

- Usage of gNB beams for FR1 MIMO OTA testing

    - Beam1_H, Beam1_v, Beam2_H, Beam2_v are mapped to CE input ports 1, 2, 3 and 4 in 4x4 case and Beam1_H, Beam1_v are mapped to CE input ports 1 and 2 in 2x2 case. 2 strongest beams selected for 4X4 and 1 strongest selected for 2X2.

    - The beams are not combined in the actual test. The gNB emulator ports are connected to each CE input port, where each port corresponds a channel model realization with one polarization of one beam.

•	System validation

- For spatial correlation validation, only Vertical validation is required.

- gNB Beams Usage Criteria for FR1 MIMO OTA Channel Model Validation

    - Option 1: Beam specific: The two strongest beams are measured separately applied for all channel model validation factors (agreed as baseline, further define validation pass/fail limit based on this approach)

    - Option 2: Combined beams: The two strongest beams are combined at the input of the channel emulator for validation of spatial correlation. The PDP validation is done separately for each beam in this case.

- For FR1 and FR2 channel model validation, adopt 40dB threshold for cluster power.

    - Further study the impact of 40dB threshold on UE throughput performance with CDL-A channel model for FR2

- Conclude the existing PSP validation procedure with 5cm radius as baseline.

- Channel model validation limit

    - Further discuss and define the reference before RAN4 specify the actual values.

•	Test Parameters for FR1 performance requirements

- RAN4 confirms that the CDL-C UMa channel model is adopted to define FR1 4x4 MIMO OTA requirements

- For FR1 2x2 MIMO OTA requirements, the channel model is FFS. Candidate options are: CDL-A UMi, CDL-C UMa, CDL-C UMi. Other options are not precluded.

- PRS-EPRE-MAX for band frequency <3GHz, 40MHz bandwidth

    - [-77dBm/30kHz] (starting point)

Note: This value is pending on further verification and confirmation from TE vendors/Test labs for the feasibility. Further discuss and revise the value is not excluded.

- PRS-EPRE-MAX for band frequency >3GHz, 10MHz and 40MHz bandwidth needs further study.

    - When define the maximum downlink power, the path loss induced by different channel models need to be considered.

- Minimum number of slots for FR1 MIMO OTA testing

    - For bands >1GHz: 20k for 30kHz SCS, 10k for 15kHz SCS;

    - For bands <1GHz, [20k] for 15kHz SCS;

    - Note: The above number of slots are the recommendation value for both FR1 MIMO OTA test lab alignments and FR1 MIMO OTA UE performance requirements introduction

    - Test labs are encouraged to bring more analysis/measurement results with different number of slots for comparison purpose.

•	NR MIMO OTA performance requirements

- Figure of Merit for FR1 MIMO OTA performance requirements

    - Restriction of Pmode at 70%TP for 40MHz CHBW.

      - For 40MHz CHBW, the 11 of total 12 PMODE should reach 70%TP as a starting point based on the current assumption of PRS-EPRE-MAX value

    - Additional restriction of Pmode for 10MHz and 40MHz CHBW.

      - Adopt Pmode at 90%TP as an additional restriction.

      - 90% TP, [TBD] of total 12 as starting point

      - Companies are encouraged to bring more analysis and measurement data.

- Figure of Merit for FR2 MIMO OTA performance requirements

    - How to treat the orientations that cannot reach 70% TP

      - how to treat the missing points can not reach 70%TP for MASC calculation is FFS.

      - additional criterion on how to treat the number of missing points should be further studied.

2.4.2	Remaining Open issues

- Further study the proper Channel model for FR1 2x2 MIMO OTA requirements

- Pass/fail limit and reference figure of channel model validation.

  - Channel model validation results for FR2 channel models are encouraged.

- Further discuss testing parameters for requirements (e.g. Maximum downlink power for bands>3GHz)

- Specify the FR1/FR2 MIMO OTA requirements

  - Further discuss the Figure of Merit for FR1 and FR2

  - Measurement results of FR1 or FR2 UEs are encouraged for discussion.

  - FR2 simulation results of UE performance are encouraged.

- Analysis on MU evaluation of FR2 blocking issue.

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

N/A

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#98-e

- R4-2103946	Email discussion summary for [98e][329] NR_MIMO_OTA, Moderator(CAICT)

- R4-2103913	WF on NR MIMO OTA, vivo, CAICT

- R4-2103914	WF on TP work split and Work Plan, vivo, CAICT, OPPO

- R4-2103915	WF on FR2 MIMO OTA simulation assumptions, Huawei, HiSilicon

- R4-2101822	TS 38.151 v0.2.0 NR MIMO OTA requirements, vivo

- R4-2103971	TP to TS 38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements, vivo, CAICT

- R4-2103970	TP to TS 38.151 v0.1.0 on FR2 test system for requirements, vivo, CAICT

- R4-2103969	TP to TS 38.151 v0.1.0 on FR2 Channel model and RMC, vivo, CAICT

- R4-2101827	Discussions on Channel models mapping for FR1 MIMO OTA requirement, vivo

- R4-2101941	FR1 MIMO OTA measurement results and channel model mapping, CAICT, Keysight, ETS-Lindgren

- R4-2101758	Consideration on how to treat the missing orientations for FR2, OPPO

- R4-2102497	Discussion on FR2 MIMO OTA performance requirements, Qualcomm Incorporated

- R4-2102719	Simulation assumption summary for NR FR2 MIMO OTA, Huawei, HiSilicon

- R4-2100892	Discussion on FR1 test parameters and Figure of Merit, Samsung

- R4-2101826	Discussion on testing parameters for NR MIMO OTA requirement, vivo

- R4-2102729	Consideration on FR2 MIMO OTA UE requirement, Huawei, HiSilicon

- R4-2101757	Consideration on 3D-MPAC probe locations configuration for FR2 MIMO OTA, OPPO

- R4-2102400	Analysis on number of test points vs uncertainty of FR2 MIMO OTA requirements, Huawei, HiSilicon

- R4-2100845	Channel Model Validation Bounds, Spirent Communications

- R4-2102613	gNB Beams Usage Criteria for NR FR1 MIMO OTA Channel Model Validation, Keysight Technologies UK Ltd, China Mobile

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template