3GPP TSG-RAN Meeting #91e	RP-210635

# Electronic meeting, 21th March - 26th March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.807 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Regulatory update for the 52.6 to 71GHz frequency range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RP |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_beyond_52GHz |  |  |  |  |  | Date: |  |  | 14-01-2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TR 38.807 was instantiated under the RAN SI to collect and capture the corresponding regulatory related information for the frequency range from 52GHz to 116GHz. The corresponding RAN SI was formally completed in December 2019, but after that more related regulatory related decisions were made for the frequency range of 52-71GHz. To facilitate further normative work in RAN WGs, the corresponding updates are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General corrections in section 4.1 regarding outcome of WRC19. General corrections in section 4.2.1 on the global overview of licensing modes in 52-71GHz frequency range. Updated technical parameters of 52-71GHz in ITU region 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 3GPP TR 38.807 will contain old and outdated regulatory information for the 52-71GHz frequency range. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2.1, 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TD RP-181435: "New SID: Study on NR beyond 52.6 GHz".

[3]	3GPP TR 38.805: "Study on New Radio access technology; 60 GHz unlicensed spectrum".

[4]	3GPP TR 38.913: "Study on New Radio access technology; Next Generation Access Technologies".

[5]	WRC-15 Resolution 238: "Studies on frequency-related matters for International Mobile Telecommunications identification including possible additional allocations to the mobile services on a primary basis in portion(s) of the frequency range between 24.25 and 86 GHz for the future development of International Mobile Telecommunications for 2020 and beyond".

[6]	International Telecommunication Union, "CPM Report to WRC-19: Chapter 2, agenda item 1.13, Method C2, "	https://www.itu.int/md/R15-CPM19.02-R-0001/en

[7]	APG19-3/INP-35, Australia, "PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5 AND 9.1.8)", APG19-3, March, 2018, Australia

[8]	APG19-3/INP-87, China, "PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5, 9.1.8) ", APG19-3, March, 2018, Australia

[9]	Independent Communications Authority of South Africa. Standard procedures and guideline regarding the use of radio frequency spectrum in the E-band (71 – 76 GHz paired with 81 – 86 GHz). April 28, 2017.

[10]	Independent Communications Authority of South Africa. National Radio Frequency Plan 2018. May 25, 2018.

[11]	Independent Communications Authority of South Africa. Electronic Communications Act (781/2016): Amendment to the Radio Frequency Spectrum Regulations 2015. No. 404436, November 22, 2016.

[12]	Independent Communications Authority of South Africa. Radio Frequency Spectrum Regulations (279/2015): Amendment to the Radio Frequency Spectrum Regulations, 2015. November 22, 2016.

[13]	Canada Spectrum Management and Telecommunications. Canadian Table of Frequency Allocations 2018 Edition. April 2018.

[14]	Canada Spectrum Management and Telecommunications. Technical Requirements for Fixed Line-of-Sight Radio Systems Operating in the Bands 71-76 GHz and 81-86 GHz. May 2017.

[15]	MIIT, "Notice on wireless technical applications with very low transmission power (short distance) in 60GHz. 2006-082" (关于60GHz 频段微功率 (短距离) 无线电技术应用有关问题的通知 2006-082) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176263/part/3176264.pdf

[16]	MIIT, "Technical requirement of wireless devices with very low transmission power (short distance 2005-423)." (微功率 (短距离) 无线电设备的技术要求 2005-043) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176241/part/3176242.pdf

[17]	ARIB STD-B43 v2.1, "PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION," Jan. 2018.

[18]	ARIB STD-T69 v4.0, "LOW POWER DATA COMMUNICATION SYSTEM/MILLIMETER-WAVE VIDEO TRANSMISSION EQUIPMENT," Sept. 2016.

[19]	ARIB STD-T117 v1.0, "LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS," Sept. 2016.

[20]	ARIB STD-T48 v2.2, "MILLIMETER-WAVE RADAR EQUIPMENT FOR SPECIFIED LOW POWER RADIO STATION," Dec. 2015.

[21]	ARIB STD-T111 v1.1, "79 GHz BAND HIGH-RESOLUTION RADAR," Mar. 2017.

[22]	Republic of Korea Communications Agency. Republic of Korea Frequency Allocations Status. May 2018.

[23]	Republic of Korea National Radio Research Agency. National Radio Research Agency Publication 2014-12. July 02, 2014.

[24]	Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-3. July 02, 2014.

[25]	Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-4. July 02, 2014.

[26]	NFAP-18	"National Frequency Allocation Plan – 2018", Government of India Ministry of Communications.

[27]	IMDA (Infocomm Media Development Authority), IMDA TS SRD, April 2018. https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/ict-standards/telecommunication-standards/radio-comms/imdatssrd.pdf?la=en

[28]	ACMA (Australian Communications and Media Authority), "Radiocommunications (Low Interference Potential Devices) Class Licence 2015", 2018. https://www.legislation.gov.au/Details/F2018C00500

[29]	No. 2018-71 Ministry of Science and ICT notice

[30]	No. 2018-26 National Radio Research Agency notice

[31]	No. 2018-36 Ministry of Science and ICT notice

[32]	Ministry of Transportation and Communications, "TABLE OF RADIO FREQUENCY ALLOCATIONS OF THE REPUBLIC OF CHINA", Feb. 2017, http://www.motc.gov.tw/post/home.jsp?id=364&parentpath=0.

[33]	China, "People's republic of China Regulations on the Radio Frequency Allocation", The People's Posts and Telecommunications Press, 2010.

[34]	IMDA (Infocomm Media Development Authority), "Singapore Spectrum Allocation Chart", https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/frameworks-and-policies/spectrum-management-and-coordination/spectrumchart.pdf?la=en

[35]	ACMA, "Australian Radiofrequency Spectrum Plan 2017-including general information", https://www.acma.gov.au/theacma/australian-radiofrequency-spectrum-plan-spectrum-planning-acma

[36]	The Wireless Planning & Coordination (WPC) Wing of the Ministry of Communication & Information Technology, "License Exemption for SRD Device GSR 1047(E) dated 18/10/ 2018"

[37]	International Telecommunication Union, "ITU-R, The Radio Regulations," Edition of 2016, https://www.itu.int/pub/R-REG-RR (Edition of 2016)

[38]	3GPP TR 22.826: "Study on communication services for critical medical applications"

[39]	3GPP TR 33.853: "Technical report on key issues and potential solutions for Integrity protection of the User Plane"

[40]	CEPT ECC, "ERC Recommendation 12-12: Radio frequency channel, arrangement for Fixed Service Systems operating in the band 55.78 to 57.0 GHz," January 2015.

[41]	CEPT ECC, "THE EUROPEAN TABLE OF FREQUENCY ALLOCATIONS AND APPLICATIONS IN THE FREQUENCY RANGE 8.3 kHz to 3000 GHz (ECA TABLE)", March 2019.

[42]	CEPT ECC, "ERC Recommendation 70-03: Relating to the use of Short Range Devices (SRD)," June 2019.

[43]	CEPT ECC, "Recommendation 02-05: Unwanted Emissions," March 2012.

[44]	CEPT ECC, "ERC Recommendation 74-01: Unwanted emissions in the spurious domain," May 2019.

[45]	Official Journal of the European Union, " COMMISSION IMPLEMENTING DECISION (EU) 2017/1483 of August 2017 amending Decision 2006/771/EC on harmonisation of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC," August 2017.

[46]	CEPT ECC, "ECC RECOMMENDATION (05)02: USE OF THE 64-66 GHz FREQUENCY BAND FOR FIXED SERVICE," February 2009.

[47]	CEPT ECC, "ECC Decision (09)01: Harmonised use of the 63.72-65.88 GHz frequency band for Intelligent Transport Systems (ITS)," March 2009.

[48]	CEPT ECC, "ECC RECOMMENDATION (05)07: RADIO FREQUENCY CHANNEL ARRANGEMENTS FOR FIXED SERVICE SYSTEMS OPERATING IN THE BANDS 71-76 GHz AND 81-86 GHz," May 2013.

[49]	CEPT ECC, "ECC Decision (04)03: The frequency band 77-81 GHz to be designated for the use of Automotive Short Range Radars," March 2015.

[50]	CEPT ECC, "EC Decision 2004/545/EC: Commission Decision of 8 July 2004 on the harmonisation of radio spectrum in the 79 GHz range for the use of automotive short-range radar equipment in the Community (notified under document number C(2004) 2591)(Text with EEA relevance)," August 2004.

[51]	CEPT ECC, "ECC Recommendation (18)02: Radio frequency channel/block arrangements for Fixed Service systems operating in the bands 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz and 111.8-114.25 GHz," September 2018.

[52]	ETSI, " ETSI EN 302 217-2: Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1,3 GHz to 86 GHz; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU," June 2016.

[53]	CEPT ECC, "ECC RECOMMENDATION (05)07: RADIO FREQUENCY CHANNEL RRANGEMENTS FOR FIXED SERVICE SYSTEMS OPERATING IN THE BANDS 71-76 GHz AND 81-86 GHz," May 2013.

[54]	CEPT ECC, "ECC Report 80: ENHANCING HARMONISATION AND INTRODUCING FLEXIBILITY IN THE SPECTRUM REGULATORY FRAMEWORK," March 2006.

[55]	RSS-210, "Licence-Exempt Radio Apparatus: Category I Equipment", Issue 10, December 2019, URL: https://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf01320.html.

[56]	F2019C00681, "Radiocommunications (Low Interference Potential Devices) Class Licence 2015", August 2019, URL: https://www.legislation.gov.au/Details/F2019C00681

********** NEXT CHANGED SECTION **********

# 4	Operational Requirements

## 4.1	Overview of Global Spectrum Availability

In 2015, the international telecommunication union (ITU) proposed 11 millimetre-wave bands between 24 and 86 GHz to be studied towards WRC-19 for possible identification for IMT (see Table 4.1-1). The WRC-19 [5] concluded that the following frequency bands should have an IMT identification to enable 5G deployments: 24.25– 27.5 GHz, 37–43.5 GHz, 45.5–47 GHz, 47.2–48.2 GHz, 66–71 GHz.

Table 4.1-1 Bands studied under WRC-19 AI 1.13

| Candidate frequency bands which have allocations to the mobile service on a primary basis | Candidate frequency bands which required a co-primary mobile allocation |
| --- | --- |
| 24.25-27.5 GHz [Note1] | 31.8-33.4 GHz, |
| 37-40.5 GHz | 40.5-42.5 GHz |
| 42.5-43.5 GHz | 47-47.2 GHz |
| 45.5-47 GHz |  |
| 47.2-50.2 GHz |  |
| 50.4-52.6 GHz |  |
| 66-76 GHz |  |
| 81-86 GHz |  |
| NOTE 1: When conducting studies in the band 24.5-27.5 GHz, to take into account the need to ensure the protection of existing earth stations and the deployment of future receiving earth stations under the EESS (space-to-Earth) and SRS (space-to-Earth) allocation in the frequency band 25.5-27 GHz |  |

Table 4.1-2 provides information on the allocation within the frequency range 52.6 GHz to 116 GHz in ITU Radio Regulation [37]. The column with comments contains (a subset of) information on protection requirements for incumbent services. For the full details please refer to the Radio Regulations

Table 4.1-2: Frequency bands in the range 52.6 to 116 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 52.6-54.25 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 54.25-55.78 | No | No | EESS (passive) and SRS (passive) |
| 55.78-59 | Yes | Yes | EESS (passive) and SRS (passive) This band available for high-density applications in the fixed service, footnote 5.547 |
| 59-59.3 | Yes | Yes | EESS (passive) and SRS (passive) Radiolocation |
| 59.3-64 | Yes | Yes | Radiolocation |
| 64-65 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 65-66 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 66-71 | Yes | No | The frequency band 66-71 GHz is identified for use by administrations wishing to implement the terrestrial component of International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which this frequency band is allocated and does not establish priority in the Radio Regulations. |
| 71-76 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in clause 2.2. |
| 76-81 | No | No | Radiolocation |
| 81-86 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in clause 2.2. |
| 86-92 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 92-94 | Yes | Yes | Radiolocation |
| 94-94.1 | No | No | Radiolocation |
| 94.1-95 | Yes | Yes | Radiolocation |
| 95-100 | Yes | Yes | Radiolocation |
| 100-102 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 102-105 | Yes | Yes | N/A |
| 105-109.5 | Yes | Yes | SRS (passive) |
| 109.5-111.8 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 111.8-114.25 | Yes | Yes | SRS (passive) |
| 114.25-116 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |

Within the range 52.6 to 116 GHz, the frequency bands 66-76 GHz (including 66-71 and 71-76 GHz) and 81-86 GHz were studied under WRC-19 Agenda Item 1.13 for potential IMT identification. The frequency band 66-71 GHz is identified for use by administrations wishing to implement the terrestrial component of International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which this frequency band is allocated and does not establish priority in the Radio Regulations. Resolution 241 (WRC-19) applies.

For 71-76 GHz, studies were carried out for the FS, RLS and FSS (space-to-Earth) indicating that sharing with FS and FSS is feasible. However, additional limits of the IMT BS and UE unwanted emissions is needed to protect RLS in the adjacent frequency band 76-81 GHz.

For 81-86 GHz, studies were carried out for the FS, FSS (Earth-to-space), RAS (in band and adjacent band), EESS (passive) and RLS. Studies are not needed for the SRS (passive), as this service is dealing with sensors around other planets and no interference issue is expected. Studies were also not carried out for the MSS. The results of those studies indicate that sharing with FS, FSS and RAS (in band and adjacent band) is feasible. Notice that additional limits of the IMT BS and UE unwanted emissions would be needed to ensure protection of EESS (passive) in the adjacent frequency band 76-81 GHz and RLS in the adjacent frequency band 86-82 GHz. Table 4.1-3 and Table 4.1-4 depicts the limits of the IMT BS and UE maximum unwanted emissions levels according to the individual compatibility studies.

Table 4.1-3: Limits of unwanted emission levels from IMT BS and UE operating on 71-76 and 81-86 GHz frequency bands into the 76-81 GHz frequency band according to WRC-19 compatibility studies

| Limits of unwanted emissions into 76-81 GHz from IMT stations | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/] | [TBD/−33] |
| UE | [TBD/−20] | [TBD/−35] |

Table 4.1-4: Limits of unwanted emission levels from IMT BS and UE operating on 81-86 GHz into 86-92 GHz frequency band according to WRC-19 compatibility studies

| Limits of unwanted emissions into 86-92 GHz from IMT stations | dB(W/100 MHz) |
| --- | --- |
| BS | [TBD/-43.6/-49.9/-31.3/-20] |
| UE | [TBD/-43.5/-49.8/-31.2/-19.9] |

## 4.2	Country / Regional Specific Spectrum Availability and Spectrum Regulatory Requirements

### 4.2.1	Country / Regional Specific Licensing situation

Table 4.2-1 and 4.2-2 provide an overall summary of the current licensing situation for data communication (Mobile and Fixed, excluding radar-based telemetry) between frequencies 52.6GHz and 100GHz for various countries under ITU region 1, 2, and 3. Symbol 'U', and 'L', represent unlicensed spectrum and licensed spectrum (including light licensing and other licensing forms), respectively. Notice that Table 4.2-1 and 4.2-2 does not include information about the service allocated to each frequency band. It neither includes information about the frequency bands currently studied in AI 1.13 WRC-19 which are listed in Table 4.1-1. Also, it is worthwhile noticing that type of licensing for each frequency band is a national decision.

Table 4.2.1-1: Current Licensing situation for various countries between 52.6GHz and 71GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 |  | 54.25-55.78 |  | 55.78-56.9 |  | 56.9-57 | 57-58.2 |  | 58.2-59 |  | 59-59.3 |  | 59.3-64 | 64-65 |  |  |  | 65-66 |  |  | 66-71 |  |
| ITU Region 1 | Europe/CEPT |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ITU Region 2 | USA |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Canada |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Brazil |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mexico |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Korea |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Singapore |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.2.1-2: Current Licensing situation for various countries between 71GHz and 100GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 71-741) | 74-761) | 76-77 | 77-81 | 81-841) | 84-861) | 86-92 | 92-94 | 94-94.1 | 94.1-95 | 95-100 |
| ITU Region 1 | Europe/CEPT | L (Fixed) |  |  |  | L (Fixed) |  |  | L (Fixed) | L (Fixed) |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 2 | USA | L (Fixed /Mobile) |  |  |  | L (Fixed /Mobile) |  |  | L (Fixed /Mobile) |  | L (Fixed /Mobile) |  |
|  | Canada | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Brazil | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Mexico | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  |  |  |  |  |  |
|  | Japan | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Korea | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan | L(Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Singapore | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Candidate frequency band for IMT identification under WRC-19 AI 1.13 |  |  |  |  |  |  |  |  |  |  |  |  |

### 4.2.2	ITU Region 1

#### 4.2.2.1	Europe and CEPT

General information for usages in CEPT can be found in [41]. When it comes to Regulation about Short Range Devices additional information can be found in [42].  Unwanted emission limits can be found in [43] and [44]. The 57-71 GHz range, encompassing the 66-71 GHz range, which has been identified by the WRC-19 for IMT, is already subject to a permanent European Commission mandate to CEPT covering Short-Range Devices (SRD). ERC/REC 70-03 sets out the CEPT position on common spectrum which can be designated for Short Range Devices (SRD) applications and is a reference document to assist in preparing national spectrum regulations. The position set out in this Recommendation (ERC Recommendation 70-03) is subject to continuous review.  ERC Recommendation 70-03 and EC Decision for SRDs (EU) 2019/1345 already have provisions to allow higher powered Wideband Data Transmission use in the 57-71 GHz band. The spectrum regulatory deliverables were both revised and updated during the study period for WRC-19 to include both increased power and spectrum availability to allow use of the band by a more diverse set of uses and applications.

The results of the studies carried out within CEPT between wideband data transmission use and the incumbent services and applications in 57-66GHz were analysed to include the possibility of expanding the existing regulations to cover 66-71 GHz. As a result of this additional analysis both CEPT and EC developed the revised regulations and technology neutral technical conditions contained in both SRDs (EU) 2019/1345 and ERC Recommendation 70-03 that were approved last year. The scope of this work was not to look at introducing licenced mobile services into the band as the assumption was that 5G services in 66-71 GHz would have to share and meet the same technical requirements as the existing services operating in the band under a general authorisation regime.

ECC WGFM have been tasked by the ECC to develop least restrictive harmonised technical conditions suitable for next-generation (5G) terrestrial wireless systems for priority frequency bands above 24 GHz. This task is to review and revise, only if necessary, the EU-harmonised technical conditions for use of the 66-71 GHz frequency band with a view to use of this band for next-generation (5G) wireless broadband electronic communications services, taking into account relevant scenarios for shared spectrum use and the further development of all relevant services within the band. In this regard, ensure coherence of results and reporting to the European Commission, taking into account the ongoing work on the permanent mandate to CEPT on short-range devices, in accordance with the latest Commission guidance.

ECC WGFM at their October 2020 meeting concluded the following:

Based on our analysis the existing technical conditions contained in both SRDs (EU) 2019/1345 and ERC Recommendation 70-03 are sufficient to allow the use of the 66-71GHz frequency band by next generation (5G) wireless broadband electronic communication services. We also consider that the current regulations have been developed taking account of the CEPT Roadmap for 5G and the assumptions are that 5G services in this band would operate under the current general authorisation framework and would implement the appropriate technical conditions to co-exist with other users of the band including current Fixed and mobile Wideband technologies operating under the current regulations.

Table 4.2.2.1-1 and Table 4.2.2.1-2 provide summary of frequency allocations in the range of 52.6 and 114.25 GHz and regulatory requirements for existing Mobile and Fixed services in the frequency bands between 52.6 GHz and 114.25 GHz, respectively.

Table 4.2.2.1-1: Frequency bands in the range 52.6 to 100 GHz

| Frequency range in GHz | Comments | Region |
| --- | --- | --- |
| 52.6 to 54.25 | EESS service, all emissions are prohibited (footnote 5.340 of [37]) | ITU/CEPT |
| 55.78 to 57 | Recommended to be used by fixed services [40](footnote 5.547 of [37]) | ITU/CEPT |
| 57 to 66 | Several usage of Short range devices [42][45] 57-64 GHz recommended by CEPT for fixed service limited to point to point [47] 64-66 GHz recommended by CEPT for fixed service limited to point to point [46] 63-64 GHz ITS band in Europe [47]) | EC/CEPT |
| 66 to 71 | (EU) 2019/1345 and ERC Recommendation 70-03 | EC/CEPT |
| 71 to 76 and 81 to 86 | Recommended by CEPT for fixed services [48] | CEPT |
| 77 to 81 | Automotive RADAR in Europe [49][50] | EC/CEPT |
| 86 to 92, 100 to 102, 109.5 to 111.8 | EESS service, all emissions are prohibited (footnote 5.340 of [37]) | ITU/CEPT |
| 92 to 94, 94.1 to 100, 102 to 109.5 and 111.8 to 114.25 | Recommended by CEPT for fixed services [51]) | CEPT |

Table 4.2.2.1-2: Europe/CEPT – Current Regulatory Requirements for spectrum between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 57 – 71 | C1: 40 dBm mean EIRP, 23 dBm/MHz mean EIRP density. C2: 40 dBm mean EIRP, 23 dBm/MHz mean EIRP density, and maximum transmit power 27dBm and the antenna port or ports. C3: 55 dBm mean EIRP, 38 dBm/MHz mean EIRP density, and maximum transmit power 27dBm and transmit antenna gain >= 30 dBi. | Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment | Not specified | Short Rage Devices - Wide Data Transmissions [42] | C1: Fixed outdoor installations are not allowed. C3: Applies only to fixed outdoor installations. |
| 71-76, 81-86 | Maximum power limit= 30dBm Maximum EIRP= 85dBm (55 dBW) Minimum antenna gain= Pout (dBm) + 15; or 38 (whichever is the greater) maximum antenna gain= 85-Pout(dBm) Complete set of specification (part 2-2 and part 3 of [52]) | Access to the frequency bands by Authorization regimes or by Block assignment/auction regimes. | An aggregation of 250 MHz channels for wider channels as defined in [53] (channel arrangements for channel sizes ranging from 250 to 4750 MHz) | Fixed Services defined in [53] To assist the planning of fixed links, self-coordination approach, similar to the "light licensing", described in [54] can be considered. |  |
| 92-94, 94.1-100, 102-109.5, 111.8-114.25 | Not defined [51] | Not defined [51] | Radio frequency channel/block arrangements should be derived by aggregation of 250 MHz channels. Inside a specific sub-band no upper limit is given to block size, provided that the blocks´ duplex separation is maintained. Details in Annex 3 of [51] | Fixed Services defined in [51]. The choice of the appropriate assignment method and licensing regime for Fixed Service remains a decision for national administrations, based on several technical and not technical factors | To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in [54] can be considered. |

********** NEXT CHANGED SECTION **********

### 4.2.3	ITU Region 2

#### 4.2.3.1	USA

Table 4.2.3.1-1 shows the spectrum allocations for USA for frequency between 52.6GHz and 100GHz. Primary services are listed in upper case, e.g., MOBILE, and secondary services are listed with Capitalization, e.g. Amateur. The FCC rules part(s) and additional requirements are rules and regulations that pertain to specified frequency ranges and detailed description of the rules and regulations are listed in FCC code of federal regulations Title 47 Part 2.

Table 4.2.3.1-1: USA spectrum allocation between 52.6 GHz and 100 GHz

| Frequency [GHz] | Allocations | Governing FCC Rule Part(s) |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) | General Rules and Regulations (2.105 -US246) |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | Satellite Communications (25) General Rules and Regulations (2.105 -5.556A) |
| 55.78-56.9 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | Satellite Communications (25) General Rules and Regulations (2.105 -US353, US532, US379, 5.556A, 5.558) |
| 56.9-57 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | General Rules and Regulations (2.105 - US532, 5.558) |
| 57-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | RF Devices (15) Satellite Communications (25) General Rules and Regulations (2.105 -US532, 5.556A, 5.558) |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | RF Devices (15) General Rules and Regulations (2.105 -US353, US354) |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE RADIOLOCATION SPACE RESEARCH (passive) | RF Devices (15) General Rules and Regulations (2.105 -US353, 5.558, 5.559) |
| 59.3-64 | FIXED MOBILE RADIOLOCATION | RF Devices (15) ISM Equipment (18) General Rules and Regulations (2.105 -5.138, US353, 5.559, 5.558) |
| 64-65 | FIXED MOBILE except aeronautical mobile | RF Devices (15) |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | RF Devices (15) Satellite Communications (25) |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | RF Devices (15) Satellite Communications (25) General Rules and Regulations (2.105 -5.554, 5.553, 5.558) |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) | Fixed Microwave (101) General Rules and Regulations (2.105 -US389) |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US389) |
| 76-77 | RADIO ASTRONOMY RADIOLOCATION Amateur Space research (space-to-Earth) | RF Devices (15) Personal Radio (95) Amateur Radio (97) General Rules and Regulations (2.105 -US342) |
| 77-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | RF Devices (15) Personal Radio (95) Amateur Radio (97) General Rules and Regulations (2.105 -5.560, US342) |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342, US389, US297) |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342, US389) |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US246, US74) |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342) |
| 94-94.1 | RADIOLOCATION Radio astronomy | RF Devices (15) General Rules and Regulations (2.105 -5.562A) |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342) |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE | General Rules and Regulations (2.105 -5.554, US342) |

Among the spectrum allocations for U.S.A., frequency ranges 57GHz to 71GHz are available for mobile use as part of unlicensed spectrum regulated by Title 47 Part 15 of the FCC regulations. Frequency ranges 71GHz to 74GHz and 81GHz to 86GHz are available for fixed use as part of licensed spectrum using universal licensing system regulated by Title 47 Part 101 of the FCC regulations. Frequency ranges 92GHz to 95GHz (TBC) are available for fixed indoor wireless use as part of licensed spectrum using universal licensing system regulated by Title 47 Part 101 of the FCC regulations.

The 70/80/90GHz bands in US are licensed on the basis of non-exclusive nationwide licenses. There is no limit to the number of non-exclusive nationwide licenses that may be granted for these bands, and these licenses will serve as a prerequisite for registering individual links. An entity may request any portion of the 71-76GHz and 81-86GHz bands, up to 5 gigahertz in each segment for a total of 10 gigahertz. Licensees are also permitted to register smaller segments. The 92-95GHz band is divided into three segments: 92.0-94.0GHz and 94.1-95.0 GHz for non-government and government users, and 94.0-94.1 GHz for Federal Government use. Pairing is allowed and segments may be aggregated without limit (possible 12.9 gigahertz maximum aggregation). Licensees may use the 70GHz, 80GHz and 90GHz bands for any point-to-point, non-broadcast service. The segments may be unpaired or paired, but paring will be permitted only in a standardized manner (e.g., 71-72.25GHz may be paired only with 81-82.25GHz, and so on).

Table 4.2.3.1-2 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in U.S. It should be noted that no channel occupancy time, occupied channel bandwidth, and listen-before-talk requirements exist for unlicensed operations in above 52.6GHz.

Table 4.2.3.1-2: USA regulatory requirements for frequency between 52.6GHz and 100GHz that are available for fixed and mobile use

| Frequency band [GHz] | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- |
| 57 – 71 | Max avg, EIRP 40dBm Max peak EIRP 43dBm If emission-BW is less than 100 MHz, max peak conducted output power is {500mW × emission-BW / 100MHz} [Note 1] Otherwise, max peak conducted output power is 500mW | No requirements. | No specified requirements. | Equipment other than fixed outdoor. [Note 4] | Unlicensed. [Note 2] [Note 3] [Note 10] |
| 57 – 71 | Max avg. EIRP (82 – 2N) dBm Max peak EIRP (85 – 2N) dBm. N = max(0, 51 dBi – antenna-gain) If emission-BW is less than 100 MHz, max peak conducted output power is {500mW × emission-BW / 100MHz} [Note 1] Otherwise, max peak conducted output power is 500mW | No requirements. | No specified requirements. | Fixed outdoor equipment | Unlicensed. [Note 10] |
| 71 – 76 | Max EIRP 55dBW, Max Tx Power 5dBW Max Tx PSD 150mW/100MHz [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 5 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly licensed |
| 81 – 86 | Max EIRP 55dBW Max Tx power 5dBW Max Tx PSD 150mW/100MHz [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 5 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly licensed |
| 92 – 94 | Max EIRP 55dBW [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 2 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly licensed |
| 94 – 94.1 | Max EIRP 55dBW [Note 8] [Note 9] | N/A |  | Any point-to-point, non-broadcast service (Government use only) | Lightly Licensed |
| 94.1 – 95 | Max EIRP 55dBW [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 0.9 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly Licensed |
| 92 – 95 | Max avg. PSD 9uW/sq.cm [Note 6] Max peak PSD 18uW/sq. cm [Note 6] Max Tx power 500mW | Sharing and coordination among non-government licensees and between non-government and government services. | No specified requirements. | Fixed Indoor Only | Unlicensed. [Note 2] [Note 7] [Note 10] |
| 95 – 100, 102 – 109.5, 111.8 – 114.25 | Rules being defined by FCC as part of the Spectrum Horizons 95-275GHz proceeding | Rules being defined by FCC as part of the Spectrum Horizons 95-275GHz proceeding | Rules being defined by FCC as part of the Spectrum Horizons 95-275GHz proceeding | There are passive services like radio astronomy service (RAS), Earth exploration-satellite service (EESS) (passive), and space research service (SRS) (passive) in the gaps (100-102GHz and 109.5-111.8 GHz) between bands considered | Licensed [Note 12] |
| Note 1: Emission bandwidth is defined as the instantaneous frequency range occupied by a steady state radiated signal with modulation, outside which the radiated power spectral density never exceeds 6 dB below the maximum radiated power spectral density in the band, as measured with a 100kHz resolution bandwidth spectrum analyzer. The center frequency must be stationary during the measurement interval, even if not stationary during normal operation (e.g., for frequency hopping devices). Peak transmitter conducted output power shall be measured with an RF detector that has a detection bandwidth that encompasses the 57-71GHz band and that has a video bandwidth of at least 10MHz. Note 2: The power density of any emissions outside the 57-71GHz band shall consist solely of spurious emissions. Radiated emissions below 40GHz shall not exceed the general limits of 5 uV/m measured at a distance of 3 m. Between 40 GHz and 200 GHz, the level of these emissions shall not exceed 90 pW/cm2 at a distance of 3m. The levels of the spurious emissions shall not exceed the level of the fundamental emission. Note 3: Equipment is presumed to operate over the temperature range −20 to + 50 degrees Celsius with an input voltage variation of 85% to 115% of rated input voltage, unless justification is presented to demonstrate otherwise. Note 4: Operation on aircraft is permitted under the following conditions: (1) when the aircraft is on the ground, and (2) while airborne, only in closed exclusive on-board communication networks within the aircraft, with the following exceptions: (i) Equipment shall not be used in wireless avionics intra-communication (WAIC) applications where external structural sensors or external cameras are mounted on the outside of the aircraft structure. (ii) Equipment shall not be used on aircraft where there is little attenuation of RF signals by the body/fuselage of the aircraft. These aircraft include, but are not limited to, toy/model aircraft, unmanned aircraft, crop-spraying aircraft, aerostats, etc. Note 5: Licensees are also permitted to register smaller bandwidth segments and segments may be aggregated without limit (up to 12.9 gigahertz maximum aggregation). Note 6: Measured at 3 meters from the radiating structure. Note 7: The use of outdoor mounted antennas, e.g., antennas mounted on the outside of a building or on a telephone pole, or any other outdoors infrastructure is prohibited. The emissions from equipment operated under this section shall not be intentionally directed outside of the building in which the equipment is located, such as through a window or a doorway. Spurious emission Between 40 GHz and 200 GHz, the level of these emissions shall not exceed 90 pW/cm2 at a distance of 3 meters. Note 8: The mean power of emissions must be attenuated below the mean output power of the transmitter in any 1 MHz band by A, where A = max{56, 11 + 0.4×(P-50) + 10×log10(B) } dB and A = Attenuation (in decibels) below the mean output power level, P = Percent removed from the center frequency of the transmitter bandwidth and between 50 and 250 percent, and B = Authorized bandwidth in MHz. Attenuation to an absolute power of less than -13dBm/1MHz is not required. Note 9: Unless otherwise authorized upon specific request by the applicant, each station authorized must employ a directional antenna adjusted with the center of the major lobe of radiation in the horizontal plane directed toward the receiving station with which it communicates: provided, however, where a station communicates with more than one point, a multi- or omni-directional antenna may be authorized if necessary. Fixed stations must employ transmitting and receiving antennas (excluding second receiving antennas for operations such as space diversity) meeting the appropriate performance indicated in Table 4.2.3.1-3. Other than frequency 92 to 95 GHz, the antenna gain less than 50 dBi (but greater than or equal to 43 dBi) is permitted only with a proportional reduction in maximum authorized EIRP in a ratio of 2 dB of power per 1 dB of gain, so that the maximum allowable EIRP (in dBW) for antennas of less than 50 dBi gain becomes + 55−2×(50-G), where G is the antenna gain in dBi. In addition, antennas in these bands must meet two additional standards for minimum radiation suppression: At angles between 1.2 and 5 degrees from the centerline of the main beam, co-polar discrimination must be G−28, where G is the antenna gain in dBi; and at angles of less than 5 degrees from the centerline of main beam, cross-polar discrimination must be at least 25 dB. Note 10: Any transmitter that has received the necessary FCC equipment authorization may be mounted in a group installation for simultaneous operation with one or more other transmitter(s) that have received the necessary FCC equipment authorization, without any additional equipment authorization. However, no transmitter may be equipped with external phase-locking inputs that permit beam-forming arrays to be realized. Note 11: First, a licensee applies for a non-exclusive nationwide license; second, the licensee registers individual point-to-point links with a database manager. Registration process verifies potential for harmful interference to or from all previously registered non-government links. Note 12: On February 22nd, 2018, the FCC adopted their Spectrum Horizons Notice of Proposed Rulemaking (NPRM) that seeks comment on bands above 95GHz; A mmW coalition has been active in this proceeding and has petitioned FCC to include these passive bands (100-102GHz and 109.5-111.8 GHz), and other passive bands above 114.25GHz for consideration for terrestrial fixed and mobile use. If approved, then the whole 95-114.25GHz range could be available in USA. |  |  |  |  |  |

Table 4.2.3.1-3: Transmit and receive antenna performance for fixed stations operating in E-band

| Frequency (GHz) | Max BW to 3 dB points (included angle in degrees) | Min antenna gain (dBi) | Min radiation suppression to angle in degrees from centerline of main beam in (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5° to10° | 10° to 15° | 15° to 20° | 20° to 30° | 30° to 100° | 100° to 140° | 140° to 180° |
| 71 - 76 (co-polar) | 1.2 | 43 | 35 | 40 | 45 | 50 | 50 | 55 | 55 |
| 71 - 76 (cross-polar) | 1.2 | 43 | 45 | 50 | 50 | 55 | 55 | 55 | 55 |
| 81 – 86 (co-polar) | 1.2 | 43 | 35 | 40 | 45 | 50 | 50 | 55 | 55 |
| 81 – 86 (cross-polar) | 1.2 | 43 | 45 | 50 | 50 | 55 | 55 | 55 | 55 |
| 92 - 95 | 0.6 | 50.0 | 36 | 40 | 45 | 50 | 55 | 55 | 55 |

#### 4.2.3.2	Canada

Table 4.2.3.2-1 shows the spectrum allocations for Canada for frequency between 52.6GHz and 100GHz based on [13]. Primary services are listed in upper case, e.g., MOBILE, and secondary services are listed with Capitalization, e.g. Amateur.

Table 4.2.3.2-1: Canada spectrum allocation between 52.6GHz and 100GHz

| Frequency [GHz] | Allocations | Notes and ITU Radio Regulations Footnotes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) | 5.3.40, 5.556 |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | 5.556A |
| 55.78-56.9 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | 5.557A, 5.556A, 5.5558, 5.547 |
| 56.9-57 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | 5.558A, 5.558, 5.547 |
| 57-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | 5.556A, 5.558, 5.547 |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | 5.547, 5.556 |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE INTER-SATELLITE RADIOLOCATION SPACE RESEARCH (passive) | 5.556A, 5.558, 5.559 |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION | 5.558, 5.559, 5.138 |
| 64-65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile | 5.547, 5.556 |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | 5.547 |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.553, 5.558, 5.554 |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) | 5.561 |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 77.5-78 | AMATEUR AMEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) | 5.149 |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) | 5.149, 5.560 |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | 5.338A, 5.149, 5.561A |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | 5.338A, 5.149 |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.340 |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.338A, 5.149 |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy | 5.562, 5.562A |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.149 |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.149, 5.554 |

Canada has designed frequency bands 71-76GHz, 81-86GHz, and 92-95GHz for use by fixed service. Frequency bands 71-76GHz, 81-86GHz are subject to licensing and frequency band 92-95GHz will be subject to future revision of the Canada's Standard Radio System Plan (SRSP). Figure 4.2.3.2-1 illustrates the band plan and associated usage for the bands 71-76 and 81-86GHz. The channel center frequencies are also described in Table 4.2.3.2-2 and 4.2.3.2-3. The frequency ranges 71.125 to 74.375 GHz and 81.125 to 84.375GHz provide for thirteen pairs of 250MHz channels (A1/A1' to A13/A13') for systems employing FDD. The frequency ranges 74.375GHz to 75.875GHz and 84.375GHz to 85.875GHz provide for 6 pairs of 250MHz channels (A14/A14' to A19/A19') for systems employing either FDD or TDD.

Figure 4.2.3.2-1: Canada's 71-76GHz and 81-86GHz band plan and associated usage (from [14])

Table 4.2.3.2-2: 71-76GHz and 81-86GHz band plan and associated usage for systems using 250MHz wide channels for Canada

| Channel | Centre frequency (MHz) | Duplex |
| --- | --- | --- |
| A1/A1' | 71250/81250 | FDD |
| A2/A2' | 71500/81500 | FDD |
| A3/A3' | 71750/81750 | FDD |
| A4/A4' | 72000/82000 | FDD |
| A5/A5' | 72250/82250 | FDD |
| A6/A6' | 72500/82500 | FDD |
| A7/A7' | 72750/82750 | FDD |
| A8/A8' | 73000/83000 | FDD |
| A9/A9' | 73250/83250 | FDD |
| A10/A10' | 73500/83500 | FDD |
| A11/A11' | 73750/83750 | FDD |
| A12/A12' | 74000/84000 | FDD |
| A13/A13' | 74250/84250 | FDD |
| A14/A14' | 74500/84500 | FDD, TDD |
| A15/A15' | 74750/84750 | FDD, TDD |
| A16/A16' | 75000/85000 | FDD, TDD |
| A17/A17' | 75250/85250 | FDD, TDD |
| A18/A18' | 75500/85500 | FDD, TDD |
| A19/A19' | 75750/85750 | FDD, TDD |

Table 4.2.3.2-3: 71-76GHz and 81-86GHz band plan and associated usage for systems using aggregation of multiple 250MHz wide channels for Canada

| Channel | Bandwidth (MHz) | Centre frequency (MHz) | Duplex |
| --- | --- | --- | --- |
| B1/B1' | 500 | 71375 / 81375 | FDD |
| B2/B2' | 500 | 71875 / 81875 | FDD |
| B3/B3' | 500 | 72375 / 82375 | FDD |
| B4/B4' | 500 | 72875 / 82875 | FDD |
| B5/B5' | 500 | 73375 / 83375 | FDD |
| B6/B6' | 500 | 73875 / 83875 | FDD |
| B7/B7' | 500 | 74375 / 84375 | FDD |
| B8/B8' | 500 | 74875 / 84875 | FDD, TDD |
| B9/B9' | 500 | 75375 / 85375 | FDD, TDD |
| C1/C1' | 750 | 71500 / 81500 | FDD |
| C2/C2' | 750 | 72250 / 82250 | FDD |
| C3/C3' | 750 | 73000 / 83000 | FDD |
| C4/C4' | 750 | 73750 / 83750 | FDD |
| C5/C5' | 750 | 74500 / 84500 | FDD |
| C6/C6' | 750 | 75250 / 85250 | FDD, TDD |
| D1/D1' | 1000 | 71625 / 81625 | FDD |
| D2/D2' | 1000 | 72625 / 82625 | FDD |
| D3/D3' | 1000 | 73625 / 83625 | FDD |
| D4/D4' | 1000 | 74625 / 84625 | FDD |
| E1/E1' | 1250 | 71750 / 81750 | FDD |
| E2/E2' | 1250 | 73000 / 83000 | FDD |
| E3/E3' | 1250 | 74250 / 84250 | FDD |
| F1/F1' | 1500 | 71875 / 81875 | FDD |
| F2/F2' | 1500 | 73375 / 83375 | FDD |
| G1/G1' | 1750 | 72000 / 82000 | FDD |
| G2/G2' | 1750 | 73750 / 83750 | FDD |
| H1/H1' | 2000 | 72125 / 82125 | FDD |
| H2/H2' | 2000 | 74125 / 84125 | FDD |
| I1/I1' | 2250 | 72250 / 82250 | FDD |
| I2/I2' | 2250 | 74500 / 84500 | FDD |
| J1/J1' | 2500 | 72375 / 82375 | FDD |
| K1/K1' | 2750 | 72500 / 82500 | FDD |
| L1/L1' | 3000 | 72625 / 82625 | FDD |
| M1/M1' | 3250 | 72750 / 82750 | FDD |
| N1/N1' | 3500 | 72875 / 82875 | FDD |
| O1/O1' | 3750 | 73000 / 83000 | FDD |
| P1/P1' | 4000 | 73125 / 83125 | FDD |
| Q1/Q1' | 4250 | 73250 / 83250 | FDD |
| R1/R1' | 4500 | 73375 / 83375 | FDD |

Table 4.2.3.2-4 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in Canada. In RSS-210 [55], Canada has extended license-exempt usage of 57-64GHz to the 57-71GHz frequency range.

Table 4.2.3.2-4: Canada regulatory requirements for frequency between 52.6GHz and 100GHz that are available for fixed and mobile use

| Frequency band [GHz] | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- |
| 57 – 71 | Max Peak conducted power: - If BW >100MHz, 500mW - If BW<100MHz; 500mW (Emission BW/100MHz) In EIRP: Indoor: 40dBm avg/43dBm Peak Outdoor PtP: 82dBm when Gant>51dBi; 82-2× (51-Gant) when Gant≤51dBi | No requirements. | No specified requirements. | Indoor/outdoor | Unlicensed. |
| 71 – 76 | Max power 5dBW. Max PSD -15dBW/MHz Minimum antenna gain 38 dBi Max EIRP 55dBm (Gant > 55dBi), Max EIRP 55 – (55-Gant) dBm (55dBi ≥ Gant > 45 dBi), and Max EIRP 45 – 2× (45-Gant) (45dBi ≥ Gant > 38 dBi), where Gant: antenna gain. [Note 1] [Note 2] | N/A |  | fixed point-to-point | Licensed |
| 81 – 86 | Max power 5dBW. Max PSD -15dBW/MHz Minimum antenna gain 38 dBi Max EIRP 55dBm (Gant > 55dBi), Max EIRP 55 – (55-Gant) dBm (55dBi ≥ Gant > 45 dBi), and Max EIRP 45 – 2×(45-Gant) (45dBi ≥ Gant > 38 dBi), where Gant: antenna gain. [Note 1] [Note 2] | N/A |  | fixed point-to-point | Licensed |
| Note 1: For unwanted emission limits described in Table 4.2.3.2-5 shall be applied. The power spectral density, delivered to the antenna input, outside the edges of the bands 71-76 GHz and 81-86 GHz shall be limited to a maximum of −55 dBW/MHz. Additionally, emissions into the band 86-92 GHz are furthermore limited. The power spectral density, delivered to the antenna input, shall be limited to a maximum of −41 dBW/100 MHz at 86 GHz and reducing proportionally to −55 dBW/100 MHz at and beyond 87 GHz Note 2: Antenna pattern must satisfy Envelop A or B specified in Table 4.2.3.2-6. The co-polarized radiation pattern envelope in the horizontal plane of the antenna must remain within Envelope A for both vertical and horizontal polarizations. The deployment of a radio system using an RF channel bandwidth greater than 2000 MHz and an antenna radio pattern remaining within Envelope B is not permitted. The values for the radiation pattern envelopes are shown table below and specified as off-axis antenna gain in units of dBi, as opposed to dBs of attenuation from the main lobe gain. In order to convert the pattern values to attenuation from the main lobe, the antenna main beam gain in dBi can be subtracted from the off-axis dBi values of the applicable envelope. . |  |  |  |  |  |

Table 4.2.3.2-5: Unwanted emission limits for E-band in Canada

| Maximum power spectral density (dBW/MHz) | Frequency separation (% of channel bandwidth) |
| --- | --- |
| −30 | 50 |
| −45 | 57.5 |
| −45 | 70 |
| −65 | 125 |
| −75 | 250 or 150 + 500 MHz [NOTE] |
| NOTE: The 250% value is applied for a channel bandwidth that is less than or equal to 500 MHz. For a bandwidth greater than 500 MHz, the 150%+500 MHz value is applied. |  |

Table 4.2.3.2-6: Antenna pattern for envelop A and B for E-band in Canada

| Azimuth (in degrees) from main lobe peak | Antenna directivity co-polarization (dBi) |  |
| --- | --- | --- |
|  | Envelope A | Envelope B |
| 5° | 16 | 25 |
| 10° | 9 | 17.5 |
| 15° | 5 | 10 |
| 20° | 1 | 7 |
| 40° | −0.33 | 2 |
| 50° | −1 | 0.66 |
| 70° | −4 | −2 |
| 88.75° | −16.18 | −7 |
| 90° | −17 | −7 |
| 100° | −17 | −7 |
| 100° | −17 | −10 |
| 180° | −17 | −10 |

********** NEXT CHANGED SECTION **********

#### 4.2.4.7	Australia

Table 4.2.4.7-1 shows the spectrum allocations for Australia for frequency between 52.6GHz and 114.25GHz based on [35]. Primary services are listed in upper case, e.g., MOBILE, and secondary services are listed with Capitalization, e.g. Amateur.

Table 4.2.4.7-1: Australia spectrum allocation between 52.6GHz and 114.25GHz

| Frequency [GHz] | Allocations | Notes and ITU Radio Regulations Footnotes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION–SATELLITE (passive) SPACE RESEARCH (passive) RADIO ASTRONOMY | 5.340 |
| 54.25-55.78 | EARTH EXPLORATION–SATELLITE (passive) INTER–SATELLITE SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE |
| 55.78-56.9 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE SPACE RESEARCH (passive) | 5.557A for Fixed, 5.556A for INTER-SATELLITE, 5.558 for Mobile |
| 56.9-57 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE SPACE RESEARCH (passive) | 5.558A for INTER-SATELLITE, 5.558 for Mobile, 5.547 |
| 57-58.2 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE, 5.558 for Mobile, 5.547 |
| 58.2-59 | EARTH EXPLORATION–SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) RADIO ASTRONOMY | 5.547 |
| 59-59.3 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE RADIOLOCATION SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE, 5.558 for Mobile, 5.559 for RADIOLOCATION |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION | 5.558 for MOBILE, 5.559 for INTER-SATELLITE, |
| 64-65 | FIXED MOBILE except aeronautical mobile INTER-SATELLITE RADIO ASTRONOMY | 5.547, 5.556 |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | 5.547 |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.553 for MOBILE, 5.558 for MOBILE |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) |  |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 77.5-78 | AMATEUR AMEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) | 5.559B for RADIOLOCATION |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) |  |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) |  |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | 5.338A for FIXED |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | 5.338A for FIXED, 5.561B for FIXED-SATELLITE (Earth-to-space) |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.338A for FIXED |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy |  |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION |  |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |
| 100-102 | SPACE RESEARCH (passive) EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY |  |
| 102-105 | FIXED MOBILE RADIO ASTRONOMY |  |
| 105-109.5 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.562B for SPACE RESEARCH (passive) |
| 109.5-111.8 | RADIO ASTRONOMY SPACE RESEARCH (passive) EARTH EXPLORATION-SATELLITE (passive) |  |
| 111.8-114.25 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.562B for SPACE RESEARCH (passive) |

ACMA (Australian Communications and Media Authority) publishes sensors using radar which can be applied in some different bands from 52.6GHz to 85GHz in Australia [28]. Moreover, details of their regulatory requirements can be found in Table 4.2.4.7-2. In August 2019, ACMA released a new update to low interference potential devices [56], in which the 57-71GHz range has been allocated for Frequency hopping, WiFi and RLAN transmitters.

Table 4.2.4.7-2: Regulatory requirements for available spectrum between 52.6 GHz and 100 GHz in Australia

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 61-61.5 | Maximum EIRP: 100 mW | No requirement | Not specified | Transmitters for non-specific applications |  |
| 59–63 | Maximum EIRP: 50 W The maximum transmitter power must not exceed 20 mW. | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must not be operated on board an aircraft. The transmitter must only be used outdoors. The transmitter must not cause spurious emissions outside the band at or greater than –30 dBm/MHz. |
| 57–71 | Maximum EIRP: 20 W | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must comply with FCC Rules Title 47 Part 15 Section 255. |
| 75-85 | Maximum EIRP: 75 nW | No requirement | Not specified | Same as 60-61GHz | The maximum EIRP applies outside the shielded room enclosure. The transmitter must meet the requirements of ETSI Standard EN 302 372. |
| 76-77 | Maximum EIRP: 25 W | No requirement | Not specified | Same as 60-61GHz |  |
| 77-81 | See Note1 below | No requirement | Not specified | Same as 60-61GHz | The transmitter must meet the requirements of ETSI Standard EN 302 264. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| 57-64, 75-85 | See Note2 below | See Note3 below | Not specified | Same as 60-61GHz | The transmitter must be operated in a position such that emissions are directed towards: (i) the ground; or (ii) the floor or a wall of a building or similar structure. The transmitter must comply with ETSI Standard EN 302 729. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| NOTE 1: Section 4.3.3.3 in ETSI EN 302 264 V2.1.1. The peak power for EUT with fixed beam or scanning antenna shall not be greater than 55 dBm. NOTE 2: Section 4.3.4.3 in ETSI EN 302 729 V2.1.1. 57-64 GHz: Maximum peak power limit EIRP is 35dBm, measured in 50MHz within main beam. 75-85 GHz Maximum peak power limit EIRP is 34dBm, measured in 50MHz within main beam. NOTE 3: Section 4.7 in ETSI EN 302 729 V2.1.1 |  |  |  |  |  |