3GPP TSG-RAN5 Meeting #91-e	R5-212988

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1257 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating 6.2D.3 A-MPR for UL MIMO for band n1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The A-MPR for UL MIMO testing in clause 6.2D.3 for NS_05, NS_05U, NS_48, NS_49 and NS_100, which applies for band n1, are absent and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding testing for NS_05, NS_05U, NS_48, NS_49 and NS_100 in clause 6.2D.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR for UL MIMO can’t be tested for band n1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

6.2D.3.4	Test description

6.2D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2D.3.4.1-1: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 3 | 2496 + BWChannel /2 + |  |  |  | CP-OFDM QPSK |  | Inner Full |
| 4 | MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 5 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 6 | High |  |  |  | CP-OFDM QPSK |  | Inner Full |
| 7 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 8 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 9 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 10 | 2496 + BWChannel /2 + |  |  |  | CP-OFDM 16 QAM |  | Inner Full |
| 11 | MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 12 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 13 | High |  |  |  | CP-OFDM 16 QAM |  | Inner Full |
| 14 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 15 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 16 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 17 | 2496 + BWChannel /2 + MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 18 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 19 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 20 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 21 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 22 | 2496 + BWChannel /2 + MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 23 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-2: Test Configuration table for NS_35

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_35 |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default | N/A | CP-OFDM QPSK | Outer Full |
| 4 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 10 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |

Table 6.2D.3.4.1-3: Test Configuration table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 |  | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 |  | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 |  | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 |  | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 |  | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 |  | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 |  | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 35 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 37 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 38 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 39 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 40 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 41 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 42 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 43 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 44 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 45 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 46 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 47 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 48 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 49 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 50 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-4: Test Configuration table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |  |
| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 |  | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 |  | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 |  | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 |  | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 |  | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 |  | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 |  | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 35 | 1935 | 10 | Default |  | A4 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 37 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 38 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 39 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 40 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 41 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 42 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 43 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 44 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 45 | 1950 | 20 | Default |  | A6 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 46 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 47 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 48 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 49 | 1935 | 10 | Default |  | A4 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 50 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 51 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 53 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 54 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 55 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 56 | 1950 | 20 | Default |  | A6 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-5: Test Configuration table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 5 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 6 | Default | 40 | Default |  |  | QPSK | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 7 | Default | 40 | Default |  |  | QPSK | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 8 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 9 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 10 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 11 | Default | 50 | Default |  |  | QPSK | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 12 | Default | 50 | Default |  |  | QPSK | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 13 | Default | 50 | Default |  |  | QPSK | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 14 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 15 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 16 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 17 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 19 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 20 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 21 | Default | 40 | Default |  |  | 256 QAM | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 22 | Default | 40 | Default |  |  | 256 QAM | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 23 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 24 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 25 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 26 | Default | 50 | Default |  |  | 256 QAM | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 27 | Default | 50 | Default |  |  | 256 QAM | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 28 | Default | 50 | Default |  |  | 256 QAM | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 29 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 30 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-6: Test Configuration table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 5 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 7 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 8 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 10 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 11 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 12 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 14 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 15 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 16 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 17 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 18 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 19 | Default | 50 | Default |  |  | QPSK | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 20 | Default | 50 | Default |  |  | QPSK | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 22 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 23 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 24 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 26 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 27 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 28 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 29 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 31 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 32 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 33 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 34 | Default | 40 | Default |  |  | 256 QAM | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 35 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 36 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 37 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 38 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 39 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 40 | Default | 50 | Default |  |  | 256 QAM | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 41 | Default | 50 | Default |  |  | 256 QAM | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 42 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-7: Test Configuration table for NS_100

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_100 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | Low | Default | Default |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void. NOTE 4: Void |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table 6.2D.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3.4.3.

6.2D.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

6.2D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

6.2D.3.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.2D.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2D.3.4.3.3	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3.4.3.4	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3.4.3.5	Message contents exceptions for network signalled value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2D.3.4.3.6	Message contents exceptions for network signalled value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2D.3.4.3.7	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-3. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 2 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 3 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 4 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 1.5 | 0 | 0 | 24.5 | 3 | 3 | 28+TT | 21.3-TT |
| 7 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 8 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 9 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 10 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.0-TT |
| 14 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 15 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 16 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 17 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 18 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 19 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 20 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 21 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 22 | 26 | 0 | 7.5 | 0 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 23 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 24 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 2 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 6 | 23 | 1.5 | 0 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
| 7 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 8 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 12 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 13 | 23 | 2.0 | 0 | 0 | 21.0 | 5 | 3 | 25+TT | 16.0-TT |
| 14 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 15 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 21 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 22 | 23 | 0 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 11 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 16 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 17 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 18 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 22 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 27 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 29 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 30 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 5 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 11 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 16 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 17 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 30 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 31 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 32 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 35 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 36 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 38 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 39 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 40 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 41 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 42 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

<End of change>