3GPP TSG RAN meeting #94e										RP-213124

Electronic Meeting, Dec. 6-17, 2021

## Status Report to TSG

Agenda item: 9.3.4.7

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR1_enh |  |  |  |  |
| Unique ID | 890062 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-212527 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 80% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Liu Ye |
|  | Company | Huawei, HiSilicon |
|  | Email | leo.liuye@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #106bis-e:

Agreement

For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, introduce a new RRC parameter to configure between 1) and 2)

  - The state of Tx chains supporting 2Tx transmission on the carrier is assumed.

  - 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

R1-2110698	Reply LS on SCell dropping issue of CA

Answers are provided in the reply LS for the questions raised by RAN4.

RAN1 #107-e:

Agreement

For a UE capable of 2Tx-2Tx switching and configured with UL Tx switching via uplinkTxSwitching, to differentiate the switching delay for 1Tx-2Tx switching from that for 2Tx-2Tx switching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

  - If 1Tx-2Tx mode is derived by the new RRC parameter, then there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. RAN1 assume the uplink is configured with RRC parameter “carrier1” by RAN2.

  - The default value of the new RRC parameter is 1Tx-2Tx switching mode.

  - In a configured switching mode, the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode.

  - Note: This RRC parameter doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #116-e:

Postpone the discussion on RRC configuration of 1Tx-2Tx switching with 2CCs on band B to next meeting as RAN1 plans to complete functionality design in Nov meeting and will give RAN2 inputs on RRC parameters and other related agreements.

#### 2.2.2	Remaining Open issues

2Tx switching between carrier 1 and carrier 2, Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

- Update the RRC configuration if needed

- Update the UE capabilities if needed

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #101-e meeting

- UL MIMO configuration for SUL band configurations

  - CR R4-2117708 was agreed, which removes the remaining restriction for supporting UL MIMO in SUL bands.

- 2Tx switching between carrier 1 and carrier 2, Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

  - WF on UL MIMO coherence for Rel-17 Tx switching was approved, in which it is confirmed that the Rel-16 UE capability for UL MIMO coherence on 2CC 1Tx-2Tx switching can be reused for 3CC 1Tx-2Tx, while for 2Tx-2Tx switching, new per band per BC UL-MIMO coherence capability is needed. It also clarifies what’s the applicable UE capability for the cases if the UL-MMO coherence capability is absent for 1Tx-2Tx and 2Tx-2Tx.

  - LS on UL-MIMO coherence for Tx switching was sent to RAN2 for the agreements captured in the WF.

- HPUE for TDD intra-band contiguous UL CA with UL MIMO

  - WF on PC2 intra-band contiguous UL CA with UL MIMO was approved in R4-2119954.

    - MPR: values for 23+23dBm were agreed; for 26+26dBm, reuse 1Tx PC2 UL CA MPR; for 23+26dBm, FFS the applicable MPR based on UE capabilities.

    - It was agreed that a UE supports the ULCA+MIMO feature, in addition to meeting the feature’s requirements (suffix ‘H’), it must meet the general requirements as well as the requirements for ULCA (suffix ‘A’) and the requirements for ULMIMO (suffix ‘D’).

- HPUE for TDD intra-band non-contiguous UL CA

  - WF R4-2119955 was approved.

    - carrier/image leakage exceptions are removed from 38.101-1

    - MPR values for PC2 and PC3 1LO architecture were agreed

    - FFS on signaling to distinguish the applicable MPR requirements for different architectures, reuse of existing signalling (TxD, DualPA, Full UL power, ModifiedMPR) is prioritized, while introduction of additional signalling is not precluded

- Solution for SCell dropping

  - WF R4-2119956 was approved, in which aspects to be studied by the proposed solutions are listed

#### 2.4.2	Remaining Open issues

- UL MIMO configuration for SUL band configurations

  - Any issues need to be improved.

- HPUE for TDD intra-band contiguous UL CA with UL MIMO

  - FFS on signaling for applicable MPR for 23+26dBm

  - Specify AMPR requirements for example BC if any.

  - CR on HPUE intra-band contiguous UL CA with UL MIMO.

- HPUE for TDD intra-band non-contiguous UL CA

  - FFS on signaling for applicable MPR for different architectures

  - Specify AMPR requirements for example BC if any.

  - CR on HPUE intra-band non-contiguous UL CA

- Solution for SCell dropping

  - Specific solution to be further discussed based on the identified aspects in the WF

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #101-e:

[01] R4-2119919, Email discussion summary for [101-e][119] NR_RF_FR1_enh_IntraHPUE, Huawei, HiSilicon

[02] R4-2117190, Discussion on UL MIMO coherence for Rel-17 Tx switching,	China Telecom

[03] R4-2117526, PC2 Intra-band UL NC CA 1LO MPR	Nokia, Nokia Shanghai Bell

[04] R4-2117633, On enabling 23+26 PC2 UEs for ULCA and ULCA+MIMO, Qualcomm Incorporated

[05] R4-2117643, PC2 and PC3 MPR for 1LO architecture, Qualcomm Incorporated

[06] R4-2117708, CR for 38.101-1 to remove UL MIMO restriction for SUL carrier, CMCC

[07] R4-2117752, Discussion on UL-MIMO coherence capability, Xiaomi

[08] R4-2117990, Views on SCell dropping for UL CA, Apple

[09] R4-2118115, MPR for PC2 intra-band contiguous UL CA with UL MIMO support, Skyworks Solutions Inc.

[10] R4-2118129, More on resolving the Scell dropping (power reduction) problem by power limits, Ericsson

[11] R4-2118132, LS to RAN1 and RAN2 on UE-specific power limits for serving cells of UL CA, Ericsson

[12] R4-2118290, Discussion on Scell dropping, vivo

[13] R4-2118873, R17 FR1 SCC drop, OPPO

[14] R4-2118876, R17 FR1 UL CA MPR discrimination, OPPO

[15] R4-2118877, R17 FR1 Tx switching coherence, OPPO

[16] R4-2119090, On UL MIMO coherence capability for Rel-17 Tx switching, Huawei Technologies France

[17] R4-2119213, PC2 NC ULCA MPR and transceiver impairments for 1LO architectures, Skyworks Solutions Inc.

[18] R4-2119491, 2tx dualPA use with NC UL CA, Qualcomm Incorporated

[19] R4-2119492, CR to remove LO exceptions, Qualcomm Incorporated

[20] R4-2119493, Delta Pcell parameter to solve SCell dropping issue, Qualcomm Incorporated

[21] R4-2119499, CR to remove LO exceptions, Qualcomm Incorporated

[22] R4-2119514, MPR for PC2 intra-band contiguious CA with 2Tx, Huawei, HiSilicon

[23] R4-2119515, RF requirements for PC2 intra-band UL CA with UL MIMO, Huawei, HiSilicon

[24] R4-2119516, draft CR for TS 38.101-1 correction of clauses of suffix H for CA with UL MIMO (R17), Huawei, HiSilicon

[25] R4-2119518, On Scell dropping, Huawei, HiSilicon

[26] R4-2119833, Introduction of power limits for serving cells of UL CA, Ericsson

[27] R4-2119834, Introduction of power limits for serving cells of UL CA, Ericsson

[28] R4-2120039, LS on UL-MIMO coherence for Rel-17 Tx switching, China Telecom

RAN1 106bis-e:

[01] R1-2110566, [106bis-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching, Moderator (China Telecom)

[02] R1-2110687, RAN1 agreements for Rel-17 Tx switching,	WI rapporteur (China Telecom)

[03] R1-2108743, Discussions on enhancements for UL Tx switching, Huawei, HiSilicon

[04] R1-2108839, Remaining issues for Rel-17 UL Tx switching, ZTE

[05] R1-2108949, Remaining issues on Rel-17 Tx switching, vivo

[06] R1-2109050, Discussion on Rel-17 Tx switching enhancement, OPPO

[07] R1-2109246, Remaining issues on Rel-17 uplink Tx switching, China Telecom

[08] R1-2109269, Discussion on Rel-17 UL Tx Switching, CMCC

[09] R1-2110163, Discussion on R17 UL Tx switching, Qualcomm Incorporated

[10] R1-2108701, LS on scell dropping issue of CA, RAN4, Huawei

[11] R1-2110668, Summary of email discussion [106bis-e-NR-SCell-Dropping] on reply LS to R1-2108701, Moderator (Huawei)

[12] R1-2110656, Draft reply LS on Scell dropping issue of CA, Moderator (Huawei)

[13] R1-2110698, Reply LS on SCell dropping issue of CA, RAN1, Huawei

RAN1 107-e:

[107-e-NR-R17-TxSwitching-01] Email discussion on RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) – Jianchi (China Telecom)

[01] R1-2110795, Discussions on enhancements for UL Tx switching, Huawei, HiSilicon

[02] R1-2110904, Remaining issues for Rel-17 UL Tx switching, ZTE

[03] R1-2110972, Remaining issues on Rel-17 Tx switching enhancements, vivo

[04] R1-2111288, Discussion on Rel-17 Tx switching enhancement, OPPO

[05] R1-2112188, Discussion on R17 UL Tx switching, Qualcomm Incorporated

RAN2 116-e:

[01] R2-2111578, Summary of [AT116-e] [025][NR17] UL TX Switching & 100M BW (Huawei)	Huawei, HiSilicon

[02] R2-2111059, RAN2 signalling to support R17 UL Tx switching enhancements, Huawei, HiSilicon, China Telecom, Apple

[03] R2-2111060, RRC configuration to support R17 UL Tx switching enhancements, Huawei, HiSilicon, China Telecom, Apple

[04] R2-2111061, Running CR to TS38.331 to support Tx switching enhancements, Huawei, HiSilicon, China Telecom, Apple, CATT

[05] R2-2110424, Running CR to TS 38.306 to support Tx switching enhancements, China Telecom, Huawei, HiSilicon, Apple, CATT

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template